3GPP TSG RAN WG1 #120bis			R1-2501881

Wuhan, China, April 7th – 11th, 2025

Agenda item:	9.11.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Document for:	Discussion and decision

Introduction

In RAN#106, revised WI on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 was approved [1]. One objective is to specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap UEs and eRedCap UEs.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

This contribution provides our views for enhancements for the HD collisions case 4.

Discussion

During last RAN1 meetings, the following agreement of collision handling for case 4 was agreed. We support to confirm it with the following change.

Working assumption For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized. - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases - This is signaled by one UE specific RRC parameter - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

UE in RRC connected mode only monitor Type-2 PDCCH CSS to get short message, however, UE would not receive PDSCH scheduled by Type-2 PDCCH CSS. So Type-2 can be removed. According to PDSCH scheduled by Type-1 PDCCH CSS, it may happen during CBRA triggered by UE or PDCCH order and CFRA. Under any of those random access cases, up to UE implementation for case 4 is a good choice. Thus Type-1 can be kept.

For PDCCH ordered PRACH transmission, some suggest to set it as an exception case. We share a different view. Since PDCCH ordered PRACH always happens when downlink data arrival while UE is out of sync. So there is low possibility that gNB scheduled a unicast PDSCH and PDCCH ordered PRACH. For PDSCH carrying SI, it has already covered by the exception case, which is up to UE implementation. So PDCCH ordered PRACH is not an exception case.

For other use cases, default priority rule for collision case 4 can prioritize UL. Because it needs more processing to cancel UL transmission than DL reception. That is why there is UL cancellation timeline but no DL cancellation, since it is totally UE implementation to skip a DL reception. Furthermore, UE also needs to cancel UL according to the condition that whether or not UL cancellation timeline can be satisfied. The satisfaction status cannot be known by gNB, so there is no help to prioritize DL due to the final decision is still depend on timeline. Considering all this, UL transmission can with prioritized.

- Confirm the following WA with some changes:

Working assumption

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized.

  - Network is allowed to indicate [DL or UL]  overriding [DL or UL]  for all cases

    - This is signaled by one UE specific RRC parameter

  - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

When network indicates DL overriding UL in RRC-Connected mode for collision case 4, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with PDSCH scheduled by Type-0/0A/1-PDCCH CSS.

- When network indicates DL overriding UL in RRC-Connected mode for collision case 4, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with PDSCH scheduled by Type-0/0A/1-PDCCH CSS.

It was agreed to further study TA report enhancements as the following.

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

Our view is no enhancement of TA report is introduced in Rel-19 NTN.

One reason is the enhancements of TA reporting cannot solve TA mismatch issue. It can only mitigate the TA mismatch partially, so it is not a complete solution.

- For first two bullets, finer TA report granularity and smaller TA offset threshold, it is uncertain time for the actual PUSCH with TAR MAC-CE. So the practical effects is limited.

- For triggering a TA report by UE, it is too late to detect a collision of case 4 and then report TA. The collision has already happened. Furthermore, trigger a TA report by UE requires a lot of discussion and specification impacts, e.g. the definition of event, container of TAR, resource allocation, etc. It is impossible to finish it within the current TU allocation.

- For TA reporting with a new triggering mechanism from gNB, it is hard to understand how gNB know when to trigger a TA. If the trigger is gNB find a UL is missed, a collision has already there and it is too late to trigger TAR.

- For TA drifting rate reporting, gNB can obtain TA drift rate according to the previous TA reports, again TA drift rate at UE side is also acquired at same way. Therefore, it really does not necessary to report TA drifting rate.

- No enhancement of TA report is introduced in Rel-19 NTN.

Conclusion

In this contribution, we made the following observations and proposals.

- Confirm the following WA with some changes:

Working assumption

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized.

  - Network is allowed to indicate [DL or UL]  overriding [DL or UL]  for all cases

    - This is signaled by one UE specific RRC parameter

  - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

- When network indicates DL overriding UL in RRC-Connected mode for collision case 4, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with PDSCH scheduled by Type-0/0A/1-PDCCH CSS.

- No enhancement of TA report is introduced in Rel-19 NTN.

References

RP-243300  Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

R1-2501605	Final Summary for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Moderator (CATT)