3GPP TSG RAN WG1 Meeting #120-bis	R1-2502131

Wuhan, China, April 7th – 11th, 2025

Source:	Fujitsu

Title:	Discussion on uplink capacity/cell throughput enhancement for FR1-NTN

Agenda Item:	9.11.3

Document for:	Discussion

Introduction

In the WID for Rel-19 NR NTN [1], the detailed objectives for uplink capacity/cell throughput enhancements for FR1-NTN are described as follows:

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we provide our views on the details of UL capacity/cell throughput enhancement by using OCC.

OCC details for PUSCH in Rel-19 NR NTN

In the RAN1#119 [2], the following agreement is made.

Agreement in RAN1#119 RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

In this section, we discuss the OCC indication mechanism for inter-slot OCC and the feasibility of applying inter-slot OCC to existing techniques such as TBoMS.

OCC indication mechanisms

In the RAN1#120 [3], there is no agreement for OCC indication mechanism, however, FL provided several FL recommendations in the FL summary [4]. For the indication of occ-Length, following FL recommendations are provided.

FL recommendation 3-2: For occ-Length for OCC with DG PUSCH, proponents of each option are encouraged to contribute details for the next meeting - Option 1: occ-Length applicable to PUSCH is configured by RRC higher-layer parameter - Option 2: Max value of occ-Length is configured by RRC higher-layer parameter, and occ-Length applicable to PUSCH is determined based on the configured repetition number - Option 3: Candidates of occ-Length are configured by RRC higher-layer parameter, and occ-Length applicable to PUSCH is indicated by scheduling DCI for DG PUSCH FL recommendation 3-3: For occ-Length for OCC with CG PUSCH type 2, proponents of each option are encouraged to contribute details for the next meeting - Option 1: occ-Length applicable to PUSCH is configured by RRC higher-layer parameter - Option 2: Max value of occ-Length is configured by RRC higher-layer parameter, and occ-Length applicable to PUSCH is determined based on the configured repetition number - Option 3: Candidates of occ-Length are configured by RRC higher-layer parameter, and occ-Length applicable to PUSCH is indicated by CG PUSCH type 2 activation DCI FL recommendation 3-4: For occ-Length for OCC with CG PUSCH type 1, occ-Length applicable to PUSCH is configured by RRC higher-layer parameters, proponents of each option are encouraged to contribute details for the next meeting

For the indication of occ-Length, three options in the above FL recommendations can be considered for OCC with DG PUSCH and CG PUSCH type 2. Option 1 specifies occ-Length in RRC signalling, and gNB can indicate OCC length for each UEs via RRC. In addition, for Option 1, the RRC signalling to specify occ-Length should be UE specific signalling because cell specific signalling cannot manage OCC application for each UEs. Option 2 utilizes RRC signalling to indicate the max value of the supported OCC length and a UE uses the maximum supported OCC length less than the actual number of PUSCH repetitions. In addition, for Option 2, the RRC signalling should be cell specific signalling (e.g., SIB19) because the maximum supported OCC length is fixed among UEs. Option 3 utilizes DCI to select one value in RRC (i.e., PUSCH-TimeDomainResourceAllocationList-r16) which contains the new parameter occ-Length for each UEs directly. In our understanding, Option 2 is the best way to indicate occ-Length from the perspective of signalling overhead and gNB workload. For example, when NW indicates the largest value of OCC length in SIB19, the UE can assume that other OCC lengths smaller than the indicated value are also available, i.e. when 4 is indicated, 1, 2, and 4 are available for OCC. This mechanism can reduce the gNB workload because the gNB is no longer required to manage OCC for each UEs with UE specific signal (e.g. UE dedicated RRC). The issue here is the UE cannot recognize the actual OCC length by the UE itself. However, it can be solved by using the relationship between inter-slot OCC and PUSCH repetition type A. When the largest value among the supported OCC length is indicated in cell specific signal, the UE can interpret that applied OCC length is the maximum supported value less than the actual number of PUSCH repetitions. For example, if gNB indicates the actual repetition number is 6 and the maximum OCC length supported by the UE is 4, the UE applies OCC for first 4 slots. At this time, OCC is not applied for the remaining 2 slots, and only one of the multiplexed UEs can have such OCC length and the number of repetitions that is unmatched. Therefore, inter-slot OCC can be controlled with existing DCI which indicates the number of repetitions.

Observation 1: For indication of occ-Length with Option 2 and DG/CG type 2 PUSCH, OCC application can be controlled with existing DCI. As an example, when the largest value of supported OCC length is configured by cell specific signalling (e.g. SIB19), UE can use the maximum supported OCC length less than the actual number of PUSCH repetitions.

In comparison to Option 2, Options 1 and 3 have concerns from the perspective of signalling overhead. For Option 1, RRC Reconfiguration signalling is necessary when OCC length needs to be changed for scheduling. Consequently, the amount of signalling for OCC is increased, and it may result in unacceptable control resource overhead in NTN scenario. For Option 3, the number of lists in PUSCH-TimeDomainResourceAllocationList-r16 should be considered. In the current specification, there are 16 lists which is indicated by 4-bit fields in DCI, and these 4-bit fields need to cover all the parameters in lists which include the number of repetitions, start symbol and length in time domain, mapping type, and the number of slots for TBoMS if needed. If occ-Length is added in this list without reducing granularity of other parameters, additional bit field is required. Therefore, we support Option 2 with DG/CG type 2 to indicate occ-Length.

Proposal 1: For OCC length indication with DG/CG type 2 PUSCH, support Option 2 to reduce signalling overhead.

For the indication of occ-Length with CG type 1 PUSCH, the same method as DG/CG type 2 PUSCH can be considered: max value of occ-Length is configured by RRC higher-layer parameter, and occ-Length applicable to PUSCH is determined based on the configured repetition number. In the current specification, rep-K is used to indicate the number of repetitions with CG type 1 PUSCH, and rep-K is the same as numberOfRepetitions-r16. In summary, we propose to use the same method as DG/CG type 2 PUSCH to indicate occ-Length.

Proposal 2: For OCC length indication with CG type 1 PUSCH, support the following method.

- max value of occ-Length is configured by RRC higher-layer parameter, and occ-Length applicable to PUSCH is determined based on the configured repetition number (i.e., rep-K).

Furthermore, in the RAN1#120, following FL recommendations are provided for occ-SequenceIndex indication.

FL recommendation 3-5: For occ-SequenceIndex for OCC with DG PUSCH, proponents of each option are encouraged to contribute details for the next meeting - Indicated in scheduling DCI for DG PUSCH - Option 1: Explicitly. - Option 2: Implicitly – e.g. associated with antenna port indicated in the scheduling DCI for DG PUSCH. FL recommendation 3-6: For occ-SequenceIndex for OCC with CG PUSCH type 2, proponents of each option are encouraged to contribute details for the next meeting - Indicated in activation DCI for CG PUSCH type 2 - Option 1: Explicitly. - Option 2: Implicitly – e.g. associated with antenna port indicated in the CG PUSCH type 2 activation DCI. FL recommendation 3-7: For occ-SequenceIndex for OCC with CG PUSCH type 1, configured via RRC higher-layer parameter.

For indication of occ-SequenceIndex, we prefer to select Option 2 for OCC with DG/CG type 2 PUSCH, and it is applicable to CG type 1 PUSCH because antenna port should be indicated in ConfiguredGrantConfig. For Option 2, in other words, occ-SequenceIndex is associated with DM-RS antenna port field in DCI because DM-RS antenna port should be designed to be orthogonal among OCC UEs and antenna port may be the same among OCC UEs. For example, when OCC length 2 is applied, the following sentence could be inserted in the description of the antenna port field in TS 38.212 [5], "The UE using the DM-RS port 0 applies the OCC sequence index 1, and the UE using the DM-RS port 1 applies the OCC sequence index 2". In contrast to Option 1 which requires additional signalling, Option 2 can reduce signalling overhead due to its association of occ-SequenceIndex and DM-RS antenna port. Therefore, we propose to select Option 2 for OCC with DG/CG type 1/CG type 2.

Proposal 3: For indication of occ-SequenceIndex for OCC with DG/CG type 1/CG type 2, support Option 2 which implicitly associates occ-SequenceIndex with DM-RS antenna port.

TBoMS for OCC techniques

TBoMS would be useful because the reduction of CRC overhead is important as we assume very small TB size is mainly used for NTN communication. In addition, TBoMS is also applicable to NTN communication with OCC. The reason is, in our understanding, the application of inter-slot OCC for TBoMS is not so complicated since TBoMS with PUSCH repetition type A has already supported in Rel-17. However, when the number of slots is the same, TBoMS and repetition scheme which is already supported are similar in the amount of transmitting information bits. Hence, it should be discussed whether to support TBoMS with OCC based on simulation results compared to repetition scheme with OCC. In summary, we would like to support TBoMS with OCC, however, it can be discussed after determining OCC details due to the lack of discussion time based on simulation results.

Proposal 4: TBoMS with OCC should be discussed after determining OCC details.

Conclusion

In this contribution, we provide our views as below.

Observation 1: For indication of occ-Length with Option 2 and DG/CG type 2 PUSCH, OCC application can be controlled with existing DCI. As an example, when the largest value of supported OCC length is configured by cell specific signalling (e.g. SIB19), UE can use the maximum supported OCC length less than the actual number of PUSCH repetitions.

Proposal 1: For OCC length indication with DG/CG type 2 PUSCH, support Option 2 to reduce signalling overhead.

Proposal 2: For OCC length indication with CG type 1 PUSCH, support the following method.

- max value of occ-Length is configured by RRC higher-layer parameter, and occ-Length applicable to PUSCH is determined based on the configured repetition number (i.e., rep-K).

Proposal 3: For indication of occ-SequenceIndex for OCC with DG/CG type 1/CG type 2, support Option 2 which implicitly associates occ-SequenceIndex with DM-RS antenna port.

Proposal 4: TBoMS with OCC should be discussed after determining OCC details.

References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- Chair’s notes RAN1# 119.

- Chair’s notes RAN1# 120.

- R1-2501402, Feature lead summary #3 of AI 9.11.3 on NR-NTN uplink capacity and throughput.

- TS 38.212, Multiplexing and channel coding, V18.5.0, Jan, 2025.