3GPP TSG RAN WG1 #120-bis	R1-2502165

Wuhan, China, April 7th–April 11th, 2025

Source: 	CMCC

Title:	Discussion on on-demand SIB1 for UEs in idle/inactive mode

Agenda item:	9.5.2

Document for:	Discussion & Decision

- Introduction

In RAN#102 meeting, a new WID on enhancements of network energy saving has been approved [1], one of the objectives is on-demand SIB1 for UEs in idle/inactive mode, the scope of this objective has been discussed and revised in RAN#105 [2], the revised objective is as following,

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we provide our views on UE behavior for SIB1 reception and specific content in RAR, and give our proposals.

- Discussion on UE behavior for SIB1 reception

For discussion on UE behavior for SIB1 reception, the following agreements are agreed during RAN1#120 meeting [3],

Agreement The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS if this is indicated via UL WUS configuration - FFS: Whether UE expect PDCCH on PDCCH monitoring occasion corresponding to SSB other than the one mentioned above - FFS: Whether gNB can indicate (in UL-WUS configuration) the SSB the UE can expect PDCCH transmission on Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following: - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission. - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy. Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements Note: The above cases are for SSB on the sync raster.

In this section, we discuss on UE behavior for SIB1 reception procedure, including time domain behavior and spatial domain behavior on SIB1 reception.

Regarding time domain behavior on SIB1 reception (i.e. UE behavior when it has SIB1 request configuration of a cell and before transmitting UL WUS and detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2), we analysis the pros and cons for each alternative and the results are listed in Table 1:

Table 1. Analysis on pros and cons for each alternative on UE behavior

| Alternative | Pros | Cons |
| --- | --- | --- |
| Alt. 1 | UE can directly obtain explicit indication when OD-SIB1 is not transmitted, so that it does not need to perform blind detection. | UE has to detect MIB at first when it needs to acquire OD-SIB1, even if OD-SIB1 is already transmitted. |
| Alt. 2 | UE can directly monitor the corresponding PDCCH without any other procedure when it need to acquire OD-SIB1. | UE needs to do blind detection without explicit information. |
| Alt. 3 | UE has the maximum flexibility. | UE may transmit WUS even when OD-SIB1 is already transmitted, which will result in waste of energy for both gNB and UE. Inconsistent with existing RAN2 agreements. |
| Alt. 4 | UE does not need to do any pre-detection when it need to acquire OD-SIB1. | UE has to transmit WUS regardless of whether OD-SIB1 is already transmitted, which will result in waste of energy for both gNB and UE. Inconsistent with existing RAN2 agreements. |
| Alt. 5 | UE can directly obtain explicit indication when OD-SIB1 is not transmitted, so that it does not need to perform blind detection. | UE has to detect MIB at first when it needs to acquire OD-SIB1, even if OD-SIB1 is already transmitted. |

Based on the above analysis, we observe that:

- Alt. 3 and Alt. 4 will result in obvious energy waste for both gNB and UE side, and it is also inconsistent with existing RAN2 agreements. Thus, these alternatives should not be supported.

- Alt.1 and Alt.5 can achieve the similar effect. Moreover, compared Alt. 1/5 with Alt.2, Alt. 1/5 requires one or more detection on MIB/PBCH for each attempt while Alt. 2 requires one or more blind detection on PDCCH for each attempt. From our understanding, there is no obvious difference between these two approaches since:

- According to the UE power consumption model in TR 38.840 [5], the total power consumption on SSB detection and PDCCH blind detection are the same.

- Thanks to the hard work in Rel-15 design, the reliability for both PDCCH and PBCH are equal or larger than 99% (i.e. BLER≤1%). Thus, the impact on miss detection for both channels are relatively small.

- Therefore, for the sake of simplicity, we slightly prefer Alt. 2. And for the definition on monitoring time window of Type 0 PDCCH, we support that it can be up to UE implementation.

Proposal 1: When a UE has SIB1 request configuration of a cell and before transmitting UL WUS and detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, support Alt. 2 (UE monitors Type 0 PDCCH for SIB1 transmission) and the monitoring time window of Type 0 PDCCH is up to UE implementation.

Regarding spatial domain behavior on SIB1 reception (i.e. UE monitors SIB1 on which SSB beam), since both UL WUS and SIB1 are associated with the specific SSB beam, the following ways can be considered:

- The UE only monitors SIB1 on the SSB beam direction that is associated with the transmitted UL WUS.

- The UE monitors SIB1 on the SSB beam direction that is configured by gNB (e.g. via UL WUS configuration).

Proposal 2: For spatial domain behavior on SIB1 reception, support one of the followings:

- The UE only monitors SIB1 on the SSB beam direction that is associated with the transmitted UL WUS.

- The UE monitors SIB1 on the SSB beam direction that is configured by gNB (e.g. via WUS configuration).

- Discussion on specific content in RAR

For discussion on content in RAR, the following agreements is agreed during and RAN1#119 meeting [4]:

Agreement At least the contents of RAR in response to SI request are included in RAR in response to UL WUS.

In this section, we discuss whether other parameter is needed in RAR.

According to TS 38.321 [6], a RAR message for specific UE is assembled in a MAC PDU, and the PDU may also contains other MAC subPDUs for other UEs, as is shown in Figure 1. Moreover, the RAR message in response to SI request is a RAPID-only MAC subPDU, and there is only 2 MAC subPDU sizes (1 byte and 7 bytes) in current NR design, as is shown in Figure 2.

Figure 1. An example for MAC PDU structure in TS 38.321

Figure 2. An example for RAR subPDU structure in TS 38.321

Therefore, if other parameter is included in RAR, the size of MAC subPDU will changes accordingly. Simultaneously, a legacy UE will still decode the MAC PDU according to the current structure. Thus, the change of MAC subPDU size will cause a misalignment for legacy UE. Therefore, we prefer to not include other parameter in RAR, or leave such issue to RAN2 discussion.

Proposal 3: For the specific content in RAR, support one of the followings:

- Do not support to include other parameter in RAR.

- Up to RAN2 discussion to decide whether other parameter is included in RAR, and how to deal with the potential misalignment for legacy UE.

- Conclusions

In this contribution, we provide our views on UE behavior for SIB1 reception and specific content in RAR. The proposals are listed as below.

Proposal 1: When a UE has SIB1 request configuration of a cell and before transmitting UL WUS and detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, support Alt. 2 (UE monitors Type 0 PDCCH for SIB1 transmission) and the monitoring time window of Type 0 PDCCH is up to UE implementation.

Proposal 2: For spatial domain behavior on SIB1 reception, support the following:

- The UE only monitors SIB1 on the SSB beam direction that is associated with the transmitted UL WUS.

- The UE monitors SIB1 on the SSB beam direction that is configured by gNB (e.g. via WUS configuration).

Proposal 3: For the specific content in RAR, support one of the followings:

- Do not support to include other parameter in RAR.

- Up to RAN2 discussion to decide whether other parameter is included in RAR, and how to deal with the potential misalignment for legacy UE.

- References

- RP-234065, New WID: Enhancements of network energy savings for NR

- RP-242354, Revised WID: Enhancements of network energy savings for NR

- R1-2501382, Moderator (MediaTek), FL summary 5 for on-demand SIB1 in idle/inactive mode

- R1-2410681, Moderator (MediaTek), FL summary 5 for on-demand SIB1 in idle/inactive mode

- TR 38.840, Study on User Equipment (UE) power saving in NR

- TS 38.321, Medium Access Control (MAC) protocol specification