3GPP TSG RAN WG1 #120bis		                        R1-2502542

Wuhan, China, April 7th -11th, 2025

Source: 	Transsion Holdings

Title:	Discussion on on-demand SSB SCell operation

Agenda item:	9.5.1

Document for:	Discussion & Decision

Introduction

In RAN1#120 meeting, the following agreements[1] on on-demand SSB SCell operation were achieved：

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following. - Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst. - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB. - Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB. - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB. - Alt Time-C: The specification has no restriction with regards to overlapping Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Send an LS to RAN4 to inform them of the above agreement. Agreement - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-phySCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list Agreement For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission. Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain. Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB Agreement For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options - Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication - Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB” - Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

In this contribution, some considerations for on-demand SSB SCell operation will be discussed.

On-demand SSB SCell operation

Scenarios for on-demand SSB

In previous meeting, the scenarios for on-demand SSB were discussed and the following agreements were achieved:

Agreement（RAN1#116bis meeting） For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Conclusion（RAN1#120 meeting） The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain.

First, we explain the definitions of the scenarios and cases mentioned in the above agreements as follows::

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #2A: When UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3B: When SCell activation is completed and SCell is activated or After SCell activation is completed and SCell is activated

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

For Scenario #3B, SSB is mainly used to maintain downlink synchronization, perform L1/L3 measurements and beam management. Supporting on-demand SSB transmission enables the gNB to perform SSB transmission only when synchronization or measurement is really needed. This method not only effectively reduces the network energy consumption, but also helps the UEs to quickly obtain on-demand SSB when downlink or uplink traffic bursts occur, so as to help UE quickly complete SCell synchronization and AGC compensation, thereby significantly reducing the transmission delay of the UE. Therefore, scenario #3B can support on-demand SSB transmission.

In addition, after SCell activation is completed, there may be a duration when no traffic transmission. At this time, the network can choose to stop SSB transmission or transmit always-on SSB with a longer period to save network energy consumption.

In summary, Scenarios#3B and case#1 and Scenario#3B and case#2 can be considered.

Proposal 1  Scenarios 3B and Case #1 and Scenario #3B and Case #2 should be supported.

After discussing the scenarios for on-demand SSB, let's discuss the types of on-demand SSB. For the types of on-demand SSB, the following consensus was achieved in the RAN1#118 meeting:

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

- First, considering that a cell may be used as a SCell by one UE and as a Pcell by another UE at the same time, if the on-demand SSB of the cell is CD-SSB, it may affect the camping and system information acquisition of some UEs in the cell.

- In addition, if the on-demand SSB is CD-SSB, in order to avoid its impact, we must restrict the cell supporting on-demand SSB from being used as a Pcell by any UE. However, this restriction reduces the flexibility of cell configuration. Therefore, on-demand SSB for CD-SSB located on synchronization raster cannot be supported.

Proposal 2  On-demand SSB for CD-SSB located on synchronization raster cannot be supported.

The relationship between OD-SSB SSB and AO-SSB

- In previous meeting, the following agreements were achieved.

Agreement (RAN1#119 meeting) Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress. Agreement (RAN1#120 meeting) Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Send an LS to RAN4 to inform them of the above agreement.

- Since SSB has a significant impact on both the reference signal and the channel, it is recommended that the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern, which can effectively reduce the impact on the specification.

- Proposal 3  For the case when the center frequency locations of always-on SSB and on-demand SSB are same, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern should be  supported.

- In previous meeting, the following agreements were achieved.

Agreement(RAN1#119 meeting) Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. Agreement(RAN1#120 meeting) Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

- Given that CD-SSB is mainly used for cell identification and camping by UEs, it must be located on the synchronization raster. Therefore, it is not allowed to support the case where always-on SSB is CD-SSB and not on a synchronization raster.

- Proposal 4  It is recommended to support Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Signaling of on-demand SSB operation

- For signaling of on-demand SSB operation, the previous meeting reached the following agreement:

- Agreement(RAN1#117 meeting) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement(RAN1#118 meeting) For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

- First, when RRC signaling or MAC CE signaling is used to indicate both SCell activation and on-demand SSB transmission, the application time of RRC signaling or MAC CE signaling will be relatively long which will inevitably increase the SSB transmission delay.

- Second, when RRC signaling or MAC CE signaling is used to indicate both SCell activation and on-demand SSB transmission, if activation/deactivation of on-demand SSB transmission is required, the SCell needs to be activated/deactivated frequently, which will undoubtedly increase the energy consumption of the UE.

- In addition, since RRC signaling and MAC CE signaling generally require HARQ-ACK feedback, even if RRC signaling or MAC CE signaling separately indicates SCell activation and on-demand SSB transmission, the latency of SSB transmission will be increased. However, compared with RRC signaling and MAC CE signaling, DCI signaling does not require HARQ-ACK feedback. Therefore, the delay of SSB transmission can be effectively reduced by indicating on-demand SSB transmission via DCI signaling.

- In summary, DCI based signaling can be used to indicate on-demand SSB transmission.

- Proposal 5  DCI based signaling to indicate on-demand SSB transmission can be supported.

- Finally, since the scenarios supported by on-demand SSB transmission are based on the SCell activation status, i.e., the scenarios of on-demand SSB transmission are defined from the UE’s perspective. Therefore, if DCI-based signaling is supported to indicate on-demand SSB transmission, we prefer to design DCI as UE-specific signaling.

- Proposal 6  If DCI based signaling support on-demand SSB transmission, DCI is UE-specific.

Deactivation of on-demand SSB transmission

- In previous meeting, the following agreements were achieved.

Agreement(RAN1#118bis meeting) For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement(RAN1#119 meeting) For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

- Since on-demand SSB transmission is mainly applied to SCells, when a SCell is deactivated, its corresponding on-demand SSB transmission should also be deactivated. In other words, any operation of deactivating a SCell should also be applicable to deactivating the on-demand SSB transmission of the SCell by default. Therefore, Option 4, 4A can be used to deactivate on-demand SSB transmission.

- Proposal 7  Option 4, 4A can be used to deactivate on-demand SSB transmission.

Time domain location of on-demand SSB

In previous meeting, the following agreements were achieved.

Agreement(RAN1#119 meeting) - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement(RAN1#120 meeting) - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists

- Compared to defining the time domain location of the on-demand SSB using two independent parameters, defining the time domain location of the on-demand SSB based on the time offset between the always-on SSB and the on-demand SSB, can more effectively reduce signaling overhead. Furthermore, by constraining the time domain location of the on-demand SSB based on the time domain location of the always-on SSB, the UE can quickly perform AGC or synchronization operations through multiple SSBs, when there is a service requirement. This approach reduces the activation delay of SCells and achieves fast synchronization.

- Proposal 8  For case #2, using time offset between always-on SSB and on-demand SSB to define time domain location of on-demand SSB should be supported.

TX behavior of on-demand SSB burst

- For the TX behavior of on-demand SSB burst, the main discussion is about time instance A. Currently, when MAC CE is used to indicate on-demand SSB transmission, the relevant agreements of time instance A are as follows.

Agreement (RAN1#118bis meeting) The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing the first actually transmitted SSB index within the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement(RAN1#120 meeting) For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options - Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication - Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB” - Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

- According to section 4.3 of TS 38.213, the subcarrier spacing (SCS) of the minimum time required for the application of the secondary cell activation command is determined by the SCS used by the PUCCH carrying the HARQ-ACK information corresponding to the PDSCH carrying the secondary cell activation command. Therefore, the SCS to determines the value of T shall be the subcarrier spacing of the PUCCH for which the UE transmits ACK corresponding to the MAC-CE indicating the on-demand SSB transmission.

## 4.3 Timing for secondary cell activation / deactivation With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following: - the actions related to CSI reporting on a serving cell that is active in slot - the actions related to the SCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot - the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after in which the serving cell is active. The value of is m+3 Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3 and is a number of slots per subframe for the SCS configuration of the PUCCH transmission as defined in [4, TS 38.211].

- Proposal 9  The SCS to determine the value of T should be the subcarrier spacing of the PUCCH for which the UE transmits ACK corresponding to the MAC-CE indicating the on-demand SSB transmission.

Conclusions

In this contribution,some views about on-demand SSB SCell operation are discussed, and the following proposals are made.

Proposal 1  Scenarios 3B and Case #1 and Scenario #3B and Case #2 should be supported.

- Proposal 2  On-demand SSB for CD-SSB located on synchronization raster cannot be supported.

- Proposal 3  For the case when the center frequency locations of always-on SSB and on-demand SSB are same, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern should be  supported.

- Proposal 4  It is recommended to support Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

- Proposal 5  DCI based signaling to indicate on-demand SSB transmission can be supported.

- Proposal 6  If DCI based signaling support on-demand SSB transmission, DCI is UE-specific.

- Proposal 7  Option 4, 4A can be used to deactivate on-demand SSB transmission.

- Proposal 8  For case #2, using time offset between always-on SSB and on-demand SSB to define time domain location of on-demand SSB should be supported.

Proposal 9  The SCS to determine the value of T should be the subcarrier spacing of the PUCCH for which the UE transmits ACK corresponding to the MAC-CE indicating the on-demand SSB transmission.

References

- “Draft Report of 3GPP TSG RAN WG1 #120 v0.3.0”, RAN1#120