3GPP TSG RAN WG1 #120bis		        R1-2502605

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.4.1

Source:	Apple

Title:	On modulation aspects for Ambient IoT

Document for:	Discussion/Decision

# Introduction

In RAN#106, work item on Rel-19 ambient IoT was approved and was further revised in RAN#107 with following objectives [1]:

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

- Proximity determination via Solution 1 in TR 38.769 only.

- Device (un)availability via Direction 1 in TR 38.769 only.

The following RAN1 objectives are set:

- PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively.

- R2D and D2R signal(s)

- Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA.

- R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

- R2D transmission supports only the Manchester line code in TR 38.769

- D2R transmission supports:

  - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and

  - A corresponding small frequency shift method according to the options in TR 38.769.

- R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis).

- PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1.

- D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this contribution, we discuss details related to following:

- CP handling methods

- R2D modulation (M values)

- R2D chip duration determination

# Discussion

## CP handling methods

In RAN1#120bis, following agreement has been made related to CP handling methods [2]:

Agreement

For CP handling which retains subcarrier orthogonality, the candidate methods are clarified as follows:

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Detail design of Alt M2-1-1 or Alt M2-1-2 from the followings:

    - Alt M2-1-1

      - Candidate 1:

        - For M less than 6: Use Alt M1-1

        - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-2

      - Candidate 4:

        - Change  case to  case by changing OOK-4 M value to adjacent M +/- 1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.

  - FFS: applicable M values with or without combination of Method Type 1

For CP handling which does not retain subcarrier orthogonality, the candidate methods are clarified as follows:

- For Method Type 2 Alt M2-2 (as per TR 38.769)

  - FFS detail design of Alt M2-2 from the following:

    - Candidate 1:

      - A duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot.

For further down-selection among candidate methods which retains subcarrier orthogonality, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis:

- Method Type 1

- Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

- Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

- Note: The evaluation for CP handling at least includes a M value from {2,4,6,8}, a M value from {12, 16}, and a M value from {24, 32}

- Note: In this evaluation, the transmit power of the ON chips remains constant

In order to further discuss and down-select among the alternatives agreed during RAN1#120, in our view, we think at least 3 criteria including applicability to different M values, overhead consideration and impact to chip duration should be considered. In Table 1, below, we summarize these three aspects for all the alternatives. In addition, forward compatibility is also important when it comes to selecting one of the alternatives. Although, for the deployment scenario D1T1 in Rel-19, we think that Method Type 2, Alt M2-2, Candidate 1 can work well as we would not expect practical deployment where such co-existence would be needed and therefore subcarrier orthogonality is not critical. However, for future deployment scenarios such as D2T2 and outdoor scenarios, not ensuring subcarrier orthogonality may be problematic. Therefore, considering forward compatibility issues, Method Type 2, Alt M2-2, Candidate 1 can be precluded. Then on the aspect of having a single solution for all values of M, we think Method Type 2, Alt M2-1-1, Candidate 1 can be precluded as well since it requires combination of two methods depending on M is greater than 6 or not. Considering very limited time-frame and the presence of other candidates that work for all values of M, we should also preclude Method Type 2, Alt M2-1-1, Candidate 1. Then Method Type 2, Alt M2-1-1, Candidate 2 has an additional consideration of discussing and defining some know sequence which is added to deal with CP insertion. In our view, this adds unnecessary an additional point of discussion and considering the limited time-frame, this should be avoided. Therefore, due to potentially more effort in supporting Method Type 2, Alt M2-1-1, Candidate 2, it could be precluded as well. Among Method Type 2, Alt M2-1-2, Candidate 4 and Method Type 2, Alt M2-1-2, Candidate 5, we fundamentally don’t see any difference. Basically, both candidates would result in different M values for certain chips in certain OFDM symbols. Therefore, we could preclude one candidate among the two. Based on majority preference during RAN1#120 among the two, Method Type 2, Alt M2-1-2, Candidate 4 can be precluded.

Table 1: Summary and comparison between CP handling methods

| Method/Candidate | Details | Applicability to M values | Additional Overhead | Chip duration |
| --- | --- | --- | --- | --- |
| Method Type 1 (including Alt M1-1 and/or Alt M1-2) | OFDM symbol boundary determined by device based on TAS and no transmit-side specification | Single method applicable to all M values | No additional overhead except the legacy CP | Fixed chip duration assuming removal of CP at device side |
| Method Type 2, Alt M2-1-1, Candidate 1 | Combination of method type 1 Alt M1-1 with no transmit-side specification and Alt M2-1-1, with cyclic shift of samples before CP insertion | Two methods depending on M value | Yes, additional overhead due to cyclic shift of samples even fore CP insertion | Floating chip duration for M >=6 |
| Method Type 2, Alt M2-1-1, Candidate 2 | Insertion of known signal in the tail portion of R2D - fundamentally similar to candidate 3 | Single method applicable to all M values | Yes, additional known signal added | Floating chip duration |
| Method Type 2, Alt M2-1-1, Candidate 3 | Insertion of padding bits - fundamentally similar to candidate 2 | Single method applicable to all M values | Yes, additional padding bits at least at the end | Floating chip duration |
| Method Type 2, Alt M2-1-2, Candidate 4 | Adjusting the M value by +1 or -1 for a given OFDM symbol to avoid false rising/falling edges - fundamentally similar to candidate 5 | Single method applicable to all M values | No additional overhead except the legacy CP | Floating chip duration and different M values at least for one symbol |
| Method Type 2, Alt M2-1-2, Candidate 5 | Odd number of chips for first OFDM symbol to avoid false rising/falling edges - fundamentally similar to candidate 4 | Single method applicable to all M values | No additional overhead except the legacy CP | Floating chip duration and different M values at least for one symbol |
| Method Type 2, Alt M2-2, Candidate 1 | Nor subcarrier orthogonality (may not be forward compatible considering new use-cases/scenarios are expected in R20) | Single method applicable to all M values | No additional overhead except the legacy CP | Fixed chip duration assuming removal of CP at device side |

Observation 1: For future deployment scenarios such as D2T2 and outdoor scenarios, not ensuring subcarrier orthogonality may be problematic with Method Type 2, Alt M2-2, Candidate 1

Observation 2: Method Type 2, Alt M2-1-1, Candidate 1 requires specifying combination of two different solutions depending on value of M and this results in more effort towards an issue for which there are other solutions that may work well for all values of M

Observation 3: Method Type 2, Alt M2-1-1, Candidate 2 has an additional consideration of discussing and defining some know sequence which is added to deal with CP insertion, and this adds unnecessary an additional point of discussion

Observation 4: Among Method Type 2, Alt M2-1-2, Candidate 4 and Method Type 2, Alt M2-1-2, Candidate 5, we fundamentally don’t see any difference and both candidates would result in different M values for certain chips in certain OFDM symbols.

Proposal 1: In RAN1#120bis, at least we should remove candidates that are fundamentally same and/or that require different handling for different M values and/or are not forward compatible. Considering this, at least remove following candidates in RAN1#120bis:

- Method Type 2, Alt M2-1-1, Candidate 1

- Method Type 2, Alt M2-1-1, Candidate 2

- Method Type 2, Alt M2-1-2, Candidate 4

- Method Type 2, Alt M2-2, Candidate 1

Proposal 2: As a baseline, Method Type 1 (including Alt M1-1 and/or Alt M1-2) can be considered because of no impact to transmitter side and applicability to all values of M and fixed chip duration

- If other candidates impacting transmit sider don’t provide significant gains, then Method Type 1 (including Alt M1-1 and/or Alt M1-2) is adopted.

## R2D modulation

In RAN1#120bis, following agreement has been made related to R2D modulation [2]:

Agreement

For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission:

- The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

On the maximum value for M, to have comparable date rate as RFID, M = 24 can be the maximum value. Furthermore, if M = 32 is considered, there are potential performance issues in accurate determination of chip duration unless excessively long CAP is specified. So overall performance gain for M = 32 may not be significant compared to M = 24. On the minimum value of M, considering M = 1 could require very large overhead/duration for the CAP design (as discussed in our companion contribution [3]) and typically we don’t see a strong need to support very low data rate corresponding to M  = 1, therefore, our preference is to agree on M = 2 as the minimum value. In addition, at least M = 6/8 and M = 12/16 can be considered to cover all performance gaps.

Observation 5: To have comparable date rate as RFID, M = 24 is needed

Observation 6: If M = 32 is considered, there are potential performance issues in accurate determination of chip duration unless excessively long CAP is specified. So overall performance gain for M = 32 may not be significant compared to M = 24

Observation 7: On the minimum value of M, considering M = 1 could require very large overhead/duration for the CAP design, at least a minimum of 3 OFDM symbols, since at least 3 chips are needed in CAP

Proposal 3: Adopt minimum value of M = 2 and maximum value of M = 24 for R2D modulation

Proposal 4: Additional value of M = 6 or 8 and M = 12 or 16 can be considered. Essentially, adopt one of the following candidates set for M values for R2D modulation

- Candidate set 1: M = 2, 6, 12 and 24

- Candidate set 2: M =2, 8, 16 and 24

## R2D chip determination

In RAN1#120bis, following proposal for R2D chip determination methods was discussed in the FL summary [4]:

Proposal 6.2.1a-v1:

At least from reader perspective, depending on CP handling and/or M values (if supported), the length of a R2D chip duration (noted as ‘C’) can be derived from the followings:

- Option 1: C = (1/SCS) / M

- Option 2: C = ceil {(1/SCS) / M} or C = floor {(1/SCS) / M}

- Option 3: C = (1/SCS + CP) / M

- Option 4: C = (1/SCS) / M + CP

- Option 5: Dedicated chip duration C is introduced to ensure no additional transition edge introduced by CP

- FFS: from device perspective, whether to define the following on top of above options

  - C - delta_C<= Cd <= C + delta_C

  - Where, delta_C is a floating chip duration and FFS: delta_C/C

As discussed in RAN1#120, chip duration is correlated to CP handling method, therefore, considering our preference to support Method Type 1 (including Alt M1-1 and/or Alt M1-2), it is clear that chip duration is fixed and depends on symbol duration and corresponding value M. Therefore, Option 1 from FL  Proposal 6.2.1a-v1 can be adopted, i.e.  chip duration : C = (1/SCS) / M

Observation 8: Chip duration is correlated to CP handling method and for Method Type 1 (including Alt M1-1 and/or Alt M1-2), chip duration is fixed and depends on symbol duration and corresponding value of M

Proposal 5: If Method Type 1 (including Alt M1-1 and/or Alt M1-2) is adopted, then chip duration : C = (1/SCS) / M

# Conclusion

In this contribution, following observations/proposals have been made related to modulation aspects for ambient IoT:

Observation 1: For future deployment scenarios such as D2T2 and outdoor scenarios, not ensuring subcarrier orthogonality may be problematic with Method Type 2, Alt M2-2, Candidate 1

Observation 2: Method Type 2, Alt M2-1-1, Candidate 1 requires specifying combination of two different solutions depending on value of M and this results in more effort towards an issue for which there are other solutions that may work well for all values of M

Observation 3: Method Type 2, Alt M2-1-1, Candidate 2 has an additional consideration of discussing and defining some know sequence which is added to deal with CP insertion, and this adds unnecessary an additional point of discussion

Observation 4: Among Method Type 2, Alt M2-1-2, Candidate 4 and Method Type 2, Alt M2-1-2, Candidate 5, we fundamentally don’t see any difference and both candidates would result in different M values for certain chips in certain OFDM symbols.

Observation 5: To have comparable date rate as RFID, M = 24 is needed

Observation 6: If M = 32 is considered, there are potential performance issues in accurate determination of chip duration unless excessively long CAP is specified. So overall performance gain for M = 32 may not be significant compared to M = 24

Observation 7: On the minimum value of M, considering M = 1 could require very large overhead/duration for the CAP design, at least a minimum of 3 OFDM symbols, since at least 3 chips are needed in CAP

Observation 8: Chip duration is correlated to CP handling method and for Method Type 1 (including Alt M1-1 and/or Alt M1-2), chip duration is fixed and depends on symbol duration and corresponding value of M

Proposal 1: In RAN1#120bis, at least we should remove candidates that are fundamentally same and/or that require different handling for different M values and/or are not forward compatible. Considering this, at least remove following candidates in RAN1#120bis:

- Method Type 2, Alt M2-1-1, Candidate 1

- Method Type 2, Alt M2-1-1, Candidate 2

- Method Type 2, Alt M2-1-2, Candidate 4

- Method Type 2, Alt M2-2, Candidate 1

Proposal 2: As a baseline, Method Type 1 (including Alt M1-1 and/or Alt M1-2) can be considered because of no impact to transmitter side and applicability to all values of M and fixed chip duration

- If other candidates impacting transmit sider don’t provide significant gains, then Method Type 1 (including Alt M1-1 and/or Alt M1-2) is adopted.

Proposal 3: Adopt minimum value of M = 2 and maximum value of M = 24 for R2D modulation

Proposal 4: Additional value of M = 6 or 8 and M = 12 or 16 can be considered. Essentially, adopt one of the following candidates set for M values for R2D modulation

- Candidate set 1: M = 2, 6, 12 and 24

- Candidate set 2: M =2, 8, 16 and 24

Proposal 5: If Method Type 1 (including Alt M1-1 and/or Alt M1-2) is adopted, then chip duration : C = (1/SCS) / M

# References

- RP-250796, “Revised New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”

- 3GPP RAN1#120, “Chairman Notes”

- R1-2502607, “On timing acquisition & synchronization aspects for Ambient IoT”, Apple

- R1-2501623, “Final summary for Ambient IoT: “9.4.1 Physical channels design – modulation aspects”, Moderator (Huawei)