3GPP TSG RAN WG1 #120bis	                                   R1-2502960

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.2

Agenda Item:		9.15.2

Document for: 	Endorsement

- 9.15.2	UE features for NR MIMO Phase 5

R1-2502739	Summary of UE features for NR MIMO Phase 5	Moderator (AT&T)

#### UE-initiated/event-driven beam management

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management [for Event-2 and Mode A] | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 4 candidate values: {1, 2, … , 64}] Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Maximum number of the configured RS(s) for new beam in the RS resource set] [3. Supported values of X] symbols between the last symbol of sending first PUCCH and the first available symbol of transmission occasion of second PUSCH] | 59-1-1 FFS for other potential FG(s) | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-2 | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to have separate FGs for event 2/1/7 for mode B | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window. | 1. Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) for at least one new beam within a time window [with M>1 per one new beam], where M > 1 | 59-1-1 FFS for other potential FG(s) | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | FFS Per band | FFS n/a | FFS | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] [4. Support of Mode A for Event-1 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported report [for Mode A] | Per Band | n/a | FFS | n/a | [Component 3 candidate values: {FFS}] Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] [2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set] [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] [4. Support of Mode A for Event-7 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS | n/a | Component 1 candidate values for Q: FFS [{1, 2, ..., 8}] [Component 2 candidate values: {1, 2, ..., 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### CSI enhancements for up to 128 ports

Proposal: FG 59-2-2-4 is being deleted from the Rel. 19 UE feature list

| 59. NR_MIMO_Ph5 | 59-2-2-4 | Configuration of MR always-reported resources for Type-I | [Supported] maximum number of MR for Type-I | 59-2-2-3 | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 for Type-I | FFS | FFS | FFS | FFS | [Component candidate values: MR={1,2}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: FG 59-2-2-4a is being deleted from the Rel. 19 UE feature list

| 59. NR_MIMO_Ph5 | 59-2-2-4a | Configuration of MR always-reported resources for Rel-16 eType-II | For Rel-16 eType-II, MR={1} | 59-2-2-3 | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 for Rel-16 eType-II | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-13a-3-1 | PMI sub-bands with R=2 for extended Rel-16 eType-II codebook enhancement for up to 128 ports | 1. Support of PMI sub-bands with R=2 for extended Rel-16 eType-II codebook enhancement for up to 128 ports 2. A list of supported combinations, each combination is {Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 | 59-2-1-3 | Yes | n/a | PMI sub-bands with R=2 for extended Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-13a-3-2 | Parameter combinations 7-8 for extended Rel-16 eType-II codebook enhancement for up to 128 ports | 1. Support of parameter combinations 7-8 for extended Rel-16 eType-II codebook enhancement for up to 128 ports | 59-2-1-3 | Yes | n/a | Parameter combinations 7-8 for extended Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-13a-3-3 | Rank 3,4 for extended Rel-16 eType-II codebook enhancement for up to 128 ports | 1. Support of Rank 3,4 for extended Rel-16 eType-II codebook enhancement for up to 128 ports 2. Support of parameter combinations with M=1 3. Support of rank 1-2 [4. Support R=1] [5. A list of supported combinations, each combination is {Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1] [6 Supported processing capability] | [59-2-1-3] | Yes | n/a | Rank 3,4 for extended Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | FFS: Component 5 candidate values a. {48, 64} b. {12, …, 64} c. {64, …, 256} FFS: Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = 1 ceil(P/32) | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4ab | M=2 and R=1 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 64 ports | 1. Support M=2 and R=1 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 64ports 2. Support of parameter combinations with M=2 3. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=1 | 59-2-1-4 | Yes | n/a | M=2 and R=1 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 64ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values a. {48, 64, 128} b. {12, …, 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4bc | R=2 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 64 128 ports | 1. Support M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 64 128 ports 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=2 | 59-2-1-4 | Yes | n/a | R=2 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 64 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {12, …, 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4cd | Rank 3,4 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 64 128 ports | Support rank 3, 4 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 64 128 ports | 59-2-1-4 | Yes | n/a | Rank 3, 4 for extended Rel-17 FeType-II PS (port selection) codebook enhancement for up to 64 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5ac | N4>1 for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Support for the size of DD-basis, N4>1 2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report resource group, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report, Max # of resources and total # of Tx ports} for one CSI report setting 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR | 59-2-1-5 | Yes | n/a | N4>1 for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {1,2,4,8} b. {48, 64,128} c. {2,3,4 … 64} d. {64, …, 256} Component 3 Candidate values a. {1,2,4,8} b. {48, 64,128} c. {4,8,12} d.{64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5bd | DD unit size d=1 when A-CSI-RS is configured for CMR N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports | Support value of d=1 for the DD unit size when A-CSI-RS is configured for CMR for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-1-5 | Yes | n/a | Value of d=1 for the DD unit size when A-CSI-RS is configured for CMR N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5ce | Maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Supported maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-1-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values: {4, 8, 12} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5f | PMI subband R=2 for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Support PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports 2. A list of supported combinations, each combination is {Max # of Tx ports in one report resource group, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 | 59-2-1-5 | Yes | n/a | PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64,128} b. {2,3,4 … 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5g | X=1 based on first and last slot of WCSI for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support X=1 based on first and last slot of WCSI for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports Support of TDCQI=’1-2’ | 59-2-1-5 | Yes | n/a | X=1 based on first and last slot of WCSI, for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5h | X=2 CQI based on 2 slots for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Support X=2 CQI based on 2 slots for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports 2. Support of TDCQI=’2’ | 59-2-1-5 | Yes | n/a | X=2 CQI based on 2 slots for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5i | l = (n – nCSI,ref ) for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Support l = (n – nCSI,ref ) for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports[, when N4=1 and d>1] [2. Support of l = (n – nCSI,ref ) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports, when N4>1] | 59-2-1-5 | Yes | n/a | l = (n – nCSI,ref) for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5j | L=6 for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support L=6 for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-1-5 | Yes | n/a | L=6 for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5k | Rank 3 and 4 for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support Rank 3 and 4 for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-1-5 | Yes | n/a | Rank 3 and 4 for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5l | Processing timeline for CSI reference slot for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Aperiodic CSI report timing relaxation, w, extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports 2. Aperiodic CSI report timing relaxation, type, for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-1-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: UE reports candidate value, w, independently for each SCS in unit of symbols: {14*(KP–1)*d, 14*KP*d} Note: Kp is according to Component 12 of FG59-2-5 Note: d=4 (minimum periodicity of periodic CSI-RS) Component 2 candidate values: {CAP1, CAP2} For N4 = 1 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP1 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP2 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m + Z'2, 2Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w + Z'2, 2Z'2) Z2/Z'2 are defined in Table 5.4-2 in TS38.214 KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports M = {1,2}, is the offset between two adjacent AP CSI-RS resource groups for the CMR in slots | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5m | Maximum periodicity of CMR when configured as periodic CSI-Rs for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Supported maximum periodicity of CMR when configured as periodic CSI-Rs for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-1-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values (in slots): {4, 5, 8, 10, 20} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-6 | CSI-RS resource slot offset configuration [for periodic [and/or] aperiodic CSI-RS] CSI-RS resource time domain restriction for Type-I and Type II codebook enhancement for up to 128 ports | [Support of 2-slot CSI-RS resource aggregation] Support of two slots the K CSI-RS resources shall be configured within for Type-I and Type II codebook enhancement for up to 128 ports | FFS | yes | n/a | 2-slot resource aggregation is not supported | FFS | FFS | FFS | FFS | [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-2-1 | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook | 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | FFS | yes | n/a | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {1,2,3,4} Component 2 candidate values: FFS a. {16,32} b. {1,2,3,4 … 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-2-2 | Hybrid BF (CRI-based) with Rel-16 eType-II codebook | 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | FFS | yes | n/a | Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported | FFS Per band and Per BC | FFS n/a | FFS n/a | FFS n/a | Component 1 candidate values: {1,2} Component 2 candidate values: FFS a. {16,32} b. {1,2,3,4 … 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-3-10 | [Relaxed] timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT | [Support of relaxed] timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT, i.e., Drelax = drelax | FFS | yes | n/a | [Relaxed] timeline for joint triggering od CJTC Dd and Rel-18 eType-II CJT is not supported, i.e., Drelax =0 | FFS Per band and Per BC | FFS n/a | FFS n/a | FFS n/a | FFS: candidate values Component candidate values: 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-4 | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH | 1. Support association between {48, 64, 128} CSI-RS ports and SRS resource set for non-codebook-based PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports in a set of aggregated resources, Max # of sets of aggregated resource, and total # of Tx ports} simultaneously |  | Yes | n/a | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in in a set of aggregated resources is {48, 64, 128} The candidate value set of the max # of sets of aggregated resource is: [{2, …, 64}] [The candidate value set of total # of ports is: {48, …, 256}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-9 | NES SD Type1 timeline for Rel-19 Type-I single-panel codebook | 1. Support NES SD Type1 for Rel-19 Type-I single-panel codebook 2. Supported NES SD Type1 timeline from two timeline capabilities, for Rel-19 Type-I single-panel codebook | [59-2-1-1, 1a, 1b, 1c, 1d, or 1e] | Yes | n/a | gNB does not know UE’s CSI processing timeline NES SD Type1 for Rel-19 Type-I single-panel codebook is not supported | [Per-band and per-BC] | n/a | n/a | n/a | Component 12 candidate values: - Capability 1: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) - Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-10 | CSI processing timeline for Rel-19 Type-I SP [and Type-II codebook] | Support from two timeline capabilities, for Rel-19 Type-I single-panel codebook and Type-II codebook | [59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a] | Yes | n/a | gNB does not know UE’s CSI processing timeline | FFS | FFS | FFS | FFS | Component 1 candidate values: - Capability 1: Reuse legacy Z/Z’ values, OCPU = ceil(P/32) - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources, OCPU = 1 Note: A UE that supports [59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a] must signal this FG FFS: whether CSI processing timeline capability is reported respectively for each of Rel-19 codebooks | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### 3-antenna-port codebook-based transmissions

Proposal: FG 59-3-3b is being deleted from the Rel. 19 UE feature list

| 59. NR_MIMO_Ph5 | 59-3-3b | Maximum 2 semi-persistent and 1 periodic SRS sets for 3T3R SRS antenna switching | Support of maximum 2 semi-persistent SRS resource sets and maximum 1 periodic SRS resource set for 3T3R SRS antenna switching | FFS | Yes | n/a | Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS is supported for 3T3R SRS antenna switching | FFS | n/a | n/a | n/a | Note: The two semi-persistent SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: FG 59-3-3c is being deleted from the Rel. 19 UE feature list

| 59. NR_MIMO_Ph5 | 59-3-3c | Maximum 2 semi-persistent and 1 periodic SRS sets for 3T6R SRS antenna switching | Support of maximum 2 semi-persistent SRS resource sets and maximum 1 periodic SRS resource set for 3T6R SRS antenna switching | FFS | Yes | n/a | Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS is supported for 3T6R SRS antenna switching | FFS | n/a | n/a | n/a | Note: The two semi-persistent SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | FFS Per FSPC | FFS n/a | FFS n/a | FFS n/a | Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3[, 4]} Component 3 candidate values: {1,2,3[, 4]} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | FFS Per FSPC | FFS n/a | FFS n/a | FFS n/a | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} [Component 3 candidate values: {1,2,3}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | FFS | yes | n/a | 3TX 3T6R antenna switching is not supported | FFS Per FS | FFS n/a | FFS n/a | FFS n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-3-3a | 3T3R antenna switching | 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | FFS | yes | n/a | 3TX 3T3R antenna switching is not supported | FFS Per FS | FFS n/a | FFS n/a | FFS n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} Note: This UE feature can [only] be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch or 59-3-3 to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx Note: ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| NR MIMO Phase 5 | 59-3-4 | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based | 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based | FFS Per FS | FFS n/a | FFS n/a | FFS n/a | Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based | FFS Per FS | FFS n/a | FFS n/a | FFS n/a | Component 3 candidate values: {1,2,3[.4]} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| NR MIMO Phase 5 | 59-3-5 | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based | FFS Per FSPC | FFS n/a | FFS n/a | FFS n/a | Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based | FFS Per FSPC | FFS n/a | FFS n/a | FFS n/a | Component 3 candidate values: {1,2,3[,4]} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### Asymmetric DL sTRP/UL mTRP scenarios

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate TPC command for SRS associated with a separate SRS CLPC adjustment state closed loop index(es) | Support of DCI format 1_1 to indicate TPC command for SRS associated with a separate SRS CLPC adjustment state closed loop index(es) | FFS | yes | n/a | DCI 1_1 indicating TPC command for SRS associated with a separate SRS CLPC adjustment state closed loop index(es) is not supported | FFS Per Band | FFS n/a | FFS n/a | FFS n/a | FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states” | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-1a | PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s) | Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for joint DL/UL TCI state(s) | 23-1-1 | yes | n/a | PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS Applicable only to FR1 | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-1b | PL offset for PUCCH/PUSCH/SRS power control for separate DL/UL TCI state(s) | Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for separate DL/UL TCI state(s) | 23-10-1 | yes | n/a | PL offset for PUCCH/PUSCH/SRS power control under separate DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS n/a | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-2a | Path Loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) | Support of applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) | 23-1-1 | yes | n/a | Applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS Applicable only to FR1 | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-2b | Path Loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s) | Support of applying path loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s) | 23-10-1 | yes | n/a | Applying path loss offset on PDCCH-order PRACH under separate DL/UL TCI state(s) is not supported | FFS Per Band | FFS n/a | FFS n/a | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH | Support of two separate SRS closed loop indices separate from PUSCH | FFS | yes | n/a | Two separate SRS closed loop indexes is not supported | FFS Per Band | FFS n/a | FFS n/a | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management [FFS: whether “Support of ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH” is component or separate FG] | 23-1-1a | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management is not supported | Per FS | No | No | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### For online checking

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-4c | M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports | 1. Support M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=2 | 59-2-1-4 | Yes | n/a | M=2 and R=2 for extended Rel-17 FeType-II PS (port selection) codebook for up to 64 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64} b. {12, …, 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5d | DD unit size d=1 when A-CSI-RS is configured for CMR N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports | Support value of d=1 for the DD unit size when A-CSI-RS is configured for CMR for extended Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-1-5 | Yes | n/a | Value of d=1 for the DD unit size when A-CSI-RS is configured for CMR N4>1 for extended Rel-18 Type-II Doppler codebook for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2501732	UE features for NR MIMO Phase 5	MediaTek Inc.

R1-2501785	Discussion on UE features for NR MIMO Phase 5	ZTE Corporation, Sanechips

R1-2501830	Discussion on UE features for NR MIMO Phase 5	vivo

R1-2501980	UE features for NR MIMO Phase 5	CATT

R1-2502048	Views on UE features for MIMO Ph5	Ofinno

R1-2502134	Views on NR MIMO Phase 5 UE features	Nokia

R1-2502179	Discussion on UE features for NR MIMO Phase 5	CMCC

R1-2502228	UE features for NR MIMO Phase 5	Huawei, HiSilicon

R1-2502297	UE features for NR MIMO Phase 5	OPPO

R1-2502392	UE features for NR MIMO Phase 5	Samsung

R1-2502460	Discussion on UE features for NR MIMO Phase 5	Xiaomi

R1-2502640	Views on UE features for NR MIMO Phase 5	Apple

vR1-2502786	Discussion on UE features for NR MIMO Phase 5	NTT DOCOMO, INC.

R1-2502810	UE features for NR MIMO Phase 5	Ericsson

R1-2502865	UE features for NR MIMO phase 5	Qualcomm Incorporated