3GPP TSG RAN WG1 #120bis	                                   R1-2502962

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.6

Agenda Item:		9.15.6

Document for: 	Endorsement

- 9.15.6	UE features for NR mobility enhancements Phase 4

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

R1-2502741	Summary of UE features for NR mobility enhancements Phase 4	Moderator (AT&T)

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE | 63-6 | Yes |  | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63-x1 | Beam indication with joint DL/UL LTM TCI states | 1. Support of unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure. 2. Maximum number of configured joint LTM TCI state(s) per candidate cell 3. Support of indicating and activating a single joint LTM TCI state in a cell switch command. 4. Supported QCL source RS in the LTM TCI-state configuration 5. Maximum number of configured joint LTM TCI state(s) across candidate cells 6. Maximum number of configured cells for joint LTM TCI states | FFS | Yes |  | UE does not support Rel-19 LTM Beam indication with joint DL/UL LTM TCI states | FFS | FFS | FFS |  | Component 2 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {CSI-RS, TRS, both} Component 5 candidate values: {8, 16, 24, 32, …, 1024} Component 6 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63-x1a | MAC-CE activated joint LTM TCI states | 1. Supported QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of MAC-CE activated joint LTM TCI states per candidate cell 3. Maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cell TCI states across serving cells in the band | 63-x1a | Yes | No | UE does not support Rel-19 LTM MAC-CE activated joint LTM TCI states | FFS | FFS | FFS |  | Component 1 candidate values: {CSI-RS, TRS, both} Component 2 candidate values for K: {1,2,3,4,…,15,16} Component 3 candidate values: {1,2,3,4,…30,31,32} | Optional with capability signalling |
| 63-x2 | Beam indication with separate DL/UL LTM TCI states | 1. Support of unified TCI with separate DL/UL TCI-state indication for LTM procedure. 2. Maximum number of configured DL TCI state(s) per candidate cell 3. Maximum number of configured UL TCI state(s) per candidate cell 4. Support of indicating and activating a pair of UL/DL TCI-state in a cell switch command. 5. Supported QCL source RS in the LTM TCI-state configuration 6. Maximum number of configured separate DL LTM TCI state(s) across candidate cells 7. Maximum number of configured separate UL LTM TCI state(s) across candidate cells 8. Maximum number of configured cells for separate DL/UL LTM TCI states | FFS | Yes | No | UE does not support Rel-19 LTM Beam indication with separate DL/UL TCI states | FFS | FFS | FFS |  | Component 2 candidate values: {4, 8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate values: {4, 8, 12, 16, 24, 32, 48, 64} Component 5 candidate values: {CSI-RS, TRS, both} Component 7 candidate values: {8, 16, 24, 32, …, 1024} Component 8 candidate values: {4, 8, 12, 16, …, 512} Component 9 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signalling |
| 63-x2a | MAC-CE activated DL/UL LTM TCI states | 1. Supported QCL source RS for MAC-CE activated DL/UL LTM TCI states 2. Maximum number K1 of MAC-CE activated DL TCI states per candidate cell 3. Maximum number K2 of MAC-CE activated UL TCI states per candidate cell 4. Maximum number of MAC-CE activated LTM DL TCI states across all candidate cells and serving cell DL TCI across all serving cells 5. Maximum number of MAC-CE activated LTM UL TCI states across all candidate cells and serving cell UL TCI across all serving cells in the band | 63-x2 | Yes | No | UE does not support Rel-19 LTM MAC-CE activated DL/UL TCI states | FFS | FFS | FFS |  | Component 1 candidate values: {CSI-RS, TRS, both} Component 2 candidate values: {1, 2,3,4,5,6,7,8} Component 3 candidate values: {1, 2,3,4,5,6,7,8} Component 4 candidate values: {1,2,3,…,30,31,32} Component 5 candidate values: {1,2,3,…,30,31,32} | Optional with capability signalling |

R1-2501786	Discussion on UE features for NR mobility enhancements Phase 4	ZTE Corporation, Sanechips

R1-2501834	Discussion on UE features for NR mobility enhancements Phase 4	vivo

R1-2501984	Discussion on UE features for NR mobility enhancements Phase 4	CATT

R1-2502138	UE Features for NR mobility enhancements Phase 4	Nokia

R1-2502183	Discussion on UE features for NR mobility enhancements Phase 4	CMCC

R1-2502247	UE features for NR mobility enhancements phase 4	Huawei, HiSilicon

R1-2502299	Discussion on UE features for NR mobility enhancements	OPPO

R1-2502396	UE features for NR mobility enhancements Phase 4	Samsung

R1-2502464	Discussion on UE features for NR mobility enhancements Phase 4	Xiaomi

R1-2502563	UE features for NR mobility enhancements phase 4	Ericsson

R1-2502644	Rapporteur input on UE features for Rel-19 NR Mobility enhancements Ph4	Apple

R1-2502790	Discussion on UE features for NR mobility enhancemens Phase4	NTT DOCOMO, INC.

R1-2502869	UE features for NR mobility enhancements Phase 4	Qualcomm Incorporated

Withdrawn