3GPP TSG RAN WG1 #120bis	                                   R1-2502962

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.6

Agenda Item:		9.15.6

Document for: 	Endorsement

- 9.15.6	UE features for NR mobility enhancements Phase 4

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

R1-2502741	Summary of UE features for NR mobility enhancements Phase 4	Moderator (AT&T)

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE | 63-6 | Yes |  | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-3 | CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes |  | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-3a | CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 63. NR_Mob_Ph4 | 63-4 | CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 63. NR_Mob_Ph4 | 63-4a | CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-3 | CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated jointseparate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes |  | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated jointseparate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2501786	Discussion on UE features for NR mobility enhancements Phase 4	ZTE Corporation, Sanechips

R1-2501834	Discussion on UE features for NR mobility enhancements Phase 4	vivo

R1-2501984	Discussion on UE features for NR mobility enhancements Phase 4	CATT

R1-2502138	UE Features for NR mobility enhancements Phase 4	Nokia

R1-2502183	Discussion on UE features for NR mobility enhancements Phase 4	CMCC

R1-2502247	UE features for NR mobility enhancements phase 4	Huawei, HiSilicon

R1-2502299	Discussion on UE features for NR mobility enhancements	OPPO

R1-2502396	UE features for NR mobility enhancements Phase 4	Samsung

R1-2502464	Discussion on UE features for NR mobility enhancements Phase 4	Xiaomi

R1-2502563	UE features for NR mobility enhancements phase 4	Ericsson

R1-2502644	Rapporteur input on UE features for Rel-19 NR Mobility enhancements Ph4	Apple

R1-2502790	Discussion on UE features for NR mobility enhancemens Phase4	NTT DOCOMO, INC.

R1-2502869	UE features for NR mobility enhancements Phase 4	Qualcomm Incorporated

Withdrawn