3GPP TSG RAN WG1 #120bis	                                   						R1-2502969

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.10

Agenda Item:		9.15.10

Document for: 	Endorsement

- 9.15.10 UE features for IoT-NTN TDD mode

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

Proposal 1:

Introduce the following FG for IoT-NTN TDD mode

| Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be [48…50] ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where he unit of postponement is one PRU. [FFS Whether/how to capture NPRACH transmission in IoT-NTN TDD mode.] [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)] | [Rel-17 2-1b] | Yes | N/A | IoT-NTN TDD mode is not supported | [FFS Per UE or Per band] | [TDD only] | [N/A] | Applicable only for 1616-1626.5 MHz MSS allocated band | FFS |

Proposal 2

Capture Alt-1 below in “Mandatory/optional” column in FG 2-1 for Rel-19 IoT-NTN TDD mode.

- Alt-1: FG 2-1 is mandatory without capability signaling for UEs that indicate support of the 1616-1625.5 MHz MSS band

- Alt-2: FG 2-1 is optional with capability signaling, and Note captures “a UE must indicate the support of this FG if it supports [IoT NTN TDD mode in]1616-1625.5 MHz MSS band”

- Alt-3: FG 2-1 is optional with capability signaling

- Alt-4: FG2-1 is optional without capability signaling, and Note captures “a UE supporting 1616-1625 MHz MSS band must support this FG”.

R1-2501837	Discussion on UE features for IoT-NTN TDD mode	vivo

R1-2501944	Discussion on the UE feature for IoT-NTN TDD	ZTE Corporation, Sanechips

R1-2502058	UE features for IoT-NTN TDD mode	Iridium Satellite LLC

R1-2502141	Views on Rel-19 NTN for Internet of Things (IoT) TDD mode UE features	Nokia

R1-2502250	UE features for IoT-NTN TDD mode	Huawei, HiSilicon

R1-2502272	Discussion on UE features for IoT-NTN TDD mode	OPPO

R1-2502399	UE features for IoT-NTN TDD mode	Samsung

R1-2502647	On Rel-19 IoT-NTN TDD mode UE Features	Apple

R1-2502872	UE features for IOT-NTN TDD mode	Qualcomm Incorporated