3GPP TSG RAN WG1 #120bis	                                   						R1-2502970

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.11

Agenda Item:		9.15.11

Document for: 	Endorsement

- 9.15.11 UE features for MCE for NR Phase 3

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

Proposal

Introduce the following FG 66-1b/66-2b for multi-cell PDSCH/PUSCH scheduling by DCI format 1_3/0_3 with different carrier type and different SCS, and FFS whether to define FG(s) for same SCS and different carrier type, and for different SCS and same carrier type

| 66-1 | Multi-cell PDSCH scheduling by DCI format 1_3 with same SCS and different carrier type for the cells in the set | 1. UE supports monitoring DCI format 1_3 for DL scheduling with same SCS and different carrier type for the cells in the set. Supported carrier types for the cells in the set is {FR1 FDD, FR1 TDD} 2. Supported carrier type for scheduling cell: {FR1 FDD, FR1 TDD} FFS: Other combination(s) of carrier type FFS: other component(s) |  | Yes | N/A | Multi-cell PDSCH scheduling by DCI format 1_3 with same SCS and different carrier type for the cells in the set is not supported | [Per BC] | [n/a] | [n/a] | [n/a] |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-1a | Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and same carrier type for the cells in the set | 1. UE supports monitoring DCI format 1_3 for DL scheduling with different SCS and same carrier type for the cells in the set 2. Supported applicable combinations of carrier type for the cells in the set: {FR1 FDD, FR1 TDD, FR2-1} and carrier type for scheduling cell: {FR1 FDD, FR1 TDD} from the band combination FFS: Other combination(s) of carrier type FFS: other component(s) |  | Yes | N/A | Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and same carrier type for the cells in the set is not supported | [Per BC] | [n/a] | [n/a] | [n/a] |  | Optional with capability signalling |
| 66-1b | Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and different carrier type for the cells in the set | 1. UE supports monitoring DCI format 1_3 for DL scheduling with different SCS and different carrier type for the cells in the set 2. Supported applicable combinations of [two] sets of (carrier types, SCS) for the cells in the set: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz), (FR2-1, 120kHz)} and a set of (carrier type, SCS) for scheduling cell: {(FR1 FDD, 15kHz), (FR1 TDD, 15kHz)} from the band combination FFS: Other combination(s) of (carrier type, SCS) FFS: Whether to limit the number of cells supported in the set of cells FFS: Whether to limit FR1 TDD and/or FR2-1 to be licensed band only FFS: other component(s) |  | Yes | N/A | Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and different carrier type for the cells in the set is not supported | [Per BC] | [n/a] | [n/a] | [n/a] |  | Optional with capability signalling |
| 66-2 | Multi-cell PUSCH scheduling by DCI format 0_3 with same SCS and different carrier type for the cells in the set | 1. UE supports monitoring DCI format 0_3 for UL scheduling with same SCS and different carrier type for the cells in the set. Supported carrier types for the cells in the set is {FR1 FDD, FR1 TDD} 2. Supported carrier type for scheduling cell: {FR1 FDD, FR1 TDD} FFS: Other combination(s) of carrier type FFS: other component(s) |  | Yes | N/A | Multi-cell PUSCH scheduling by DCI format 0_3 with same SCS and different carrier type for the cells in the set is not supported | [Per BC] | [n/a] | [n/a] | [n/a] |  | Optional with capability signalling |
| 66-2a | Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and same carrier type for the cells in the set | 1. UE supports monitoring DCI format 0_3 for UL scheduling with different SCS and same carrier type for the cells in the set 2. Supported applicable combinations of carrier type for the cells in the set: {FR1 FDD, FR1 TDD, FR2-1} and carrier type for scheduling cell: {FR1 FDD, FR1 TDD} from the band combination FFS: Other combination(s) of carrier type FFS: other component(s) |  | Yes | N/A | Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and same carrier type for the cells in the set is not supported | [Per BC] | [n/a] | [n/a] | [n/a] |  | Optional with capability signalling |
| 66-b | Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and different carrier type for the cells in the set | 1. UE supports monitoring DCI format 0_3 for UL scheduling with different SCS and different carrier type for the cells in the set 2. Supported applicable combinations of [two] sets of (carrier types, SCS) for the cells in the set: {(FR1 FDD, 15kHz), (FR1 TDD, 30kHz), (FR2-1, 120kHz)} and a set of (carrier type, SCS) for scheduling cell: {(FR1 FDD, 15kHz), (FR1 TDD, 15kHz)} from the band combination FFS: Other combination(s) of (carrier type, SCS) FFS: Whether to limit the number of cells supported in the set of cells FFS: Whether to limit FR1 TDD and/or FR2-1 to be licensed band only FFS: other component(s) |  | Yes | N/A | Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and different carrier type for the cells in the set is not supported | [Per BC] | [n/a] | [n/a] | [n/a] |  | Optional with capability signalling |

Proposal

Introduce the following 2 FGs for Rel-19 multi-cell multi-PDSCH/PUSCH scheduling:

| 66-3 | Multi-cell multi-PDSCH scheduling by DCI format 1_3 | 1. UE supports monitoring DCI format 1_3 for DL scheduling of more than one PDSCH per cell in the set of cells FFS: Whether to limit this FG to particular FR and/or SCS FFS: Whether to separate this FG per FR and/or SCS FFS: other component(s) |  | Yes | N/A | Multi-cell multi-PDSCH scheduling by DCI format 1_3 is not supported |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-4 | Multi-cell multi-PUSCH scheduling by DCI format 0_3 | 1. UE supports monitoring DCI format 0_3 for UL scheduling of more than one PUSCH per cell in the set of cells FFS: Whether to limit this FG to particular FR and/or SCS FFS: Whether to separate this FG per FR and/or SCS FFS: other component(s) |  | Yes | N/A | Multi-cell multi-PUSCH scheduling by DCI format 0_3 is not supported |  |  |  |  |  | Optional with capability signalling |

R1-2501838	Discussion on UE features for MCE for NR Phase 3	vivo

R1-2501889	Discussion on UE features for MCE	Spreadtrum, UNISOC

R1-2501894	Discussion on UE feature for Rel-19 Multi-Carrier Enhancements	Lenovo (Rapporteur)

R1-2501914	Discussion on UE features for Rel-19 MCE	ZTE Corporation, Sanechips

R1-2502142	Initial views on NR Multi-carrier Enhancements Phase 2 UE features	Nokia

R1-2502251	UE features for Rel-19 MCE	Huawei, HiSilicon

R1-2502284	Discussion on UE feature for multi-cell scheduling with a single DCI	OPPO

R1-2502400	UE features for multi-carrier enhancements	Samsung

R1-2502648	On UE features for Rel-19 multi-carrier enhancements	Apple

R1-2502723	UE features for MCE for NR Phase 3	MediaTek Inc.

R1-2502792	Discussion on UE features for multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.

R1-2502873	UE features for MCE for NR Phase 2	Qualcomm Incorporated