3GPP TSG RAN WG1 #120bis	                                   						R1-2502967

Wuhan, China, April 7th – 11th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.15.8

Agenda Item:		9.15.8

Document for: 	Endorsement

- 9.15.8 UE features for NTN for NR Phase 3

For RAN1#120bis, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input.

Agreement:

Introduce the following FG for the support of 160ms SSB periodicity during initial access for Rel-19 NR-NTN:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial [cell selection or access] [FFS Other components] | FFS | FFS | N/A | FFS: UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial access | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 | Optional [with/without] capability signaling FFS: A UE that supports Rel-19 NR-NTN must support this FG |

Proposal 4.1.2-1:

Introduce the following FG for the support of PDCCH repetition with CSS for Rel-19 NR-NTN:

| 65-1-2 | PDCCH repetition for Type0 PDCCH CSS | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS [of searchSpaceZero configured within MIB pdcch-ConfigSIB1] FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition | FFS: 26-1 | FFS | N/A | PDCCH repetition for Type0 PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65-1-3 | PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS | 1. Support reception of PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS | FFS: 26-1 | FFS | N/A | FFS | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional [without] capability signaling |

Proposal 4.1.3-1:

Introduce the following FG for the support of SIB1 PDSCH repetition with CSS for Rel-19 NR-NTN:

| 65-1-4 | SIB1 PDSCH repetition within 20ms duration | 1. Support reception of SIB1 PDSCH repetition within 20 ms duration FFS: whether to merge this FG with FG(s) of PDCCH repetition for Type0 PDCCH CSS | FFS: 26-1 | FFS | N/A | SIB1 PDSCH repetition within 20ms duration is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4.1.4-1:

Introduce the following FG for the support of Msg4 PDSCH repetition for Rel-19 NR-NTN:

| 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition | FFS: 26-1 | FFS | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4.2-1:

Introduce the following FG for the support of handling of collision Case 3 and collision Case 4 for half-duplex FDD operation for Rel-19 NR-NTN (e)RedCap UE:

| 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | [1. Support handling collision Case 3] [2. Support handling collision Case 4] FFS: whether to divide this FG into two separate FGs | FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | FDD only | FFS | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission | Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4.1.4-1:

Introduce the following FGs for the support of PUSCH transmission with OCC for Rel-19 NR-NTN:

| 65-3-1 | PUSCH with inter-slot OCC of length 2 | 1. Support inter-slot OCC with OCC length 2 for DFT-s-OFDM PUSCH transmissions with repetition Type A FFS: other components | FFS: 5-14, 5-16, 5-17, 11-6, 26-1, 30-1, 30-2, 30-4a | YES |  | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | FFS | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65-3-2 | PUSCH with inter-slot OCC of length 4 | 1. Support inter-slot OCC with OCC length 4 for DFT-s-OFDM PUSCH transmissions with repetition Type A FFS: other components | 65-3-1 | YES |  | UE does not support OCC length 4 for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | FFS | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |

R1-2501835	Discussion on UE features for NTN for NR Phase 3	vivo

R1-2501942	Discussion on the UE feature for NR-NTN Phase-3	ZTE Corporation, Sanechips

R1-2502139	Views on Rel-19 NTN phase 3 UE features	Nokia

R1-2502184	Discussion on UE features for UE features for NTN for NR Phase 3	CMCC

R1-2502248	UE features for NTN for NR phase 3	Huawei, HiSilicon

R1-2502270	Discussion on UE features for NTN for NR Phase 3	OPPO

R1-2502307	On UE features for NR-NTN Phase 3	Ericsson

R1-2502397	UE features for NTN for NR Phase 3	Samsung

R1-2502645	On Rel-19 NR-NTN UE Features	Apple

R1-2502791	Discussion on UE features for Rel-19 NR NTN	NTT DOCOMO, INC.

R1-2502870	UE features for NR NTN	Qualcomm Incorporated

R1-2502931	Initial UE features list for Rel-19 NR NTN Phase 3	Rapporteur (THALES)