3GPP TSG RAN WG1 Meeting #121	R1-2503231

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.1.4.1

Source:	Futurewei

Title:	Discussion of CSI compression on AI/ML for NR air interface

Document for:	Discussion

# Introduction

Conclusion

- RAN1 concludes that both Direction A and Direction C are feasible.

- RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible.

  - This includes both 3a-1 with and without target CSI sharing.

Conclusion

- For Direction A Option 3a-1, based on evaluation results, RAN1 concludes that scalable model structure specification is feasible.

- For Direction C, based on evaluation results, RAN1 concludes that scalable model structure specification and model parameters specification is feasible.

  - Note: RAN4 feasibility is a separate study to be confirmed by RAN4.

Conclusion

- For Direction C, confirm that the specified model should be trained using synthetic data (answer to issue 8).

- For inter-vendor collaboration option 3a-1 of Direction A, confirm that the specified model structure should be determined using synthetic data.

During RAN1#120bis, companies have made good progress on inter-vendor collaboration aspects, scalable model structure for Direction A and Direction C and the corresponding training dataset generation. The conclusions shown below have been agreed [1].

Agreement

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm SGCS and NMSE as the type of performance metric that may be used for the performance target shared as additional information along with the exchanged dataset or the model parameters.

- FFS: when to use SGCS, NMSE, and which one to use or both, and relationship with the inter-vender collaboration sub-options.

- FFS: details of the format of the performance target

  - Option 1: Average performance target, e.g. average SGCS and/or average NMSE

  - Option 2: distribution of the performance target, e.g., SGCS / NMSE for 5, 10, 20, 30 percentiles, etc.

- FFS: whether multiple performance targets should be exchanged for different configurations, such as antenna ports configuration, subband configuration and payload configuration, etc., along with each exchanged dataset or model parameters

The conclusion on feasibility of scalable model structure for Direction A and Direction C is based on the evaluation results submitted by companies and observations drawn from those results.

For Option 3a-1 and Option 4-1 in Direction A, besides model parameters and dataset(s) to be transferred from NW-side to UE-side, additional information may need to be exchanged as well, in RAN1#120bis, the following related agreements were reached.

For pairing in Direction C, similar agreement shown below as Direction A was reached.

Agreement

In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then

- The same ID can be used for UE to collect UE-side target CSI for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- The same ID can be used for NW-side data collection

- FFS: whether ID/even same ID is needed for monitoring configuration

- FFS: where the ID is assigned or how the ID is specified

- Note: whether the purpose for pair will be specified will be discussed separately

For quantization alignment purpose in Direction A and Direction C, companies reached the following agreements.

Agreement

For NW-side monitoring with target CSI reporting

- Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring

- Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side

Conclusion

UE-side monitoring is feasible.

Observation

Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

In addition to above, quite some discussions on NW-side and UE-side monitoring have taken place in RAN1#120bis like the feasibility, necessity, mechanisms/options and what information is needed. In the end the following agreement/conclusion/observation were reached for monitoring even though the details regarding UE-side monitoring may need further discussion for companies to reach consensus.

Agreement

Introduce a dedicated AI/ML PU for AI/ML features for UE,

The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

Another important topic, CPU, which is general for AI/ML-based functionality was discussed during the first online session in RAN1#120bis, companies reached high-level agreement to introduce a dedicated PU that is for AI/ML features for UE as shown below. The details regarding how/whether to count AI/ML PU and legacy CPU for different AI/ML functionalities will be left to the discussion of each use case.

With the good progress in RAN1#120bis, “we concluded and/or made enough progress on nearly all fronts” based on FL’s final summary; thus, FL suggested that we will discuss overall conclusion of the study and recommendations from RAN1, scope for the normative work, and listing potential specification impact.

In this contribution, we first discuss our view on the progress on the study and discussing the objectives specified in the SID for FS_NR_AIML_air_Ph2 [2] as of RAN1#120bis, including improving tradeoff between performance and complexity/overhead and alleviating issues related to inter-vendor training collaboration. We also discussed some of the remaining issues related to potential specification impact that we have not had sufficient time to discuss related to CSI compression use case. Finally, we discuss our view regarding moving forward for CSI compression based on the study and observations drawn so far.

# Discussion on conclusions and recommendations for CSI compression using two-sided model

- For CSI compression (two-sided model), further study ways to:

  - Improve trade-off between performance and complexity/overhead

    - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

  - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

In the new SID for FS_NR_AIML_air_Ph2 [2], objectives for CSI compression using two-sided model are captured below.

In this section, we discuss the outcome of the further study as of RAN1#120bis for the specified objectives in WID and potential conclusions of the Rel-19 study for CSI compression using two-sided model.

## Improve trade-off between performance and complexity/overhead

For the objective of improving the trade-off between performance and complexity/overhead as specified in the WID [2], there are a few aspects, including extending the original Rel-18 spatial-frequency (SF) domain CSI compression using two-sided model to temporal-spatial-frequency (TSF) domain CSI compression using two-sided model, and further reducing model complexity while still achieving decent/comparable performance. In this sub-section, we will discuss these two aspects.

### Extending spatial-frequency to temporal-spatial-frequency and CSI compression plus CSI prediction

For this topic, companies have identified 5 temporal/spatial/frequency (TSF) domain CSI compression cases, in which Case 0 is the original SF domain CSI compression (serve as BL), Case 1 and Case 2 are extending SF domain CSI compression to TSF domain CSI compression, and Case 3, 4 belong to CSI compression plus CSI prediction.

Agreement

For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3.

During RAN1#118 [6], companies reach agreement to prioritize further discussion on Case 2 and Case 3 in the Rel-19 study as shown below.

Agreement

To capture all observations in section 2 of R1-2410725 to TR38.843.

For the evaluation of various cases, companies have submitted results for all 5 cases, results have been captured in “Summary of Evaluation Results for Additional study on AI/ML for NR air interface: CSI compression” [8] and is agreed to be added to the TR.

The following a few charts are excerpted from [8] for Case 2 and Case 3.

Based on the above charts, our observation is as follows.

Observation 1: For CSI compression using 2-sided model, regarding the trade-off between performance and complexity/overhead, when extending spatial-frequency (SF) domain to temporal-spatial-frequency (TSF) domain CSI compression and/or CSI compression plus CSI prediction, the majority of results indicated TSF Case 2 and Case 3 have further performance gain over Case 0 and/or the benchmark based on the results available as of RAN1#120bis.

### Further reducing model complexity for CSI compression

For this topic, many companies have submitted results into the template during RAN1#120bis. Even though the results submitted are mostly for Case 0 and some companies have missed entering some of the attributes, FL has plotted charts for encoder complexity (FLOPs and number of parameters) vs. SGCS performance and shared during offline discussion and in the final summary [3] as shown below. The first chart shows FLOPs vs. SGCS performance, and the 2nd chart shows number of parameters vs. SGCS performance.

From the above 2 charts, we can observe the following:

- For companies who submitted results for encoder models with different complexities, comparable SGCS performance is achieved with model with lower complexities, compared to the model with the highest complexity.

- Many models with FLOPs below 10M (or number of parameters < 1M) can achieve decent performance, compared with the corresponding benchmark model (the one with the highest model complexity). The complexity reduction from the most complex model can potentially be more than 10X.

We think more results will be shared by companies in RAN1#121 which will likely show similar behavior in complexity vs. performance. With the observations above, we believe complexity reduction is feasible, at least for Case 0 as of RAN1#120bis.

Observation 2: For CSI compression using 2-sided model, regarding the trade-off between performance and complexity/overhead, using model(s) with lower complexity (e.g., ~10X less complexity than the most complex model) based on agreed scalable model structure can still achieve decent performance gain over the baseline (eType II) based on the results available as of RAN1#120bis, at least for Case 0.

Based on the discussion of the above 2 sub-sections, we propose the following

Proposal 1: For CSI compression using 2-sided model, conclude that improving trade-off between performance and complexity/overhead is feasible.

## Alleviate/resolve issues related to inter-vendor training collaboration

For the objective of alleviating/resolving issues related to inter-vendor training collaboration specified in the WID, companies have identified relevant issues associated with each inter-vendor training collaboration option during RAN1#118 [6].

### Direction A

For Direction A, 4 issues were identified as shown below.

Agreement

Continue the study of following directions - on-device operation and UE side offline engineering

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

  - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues

    - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing?

    - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it?

    - [Issue 3] Is there an overhead concern, and if so, how to address it?

    - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it?

[Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing?

During RAN1#117 [4], agreement was reached with respect to potential additional information to be exchanged/shared from NW-side to UE-side:

- Exchange

  - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a)

  - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4)

  - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4)

    - Performance target (3a/5a/4)

    - Dataset or information related to collecting dataset (3a/5a)

    - Any other additional information

During RAN1#118 [6], agreement was reached related to additional information with more details related to Direction A.

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 3a-1: encoder parameter exchange, with {target CSI}

    - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Proposal 2: For AI/ML-based CSI compression using two-sided model, considering the agreements reached related to additional information to be shared from NW-side to UE-side, conclude that [Issue 1] identified for Direction A in inter-vendor training collaboration can be considered as addressed.

[Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it?

For [Issue 2], companies reached the following conclusion in RAN1#118 [6]:

- Answer to Issue 2: From RAN1 perspective, at least,

  - For option 4-1, there is no proprietary information concern of disclosing encoder input/output from NW side to UE side

  - For option 3a-1, there is no proprietary information concern of disclosing encoder parameters from NW side to UE side

  - Note: If some additional information is disclosed from network side to UE side, proprietary concerns for option 4-1 and 3a-1 can not be precluded by default.

Proposal 3: For AI/ML-based CSI compression using two-sided model, considering the agreement reached related to proprietary information concern of Direction A, conclude that [Issue 2] identified for Direction A in inter-vendor training collaboration can be considered as addressed.

[Issue 3] Is there an overhead concern, and if so, how to address it?

RAN1 has sent LS (R1-2410922) to RAN2 for the study of feasibility of parameter / dataset exchange in RAN1#119.

RAN2 (during RAN2#129bis) has responded the LS in RAN1#120bis [7]. In the reply, in summary:

- RAN2 concluded the following approaches for NW-side sharing dataset/model parameter transfer asked by RAN1:

  - Alternative 1 (non-OTA approach)

    - gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)

  - Alternative 2 (OTA approach):

    - gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)

- In each alternative, RAN2 also identified candidate solutions.

Proposal 4: For AI/ML-based CSI compression using two-sided model, based on LS reply from RAN2 for NW-side sharing dataset/model parameter transfer, conclude that [Issue 3] identified for Direction A in inter-vendor training collaboration can be considered as addressed.

[Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it?

For Direction A, companies have submitted evaluation results for performance impact due to mismatch between NW-side data distribution and UE-side data distribution with various dataset generation configurations like dataset A includes dataset B, dataset A does not include dataset B, etc. and agreement was reached in RAN1#120 [5] to capture the observations drawn based on results into the TR. In summary, the observations show:

- When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

  - Minor performance loss was observed (by 1 source) relative to case 1.

- When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

  - Minor to moderate performance loss was observed (by 3 sources) relative to case 1.

- When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

  - Majority of sources (8 out of 12) observed similar performance or minor performance loss (with different UE training alternatives) was observed relative to case 1.

- When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

  - Majority of sources (5 out of 9) observed minor performance loss, 3 sources observed moderate performance loss and 1 observed significant performance loss relative to case 1.

- When dataset A does not include dataset B, for case 2, option 4-1

  - Majority of sources (8 out of 11) observed zero to minor performance loss (with different UE training alternatives) relative to case 1.

Observation 3: For AI/ML-based CSI compression using two-sided model, performance loss concern identified by [Issue 4] for Direction A in inter-vendor training collaboration can be minimized/reduced with the help from dataset generation strategy and/or UE training alternatives.

Observation 4: For AI/ML-based CSI compression using two-sided model, based on the results and observations drawn, RAN1 has already concluded that [Issue 4] for Direction A in inter-vendor training collaboration can be addressed.

- Conclusion

  - For direction A, performance impact, if any, due to NW / UE data distribution mismatch with respect to UE side additional condition can be addressed.

From the above, our overall observation for Direction A is that companies have provided sufficient evaluation results and discussed the mechanisms/options to address the related identified issues; thus, we think Direction A can move forward to normative work.

Proposal 5: For AI/ML-based CSI compression using two-sided model, conclude that Direction A in inter-vendor training collaboration is feasible.

### Direction B

Agreement

Continue the study of following directions - on-device operation and UE side offline engineering

- …

- Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues

  - [Issue 3] Is there an overhead concern, and if so, how to address it?

  - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs?

  - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it?

  - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

There are also 4 issues identified for Direction B

[Issue 3] Is there an overhead concern, and if so, how to address it?

Companies reached the following agreement in RAN1#118bis:

- For issues listed for inter-vendor collaboration direction B, further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred.

However, no detailed discussion related to the overhead concern for Direction B has taken place as of RAN1#120bis.

[Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs?

As of RAN1#120bis, there is no detailed discussion or agreement related to [Issue 5] for Direction B of inter-vendor training collaboration.

[Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it?

The following observations related to Direction B were discussed/agreed in RAN1#120 [5] and will be captured in the TR.

- When dataset A include dataset B, for case 3 (Direction B)

  - 2 sources [ZTE,vivo] observe minor performance loss  (-0.9% ~ +0.01%) relative to case 1

- When dataset A does not include dataset B, for case 3, Direction B,

  - 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1.

  - 1 source [QC] observes significant loss (-20%) relative to case 1

The above observation indicates the majority of results (6 out of 7 sources) show minor loss to positive gain relative to case 1. However, the evaluation results and observations drawn for Direction B are relatively less compared to results submitted for other directions.

[Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

As of RAN1#120bis, there is no detailed discussion related to [Issue 7] for Direction B of inter-vendor training collaboration.

Based on the above, apparently Direction B is treated not as important as Direction A and Direction C by companies as it is not being discussed/evaluated as much; thus, we think Direction B can be de-prioritized moving forward.

Proposal 6: For AI/ML-based CSI compression using two-sided model, conclude that Direction B in inter-vendor training collaboration is deprioritized as the evaluation results submitted are relatively limited and there is no agreement reached for majority of the issues identified for Direction B.

### Direction C

Agreement

Continue the study of following directions - on-device operation and UE side offline engineering

- …

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues

  - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them?

  - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it?

  - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing?

  - Note:

    - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder.

    - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder.

    - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder.

There are 3 issues identified for Direction C.

[Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them?

In RAN#120bis, the following conclusion related to dataset for training the reference model was agreed.

Conclusion

- For Direction C, confirm that the specified model should be trained using synthetic data (answer to issue 8).

- For inter-vendor collaboration option 3a-1 of Direction A, confirm that the specified model structure should be determined using synthetic data.

Proposal 7: For AI/ML-based CSI compression using two-sided model, conclude that [Issue 8] identified for Direction C in inter-vendor training collaboration can be considered as addressed.

[Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it?

For the performance impact concern/issue identified for Direction C, companies have submitted evaluation results for performance impact due to mismatch between NW-side data distribution and UE-side data distribution with various dataset generation configurations like dataset A includes dataset B, dataset A does not include dataset B, etc. and agreement was reached in RAN1#120 [5] to capture the observations drawn based on results into the TR. In summary, the observations show:

- For case 2 (model trained on dataset S but tested on dataset B)

  - Moderate to significant performance loss was observed (by 6 sources) relative to case 1.

- For case 2A (model further finetuned on dataset B)

  - If finetune at NW and fix encoder at UE

    - For using actual field data as dataset B

      - Minor to moderate performance loss was observed (by 2 sources) relative to case 1 depending on whether dataset A / B mismatch is considered or not.

    - For using synthetic data as dataset B

      - Minor to moderate performance loss was observed (by 4 sources) relative to case 1 depending on whether dataset A / B mismatch is considered and/or whether dataset A includes dataset B.

  - If finetune at UE and fix decoder at NW

    - For using actual field data as dataset B

      - Moderate to significant performance loss was observed (by 2 sources) relative to case 1.

    - For using synthetic data as dataset B

      - Minor to moderate performance loss was observed (by 2 sources) relative to case 1 when the datasets are different by antenna layout and indoor/outdoor ratio.

      - Significant performance was observed (by 1 source) relative to case 1 when the datasets are different by TxRU mapping.

  - If finetune at both UE and NW sides

    - For using actual field data as dataset B

      - Significant performance loss was observed (by 1 source) relative to case 1 when dataset A / B mismatch is considered, and dataset A does not contain B.

      - Moderate performance loss was observed (by 1 source) relative to case 1 when dataset A / B mismatch is considered, and dataset A equals dataset B.

    - For using synthetic data as dataset B

      - When dataset A / B mismatch is modelled by different scenarios (i.e., by NW side condition)

        - If the input is in spatial-frequency domain, minor to significant performance loss was observed (by 1 source) relative to case 1 depending on whether data Set S is used in the finetune.

        - If the input is in angular-delay domain, similar performance was observed (by 1 source) relative to case 1.

      - When dataset A includes B or is equal to B or has same distribution as B

        - If the input is in spatial-frequency domain, minor to significant performance loss observed (by 3 sources) relative to case 1.

        - If the input is in angular-delay domain, similar performance was observed (by 1 source) relative to case 1.

      - When dataset A / B mismatch is modelled by Rx antenna spacing and dataset A does not contain B and dataset A has same distribution as S

        - Similar performance was observed (by 1 source) relative to case 1.

Observation 5: For AI/ML-based CSI compression using two-sided model, performance impact concern/issue identified by [Issue 9] for Direction C in inter-vendor training collaboration, there are relatively fewer results for each discussed scenario compared to Direction A; thus, it is uncertain about the performance Direction C would achieve in the field.

[Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing?

During RAN1#117 [4], agreement was reached with respect to potential additional information to be exchanged/shared from NW-side to UE-side at high level for Direction C.

Agreement

- For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact.

...

- Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters.

Besides the above, there is no other agreement related to [Issue 10] for Direction C has been reached as of RAN1#120bis.

From the above, our overall observation for Direction C is that it may require additional discussion/study and/or evaluation to confirm its value and feasibility moving forward.

Observation 6: For AI/ML-based CSI compression using two-sided model, Direction C in inter-vendor training collaboration may require additional discussion/study to confirm the feasibility/practicality and address performance concern/issue.

Proposal 8: For AI/ML-based CSI compression using two-sided model, conclude that Direction C in inter-vendor training collaboration is considered as lower priority given that some of the identified issues have not been fully addressed.

# Discussion on other topics related to potential spec impact for CSI compression

A few other topics that have been discussed during the past few meetings, but companies haven’t reached consensus yet, including UE-side performance monitoring and CQI determination during the inference.  this section, we discuss these topics.

## Discussion on other inference aspects

CQI determination

For the evaluation of CSI compression, the specific CQI determination method(s) for AI/ML can be reported by introducing an additional field in the template, e.g.,

-	Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation.

-	Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment.

-	Option 1c: CQI is calculated based on traditional codebook.

-	Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment.

o	Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW.

o	Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW.

-	Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder.

The topic of CQI determination has been discussed during Rel-18 and the following options have been identified and specified in the TR [5] and captured below.

In general, whether to use Option 1 or Option 2 depends on whether CSI reconstruction part of the 2-sided model is available at UE. As companies are evaluating inter-vendor training collaboration options (Option 3a-1/4-1 in Direction A, 3b in Direction B, and 1 in Direction C), in the case that CSI reconstruction model is available at UE, using Option 2a in determining CQI would be feasible and accurate  when using AI/ML-based CSI compression as it would better match the output of the actual decoder at NW-side.

Proposal 9: In AI/ML-based CSI compression using two-sided model, for CQI determination, if the actual or reference CSI reconstruction model is available at UE, adopt Option 2a to determine CQI at UE.

UE-side monitoring

Conclusion

UE-side monitoring is feasible.

Observation

Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

For UE-side monitoring, only high-level conclusion was agreed during RAN1#120bis as shown below.

In the past few meetings, the need of UE-side monitoring and the potential options were discussed, and this topic was also discussed during the offline discussion in RAN1#120bis, however, views from companies are still not converging. FL summarized the options as follows.

Further discuss the need of UE-side monitoring and for potential down-selection among the two

- Based on intermediate KPI (e.g., SGCS) estimation or estimation of other monitoring output

  - This includes the following flavors from previous agreement, where the distinction between Case 2-1 and Case 2-2 is up to UE implementation.

    - Based on the output of the CSI reconstruction model at the UE (Case 2-1)

    - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2)

    - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI

  - FFS: exact monitoring metric, e.g., SGCS statistics, average SGCS against threshold, events based on SGCS statistics, etc.

- Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model

- The discussion assumes having NW-side monitoring with target CSI reporting and potentially RAN4 testing being defined.

- The support is up to additional UE capability.

Conclusion

For NW-side monitoring with target CSI reporting

- Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring

- Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side.

In RAN1#120bis, companies have already concluded NW-side monitoring with target CSI reporting options as shown below.

In our view, the feasibility of UE-side monitoring and the monitoring option(s) may depend on UE capability; thus, it may be difficult to ensure the monitoring quality/accuracy consistency among different UEs. This makes the reliability of UE-side monitoring questionable even though it is agreed that UE-side monitoring is feasible. We think UE-side monitoring may be considered as supplementary when/if supported in addition to NW-side monitoring.

In terms of the UE-side monitoring options, at least from monitoring KPI perspective, the first option (using intermediate KPI (e.g., SGCS)) is well-defined while the KPI for the second option (based on precoded RS transmitted from NW based on the output of the CSI reconstruction model) is not yet discussed in detail yet; thus, the first option is preferred in our view, in particular, considering RAN1#121 is the last meeting for Rel-19 study.

Observation 7: For AI/ML-based CSI compression using two-sided model, UE-side monitoring may be considered as supplementary or optional if NW-side monitoring is supported as its support depends on UE capability.

Observation 8: For AI/ML-based CSI compression using two-sided model, out of the two options for UE-side monitoring, the option: “based on intermediate KPI” is preferred as the other option requires additional study/discussion on monitoring KPI(s).

# Discussion on conclusion of Rel-19 further study on CSI Compression

Based on the discussion in the previous sections, our view for Rel-19 further study of CSI compression (two-sided model) can be summarized as following:

- Improving the trade-off between performance and complexity/overhead

  - Evaluation results show extending SF domain to TSF domain CSI compression and/or CSI compression plus CSI prediction can significantly improve performance.

- Alleviating issues related to inter-vendor training collaboration

  - Based on the progress of discussions, evaluations, and agreements:

    - Direction A can be considered as feasible for significantly alleviating the inter-vendor training collaboration concern while achieving reasonable performance.

    - Direction B can be deprioritized

    - Direction C may require additional discussion/evaluation; thus, it should be considered as lower priority.

Observation 9: Based on the evaluations/observations drawn and study of inter-vendor training collaboration options, consider the following on moving forward for AI/ML-based CSI compression using two-sided model:

- Direction A: prioritized

- Direction B: deprioritized

- Direction C: lower priority than Direction A

From the above observation, our proposal is as follows.

Proposal 10: Conclude that for potential normative work on AI/ML-based CSI compression using two-sided model, RAN1 recommends specifying Direction A.

# Conclusions

In this contribution, we discussed our discuss our view on the progress on the Rel-19 further study on CSI compression (two-sided model), including observations drawn from results submitted by companies as of RAN1#120bis and our recommendation regarding how to move forward after Rel-19. Our observations and proposals are as follows.

For improving trade-off between performance and complexity/overhead:

Observation 1: For CSI compression using 2-sided model, regarding the trade-off between performance and complexity/overhead, when extending spatial-frequency (SF) domain to temporal-spatial-frequency (TSF) domain CSI compression and/or CSI compression plus CSI prediction, the majority of results indicated TSF Case 2 and Case 3 have further performance gain over Case 0 and/or the benchmark based on the results available as of RAN1#120bis.

Observation 2: For CSI compression using 2-sided model, regarding the trade-off between performance and complexity/overhead, using model(s) with lower complexity (e.g., ~10X less complexity than the most complex model) based on agreed scalable model structure can still achieve decent performance gain over the baseline (eType II) based on the results available as of RAN1#120bis, at least for Case 0.

Proposal 1: For CSI compression using 2-sided model, conclude that improving trade-off between performance and complexity/overhead is feasible.

For alleviating/resolving issues related to inter-vendor training collaboration:

Observation 3: For AI/ML-based CSI compression using two-sided model, performance loss concern identified by [Issue 4] for Direction A in inter-vendor training collaboration can be minimized/reduced with the help from dataset generation strategy and/or UE training alternatives.

Observation 4: For AI/ML-based CSI compression using two-sided model, based on the results and observations drawn, RAN1 has already concluded that [Issue 4] for Direction A in inter-vendor training collaboration can be addressed.

Observation 5: For AI/ML-based CSI compression using two-sided model, performance impact concern/issue identified by [Issue 9] for Direction C in inter-vendor training collaboration, there are relatively fewer results for each discussed scenario compared to Direction A; thus, it is uncertain about the performance Direction C would achieve in the field.

Observation 6: For AI/ML-based CSI compression using two-sided model, Direction C in inter-vendor training collaboration may require additional discussion/study to confirm the feasibility/practicality and address performance concern/issue.

Proposal 2: For AI/ML-based CSI compression using two-sided model, considering the agreements reached related to additional information to be shared from NW-side to UE-side, conclude that [Issue 1] identified for Direction A in inter-vendor training collaboration can be considered as addressed.

Proposal 3: For AI/ML-based CSI compression using two-sided model, considering the agreement reached related to proprietary information concern of Direction A, conclude that [Issue 2] identified for Direction A in inter-vendor training collaboration can be considered as addressed.

Proposal 4: For AI/ML-based CSI compression using two-sided model, based on LS reply from RAN2 for NW-side sharing dataset/model parameter transfer, conclude that [Issue 3] identified for Direction A in inter-vendor training collaboration can be considered as addressed.

Proposal 5: For AI/ML-based CSI compression using two-sided model, conclude that Direction A in inter-vendor training collaboration is feasible.

Proposal 6: For AI/ML-based CSI compression using two-sided model, conclude that Direction B in inter-vendor training collaboration is deprioritized as the evaluation results submitted are relatively limited and there is no agreement reached for majority of the issues identified for Direction B.

Proposal 7: For AI/ML-based CSI compression using two-sided model, conclude that [Issue 8] identified for Direction C in inter-vendor training collaboration can be considered as addressed.

Proposal 8: For AI/ML-based CSI compression using two-sided model, conclude that Direction C in inter-vendor training collaboration is considered as lower priority given that some of the identified issues have not been fully addressed.

For other topics related to potential spec impact for CSI compression:

Observation 7: For AI/ML-based CSI compression using two-sided model, UE-side monitoring may be considered as supplementary or optional if NW-side monitoring is supported as its support depends on UE capability.

Observation 8: For AI/ML-based CSI compression using two-sided model, out of the two options for UE-side monitoring, the option: “based on intermediate KPI” is preferred as the other option requires additional study/discussion on monitoring KPI(s).

Proposal 9: In AI/ML-based CSI compression using two-sided model, for CQI determination, if the actual or reference CSI reconstruction model is available at UE, adopt Option 2a to determine CQI at UE.

For conclusion on Rel-19 further study of CSI compression:

Observation 9: Based on the evaluations/observations drawn and study of inter-vendor training collaboration options, consider the following on moving forward for AI/ML-based CSI compression using two-sided model:

- Direction A: prioritized

- Direction B: deprioritized

- Direction C: lower priority than Direction A

Proposal 10: Conclude that for potential normative work on AI/ML-based CSI compression using two-sided model, RAN1 recommends specifying Direction A.

# References

R1-253111, Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface), Ad-hoc Chair (CMCC), RAN1#120bis.

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), RAN#102.

R1-2503034, Final summary of Additional study on AI/ML for NR air interface: CSI compression, Qualcomm (Moderator), RAN1#120bis.

R1-240, Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface), Ad-hoc Chair (CMCC), RAN1#117.

R1-2501546, Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface), Ad-hoc Chair (CMCC), RAN1#120.

R1-2407478, Session notes for 9.1 (Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface), Ad-hoc Chair (CMCC), RAN1#118.

R1-2503105 (R2-2503169), Reply LS on signalling feasibility of dataset and parameter sharing, RAN2, RAN2#129bis.

R1-2140725, Summary of Evaluation Results for Additional study on AI/ML for NR air interface: CSI compression, Qualcomm (Moderator), RAN1#119.