3GPP TSG RAN WG1 Meeting #121	R1-2503232

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.1.3

Source:	Futurewei

Title:	Discussion on CSI Processing Unit for AI/ML-based CSI prediction

Document for:	Discussion and decision

# Introduction

During RAN1 meeting #120bis [1], the topic of AI/ML-based Processing Unit (APU) was discussed in three agenda items under AI 9.1 (AI/ML for NR Air Interface); 9.1.4.1 (CSI Compression), 9.1.1 (BM) and 9.1.3 (CSI Prediction). The following agreements were reached.

Agreement in AI 9.1.4.1 (CSI Compression).

- Introduce a dedicated AI/ML PU for AI/ML features for UE,

  - The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

Agreement in AI 9.1.1 (Beam Management).

- For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection:

      - Option 1: only dedicated AI/ML PU is occupied, OAPU=N1 is reported by UE, and OCPU=0

      - Option 2: only legacy CPU is occupied, OCPU=M it is reported by UE.

      - Option 3: both dedicated AI/ML PU and legacy CPU are occupied, OAPU=N2 is reported by UE, And OCPU=1.

Note: The supported option by UE is reported by UE capability, if multiple options are supported.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

  - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Agreement in AI 9.1.3 (CSI Prediction).

As AI/ML PU discussions took place in CSI compression and BM agenda items first, the following applicable agreement was adopted for CSI prediction use case.

Introduce a dedicated AI/ML PU for AI/ML features for UE,

- The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

For CSI prediction using UE side model, for inference, consider following options for potential down selection

- Option 1: only dedicated AI/ML PU (OAPU) is occupied

- Option 2: only legacy CPU (OCPU) is occupied

- Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- FFS whether OAPU and OCPU are based on defined rule and/or reported by UE

- Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported.

The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

- Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

In this contribution, we continue to present our views related to AI/ML based Processing Unit (APU).

# Discussion on CSI Processing Unit

## Sharing AI/ML computing resources among different AI/ML-based features

In Meeting #120bis, RAN1 agreed to introduce a dedicated AI/ML PU for AI/ML features for UE, instead of reusing the existing CPU for AI/ML-based CSI calculation. The agreement also emphasized that

“the AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.”

It is our understanding that APU should also be used for additional use cases beyond CSI-related AI/ML use case(s).

This is because, by considering the processing resources for AI/ML as a shared pool among different AI/ML-related features (both CSI-related and non-CSI-related), more efficient management and utilization of the dedicated AI/ML processing resources within the UE can be achieved. In fact, sharing AI/ML-based computing resources among different AI/ML features and functionalities offers several significant advantages and is a better method for resource utilization due to the following reasons:

- Efficient Resource Utilization: A shared pool, i.e., considering AI/ML resources as common resources across all AI/ML features/use cases, allows for dynamic allocation of resources based on the active functionalities and their current demands, leading to better overall efficiency.

- Flexibility and Scalability: As the number of AI/ML-based features in wireless systems grows, managing dedicated resources for each can become cumbersome (if APU is only limited to CSI-related use cases, then some other methods will be needed for new AI/ML use cases introduced later to the standard). A shared resource pool provides a more flexible and scalable architecture. New AI/ML functionalities can be introduced without requiring a complete overhaul of the resource allocation strategy.

- Reduced Power Consumption: By sharing a common AI resource pool/management, UE can potentially reduce overall power consumption compared to having multiple dedicated processing units that require separate management tasks.

- Simplified UE Capability Reporting: Instead of reporting the AI/ML processing capability for each individual feature/use case separately, UE can report the characteristics of the shared AI/ML resource pool and potentially a complexity factor for each functionality. This simplifies the UE capability reporting mechanism and provides the network with a clear understanding of the overall AI/ML processing power available on the device.

- Alignment with Industry Trends: Modern mobile devices and chipsets are increasingly incorporating dedicated AI engines or accelerators that are designed to be shared across various AI applications. Adopting a shared resource framework in the wireless standard aligns with these industry trends and can facilitate the implementation and commercialization of AI/ML-based features.

- Support for Concurrent AI/ML Operations more efficiently: A shared AI/ML resource pool can enable the UE to handle concurrent AI/ML operations more efficiently as it can reduce the complexity of handling interactions among AI/ML features/operations. This is crucial as UEs might need to perform tasks like CSI prediction and beam management simultaneously.

- Considerations for AI processing unit Counting: When AI/ML tasks are executed on shared, potentially dedicated, AI processing units, it becomes more logical to account for the utilization of this shared resource separately from the main CPU used for legacy functions. This provides a more accurate picture of the processing load associated with AI/ML operations and avoids conflating it with the demands of traditional wireless protocols. The network can then manage and configure AI/ML features based on the reported capacity of this shared pool.

Observation 1: Supporting the sharing of AI/ML-based computing resources among different AI/ML features and use cases is a more efficient, flexible, and scalable approach. It optimizes resource utilization, potentially reduces power consumption, simplifies UE capability reporting, and aligns with industry trends in AI hardware design.

We therefore propose that, for UE-sided AI/ML operations, including but not limited to CSI-related use cases, RAN1 should adopt a framework where a common pool of AI/ML-based computing resources (e.g., processing units, AI engine, APU) is shared among different AI/ML features and functionalities. UE should report the capabilities of this shared AI/ML resource pool to the network, potentially including a scaling factor per functionality or model based on the associated complexity.

Proposal 1: Support the sharing of AI/ML-based computing resources (i.e., AI/ML Processing Units) among different AI/ML features and use cases, including AI/ML-based beam management, CSI compression/prediction, and positioning, for R19.

## New timeline for model inference

In RAN1 meeting 120bis [1], it was agreed that AI/ML-based CSI reporting/calculation will be counted separately as APU. One of the advantages of separate APU accounting is that it can address concerns about timeline and issues such as switching/activation delay. Separate APU accounting can help in understanding the resource utilization implications of these factors to properly determine if additional timeline considerations are needed for APU.

In the discussions, both online and offline (via emails), there was a general agreement that the introduction of APU necessitates a re-evaluation of the APU occupancy timeline. In particular, most companies acknowledge the potential impact of model inference delay. The handling of model switching/activation delay and the APU occupancy during the observation window were also identified as important aspects to consider in defining the CPU/APU occupancy timeline for AI/ML-based CSI reporting.

It is our understanding that a new or updated timeline framework tailored to the specific characteristics of AI/ML model inference is essential for the successful implementation and optimization of APU. This framework provides a more accurate understanding of the temporal resource requirements of AI/ML inference, which is crucial for efficient overall resource management, including a more realistic view of potential CPU/APU utilization durations.

When it comes to the solutions, some companies proposed to reuse or slightly modify the legacy timeline in the discussions, while others focused on defining separate timelines for the legacy CPU and the new APU.

In RAN1 meeting 120bis [1], BM and CSI prediction use cases agreed on 3 options for APU counting (under potential down selection). Take the CSI prediction agreement for example, it says,

For CSI prediction using UE side model, for inference, consider following options for potential down selection

- Option 1: only dedicated AI/ML PU (OAPU) is occupied

- Option 2: only legacy CPU (OCPU) is occupied

- Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- FFS whether OAPU and OCPU are based on defined rule and/or reported by UE

Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported.

We think whether a new timeline is needed depends on which option will be eventually supported, or multiple options will be supported.

- If Option 1 is supported, then a new timeline is needed as the legacy timeline will not present to indicate the APU occupation time (i.e., the legacy timeline is not relevant here).

- If Option 2 is supported, then the legacy timeline will be reused. However, it is not clear how this legacy timeline addresses AI/ML-related delays. In our understanding, at least it needs to be modified to consider the special needs of AI/ML-based approach.

- If Option 3 is supported, then an integrated timeline (i.e., legacy with modification + new) is needed.

- Depending on UE capability, if multiple options are supported/allowed, then the solution would be a combination of the above solutions.

Based on the analysis above, unless Option 2 is selected as the only solution for APU calculation, a newly defined timeline for APU is necessary.

Proposal 2: Define a new timeline for APU calculation and reporting.

## The considerations of the new timeline for APU

The AI/ML approach introduces some additional processes on the UE and that could add extra time to the timeline. There are at least the following time/delay need to be considered.

  - Model loading time: if the model has not been loaded into the dedicated AI engine.

  - Model unloading time: a necessary procedure if the current model needs to be unloaded in order to load the new model.

  - Model activation time: if the model is not in the active state.

  - Model deactivation time: a necessary procedure if the current activated model needs to be deactivated and unloaded in order to load and activate the new model

  - Model switching time: a procedure that contains old model deactivation + old model unloading + new model loading + new model activation.

  - Model inference time

However, it would be too complicated for the UE to measure and report all these factors/parameters, and it will also introduce lots of signaling overhead. We therefore propose that a UE considers all these factors in its timeline calculation for an APU but only reports the total time needed (i.e., sum of all the applicable ones) for the APU.

Proposal 3: UE considers model (un)loading, (de)activation, switching, and inference time when calculating the time needed for AI/ML-based processing, but only reports the total time needed as UE capability for APU-based timeline reporting.

## Timeline is UE-capability-dependent

The time required for model inference, (de)activation, and (un)loading can vary significantly based on

- the UE's processing capabilities (determined mostly by its hardware),

- assigned resources (UE may be able to assign more or less computing resource at certain point/situation),

- the specific AI/ML model used, and

- the overall AI/ML workload on the device.

Therefore, the time it takes for a UE to calculate and report the number of APUs may vary by the different types of UEs (due to the different levels of capability of UEs), and it could vary from time to time even for the same UE (due to the dynamic assignment of resources). The new timeline framework should allow the UE to report updates of its APU timeline, enabling the network to configure/control CSI reporting parameters more accurately.

Observation 2: The time required for model inference, (de)activation, and (un)loading can vary significantly over time due to many reasons. Therefore, the time it takes for a UE to calculate and report the number of APUs may vary by the different types of UEs, and it could vary from time to time even for the same UE. The new timeline framework should allow the UE to report updates of its APU timeline, enabling the network to configure/control CSI reporting parameters more accurately.

## UE capability reporting

In RAN1 meeting 120bis [1], the group agreed to 3 options for UE side model inference and further study how OAPU and OCPU are determined, as shown in the following agreement.

For CSI prediction using UE side model, for inference, consider following options for potential down selection

- Option 1: only dedicated AI/ML PU (OAPU) is occupied

- Option 2: only legacy CPU (OCPU) is occupied

- Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- FFS whether OAPU and OCPU are based on defined rule and/or reported by UE

- Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported.

The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

- Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

It is our understanding that in both Option 1 and Option 3, OAPU  should always be reported by the UE, just like in the BM case (options of BM case shown below).

  - Option 1: only dedicated AI/ML PU is occupied, OAPU=N1 is reported by UE, and OCPU=0

  - Option 3: both dedicated AI/ML PU and legacy CPU are occupied, OAPU=N2 is reported by UE, And OCPU=1.

Also note that in both Option 1 and Option 3 of the BM case, OCPU  is always based on defined rules. We think the for CSI prediction, the OCPU can also be determined by defined rules.

For Option 2, in BM use case, OCPU  is reported by the UE, as shown below.

  - Option 2: only legacy CPU is occupied, OCPU=M it is reported by UE.

We think this agreement also applies to CSI prediction, i.e., in Option 2 of CSI prediction, the OCPU  is also reported by the UE.

As the result of the above discussion, we have the following proposal.

Proposal 4: For CSI prediction using UE-sided model, regarding whether OAPU and OCPU are based on defined rules and/or reported by UE for inference:

- For Option 1 and Option 3, OAPU  is  reported by the UE, while OCPU  can be based on defined rules.

- For Option 2, OCPU  is reported by the UE.

# Conclusions

In this contribution, we present our views on specification support for AI/ML-based CSI prediction, focusing on the discussion of CSI processing unit. Based on the discussions in the previous sections we propose the following:

Observation 1: Supporting the sharing of AI/ML-based computing resources among different AI/ML features and use cases is a more efficient, flexible, and scalable approach. It optimizes resource utilization, potentially reduces power consumption, simplifies UE capability reporting, and aligns with industry trends in AI hardware design.

Proposal 1: Support the sharing of AI/ML-based computing resources (i.e., AI/ML Processing Units) among different AI/ML features and use cases, including AI/ML-based beam management, CSI compression/prediction, and positioning, for R19.

Proposal 2: Define a new timeline for APU calculation and reporting.

Proposal 3: UE considers model (un)loading, (de)activation, switching, and inference time when calculating the time needed for AI/ML-based processing, but only reports the total time needed as UE capability for APU-based timeline reporting.

Observation 2: The time required for model inference, (de)activation, and (un)loading can vary significantly over time due to many reasons. Therefore, the time it takes for a UE to calculate and report the number of APUs may vary by the different types of UEs, and it could vary from time to time even for the same UE. The new timeline framework should allow the UE to report updates of its APU timeline, enabling the network to configure/control CSI reporting parameters more accurately.

Proposal 4: For CSI prediction using UE-sided model, regarding whether OAPU and OCPU are based on defined rules and/or reported by UE for inference:

- For Option 1 and Option 3, OAPU  is  reported by the UE, while OCPU  can be based on defined rules.

- For Option 2, OCPU  is reported by the UE.

# References

3GPP RAN1 meeting #120bis, Chairman’s Notes, April 2025, Wuhan.