3GPP TSG-RAN WG1 Meeting #121	Tdoc R1-2503239

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.15.1

Source:	Ericsson

Title:	UE Features for Rel-19 AI/ML for NR Air Interface

Document for:	Discussion, Decision

# Introduction

This contribution discusses the UE features for Rel-19 AI/ML for NR Air Interface.

# Beam Management (FGs 58-1-x)

During RAN1#120bis, RAN1 agreed to support several features related to AI/ML beam management. It was however not agreed whether the feature group naming should include a possible purpose of the feature (e.g. for UE/NW-sided model inference). Including the purpose of a feature is unprecedented in RAN1 and should be avoided both for this and other AI/ML features.

For the BM features, possible purposes of the features should not be part of any feature group naming (remove “for UE/NW-sided model inference” across all features).

## Increased number of reported RSs for beam management

During RAN1#120bis, RAN1 agreed to support a feature of “Increased number of reported RSs for beam management”. It is FFS on possible values for M. The values for M could be the same as the candidate set values as for component one on feature group 2-24 (max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot).

For the increased number of reported RSs for beam management feature, support candidate values M = 8, 16, 32, 64.

## UE-side beam prediction for BM Case 1

For the UE-side beam prediction for BM-Case 1, UE capability should support both when the set B is a subset of set A and when set B is not a subset of set A. The two sub-use cases will not be fundamentally different in the UE complexity, and hence could both be supported. The AI/ML PU is an ongoing discussion, but it is clear that there need to be impact on the UE capability. In summary, our view on the needed components are:

For components of FG 58-1-2 and FG 58-1-3 in addition to the agreed components:

  - Support setB-subset-of-setA and setB-different-from-setA

  - Supported maximum number of predicted beams in each reporting instance, candidate value is 1,2 or 4

  - Support for UE-assisted performance monitoring

  - AI/ML PU (FFS on details)

## UE-side beam prediction for BM Case 2

For the UE-side beam prediction for BM-Case 2, UE capability should include at least the values for the number of time-instances that it supports to predict.

For components of FG 58-1-4 and FG 58-1-5 in addition to the agreed components:

  - Support SetB-equal-to-SetA, setB-subset-of-setA and setB-different-from-setA

  - Supported maximum number of predicted beams in each reporting instance, candidate value is 1, 2 or 4

  - Supported maximum number of predicted time instances, candidate value is 1,2,3 or 4

  - Support for UE-assisted performance monitoring

  - AI/ML PU (FFS on details)

## Data collection for UE-side beam prediction

The data collection should also include the component of the assumptions of set A/B.

For components of FG 58-1-7 5 in addition to the agreed components:

  - Support SetB-equal-to-SetA, setB-subset-of-setA and setB-different-from-setA

# Positioning (FGs 58-2-x)

## L2 FG for positioning method (FG 58-2-1)

For the positioning use case, RAN1 agreed to introduce an FG for the support of UE based positioning Case 1. However, in legacy, RAN2 specified methods capabilities, as can be seen in 38.306 and 38.822.

Another open issue was whether to limit the FG to inference. Case 1 is not supported by new specification text for data collection (i.e., training data collection is OTT), but includes LCM with basic monitoring support. The model inference phase include the procedure where the UE signals the LMF with monitoring outcome, as agreed in RAN1#120b:

Agreement (RAN1#119) For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome Agreement (RAN1#120b) For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

Since AI/ML method involves a list of LCM procedures beyond model inference, it should be described clearly that support of Case 1 includes all necessary LCM procedures, including at least model inference, model performance monitoring, functionality activation/deactivation. We also note that the FG is not a L1 feature group, and can be up to RAN2 to specify.  The need for a L1 feature depends on the outcome of the discussion on assistance data for Case 1.

Move FG 58-2-1 to L2 Feature groups, the same as other positioning methods.

Add the following as components of FG 58-2-1:

  - Support reporting the monitoring outcome with an indication that the target UE cannot perform the Case 1 positioning method.

  - Support LMF initiated activation / deactivation of UE-based positioning Case 1.

  - Support reporting the location estimate as generated by UE-based positioning Case 1.

## Need of new FG for performance monitoring

For model performance monitoring, in RAN1#118, it was concluded that Option A-4 does not need to be further discussed. In RAN1#120bis, Option A-1/2/3 were discussed and informally concluded. It is understood that the agreement of RAN1#119 was sufficient, i.e., Option A-1/2/3 are supported; the existing signaling can be reused, and no further discussion is needed.

The only remaining issue is Option B. If RAN1 agrees to support Option B, UE feature may need to be introduced correspondingly, depending on whether Option B-1 and/or Option B-2 is supported. However, the variants in the Option A group do not imply new UE capabilities, since no new signaling needs to be introduced.

RAN1 agreements related to model performance monitoring of AI/ML positioning Case 1:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. Agreement (RAN1#119) For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

UE-based positioning Case 1 does not require separate FGs for model performance monitoring. Performance monitoring is a component of Case 1 main FG 58-2-1.

## DL PRS resources FGs (58-2-3x)

In RAN1#120bis, FG 58-2-3/3a/3b were agreed as related to DL PRS Resources for UE-based positioning Case 1. Case 1 has no special requirement on DL PRS resources, and they can be fully re-used from existing methods. Thus all values in yellow in this group of FGs can be agreed.

Agree to the current placeholder values for the components in FG 58-2-3/3a/3b.

## Additional FGs

During RAN1#120bis, RAN1 discussed the possibility to have one FG for PRS QCL processing for Case 1, as well as one FG for provisioning of labels.

For the QCL source, the FG may be needed, if a separate processing capability is agreed. On the other hand, if FG 13-1 is re-used (i.e., FG 13-1 is a pre-requisite of FG 58-2-3/3a/3b), no new FG for QCL is needed and existing FG 13-7a can be reused.

RAN1 should discuss whether a new processing capability is needed for Case 1 before deciding whether to introduce FG for QCL processing of the DL PRS for Case 1.

For the provision of labels, RAN1 has not yet agreed to any standardized support for data collection in Case 1, the model is entirely up to UE implementation, thus the UE is free to reuse any of the existing positioning procedure, or any other means, to obtain a label.

Do not Introduce additional FGs for label provisioning for Case 1.

# CSI Prediction (FGs 58-3-x)

The following agreement was made in RAN1#120 for inference for CSI prediction use case:

Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

Based on the above RAN1 agreement, the following Rel-19 UE FGs were agreed in RAN1#120bis.

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 [8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) ] [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] [11. Scaling factor for active resource counting Kp] | 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] | n/a | n/a | n/a | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR [5. Support for the size of DD-basis, N4>1] [6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] [8. Supported values of the maximum number of observation number Candidate values: {FFS}] | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] | FFS | FFS | FFS | FFS: CPU/AIMLPU related information | Optional with capability signalling |

Regarding CSI processing criteria for AI based CSI prediction use case, the following agreement was made in RAN1#120bis

Agreement For CSI prediction using UE side model, for inference, consider following options for potential down selection - Option 1: only dedicated AI/ML PU (OAPU) is occupied - Option 2: only legacy CPU (OCPU) is occupied - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - FFS whether OAPU and OCPU are based on defined rule and/or reported by UE - Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Regarding which of the options to support, RAN1 should strive towards a single option to reduce unnecessary complex specification and implementations. The future high-end UEs supporting the Rel-19 prediction features will likely include an AI/ML PU, and it is unclear whether the existing CPU will be occupied in the same manner with such new unit. Hence, option 1 is the simplest and preferred solution since otherwise the NW/UE would need to manage multiple PU pools/types.

Regarding active CSI-RS resource counting, we support reusing the existing framework for the AI based CSI prediction use case.

The components 8-11 of the basic feature group 58-3-1 for Rel-19 CSI prediction using UE-sided model is defined as follows:

  - 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’)

  - 9. Value for APU occupation, when P/SP-CSI-RS is configured for CMR. FFS the candidate values.

  - 10. Value for CPU occupation, when A-CSI-RS is configured for CMR. FFS the candidate values.

  - 11. Scaling factor for active resource counting Kp. FFS the candidate values.

In RAN1#120bis meeting, the following agreements were made regarding performance monitoring:

Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism

Regarding training data collection, the following agreements were made in RAN1

Agreement For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead. - FFS on CSI report configuration Conclusion For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported. Agreement For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report - FFS on how to indicate without CSI report in CSI-ReportConfig

Additional FGs related to performance monitoring, UE-sided data collection, etc., can be added pending further RAN1 agreements.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the BM features, possible purposes of the features should not be part of any feature group naming (remove “for UE/NW-sided model inference” across all features)

Proposal 2	For the increased number of reported RSs for beam management feature, support candidate values M = 8, 16, 32, 64.

Proposal 3	For components of FG 58-1-2 and FG 58-1-3 in addition to the agreed components:

a.	Support setB-subset-of-setA and setB-different-from-setA

b.	Supported maximum number of predicted beams in each reporting instance, candidate value is 1,2 or 4

c.	Support for UE-assisted performance monitoring

d.	AI/ML PU (FFS on details)

Proposal 4	For components of FG 58-1-4 and FG 58-1-5 in addition to the agreed components:

a.	Support SetB-equal-to-SetA, setB-subset-of-setA and setB-different-from-setA

b.	Supported maximum number of predicted beams in each reporting instance, candidate value is 1, 2 or 4

c.	Supported maximum number of predicted time instances, candidate value is 1,2,3 or 4

d.	Support for UE-assisted performance monitoring

e.	AI/ML PU (FFS on details)

Proposal 5	For components of FG 58-1-7 5 in addition to the agreed components:

a.	Support SetB-equal-to-SetA, setB-subset-of-setA and setB-different-from-setA

Proposal 6	Move FG 58-2-1 to L2 Feature groups, the same as other positioning methods.

Proposal 7	Add the following as components of FG 58-2-1:

a.	Support reporting the monitoring outcome with an indication that the target UE cannot perform the Case 1 positioning method.

b.	Support LMF initiated activation / deactivation of UE-based positioning Case 1.

c.	Support reporting the location estimate as generated by UE-based positioning Case 1.

Proposal 8	UE-based positioning Case 1 does not require separate FGs for model performance monitoring. Performance monitoring is a component of Case 1 main FG 58-2-1.

Proposal 9	Agree to the current placeholder values for the components in FG 58-2-3/3a/3b.

Proposal 10	RAN1 should discuss whether a new processing capability is needed for Case 1 before deciding whether to introduce FG for QCL processing of the DL PRS for Case 1.

Proposal 11	Do not Introduce additional FGs for label provisioning for Case 1.

Proposal 12	The components 8-11 of the basic feature group 58-3-1 for Rel-19 CSI prediction using UE-sided model is defined as follows:

	8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’)

	9. Value for APU occupation, when P/SP-CSI-RS is configured for CMR. FFS the candidate values.

	10. Value for CPU occupation, when A-CSI-RS is configured for CMR. FFS the candidate values.

	11. Scaling factor for active resource counting Kp. FFS the candidate values.

Proposal 13	Additional FGs related to performance monitoring, UE-sided data collection, etc., can be added pending further RAN1 agreements.

# References

RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, 3GPP TSG RAN Meeting #107, Incheon, S. Korea, March 12-14, 2025.

TR 38.822 v18.0.0, “User Equipment (UE) feature list”.