3GPP TSG-RAN WG1 Meeting #121	 R1-2503255

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	9.15.1

Source:	Huawei, HiSilicon

Title:	UE features for AI/ML for NR air interface

Document for:	Discussion and Decision

# Introduction

In RAN1#120bis meeting, the basic framework of UE features for AI/ML for NR air interface was agreed in [1]. Regarding on the remaining issues to be discussed, we provide our views in this contribution.

# UE feature for beam management

The basic feature groups of AI/ML for beam management are agreed in last meeting as summarized in the Annex. Regarding on the yellow highlighting parts, we provide our views in the following.

## 58-1-1

Notation of 58-1-1

During the RAN1#120bis discussion, there is controversy on the notation of 58-1-1 on whether it should restrict the reported RS for BM should only be used for NW side inference, so there is square bracket on [for NW-sided model inference].

Firstly, it has been agreed in Agenda 9.1.1 that the purpose of “NW-sided model, for inference” will not be specified in RAN1 specifications (as shown in below). Thus, there is no basis to specify the content in the square bracket in RAN1 UE feature.

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report

Secondly, UE feature is applied to report the UE capability, not to restrict the gNB behavior. Even there is any restriction to gNB, it is not appropriate to be captured in the UE feature specification.

Therefore, the “for NW-sided model inference” should be removed from the notation.

Number of measured beams

During the evaluations, it became evident that most companies used 64 beams or even more for the size of Set A. For future proof, gNBs will be able to transmit even many more highly directive narrow beams than 64 and they may sweep even larger sizes of Set A, e.g., the number of beams in Set A could be up to 256. The legacy configuration for the total number of CSI-RS resources (up to 64) do not seem sufficient to support a reasonable size of AI/ML-based beam measurement including Set A, which may exceed 64.

Regarding the currently configurable CSI-RS resources and UE capability for beam management, the maximum number of beams that can be measured/configured to UE is up to 64 (TS 38.331).

maxNrofNZP-CSI-RS-ResourcesPerSet INTEGER ::= 64 … BeamManagementSSB-CSI-RS ::= SEQUENCE { maxNumberSSB-CSI-RS-ResourceOneTx ENUMERATED {n0, n8, n16, n32, n64}, maxNumberCSI-RS-Resource ENUMERATED {n0, n4, n8, n16, n32, n64}, maxNumberCSI-RS-ResourceTwoTx ENUMERATED {n0, n4, n8, n16, n32, n64}, supportedCSI-RS-Density ENUMERATED {one, three, oneAndThree} OPTIONAL, maxNumberAperiodicCSI-RS-Resource ENUMERATED {n0, n1, n4, n8, n16, n32, n64} }

It can be seen in the table above that three types of RS numbers can be supported to the UE:

- maxNrofNZP-CSI-RS-ResourcesPerSet, is the maximum number of CSI-RS resource per resource set. This will restrict a larger size of Set A.

- maxNumberCSI-RS-Resource, is the maximum number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured for RSRP measurement. It would limit the number of beams in total that can be configured for CSI measurement, which is due to the limitation of managed number of Tx beams at the UE. This will also restrict a larger size of Set A.

The parameter of maxNumberCSI-RS-Resource is also reflected in the FG 2-24 as UE capability shown in below.

| 2-24 | SSB/CSI-RS for beam measurement | 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a | beamManagementSSB-CSI-RS { 1. maxNumberSSB-CSI-RS-ResourceOneTx 2. maxNumberCSI-RS-Resource 3. maxNumberCSI-RS-ResourceTwoTx 4. supportedCSI-RS-Density 5. maxNumberAperiodicCSI-RS-Resource } | Mandatory with capability signalling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signalling. Component-2, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-3, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} |
| --- | --- | --- | --- | --- | --- |

From our view, considering the number of beams for the current RF module is much more than 64, it is essential to extend the maxNumberCSI-RS-Resource to up to 256 to enable the measurement of Set A. Regarding maxNumberSSB-CSI-RS-ResourceOneTx and maxNumberCSI-RS-ResourceTwoTx, as it only restricts the number of beams configured within per slot, the legacy UE capability may not need to be enhanced. Thus, an additional component (Component 4) should be added to increase the max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP.

Accordingly, the maximum number of reported L1-RSRPs for NW-side model needs to be also increased, e.g., up to 64.

Proposal 1: Update FG 58-1-1 as the cyan-highlighted parts in Table 1.

Table 1 Updates for FG 58-1-1

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management [for NW-sided model inference] | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 4. Increase the max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP | FFS | yes | n/a | Increased number of reported beams for beam management [for NW-sided model inference] is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values : {FFS 8, 16, 32, 64} Component 4 candidate values: {128, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 58-1-2 and 58-1-3

58-1-2/3 correspond to UE side inference for BM-Case 1.

Notation of 58-1-2/58-1-3

Regarding the notation of the two FGs, there is square bracket on [for inference]. Since similar notation of “for UE-sided inference” is already agreed for CSI prediction FG 58-3-1, we think the same principle can be applied to BM, i.e., the “for inference” can be confirmed.

In the following, we provide our views to the components.

58-1-2 components

- Component 2: it is not clear on the meaning of mapping pattern. If it means the case of Set B is a subset of Set A, we doubt whether it will impact UE capability, since such mapping only refers the mapping between logic RS IDs, while how the physical beams are mapped is up to NW and not known by UE. Therefore, reporting this mapping pattern does not make sense.

- Component 3/4/5: In legacy FG 2-35, it already incorporates the max number of CSI reports for A-CSI, SP-CSI and P-CSI reports. Regarding AI/ML based CSI reports, the number of AI/ML calculation/memory resources can be reflected by PU mechanism, which will not impact the number of CSI reports. Therefore, we do not support Component 3/4/5.

- Component 6: Regarding the FFS part, RS type of Set A can also be SSB or CSI-RS.

- Component 7/7a/7b: Component 7 can fully reflect the information of 7a/7b and is more flexible than 7a/7b, e.g., Component 7 can preclude some unreasonable combinations of Set B size and Set A size, e.g., {4, 64}, {24, 32}. Therefore, we prefer to keep Component 7 over 7a/7b.

- Component 8/9: OK to support.

- Component 10: Can be discussed after the monitoring FG is clear.

- Component 11: Not sure whether setB-different-from-setA only refers to wide-to-narrow, or it can also be narrow-to-narrow. Can be further discussed.

- Component 12: OK to support. If the value of K is too large, it may impact the confidence of some predicted beams with lower ranking. E.g., for a classification model, if the reported Top-K beams of {RS#1, RS#2, RS#3, RS#4} have the probability values of {0.8, 0.1, 0.05, 0.01}, the confidence of RS#3 and RS#4 being the 3rd/4th strongest beams may be questionable. To ensure the confidence of predicted beams, UE may prefer not to report K=4 but rather K=2.

- The first FFS on maximum total number of CSI reports across different AI/ML based use-cases: Can be discussed after the PU part has progress.

- The second FFS and third FFS: can be discussed after Component 1-12 are clear.

58-1-3 components

- Component 2: Similar to Component 12 of FG 58-1-2, it can be supported.

Proposal 2: Update FG 58-1-2 and FG 58-1-3 as the cyan-highlighted parts in Table 2.

Table 2 Updates for FG 58-1-2 and FG 58-1-3

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A 3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] [12. Supported maximum number of predicted beams in each reporting instance] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for component 12 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | 1.Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] [2. Supported maximum number of predicted beams in each reporting instance] FFS: other components | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported | FFS: candidate values for component 2 | Optional with capability signalling |

## 58-1-4 and 58-1-5

58-1-4/5 correspond to UE side inference for BM-Case 2.

Notation of 58-1-4/58-1-5

Similar to our analysis to 58-1-2/3, the “for inference” can be confirmed.

In the following, we provide our views to the components.

58-1-4 components

- Component 2/3/4/5/6/7/7a/7b/8/9/10: They are same to BM-Case 1, so same handling to 58-1-2 can be applied.

- Component 11: Similar to Component 12 of FG 58-1-2, it can be supported.

- Component 12 and Component 14: The supported time gap between predicted time instance can be set as {10ms, 20ms, 40ms, 80ms, 160ms}, while the supported number of predicted time instances can be set as {1, 2, 4, 8}. In addition, the two Components can be jointly reported as combination, since the time gap times number of predicted time instances refer to the longest time instance for prediction, which may be restricted due to its accuracy.

- Component 13: Whether the total number of predicted beams will impact the UE capability can be further clarified.

- Component 15: OK to support. The time gap between observation instances may also impact the performance and the design of the model to be trained.

- The first/second/third FFS: Same as FG 58-1-2, they can be discussed later.

- The fourth FFS: Similar to Component 11 of FG 58-1-2, it can be discussed after clarification.

- The fifth FFS: As analyzed earlier, at least Component 12 and 14 can be jointly reported as a combination.

58-1-5 components

- Component 2: It seems duplicated with Component 12 of FG 58-1-4, and can be removed.

Proposal 3: Update FG 58-1-4 and FG 58-1-5 as the cyan-highlighted parts in Table 3.

Table 3 Updates for FG 58-1-4 and FG 58-1-5

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case2] [4. Maximum number of inference report(s) activated for BM-Case2] [5. Maximum number of inference report(s) triggered for BM-Case2] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A FFS: component 7 or component 7a+7b or 7+7a+7b ]8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] ]9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] [11. Supported maximum number of predicted beams in each predicted time instance] [12. Supported combinations of maximum number of predicted time instances and time gap between predicted time instances] [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [14. Supported value(s) of time gap between predicted time instances] [15. Supported value(s) of setB periodicity] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection FFS: whether/how to report each of supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) FFS: whether/how to merge some of above components to report combination(s) of supported values | FFS | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] [2. Maximum number of future time instance] FFS: other components | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |

## 58-1-7

Max number of measured beams for Set A

For data collection, similar to the discussion for FG 58-1-1, UE needs to measure Set A to obtain the label. Thus, an additional component (Component 4) should be added to increase the max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP for Set A.

FG for BM-Case 1 and BM-Case 2

For Component 3 of FG 58-1-7, the candidate cases include {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}. From our view, UE capability of supporting BM-Case 1 and BM-Case 2 should be separated also from the data collection perspective, since it may impact the configuration on the resources of Set B and Set A by gNB. E.g., from NW perspective, the flavors on the resources of Set B and Set A (e.g., number of beams, periodicity of Set B/Set A, time offset between Set B and Set A, etc.) may be different between BM-Case 1 and BM-Case 2. To ensure the aligned configuration between training and inference, gNB should be clear on the intended BM-Case 1 and BM-Case 2 by UE early in the training phase.

Therefore, FG 58-1-7 should be applied for BM-Case 1, and an additional FG, namely 58-1-7a, should be introduced for BM-Case 2.

Proposal 4: Update FG 58-1-7 as the cyan-highlighted parts in Table 4.

Table 4 Updates for FG 58-1-7

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] FFS: components for maximum number of resources 4. The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP for Set A | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | FFS: separate rows/FGs for BM case 1 and BM case 2 Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 Component 4 candidate values: {32, 64, 128, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-7a | Data collection for UE-side beam prediction for BM case 2 | 1. Support of UE-side beam prediction for BM case 2 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] 4. The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP for Set A | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported for BM case 2 | Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 Component 4 candidate values: {32, 64, 128, 256} | Optional with capability signalling |

# UE feature for positioning accuracy enhancement

The basic feature groups of AI/ML for positioning accuracy enhancement are agreed in last meeting as summarized in the Annex. Regarding on the yellow highlighting parts, we provide our views in the following.

Notation of 58-2-1

Since UE-based positioning Case 1 is the only positioning feature in Rel-19, it is fine not to add “for inference”. We are also fine to add it to be aligned with other UE features.

Prerequisite

Regarding the prerequisite, the UE needs to support the reception of DL-PRS resources and be able to process DL-PRS. But the common DL PRS Processing Capability in FG 13-1 has the Field name and Parent IE in TS 38.331 as listed in the below table:

| Field name in TS 38.331 | Parent IE in TS 38.331 |
| --- | --- |
| 1 supportedBandwidthPRS-r16 2 dl-PRS-BufferType-r16 3 durationOfPRS-Processing-r16 4 maxNumOfDL-PRS-ResProcessedPerSlot-r16 | PRS-ProcessingCapabilityPerBand-r16 LPP |

Since UE-based positioning Case 1 is a UE feature independent of other positioning method, it should also be independent of PRS capability designed for other Rel-16 related positioning methods. It means UE supporting UE-based positioning Case 1 does not need to support other Rel-16 related positioning methods. Therefore, there is no prerequisite for FG 58-2-1 to FG 58-2-3b.

Components

The square brackets for the components of FG 58-2-3, 3a, and 3b can be removed.

Proposal 5: Update FG 58-2-1 to FG 58-2-3b as the cyan-highlighted parts in Table 5.

Table 5 Updates for FG 58-2-1 to FG 58-2-3b

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | N/A | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | N/A | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Need for location server to know if the feature is supported. | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | [1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}] | N/A | N/A | N/A |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands] | N/A | N/A | N/A |  | Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | [1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] | N/A | N/A | N/A |  | Need for location server to know if the feature is supported. [Note: the reported value is the total number across all bands in the corresponding BC] Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

# UE feature for CSI prediction

The basic feature groups of AI/ML for CSI prediction are agreed in last meeting as summarized in the Annex. Regarding on the yellow highlighting parts, we provide our views in the following.

Additional FGs for inference

In addition to the agreed FGs, the enhanced FGs for Rel-19 CSI prediction may borrow the corresponding enhanced FGs of Rel-18 CSI prediction (i.e., 40-3-2-1b, 40-3-2-2, 40-3-2-3, 40-3-2-3a, 40-3-2-7, 40-3-2-7a, 40-3-2-8, 40-3-2-9, 40-3-2-11, and 40-3-2-12).

Components for 58-3-1

- Component 8: OK to support.

- Component 9/10/11: Can be discussed after the PU part has progress.

Components for 58-3-2

- Component 5: OK to support.

- Component 6: It seems redundant with Component 3, and can be clarified further.

- Component 8: It needs more clarification: for A-CSI-RS, the max number of CSI-RS resources can be reflected by an additional FG, similar to FG 40-3-2-1b of Rel-18 CSI prediction. For P/SP-CSI-RS, there seems no need to reflect this UE capability since it is UE implementation.

Proposal 6: Update FG 58-3-1 and FG 58-3-2 as the cyan-highlighted parts in Table 6.

- Additional FGs could be added to emulate the additional FGs of Rel-18 CSI prediction FG 40-3-2-1b, 40-3-2-2, 40-3-2-3, 40-3-2-3a, 40-3-2-7, 40-3-2-8, 40-3-2-9, 40-3-2-11, and 40-3-2-12.

Table 6 Updates for FG 58-3-1 and FG 58-3-2

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 [8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’)] [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] [11. Scaling factor for active resource counting Kp] | 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] |  | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR [5. Support for the size of DD-basis, N4>1] [6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] [8. Supported values of the maximum number of observation number Candidate values: {FFS}] | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] |  | FFS: CPU/AIMLPU related information |  |

# Conclusions

In this contribution, we discussed the FGs for Rel-19 NR_AIML_air. The following proposals are provided.

Proposal 1: Update FG 58-1-1 as the cyan-highlighted parts in Table 1.

Proposal 2: Update FG 58-1-2 and FG 58-1-3 as the cyan-highlighted parts in Table 2.

Proposal 3: Update FG 58-1-4 and FG 58-1-5 as the cyan-highlighted parts in Table 3.

Proposal 4: Update FG 58-1-7 as the cyan-highlighted parts in Table 4.

Proposal 5: Update FG 58-2-1 to FG 58-2-3b as the cyan-highlighted parts in Table 5.

Proposal 6: Update FG 58-3-1 and FG 58-3-2 as the cyan-highlighted parts in Table 6.

- Additional FGs could be added to emulate the additional FGs of Rel-18 CSI prediction FG 40-3-2-1b, 40-3-2-2, 40-3-2-3, 40-3-2-3a, 40-3-2-7, 40-3-2-8, 40-3-2-9, 40-3-2-11, and 40-3-2-12.

# References

R1-2502959, “Session Notes of AI 9.15.1”, RAN1#120bis, April 7 – 11, 2025.

# Annex: UE feature list of NR_AIML_air Agreed in RAN1#120bis

#### Beam management

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agree

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management [for NW-sided model inference] | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 FFS: whether/how to have separate FG for some of above components or how to merge components | FFS | yes | n/a | Increased number of reported beams for beam management [for NW-sided model inference] is not supported | FFS | FFS | FFS | FFS | Component 3 candidate values: {FFS} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] [12. Supported maximum number of predicted beams in each reporting instance] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for component 12 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | - Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] [2. Supported maximum number of predicted beams in each reporting instance] FFS: other components | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: candidate values for component 2 | Optional with capability signalling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case2] [4. Maximum number of inference report(s) activated for BM-Case2] [5. Maximum number of inference report(s) triggered for BM-Case2] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] [11. Supported maximum number of predicted beams in each predicted time instance] [12. Supported maximum number of predicted time instances] [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [14. Supported value(s) of time gap between predicted time instances] [15. Supported value(s) of setB periodicity] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection FFS: whether/how to report each of supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) FFS: whether/how to merge some of above components to report combination(s) of supported values | FFS | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] [2. Maximum number of future time instance] FFS: other components | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] FFS: components for maximum number of resources | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | FFS | FFS | FFS | FFS | FFS: separate rows/FGs for BM case 1 and BM case 2 Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### Positioning

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | FFS | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | [1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | [1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. [the reported value is the total number across all bands in the corresponding BC] Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

#### CSI prediction

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 [8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) ] [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] [11. Scaling factor for active resource counting Kp] | 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] | n/a | n/a | n/a | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR [5. Support for the size of DD-basis, N4>1] [6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] [8. Supported values of the maximum number of observation number Candidate values: {FFS}] | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] | FFS | FFS | FFS | FFS: CPU/AIMLPU related information | Optional with capability signalling |