3GPP TSG-RAN WG1 Meeting #121	R1-2503258

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on startSFN for positioning UTW

Document for:	Discussion and Decision

Introduction

RAN1 received an LS from RAN2 on the startSFN for positioning UTW in R1-2503206 [1], with the content incorporated below.

1. Overall Description: According to the RAN1 higher layers parameter list, RAN2 implemented the parameters startSFN, periodicityAndOffset and duration in IE SRS-PosUplinkTransmissionWindowConfig in TS 38.331. However, there are different understandings in RAN2 on the need and interpretation of the parameter startSFN, which leads to different understanding on the actual time start of the UTW. Therefore, RAN2 would like to ask RAN1 the following questions with regards to the configuration of the parameter startSFN for the UTW: Q1: Is the parameter startSFN needed for the UTW? Q2: If answer to Q1 is ‘yes’, how this parameter should be interpreted?

- startSFN was agreed at RAN1#114 and later included in the higher layer parameter sent to RAN2 after RAN1#115, assuming RAN2 will take care and make a proper configuration accordingly.

Agreement The UL time window for UL SRS for positioning with Tx hopping can be configured to be periodic with configurable starting SFN, slot and symbol number, periodicity, duration - FFS values for starting SFN, slot and symbol number, periodicity and duration

Based on the current signaling design from RAN2, this paper will discuss the issue raised by RAN2.

Discussion

- According to our understanding, the intention of configuring SFN and the slot number is to configure the slot offset within a periodicity, which does not actually require an explicit SFN field.

- Moreover, from the current ASN.1 of TS 38.331, the encoding of SFN and slot number is already included in the slot offset of the field periodicityAndOffset, and thus adding startSFN is not needed.

#### – SRS-PosTx-Hopping The IE SRS-PosTx-Hopping specifies the frequency hopping configuration for SRS for Positioning transmission. SRS-PosTx-Hopping information element -- ASN1START -- TAG-SRS-PosTx-Hopping-START SRS-PosTx-Hopping-r18 ::= SEQUENCE { srs-PosConfig-r18 SRS-PosConfig-r17, bwp-r18 BWP OPTIONAL, -- Need R inactivePosSRS-TimeAlignmentTimer-r18 TimeAlignmentTimer OPTIONAL, -- Need M inactivePosSRS-RSRP-ChangeThreshold-r18 RSRP-ChangeThreshold-r17 OPTIONAL, -- Need M srs-PosUplinkTransmissionWindowConfig-r18 SetupRelease { SRS-PosUplinkTransmissionWindowConfig-r18 } OPTIONAL, -- Need M ... } SRS-PosUplinkTransmissionWindowConfig-r18 ::= SEQUENCE { startSFN-r18 INTEGER(0..1023), windowPeriodicityAndOffset-r18 CHOICE { periodicityAndOffset-r18 SRS-PeriodicityAndOffset-r16, periodicityAndOffset-Ext-r18 SRS-PeriodicityAndOffsetExt-r16 }, duration-r18 ENUMERATED {s1,s2,s4,s6}, ... } -- TAG-SRS-PosTx-Hopping-STOP -- ASN1STOP

- Observation: The intention of using starting SFN and slot number by RAN1 is already fulfilled by using the encoding scheme of periodicityAndOffset as the SRS periodicity and offset, and there is no need to further explicitly include the field startSFN anymore.

- However, two options emerge in RAN2 discussion R2-2502081 as follows:

RAN1 does not provide clear clarification to this parameter either in their agreement nor RRC parameter list. So there are two different understanding options regarding to the actual time start of the UTW: Option 1: the UTW starts to allocate and activate at the slot#0 of the configured parameter startSFN; Option 2: the UTW starts to allocate at SFN#0 slot#0, but start to activate from the configured parameter startSFN. The difference between option 1 and option 2 is, when the UTW periodicity cannot be divided by the number of slots in a SFN (e.g., UTW periodicity is 4 slots and a SFN contains 10 slots), option 1 and option 2 will cause different actual activated UTW time location at UE side, see the below figure:

- Based on our understanding, these interpretations are assuming that a periodic UTW has an active time, which was not part of any RAN1 agreement or assumption. Introducing the activation mechanism in RRC causes the problem of potential ambiguity for any RRC configuration, and should not be introduced at this stage.

- With that, we suggest the following proposal for the reply to RAN2.

- Proposal: Reply to the two questions to RAN2 as follows.

Q1: Is the parameter startSFN needed for the UTW? RAN1 reply: No. RAN1 understands the startSFN is already included in periodicityAndOffset. - Q2: If answer to Q1 is ‘yes’, how this parameter should be interpreted? - RAN1 reply: Not applicable.

Conclusion

This paper discussed the two questions from RAN2, based on the RAN1 discussion/agreements and the current signaling design from RAN2, which leads to the following proposal:

- Proposal: Reply to the two questions to RAN2 as follows.

Q1: Is the parameter startSFN needed for the UTW? RAN1 reply: No. RAN1 understands the startSFN is already included in periodicityAndOffset. - Q2: If answer to Q1 is ‘yes’, how this parameter should be interpreted? - RAN1 reply: Not applicable.

Reference

- R1-2503206, “LS on the startSFN parameter for positioning SRS with TX frequency hopping”, RAN1 #121, St Julian's, Malta, May, 2025