3GPP TSG-RAN WG1 Meeting #121	R1-2503275

# St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.2.1

Source:	Huawei, HiSilicon

Title:	On UE initiated/event-driven beam management

Document for:  Discussion and Decision

# Introduction

In RAN1#120-bis, some agreements pertaining to UEI beam management were achieved [1] while there are still some remaining issues that need to be further discussed. In this paper, we provide our views on these issues.

# Event-triggered evaluation

## Condition(s) of resetting the counter

In RAN#120-bis [1], following revised Candidate#2 was supported as one condition for resetting the counting while some other candidate conditions still need to be discussed:

Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary) Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

It what follows, we provide our views on other condition(s) under which the counter should be reset. Regarding Candidate#1, our understanding is that RRC reconfiguration/update for new beam can only add or remove resources from the new beam resource set. If the new beam RS corresponding to the counter is removed from the new beam resource set, its corresponding counter should be reset and the counter whose corresponding new beam is not updated should keep counting.

Regarding Candidate#3, we think the counter of the beam that has triggered the report should be reset to zero to avoid the possibility of re-triggering the event immediately after UEI beam report is transmitted or, if supported, immediately after the prohibit timer expires. In turn, counters associated with other new beams should keep counting after UEI beam report since, if they trigger the event, the triggering beam in the corresponding report would be different from that of the earlier report.

Regarding Candidate#4, there is no NW response in Mode-B. In Mode-A, DCI in step-2 can be viewed as a NW response. However, if the counter is reset after receiving DCI in step-2 but the beam reporting in step 3 fails or dropped, re-evaluating the event by counting from 0 and re-triggering the beam report would cause a long latency.

Regarding Candidate#5, when a time window is expired, the counter corresponding to the associated new beam should be reset to zero since, according to the definition of Event-2, the report is triggered only when the number of event-2 instances within the time window reaches the threshold.

Regarding Candidate#6, when the threshold for event evaluation is updated, we think the whole procedure should restart. Therefore, all counters associated with the new beams should be reset to zero.

Regarding Candidate#7, many RRC parameter(s) in CSI report configuration have no impact on the counter. For example, RRC parameters related to reporting content, reporting mode, and reporting resource are not related to event determination. Hence, even if they are reconfigured, counters should not be reset.

Proposal 1: In addition to revised Candidate#2, support the following conditions of resetting the counter:

- The new beam RS corresponding to a counter is removed from the new beam resource set by RRC reconfiguration or, if supported, by MAC-CE signaling;

  - In this case, only the counter corresponding the removed new beam RS is reset.

- UEI beam report is transmitted;

  - In this case, only reset the counting of new beam(s) fulfilling triggering condition and reported by the UEI beam report.

- A time window expires.

  - In this case, only the counter corresponding to the associated new beam should be reset.

- The threshold for event evaluation is re-configured by RRC signaling

  - In this case, the counters corresponding to all new beams should be reset.

## Prohibit timer

In RAN1#120bis, the following options for prohibit timer were provided:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement.

For Candidate#1, two options were provided for down-selection. A problem with Option-1B for Mode-A is that, if the first PUCCH is not received successfully and/or UE does not receive DCI scheduling UL resource for the beam report, UE cannot re-transmit the PUCCH since the prohibit timer has already started. As a result, the beam report cannot be transmitted in time. In addition, for Mode-B, PUCCH repetition is useful to compensate for the lack of NW feedback and improve the reliability. However, it is not clear whether/how PUCCH repetition is supported if Option-1B is agreed. On the other hand, in Option-1A, the prohibit timer starts only after the whole beam reporting procedure is completed. This facilitates a seamless support of PUCCH repetition and, if necessary, PUCCH re-transmission. In addition, we think the prohibit timer should be associated with the triggering beam so the first PUCCH cannot be transmitted only when the same beam that has triggered the last transmitted PUCCH re-triggers the event.

For Candidate#2, Option-2A has similar design principles as Option-1A. However, we think Option-1A is clearer which has similar procedure as legacy SR. For Option-2B, it has the same problems as Option-1B as discussed above.

Based on the above analysis, we have the following proposal:

Proposal 2: Regarding the prohibit timer for both Mode-A and Mode-B, support Option-1A with the following modification:

- In the case that triggering-event associated with the CSI report configuration is determined by the same triggering beam(s) as the last transmitted PUCCH, if the prohibit timer is NOT running, the UE can transmit the first PUCCH;

  - At the first symbol after the end of the second PUSCH transmission, the UE starts the prohibit timer

Another issue related to the prohibit timer is that, once the prohibit timer starts running, whether the new beams’ counters should keep counting or reset to zero. We think the counter of the beam that has triggered the report should be reset to zero to avoid the possibility of re-triggering the event immediately after the prohibit timer expires. Note that the report corresponding to such re-triggered event includes the same triggering beam as in the last report and, therefore, is at least partially redundant. On the other hand, counters associated with other new beams should keep counting during the prohibit timer since, if they trigger the event, the triggering beam in the corresponding report would be different from that of the last report.

Proposal 3: When the prohibit timer starts running, only the counter of the triggering beam should be reset to zero while the other counters should continue counting.

## Measurement window

Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. Editor CR of 38.214: [if eventDetectionTimeWindowLength-r19 is provided, the UE starts a time window for such reference signal, wherein the time window does not overlap with other time windows for such reference signal.]

Regarding the measurement window, Option-4 was agreed in the last meeting in which, to evaluate the condition for initiating the report, UE starts a timer for a new beam when an Event-2 instance for the new beam occurs and the timer for the new beam is not running. However, in the Editor CR, how to capture the above agreement is still open. A main controversial point is whether to use ‘timer’ or ‘time window’. In our view, it is better to use ‘timer’ due to following reasons.

- Timer is closer to the agreement wording than time window.

- Timer is widely used in current specifications and it is easy to describe timer-based report initiation. Using time window may result in further discussions such as how to avoid two overlapping time windows for the same new beam, which proved to be an involved and controversial subject during the Editor’s CR discussions.

- Even though timer is used more in RAN2 specifications, there are some timer descriptions in RAN1 spec. For example, ‘searchSpaceSwitchTimer’ is a timer defined and used in 38.213 for search space group switching as follows:

Excerpt from 38.213 Clause 10.4 A UE can be provided, by searchSpaceSwitchTimer, a timer value for a serving cell that the UE is provided searchSpaceGroupIdList or, if provided, for a set of serving cells provided by cellGroupsForSwitchList. The UE decrements the timer value by one after each slot based on a reference SCS configuration that is the smallest SCS configuration _ among all configured DL BWPs in the serving cell, or in the set of serving cells. The UE maintains the reference SCS configuration during the timer decrement procedure. […] - the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

Based on above discussion, we do not see any problem using the word ‘timer’ in RAN1 spec and  suggest adopting following TP #1.

TP#1: For a UE configured with a CSI-ReportConfig with eventType-r19 for periodic reference signals [with the same periodicity configured by the newBeamResourceSet-r19] and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet-r19, if eventType-r19 is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType-r19 is set to ‘event1’, [if eventDetectionTimeWindowLength-r19 is provided, the UE starts a timer window for such reference signal if the timer for such reference signal is not running., wherein the time window does not overlap with other time windows for such reference signal.]

Proposal 4: Regarding the time window for report initiation, adopt TP #1.

# Reporting content

## Event-1

Agreement On UE-initiated/event-driven beam reporting, for Event 1, support Option-2 for RS measurement: - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - In such case, the report format for Event-2 is reused unless critical technical issue is identified - FFS: How to report the RSRP value of the current beam - FFS: RSRP of the current beam is always reported

According to the above agreement, RS resource set for new beam can be configured for Event-1. We think it is not necessary to always report new beams RSRPs as the new beam measurement is not the main purpose of specifying Event-1. In some cases, UE may not need to perform new beam measurement and reporting for Event-1 to reduce measurement and reporting overhead, while, in other cases, UE may need to report new beams’ measurement results to provide more information for better performance. Hence, we prefer to introduce an RRC parameter to enable/disable the RSRP report of new beams for a better flexibility.

Proposal 5: Support to introduce an RRC parameter to enable/disable the RSRP report of new beams for Event-1.

## Event-7

Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

In RAN1#120bis [1], ‘1-bit’ condition met indicator has been supported for Event-2. We think such design can be applied for Event-7 to notify gNB which new beam(s) satisfy the condition of Event-7.

Proposal 6: Support to introduce the ‘1-bit’ condition met indicator for Event 7.

- Same interpretation on the ‘1-bit’ condition met indicator as Event-2 can be reused.

# Beam report transmission procedure

## The first channel for both Mode A and Mode B

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases: - Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH - Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Option-1: LRR > first PUCCH > normal SR Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities. Conclusion There is no RAN1 consensus on the following. No spec change needed. On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support one of the following rule for the Case-3: the 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain - Option-1: Per CSI report configuration. - Option-2: Up to implementation. - Option-3: UE transmits the first PUCCH with higher priority (e.g., PUCCH resource with lower PUCCH resource ID, or event-type) - Option-4: Multiplexing all 1-bit indications Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support the following rule for the Case-4: the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ. - Option-1: Follow legacy SR multiplexing rule. - Note: There is no further enhancement on additional phase offset/constellation point(s).

For the multiplexing/dropping rule(s) of 1-bit first PUCCH collided/overlapped with a PUSCH (case 2), above three options were provided for down-selection in RAN1#120-bis [1]. For Option-3, considering that gNB does not know whether the 1-bit first PUCCH is transmitted, piggybacking the 1-bit first PUCCH into PUSCH would require blind detection based on two different hypotheses and is computationally complex. Between Option-1 and Option-4, we think dropping the PUSCH is more reasonable. When the first PUCCH is transmitted to report based on Event-1 or Event-2, from UE respective, the candidate beam for the UL transmission is not reliable anymore. Therefore, when the 1-bit first PUCCH is overlapped with a PUSCH, PUSCH can be dropped since its performance is affected by the UL transmission beam.

Proposal 7: When the 1-bit first PUCCH is collided/overlapped with a PUSCH, PUSCH should be dropped.

Besides Case-1 to Case-4 addressed in above listed agreements/conclusion, the following collision/prioritization cases have not been discussed and need to be further considered.

- Case-5: The 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report.

- Case-6: The 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report and/or SR/LRR.

- Case-7: The 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ-ACK information and SR/LRR.

Similar to Case-5, when SR/LRR is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report, log2K+1 bits are appended to HARQ-ACK information bits or prepended to CSI report bits to indicate one of the K SR/LRRs configured in the same slot as the PUCCH format 2/3/4. If no positive SR/LRR is going to be transmitted, the log2K+1 bits are set to all zero. To address Case-5, we think a similar design can be reused. In particular, log2L+1 bits can be appended to HARQ-ACK information bits or prepended to CSI report bits to indicate one of the L 1-bit first PUCCHs configured in the same slot as the PUCCH format 2/3/4. If no1-bit first PUCCH is going to be transmitted, the log2L+1 bits are set to all zero.

Proposal 8: When 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report, log2L+1 bits are appended to HARQ-ACK information bits or prepended to CSI report bits to indicate one of the L 1-bit PUCCHs configured in the same slot as the PUCCH format 2/3/4.

For Case-6, if L first 1-bit PUCCHs and K SR/LRRs overlap with a PUCCH format 2/3/4 carrying HARQ-ACK/CSI, we think log2L+1 bits for L first 1-bit PUCCHs can be prepended to log2K+1 bits for K SR/LRRs and both sets of bits are appended to the HARQ-ACK/CSI bits.

In contrary to some other companies’ views, we do not think log2K+L+1 bits can be used to signal L first 1-bit PUCCHs and K SR/LRRs: SR and LRR are configured for requesting data transmission while 1-bit PUCCH is configured for requesting/notifying a UEI beam report. Therefore, UE may request UL data transmission and UEI beam report at the same time. If log2K+L+1 bits are supported to signal both SR/LRR and UEI beam report, only one of SR/LRR or UEI beam report can be indicated.

Proposal 9: When 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report and SR/LRR, log2L+1 bits are prepended to log2K+1 bits for K SR/LRRs to indicate one of the L first 1-bit PUCCHs configured in the same slot as the PUCCH format 2/3/4 and the SR/LRR.

In legacy, if SR/LRR is collided with PUCCH format 0/1 carrying HARQ-ACK information, the following complicated set of rules specifies the UE behavior.

- If SR/LRR collides with PUCCH format 0 carrying HARQ-ACK information, UE will transmit a ZC sequence with specific sequence cyclic shift as described in clause 9.2.5.1 of  38.213 [2] to indicate a combination of SR/LLR and HARQ-ACK.

- If SR in a resource using PUCCH format 0 and HARQ-ACK information bits in a resource using PUCCH format 1 are in the same slot, SR will be dropped.

- If SR in a resource using PUCCH format 1 and HARQ-ACK information bits in a resource using PUCCH format 1 are in the same slot, UE will transmit one of the PUCCH resources according to whether SR is positive or negative. If SR is positive, the UE transmits a PUCCH with HARQ-ACK information bits in the SR resource. If SR is negative, the UE transmits a PUCCH with HARQ-ACK information bits in the HARQ-ACK resource.

If both SR/LRR and the 1-bit first PUCCH collide with PUCCH format 0/1 carrying HARQ-ACK information, above rules are not directly applicable and new dropping/multiplexing rules should be designed case by case. Furthermore, since the 1-bit first PUCCH can be PUCCH format 0/1, we also need to consider dropping/multiplexing rules for various PUCCH format combinations. Considering the limited remaining time in this release, a more practical solution is to avoid Case-7 by specifying that SR/LRR and the first 1-bit PUCCH cannot be configured in the same slot where PUCCH format 0/1 carrying HARQ-ACK information is configured.

Proposal 10: UE does not expect that SR/LRR and the first 1-bit PUCCH are configured in the same slot where PUCCH format 0/1 carrying HARQ-ACK information is configured.

## Design of Step-2 for Mode A

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, the following Option-1 is supported. - Option-1: A CSI trigger state corresponding to UE-initiated/event-driven beam reporting can NOT be associated with legacy AP-CSI report configuration.

In RAN1#120 [3], CSI trigger state dedicated to UE-initiated/event-driven beam reporting was supported. In legacy framework, when the number of configured trigger states are larger than 2n-1, where n is the length of CSI request field in DCI, gNB will activate at most 2n-1 configured trigger states by MAC CE and indicate one of the activated trigger states by DCI. Similar to legacy, gNB cannot indicate a trigger state associated with a UEI beam report in DCI if the trigger state has not been activated. If UE transmits a first PUCCH that is associated with an inactive trigger state, one potential solution is that gNB activates the trigger state before sending the DCI that schedules the second PUSCH. However, this is a complicated procedure as it involves the transmission of another DCI for scheduling the MAC CE, MAC CE transmission, HARQ-ACK procedure of MAC CE, and MAC CE signal processing. To avoid this problem, we think when the number of configured trigger states is larger than 2n-1, UE should not transmit a first PUCCH if its associated trigger state is not activated.

Proposal 11: When the number of configured trigger states is larger than 2n-1 where n is the length of CSI request field in DCI, UE should not transmit a first PUCCH if its associated trigger state is not activated.

# UEI beam report configuration framework

## CSI report configuration rules

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification. - For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource.

Regarding the above FFS, we think multiple CSI report configurations associated with a same CSI-AperiodicTriggerState should also be associated with a same PUCCH resource. To explain this, let us assume that CSI-ReportConfig#1 and CSI-ReportConfig#2 are associated with PUCCH#1 while CSI-ReportConfig#3 and CSI-ReportConfig#4 are associated with PUCCH#2 and, at the same time, all four CSI-ReportConfigs are associated with the same trigger state#1 (see left sub-figure of Figure 1). If UE sends PUCCH#1 for the report associated with CSI-ReportConfig#1 and, before receiving the corresponding scheduling DCI, sends PUCCH#2 for the report associated with CSI-ReportConfig#3, once gNB sends a scheduling DCI with the trigger state#1, UE will send its report associated with either CSI-ReportConfig#1 or CSI-ReportConfig#3  which include a CSI report configuration indicator to indicate the CSI report configuration corresponding to the report (see right sub-figure of Figure 1). However, based on RAN1#120 agreement, the CSI report configuration indicator is a local ID of a CSI-ReportConfig among all CSI-ReportConfigs that are associated with the same PUCCH. Therefore, based on the CSI report configuration indicator, gNB cannot pinpoint the CSI report configuration associated with the received report. For example, if the CSI report configuration indicator is set to zero, gNB can only infer that either the report of CSI-ReportConfig#1 or the report of CSI-ReportConfig#3 is sent and, if the local ID is one, it can only infer that either the report of CSI-ReportConfig#2 or the report of CSI-ReportConfig#4 is sent.

Figure 1. CSI report configurations associated with different PUCCH resources corresponding to the same CSI-AperiodicTriggerState

Above discussion motivates the following proposal:

Proposal 12: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource.

As multiple events have been supported, RAN1 should discuss whether one or multiple events can be configured with one CSI report configuration. Considering the maximum number of CSI report configurations per cell is limited, we think one CSI report configuration associated with multiple events should be supported. If a CSI report configuration is associated with multiple events, an event information such as event id should be included in the beam report to enable gNB to determine which event has occurred and to correctly interpret each field of the beam report.

Proposal 13: Support one or multiple events to be associated with one CSI report configuration.

- Event ID should be included in the report when the CSI report configuration is associated with multiple events.

## Cross CC UEI beam report/measurement

Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

In legacy framework, PUCCH resource carrying CSI report can only be transmitted on the cell where the CSI report configuration is included. Therefore, cell id of the PUCCH resource that carries the report is not included in the CSI report configuration. However, for both Mode-A and Mode-B UEI beam reports, RAN1 has agreed that PUCCH and the PUSCH can be from different or same PUCCH groups in the same cell group. This implies that PUCCH and PUSCH can belong two different cells in the same cell group. Further, RAN1 has agreed for Mode B that the cell of the PUSCH should be the same as the cell of the CSI report configuration which means that the cell of the PUCCH can be different from the cell of the CSI report configuration. Therefore, cell of the PUCCH should be indicated in the UEI beam report configuration, otherwise, UE does not know the PUCCH indicated by the PUCCH ID in the CSI report configuration is located on which cell. Note that only SpCell of the cell group or a PUCCH-SCell can be configured with PUCCH. Further, the RRC parameter pucch-Cell is already used in PDSCH-ServingCellConfig to indicate whether the PUCCH that carries HARQ-ACK is on the SpCell, if the parameter is absent, or on a specific PUCCH-SCell, if the parameter is configured. A similar RRC parameter can be used to indicate the cell on which the PUCCH for UEI beam report is located.

Proposal 14: In CSI report configuration for UEI beam report, introduce an RRC parameter to indicate SpCell or a PUCCH-SCell as the cell of the configured PUCCH.

- Note: This has a similar functionality as pucch-Cell used in PDSCH-ServingCellConfig to indicate whether HARQ-ACK PUCCH is on the SpCell or a specific PUCCH-SCell.

# Others

## Priority rules for UEI beam report

As described in Clause 5.2.5 in TS 38.214 [4], in legacy CSI report mechanism, UE can be configured with multiple CSI report configurations each of which associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where the description of the parameters is provided in Table 1. A CSI report with a higher priority value has a lower priority. If two CSI reports collide, they are multiplexed or one of them is dropped based on the priority values.

Table 1. Parameter description of CSI report priority formula

| Parameter | Description |
| --- | --- |
| y | y = 0 for aperiodic CSI reports on PUSCH y = 1 for semi-persistent CSI reports on PUSCH y = 2 for semi-persistent CSI reports on PUCCH y = 3 for periodic CSI reports on PUCCH |
| k | k = 0 for CSI reports carrying L1-RSRP or L1-SINR k =1 for CSI reports not carrying L1-RSRP or L1-SINR; |
| c | the serving cell index |
| s | CSI report configuration ID |
| Ncells | Maximum number of serving cells |
| Ms | Maximum number of CSI report configurations |
| PriiCSIy,k,c,s | a priority value of a CSI report |

When UEI and legacy CSI report configurations are configured simultaneously, it is possible that a UEI report collides with a legacy report or two UEI reports collide with each other. If no priority rule is introduced, it may result in a reporting detection failure. Therefore, if multiple UEI and legacy CSI report configurations are associated with the same PUSCH resource, a priority/dropping rule should be devised for the gNB to know the sequence of the reported CSI(s) in the PUSCH resource and/or the dropped CSI reports if the assigned resource is inadequate.

According to the priority rule of legacy CSI reports, a CSI report with more reporting opportunities has a lower priority since the dropped CSI reports in one occasion can be retransmitted in the next occasion. In particular, periodic and semi-persistent CSI reports have multiple consecutive reporting opportunities while an aperiodic CSI report is a one-shot report. Therefore, aperiodic CSI reports have the highest priority. Between periodic and semi-persistent CSI reports, semi-persistent CSI reports have a higher priority as they are activated/de-activated. We think a similar logic can be used to define the priority rule between UEI and legacy CSI reports. In particular, considering UEI beam report is a one-shot report, it should have a higher priority than legacy periodic and semi-persistent CSI reports.

Proposal 15: UE initiated beam reports should have a higher priority than legacy periodic and semi-persistent CSI reports.

To discuss the priority between UEI beam report and legacy aperiodic CSI reports, we think Mode A and Mode B based beam report should be considered separately. For Mode A based report, if the beam report is dropped, a 3-step procedure of UE request, gNB scheduling, and UE reporting, is required for a retransmission. However, for Mode B based report and legacy aperiodic CSI reports, only a 2-step procedure is required for a report retransmission. Therefore, we think Mode A based beam report should have a higher priority than both Mode B based beam report and legacy aperiodic CSI report considering the complexity of its retransmission procedure.

Proposal 16: Mode A based beam report should have a higher priority than both Mode B based report and legacy aperiodic CSI reports.

Compared to the legacy aperiodic CSI report, the UL resource carrying Mode B based beam report is preconfigured. Hence, if a Mode B based beam report is dropped, UE can retransmit the measurement result in another preconfigured UL resource after re-sending the notification in a first channel. This means that the reporting opportunity is more accessible for Mode B based beam reports than for legacy aperiodic CSI reports. Therefore, legacy aperiodic CSI reports should be prioritized over Mode B based report.

Proposal 17: Legacy aperiodic CSI reports should be prioritized over Mode B based beam report.

Based on the above discussions, to incorporate the priority of Mode A and Mode B based beam reports while keeping the specification efforts minimal, we suggest to reuse the existing legacy CSI priority formula but to redefine y to include both legacy and UEI reports:

PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms·k+Ms∙c+s

where

y=0 for Mode A based beam report,

y=1 for aperiodic CSI reports to be carried on PUSCH,

y=2 for Mode B based beam report,

y=3 for semi-persistent CSI reports to be carried on PUSCH,

y=4 for semi-persistent CSI reports to be carried on PUCCH, and

y = 5 for periodic CSI reports to be carried on PUCCH.

Proposal 18: Support the following priorities in the descending order for CSI reports: Mode A based beam report, aperiodic CSI reports carried on PUSCH, Mode B based beam report, semi-persistent CSI reports carried on PUSCH, semi-persistent CSI reports carried on PUCCH, periodic CSI reports carried on PUCCH.

## CPU occupancy time of UEI CSI report

If CSI report is configured with reportQuantity not set to ‘none’ in legacy framework, an aperiodic CSI report or an initial semi-persistent CSI report on PUSCH occupies the CPU from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report as shown in Fig2(a). In turn, periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH) occupies the CPU from the first symbol of the earliest one of channel/interference measurement resources until the last symbol of the configured PUSCH/PUCCH carrying the report as shown in Fig2(b).

(a)

(b)

Figure 2. CPU occupancy time of legacy CSI reporting

For UEI beam reporting, whether and when to transmit the beam report is determined by UE. This means that UE does not always report on the pre-configured UL resource and, therefore, the CPU occupancy time should be re-defined.

For both Mode A and Mode B based beam reporting, if the report notification/request is sent on the pre-configured first PUCCH resource, similar to the legacy CSI report, the end of the CPU occupancy time can be the last symbol of the scheduled or pre-configured PUSCH resource carrying the report. In turn, if the report notification/request is not sent on the pre-configured first PUCCH resource, CPU will not be occupied after the last symbol of this resource. Therefore, in this case, we think the end of CPU occupancy time can be the last symbol of the pre-configured PUCCH resource.

Figure 2. CPU occupancy time of UEI beam reporting

Proposal 19: The end of CPU occupancy time for UEI beam reporting can be determined based on whether the notification/request is transmitted on the first PUCCH resource.

- If the notification/request is sent on the first PUCCH resource, the end of CPU occupancy time can be the last symbol of the scheduled or pre-configured PUSCH carrying the report.

- If notification/request is not transmitted on the first PUCCH resource, the end of CPU occupancy time can be the last symbol of the first PUCCH resource.

# Summary and conclusion

Proposal 1: In addition to revised Candidate#2, support the following conditions of resetting the counter:

- The new beam RS corresponding to a counter is removed from the new beam resource set by RRC reconfiguration or, if supported, by MAC-CE signaling;

  - In this case, only the counter corresponding the removed new beam RS is reset.

- UEI beam report is transmitted;

  - In this case, only reset the counting of new beam(s) fulfilling triggering condition and reported by the UEI beam report.

- A time window expires.

  - In this case, only the counter corresponding to the associated new beam should be reset.

- The threshold for event evaluation is re-configured by RRC signaling

  - In this case, the counters corresponding to all new beams should be reset.

Proposal 2: Regarding the prohibit timer for both Mode-A and Mode-B, support Option-1A with the following modification:

- In the case that triggering-event associated with the CSI report configuration is determined by the same triggering beam(s) as the last transmitted PUCCH, if the prohibit timer is NOT running, the UE can transmit the first PUCCH;

  - At the first symbol after the end of the second PUSCH transmission, the UE starts the prohibit timer

Proposal 3: When the prohibit timer starts running, only the counter of the triggering beam should be reset to zero while the other counters should continue counting.

Proposal 4: Regarding the time window for report initiation, adopt TP #1.

Proposal 5: Support to introduce an RRC parameter to enable/disable the RSRP report of new beams for Event-1.

Proposal 6: Support to introduce the ‘1-bit’ condition met indicator for Event 7.

- Same interpretation on the ‘1-bit’ condition met indicator as Event-2 can be reused.

Proposal 7: When the 1-bit first PUCCH is collided/overlapped with a PUSCH, PUSCH should be dropped.

Proposal 8: When 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report, log2L+1 bits are appended to HARQ-ACK information bits or prepended to CSI report bits to indicate one of the L 1-bit PUCCHs configured in the same slot as the PUCCH format 2/3/4.

Proposal 9: When 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report and SR/LRR, log2L+1 bits are prepended to log2K+1 bits for K SR/LRRs to indicate one of the L first 1-bit PUCCHs configured in the same slot as the PUCCH format 2/3/4 and the SR/LRR.

Proposal 10: UE does not expect that SR/LRR and the first 1-bit PUCCH are configured in the same slot where PUCCH format 0/1 carrying HARQ-ACK information is configured.

Proposal 11: When the number of configured trigger states is larger than 2n-1 where n is the length of CSI request field in DCI, UE should not transmit a first PUCCH if its associated trigger state is not activated.

Proposal 12: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource.

Proposal 13: Support one or multiple events to be associated with one CSI report configuration.

- Event ID should be included in the report when the CSI report configuration is associated with multiple events.

Proposal 14: In CSI report configuration for UEI beam report, introduce an RRC parameter to indicate SpCell or a PUCCH-SCell as the cell of the configured PUCCH.

- Note: This has a similar functionality as pucch-Cell used in PDSCH-ServingCellConfig to indicate whether HARQ-ACK PUCCH is on the SpCell or a specific PUCCH-SCell.

Proposal 15: UE initiated beam reports should have a higher priority than legacy periodic and semi-persistent CSI reports.

Proposal 16: Mode A based beam report should have a higher priority than both Mode B based report and legacy aperiodic CSI reports.

Proposal 17: Legacy aperiodic CSI reports should be prioritized over Mode B based beam report.

Proposal 18: Support the following priorities in the descending order for CSI reports: Mode A based beam report, aperiodic CSI reports carried on PUSCH, Mode B based beam report, semi-persistent CSI reports carried on PUSCH, semi-persistent CSI reports carried on PUCCH, periodic CSI reports carried on PUCCH.

Proposal 19: The end of CPU occupancy time for UEI beam reporting can be determined based on whether the notification/request is transmitted on the first PUCCH resource.

- If the notification/request is sent on the first PUCCH resource, the end of CPU occupancy time can be the last symbol of the scheduled or pre-configured PUSCH carrying the report.

- If notification/request is not transmitted on the first PUCCH resource, the end of CPU occupancy time can be the last symbol of the first PUCCH resource.

# References

[1] Report of RAN1#120-bis, Chairman, April 7th – 11th, 2025.

[2] 3GPP TS 38.213 version 18.5.0.

[3] Report of RAN1#120, Chairman, February 17-21, 2025.

[4] 3GPP TS 38.214 version 18.5.0.