3GPP TSG RAN WG1 #121	R1-2503376

St Julian’s, Malta, May 19th – 23th, 2025

Source:	vivo

Title:	Remaining issues on measurements related enhancements for LTM

Agenda Item:	9.9.1

Document for:	Discussion and Decision

Introduction

For LTM, RAN1-related issues mainly focus on event-triggered beam reporting, CSI-RS-based beam measurement and reporting, and CSI-RS-based CSI acquisition. With the progress in previous meetings, the whole procedure of event-triggered beam reporting is clear. For CSI-RS-based beam measurement and reporting, everything is complete. For CSI-RS-based CSI acquisition, some details need to be discussed further and determined, such as the selection of CSI-RS resources on candidate cell(s) to perform CSI measurement. Besides the above three aspects, some operations within the intra-CU conditional LTM procedure have been agreed in RAN2, like early RACH triggered by PDCCH order and early Candidate TCI state activation. It needs to be identified whether these operations will have an impact on the RAN1 specification.

In this contribution, we provide our views on the above four aspects.

Discussion

CSI-RS-based measurement for LTM

In previous meetings, beam measurement and CSI measurement based on periodic and semi-persistent CSI-RS resources before the reception of cell switch command were agreed.  Periodic CSI-RS resources can be applicable for both gNB scheduled measurement reporting and event-triggered reporting, while semi-persistent CSI-RS resources can only be applicable for gNB scheduled measurement reporting. According to the discussion, the procedure of CSI-RS based beam measurement and reporting is clear and some details about CSI-RS based CSI acquisition still need further discussion, such as deactivation of semi-persistent CSI-RS resources upon the reception of cell switch command, selection of CSI-RS resources for CSI measurement on candidate cell(s) before the reception of cell switch command.  In this section, we provided our views on these issues.

- SP CSI-RS

According to the reply LS from RAN3[1], RAN3 confirmed the feasibility of specifying the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) for beam management. If semi-persistent CSI-RS is supported for beam management on candidate cell(s), the legacy activation/deactivation mechanism based on MAC CE should be reused as much as possible. In addition, after the reception of the LTM cell switch command, the active semi-persistent CSI-RS resource associated with candidate cell(s) other than the target cell should be deactivated without requiring interface interaction and beam tracking on the target cell will not be affected meanwhile. Furthermore, the above mechanism can also be applicable for semi-persistent CSI-RS resources for CSI acquisition on candidate cell(s).

- If semi-persistent CSI-RS resources are used for beam management and CSI acquisition on candidate cell(s), besides the legacy activation/deactivation mechanism by MAC CE, the active semi-persistent CSI-RS resources associated with candidate cell(s) other than the target cell should be deactivated after the reception of LTM cell switch command.

- CSI acquisition before/after CSC

In the last meeting [2], the following agreement was made about CSI-RS based CSI acquisition on candidate cell(s).

Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported - The supported max number of CSI-RS ports is up to separate UE capability Agreement For the container of LTM CSI report to a target cell, UCI is used with a solution to avoid blind detection at gNB if valid CSI is not available - FFS: the definition of valid CSI Agreement For the solution to avoid blind detection for LTM CSI report if valid CSI is not available, - Option 1: CSI is reported when valid CSI is not available - Option 1-1: one CQI codepoint is used to indicate that valid CSI is not available - Option 1-1-1: Codepoint with the lowest CQI index is reported

- Selection of CSI-RS resources on candidate cell(s) before/after CSC

According to the above agreements made in the last meeting, only one CSI report configuration for a candidate cell to acquire the corresponding CSI which can be associated with multiple CSI-RS resources for channel measurement. After receiving the CSI report configuration, UE could determine the CSI-RS resources and then perform measurement based on UE capability. However, whether UE measures a subset or all the CSI-RS resources associated with the CSI report configuration is yet to be determined. From the perspective of UE computation overhead, it is more reasonable to only measure CSI-RS resource(s) associated with the beam(s) that can be used for data transmission after cell switch potentially. Therefore, if CSI measurement on candidate cell(s) before the reception of the cell switch command is supported, only CSI-RS resources associated with activated Candidate TCI state(s) within the configured CSI-RS resources can be measured by UE before the reception of the cell switch command. Otherwise, if CSI measurement on candidate cell(s) before the reception of the cell switch command is not supported or after the reception of the cell switch command, only CSI-RS resources associated with the Candidate TCI state indicated in the cell switch command can be measured by UE.  In the unified TCI framework, CSI-RS resources for CSI are not allowed to be configured as the QCL source RS of TCI state for PDCCH and PDSCH. Hence, the above CSI-RS resources that UE measures need to be QCLed with the reference signal in the indicated/activated Candidate TCI state.

- If CSI measurement before the reception of cell switch command is supported, UE only measures CSI-RS resources on candidate cell(s) associated with activated Candidate TCI state(s) before the cell switch command is received;

- If CSI measurement before the reception of cell switch command is not supported or after the reception of the cell switch command, UE only measures the CSI-RS resource(s) associated with the TCI state indicated in the cell switch command;

The above CSI-RS resources that UE measures are the CSI-RS resources that are QCLed with the reference signal in the indicated/activated Candidate TCI state within the multiple CSI-RS resources with the CSI report configuration.

For CSI reporting on the target cell, as the reported CSI measurement result is used for the network to schedule downlink transmission, reporting the CSI measurement result acquired by the CSI-RS resource(s) associated with the indicated Candidate TCI state in the cell switch command is sufficient.

UE only reports CSI measurement result acquired by the CSI-RS resource(s) associated with the indicated Candidate TCI state in the cell switch command to the target cell.

In the current specification, the reported CRI is used to indicate the network which CSI-RS resource UE selects to calculate CSI when multiple CSI-RS resources are provided. In LTM, if the number of CSI-RS resources associated with the indicated TCI state in the CSC is larger than 1, CRI is reported; Otherwise, the network could determine that the reported CSI measurement result is acquired by the CSI-RS resource(s) associated with the indicated Candidate TCI state in the cell switch command, so the CRI is not reported.

If the number of CSI-RS resources associated with the indicated TCI state in the CSC is larger than 1, CRI is reported; Otherwise, CRI is not reported.

- The definition of valid CSI

In the last meeting, the issue of an invalid CSI report was discussed and the definition of a valid CSI report was left to study further. According to the above agreement, the initial CSI report for target cell is an aperiodic reporting and triggered by the cell switch command. In the current specification, an aperiodic CSI reporting triggered by PDCCH is valid when the following conditions are met:

- Condition 1: slot n- ncsi_ref corresponds to a valid downlink slot, where nCSI_ref is the smallest value greater than or equal to ,, slot n’ is the uplink resource that carries the initial CSI report;

- Condition 2: the first symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting  after the end of the last symbol in time of the latest of aperiodic CSI-RS resource for channel measurements and/or interference measurement for a CSI-ReportConfig;

- Condition 3: the first symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where  is defined as the next uplink symbol with its CP starting  after the end of the last symbol of the PDCCH triggering the CSI report(s);

For the initial CSI report for target cell, we think the definition of a valid CSI report should be discussed case by case as follows:

- Case-1: CSI measurement performed before CSC

  - CSI measurement performed before CSC requires at least one valid downlink slot exists before the reception of CSC;

  - The first uplink symbol of the CG occasion carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of  , where is defined as the next uplink symbol with its CP starting  after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell.

- Case-2: CSI measurement performed after CSC and CG-based CSI reporting

  - CSI reporting after CSC requires at least one valid downlink slot exists before the CSI reference resource determined based on the uplink resource carrying the report and the CSI reference resource should be later than the reception of the CSC;

  - The first uplink symbol of the CG occasion carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of  , where is defined as the next uplink symbol with its CP starting  after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell.

- Case-3: CSI measurement performed after CSC and DG/Msg3-based CSI reporting

  - CSI reporting after CSC requires at least one valid downlink slot exists before the CSI reference resource determined based on the uplink resource carrying the report and the CSI reference resource should be later than the reception of the CSC;

  - The first uplink symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of  , where is defined as the next uplink symbol with its CP starting  after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell.

  - The first symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where  is defined as the next uplink symbol with its CP starting  after the end of the last symbol of the PDCCH scheduling the uplink resource;

The definition of valid CSI for LTM is listed below.

| Cases | Conditions |
| --- | --- |
| Case-1: CSI measurement performed before CSC | - At least one valid downlink slot exists before the reception of CSC; - The first uplink symbol of the CG occasion carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell. |
| Case-2: CSI measurement performed after CSC and CG-based CSI reporting | - At least one valid downlink slot exists before the CSI reference resource determined based on the uplink resource carrying the report and the CSI reference resource should be later than the reception of the CSC; - The first uplink symbol of the CG occasion carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell. |
| Case-3: CSI measurement performed after CSC and DG/Msg3-based CSI reporting | - At least one valid downlink slot exists before the CSI reference resource determined based on the uplink resource carrying the report and the CSI reference resource should be later than the reception of the CSC; - The first uplink symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell; - The first symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol of the PDCCH scheduling the uplink resource. |

- Uplink reporting resource determination

Another issue is the determination of uplink resource carrying the report, according to the discussion in the last meeting, the following options can be considered:

- Option-1: PUSCH carrying Msg3 for RACH-based LTM;

- Option-2: PUSCH dynamically scheduled by the network to carry the first UL message for DG-based RACH-less LTM;

- Option-3: PUSCH configured by the network to carry the first UL message for CG-based RACH-less LTM;

- Option-3’: PUSCH configured by the network to carry the first UL message for CG-based RACH-less LTM, and the first UL message and the initial CSI report can be separately transmitted. Only when the two messages are transmitted successfully, the CG-PUSCH configuration can be released.

- Option-4: Dedicated PUSCH configured by the network to carry the initial CSI report.

For Option-1, Option-2 and Option-3, in addition to the legacy timeline requirement of the first UL message/Msg3, the PUSCH occasion carrying the first UL message also needs to satisfy the CSI computation and reporting timeline. For example, in CG-based RACH-less LTM, UE transmits the first UL message on the valid CG occasion, where the valid CG occasion satisfies the following conditions:

- It does not overlap with a valid PRACH occasion as described in clause 8.1 of TS38.213[3];

- There is one valid downlink slot for the CSI reference resource corresponding to the slot of the CG occasion.

A similar rule also applies to Msg3 PUSCH and DG-PUSCH. However, it may lead to a longer cell switch latency if a long time for CSI measurement is needed. To avoid it, modified Option-3 is proposed above as Option-3’. In Option-4, the network configures dedicated periodic PUSCH resources to carry the initial CSI report. And, the UE only needs to transmit the initial CSI report on the first PUSCH occasion which satisfies the CSI computation and reporting timeline. In this case, the initial CSI report can be carried as UCI or MAC CE. After UE sends the initial CSI report, the PUSCH configuration will be released. However, Option-4 would cause a waste of resources and increase detection complexity at the network side. Therefore, we prefer modified Option-3 to carry the initial CSI report for CG-based RACH-less LTM. While for RACH-based LTM and DG-based RACH-less LTM, either a separate design (i.e., Option-1 and Option-2) or a unified design (i.e., Option-4) is workable.

Support Option-3’ for CG-based rach-less LTM, i.e., the CG-PUSCH configured for the first UL message is shared with the initial CSI report, and only when the initial CSI report and the first UL message are transmitted, the CG-PUSCH configuration is released.

- CPU occupation rule

- If the initial CSI report is carried as UCI, the corresponding CSI processing unit occupation time and number shall be further discussed. Regardless of the CSI measurement performed before or after the reception of LTM CSC MAC CE, the occupied CPU starts from the first symbol after the PDSCH carrying the LTM CSC MAC CE, and ends with the last symbol of the uplink resource carrying the initial CSI report. The number of occupied CPUs is associated with report quantity. It can be discussed after the determination of report quantity.

The occupied CPU of the initial CSI report starts from the first symbol after the PDSCH carrying the LTM CSC MAC CE, and ends up with the last symbol of the uplink resource carrying the initial CSI report.

However, if CSI measurement can be performed before cell switch, how to define and measure the occupied UE capability by CSI measurement before cell switch needs further discussion. One potential approach is to define a new UE capability on the maximum number of CSI-RS for CSI across candidate cells that UE can measure before cell switch, which is similar to SSB measurement on candidate cells in Rel-18 LTM.  Furthermore, only the active CSI-RS resources on candidate cells within the active BWP in the source cell can be counted. Measuring the CSI on candidate cells outside the active BWP will add significant complexity at the UE. If supported, it will lead to a huge effort in RAN4.

Do not support CSI measurement on candidate cells performed outside the active BWP of the serving cell.

Support to define a new UE capability on the maximum number of CSI-RS for CSI across candidate cells that UE can measure before cell switch.

Intra-CU conditional LTM

According to the LS from RAN2 [4], the following operations have been agreed for intra-CU conditional LTM.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event-triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. - For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. - The Early TA is signaled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

For PDCCH ordered early RACH for CLTM, the triggering signaling and procedure designed in Rel-18 LTM can also be reused except for TA feedback mechanism. As no cell switch command in the CLTM procedure, TA value of candidate cell(s) is carried by a new MAC CE signaling to inform UE before the cell switch is performed. The new MAC CE signaling can be carried by PDSCH scheduled by a DCI scrambled by C-RNTI. Hence, no additional specification impact is needed. For early Candidate TCI state activation/deactivation mechanism for CLTM, the legacy activation/deactivation signaling and procedure in Rel-18 LTM can be reused directly, except the UE deactivates the activated Candidate TCI state upon receiving the LTM CSC MAC CE. Since there is no NW signaling to indicate UE to perform cell switch within CLTM procedure. Thus, whether/when to deactivate the activated TCI state(s) other than the indicated one within the CLTM procedure should be discussed. In our view, it is reasonable to deactivate the activated TCI state(s) other than the TCI state associated with the triggered beam upon CLTM procedure being triggered.

Activated Candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated upon CLTM procedure being triggered.

Conclusion

For measurements related enhancement(s) for LTM, we have the following observations and proposals:

- If semi-persistent CSI-RS resources are used for beam management and CSI acquisition on candidate cell(s), besides the legacy activation/deactivation mechanism by MAC CE, the active semi-persistent CSI-RS resources associated with candidate cell(s) other than the target cell should be deactivated after the reception of LTM cell switch command.

- If CSI measurement before the reception of cell switch command is supported, UE only measures CSI-RS resources on candidate cell(s) associated with activated Candidate TCI state(s) before the cell switch command is received;

- If CSI measurement before the reception of cell switch command is not supported or after the reception of the cell switch command, UE only measures the CSI-RS resource(s) associated with the TCI state indicated in the cell switch command;

The above CSI-RS resources that UE measures are the CSI-RS resources that are QCLed with the reference signal in the indicated/activated Candidate TCI state within the multiple CSI-RS resources with the CSI report configuration.

UE only reports CSI measurement result acquired by the CSI-RS resource(s) associated with the indicated Candidate TCI state in the cell switch command to the target cell.

If the number of CSI-RS resources associated with the indicated TCI state in the CSC is larger than 1, CRI is reported; Otherwise, CRI is not reported.

The definition of valid CSI for LTM is listed below.

| Cases | Conditions |
| --- | --- |
| Case-1: CSI measurement performed before CSC | - At least one valid downlink slot exists before the reception of CSC; - The first uplink symbol of the CG occasion carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell. |
| Case-2: CSI measurement performed after CSC and CG-based CSI reporting | - At least one valid downlink slot exists before the CSI reference resource determined based on the uplink resource carrying the report and the CSI reference resource should be later than the reception of the CSC; - The first uplink symbol of the CG occasion carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell. |
| Case-3: CSI measurement performed after CSC and DG/Msg3-based CSI reporting | - At least one valid downlink slot exists before the CSI reference resource determined based on the uplink resource carrying the report and the CSI reference resource should be later than the reception of the CSC; - The first uplink symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of CSI-RS resources associated with the CSI report of target cell; - The first symbol of the uplink resource carrying the initial CSI report including the effect of the timing advance starts no earlier than at the symbol of , where is defined as the next uplink symbol with its CP starting after the end of the last symbol of the PDCCH scheduling the uplink resource. |

Support Option-3’ for CG-based rach-less LTM, i.e., the CG-PUSCH configured for the first UL message is shared with the initial CSI report, and only when the initial CSI report and the first UL message are transmitted, the CG-PUSCH configuration is released.

The occupied CPU of the initial CSI report starts from the first symbol after the PDSCH carrying the LTM CSC MAC CE, and ends up with the last symbol of the uplink resource carrying the initial CSI report.

Do not support CSI measurement on candidate cells performed outside the active BWP of the serving cell.

Support to define a new UE capability on the maximum number of CSI-RS for CSI across candidate cells that UE can measure before cell switch.

Activated Candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated upon CLTM procedure being triggered.

References

R3-247911.

RAN1#120bis Chiarman Notes.

3GPP TS38.213: "NR; Physical layer procedures for control"

R2-2411134.