3GPP TSG RAN WG1 #121                                                                             R1-2503379

St Julian’s, Malta, May 19th – 23th, 2025

Source:	vivo

Title:	Remaining issues on support of RedCap and eRedCap UEs with NR-NTN

Agenda Item:	9.11.2

Document for:	Discussion and Decision

- Introduction

The Rel-19 NR NTN objectives are revised and provided in [1]. One of the objectives is to specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap and eRedCap UEs with NR NTN operating in the FR1-NTN band as follows.

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this contribution, we discuss the remaining issues on the following aspects of half duplex in NTN:

- Collision Case 4

- Multiple channels in one direction colliding with another direction within one collision occasion

- TA report enhancement

- Discussion

  - Collision Case 4

Agreement Update the working assumption in RAN1 #120 with the following updates: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all cases - This is signaled by one UE specific RRC parameter - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note2: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

In case 4, it has been agreed that handling of collision with PDSCH at least for system information scheduled by type 0/0A CSS is up to UE implementation. Nonetheless, according to the current spec, a connected UE should also monitor DCI scrambled by C-RNTI, MCS-C-RNTI, or a CS-RNTI in the Type-0/0A CSS. It should be further clarified how the UE handles the collision with these PDSCHs. In our view, it is desirable to be handled also by UE implementation, so that a unified behavior can be applied when monitoring the CSS. It should be noted that because of the collision handling of SI-RNTI is up to UE implementation, it is actually difficult, or impossible, to prioritize the PDSCH or PUSCH according to the network indication. This is because the UE has to either buffer the PDSCH in the slot before and during PDCCH decoding (at least for prioritizing the system information), or to prepare the UL transmission and cancel the remaining reception in the slot (when it intends to prioritize UL). Consequently, after the blind decoding the DCI, when the UE notes that it is a C-RNTI scheduling, it may not be able to revert the decision (i.e., changing DL to UL or vice versa) due to insufficient time. Given that the network may not have the clear information, the collision handling with dedicated PDSCH should also be up to UE implementation, which is similar to PDSCH scheduled by type-1/2 PDCCH CSS.

Proposal 1: The handling of collision with dedicated PDSCH scheduled by PDCCH with DCI scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI by type 0/0A PDCCH CSS is left to UE implementation.

Another remaining issue is how to handle the PDCCH ordered PRACH transmission. If the PDCCH order triggered PRACH collides with DL transmission except PDSCH scheduled by Type-0/0A/1/2 PDCCH CSS in RRC-Connected mode, the network can configure to prioritize the UL. Alternative, the network can avoid the DL scheduling when it triggers the PDCCH ordered PRACH transmission. Therefore, it can be one of the other use cases.

Proposal 2: Handling of collision with PDCCH order triggered PRACH can follow the priority rule and controlled by the network indication.

According to the above, we propose the following:

Proposal 3: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all cases

    - This is signaled by one UE specific RRC parameter

  - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note2: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

There are some remaining issues in the draft CR of R1-2503179 [2] of TS 38213 about collision case 4 in the following:

The first issue is about UE capability. In the previous meeting, the specific UE capabilities are agreed for case 3 and case 4 for R19 NTN. For Redcap UEs operating in NTN that do not support these new capabilities, their behavior should remain the same as the TN operation in previous releases. It means that the network should avoid case 3 and case 4 for these UEs. While for Redcap UEs operating in NTN that support the new capabilities, the collision can happen and the handling can be up to UE implementation. Besides, case 3 and case 4 are applicable to different states (e.g., case 3 is for RRC Connected and Inactive states, while case 4 is only for Connected state), and different scenarios (e.g., case 3 is basically for periodicity traffic while case 4 is for dynamic scheduling). Thus, whether to implement one or both features is a production and market decision. More details of UE capability can be found in our companion contribution [3]. Therefore, we propose the following:

Proposal 4: Delete the condition "operating on a non-NTN serving cell" to include UEs operating in NTN that do not support these new capabilities, and add ‘subject to UE capability’ or ‘UE may determine’ to properly reflect that the behavior is dependent on whether the UE supports the new capability.

Another issue is the UL cancellation timeline. For the UL cancellation timeline in TN in case 1, the understandings of UL timing between gNB and UE are aligned. The gNB can know the UL cancellation exactly based on the scheduling and UL cancellation timeline. However, in NTN, the exact time of the UL timing cannot be obtained by the network due to the TA pre-compensation of the UE. The UL cancellation timeline can be different from case 1, at least in the view of the gNB.

Observation 1: According to the note in the working assumption, the UL cancellation timeline is up to UE implementation.

The third issue is that the ‘other use cases’ description for case 4 is not clear. In the CR, the collision cases of PDSCH in “the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH” includes the PDSCH scheduled by a PDCCH in type 0/0A/1/2 CSS. However, it does not aligned with the agreement where this case should be up to UE implementation. Therefore, the PDSCH scheduled by a PDCCH in type 0/0A/1/2 CSS should be excluded.

Proposal 5: The handling collision case of PDSCH scheduled by a PDCCH in type 0/0A/1/2 CSS should be excluded in the ‘other use cases’ for case 4 (i.e., It should not be controlled by the RRC parameter ntnRedCapCollisionCase4UlPriority).

According to the above, we propose the following TP#1 of 38.213:

A half-duplex UE (HD-UE) in paired spectrum is not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum. This clause is applicable for communication of a HD-UE on a serving cell with paired spectrum. Procedures for a HD-UE are same as described for a UE in all other clauses of this document unless stated otherwise. A HD-UE operating on a non-NTN serving cell does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. A HD-UE operating on an NTN serving cell in the RRC_CONNECTED state determines based on its implementation whether to either receive in a set of symbols a PDSCH scheduled by a DCI format provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, or transmit in any symbol from the set of symbols a PUSCH/PUCCH/SRS scheduled by a DCI format, subjected to UE capability. When a PDCCH reception by a HD-UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1, the end of the PDCCH reception is the end of the PDCCH candidate that ends later. If a HD-UE is configured by higher layers to receive a PDCCH, or PDSCH, or CSI-RS, or DL PRS in a set of symbols, the HD-UE receives the PDCCH, or PDSCH, or CSI-RS, or DL PRS if the HD-UE does not detect a DCI format that indicates to the HD-UE to transmit a PUSCH, or PUCCH, or PRACH, or SRS in at least one symbol of the set of symbols; otherwise, the HD-UE does not receive the PDCCH, or PDSCH, or CSI-RS, or DL PRS in the set of symbols. If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols, or if a HD-UE operating on an NTN serving cell is not provided ntnRedCapCollisionCase4UlPriority and detects a DCI format indicating to the HD-UE to transmit SRS, or PUCCH, or PUSCH in a set of symbols, and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214]. - the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH. If a HD-UE operating on an NTN serving cell is not provided ntnRedCapCollisionCase4UlPriority and detects a DCI format indicating to the HD-UE to transmit SRS, or PUCCH, or PUSCH in a set of symbols, and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, other than a PDSCH scheduled by a DCI format provided by a PDCCH the HD-UE which is in RRC_CONNECTED state received according to a Type0/0A/1/2-PDCCH CSS set, then the HD-UE either - does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the [UL cancellation timeline] cannot be satisfied - does not expect to cancel the transmission of SRS in symbols from the subset of symbols if the [UL cancellation timeline] cannot be satisfied. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols. or, - does not receive the CSI-RS or PDSCH. If a HD-UE operating on an NTN serving cell is provided ntnRedCapCollisionCase4UlPriority and detects a DCI format indicating to the HD-UE transmit SRS, or PUCCH, or PUSCH in a set of symbols, and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in any symbol from the set of symbols, other than a PDSCH scheduled by a DCI format provided by a PDCCH the HD-UE which is in RRC_CONNECTED state received according to a Type0/0A/1/2-PDCCH CSS set, the HD-UE does not receive the CSI-RS or PDSCH. A HD-UE operating on a non-NTN serving cell does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE operating on a non-NTN serving cell does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except a Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring configured-grant based PUSCH transmission as described in clause 19.1 in the set of symbols for which case the HD-UE follows the procedure as in clause 5.1B.2.6 of [10, TS 38.133]. The HD-UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission as described in clause 19.1 per SI modification period.

Proposal 6: Adopt TP#1.

  - Multiple channels in one direction colliding with another direction within one collision occasion

Additionally, multiple collision cases would happen in case 3 or case 4. One potential method is to handle the collision rule sequentially. An example of multiple collision cases in case 3 is illustrated in Figure 1. A configured UL transmission collides with two configured DL reception. For the first DL and the UL, the UL is prioritized according to a prioritization rule, and the first DL is not received. Then, for the collision between the second DL and the UL, the second DL is prioritized. Consequently, the UL is cancelled. In this case, only the second DL can be received by the UE, but actually both could be received.

Another method is to perform the prioritization among all the overlapping transmission. Specifically, in this example, if one of the DL receptions is prioritized over the UL transmission, the UL is cancelled, and two DL receptions can be performed.

For the former method, the efficiency is lower than the latter one. However, the latter method requires clearly defining the multi-collision cases under different scenarios depending on the real-time situation. It is very difficult and complexity to define the priority rules, as well as the processing timeline. In our opinion, the simplest approach is to leave them to UE implementation.

Figure 1 An example of multiple collision cases in case 3

Observation 2: It is very difficult and complicated to define the prioritization rule for multiple collision cases.

Based on the above, the simple way is up to UE implementation to prioritize UL or DL. It is beneficial for UE flexibility and can avoid complex specification changes.

Proposal 7: When the collision happens at the UE side for collision case 3 and case 4, it is up to UE implementation to prioritize UL transmission or DL reception.

  - TA report enhancement

The mismatch between UE and gNB is caused by the granularity of TA reporting, infrequent TA reports and the TA drifting of the TA caused by the movement of the satellite. In the RAN1 118 meeting, the following agreement about TA reporting enhancements was approved to study to reduce the TA mismatch.

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

In the current spec, the granularity of TA reporting is 1ms and the TA offset threshold can be configured with a smaller value (e.g., 0.5ms). If a smaller TA offset threshold is configured, it would trigger UE frequently reporting TA which would lead to more power consumption of UE and would increase the signaling overhead of the system. The maximum TA mismatch caused by the limited value of the TA threshold can be controlled by the configuration of gNB. If finer granularity is introduced for the TA report, at most 1ms resource can be additional used by the UE in a switching duration. The benefit of the enhancement is very limited.

Another factor is the TA drifting due to the propagation delay of the TA report. The rough TA drift rate can be determined by the height of the orbit. In the scenario of LEO with 600km height, 3dB beam width, and 60-degree elevation angle, the propagation delay for a UE in the center of the cell in the UE-satellite-gNB link can be 600km3*108ms*sin60°=4.6ms. The timing drift rate is equal to 23.31 us/s. Therefore, the TA mismatch caused by propagation delay is 23.31 us/s*4.6ms = 0.1107226us. It can be covered by the CP which is equal to 4.7us with 15kHz SCS. In this case, the difference would not introduce symbol and slot level mismatch.

One more factor mentioned was whether TA reporting is supported by the UE. In our opinion, the maximum TA variation can be estimated by the gNB based on the maximum propagation delay, and the timing drift caused by the movement of satellite and the movement of the UE. It can be up to gNB implementation based on the conservation schedule for that kind of low-capability UE.

Observation 3: If the TA report is supported by a UE, a smaller TA offset threshold leads to more power consumption of UE and increases the signaling overhead of the system, and the maximum TA mismatch caused by the limited value of the TA threshold can be controlled by the configuration of gNB. The benefit of smaller granularity is limited. The TA mismatch caused by TA drifting can be covered by CP.

Observation 4: If the TA report is not supported by the UE, the maximum TA variation can be estimated by the gNB based on the maximum differential delay and the timing drift. The maximum differential delay in the cell can be calculated by the gNB. The gNB can perform scheduling based on the estimated maximum TA variation to avoid UL and DL collision.

Proposal 8: No enhancement on TA report in Rel-19.

- Conclusion

In this contribution, we provide our views on the support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN band. According to the discussions, we have the following observations and proposals:

Observation 1: According to the note in the working assumption, the UL cancellation timeline is up to UE implementation.

Observation 2: It is very difficult and complicated to define the prioritization rule for multiple collision cases.

Observation 3: If the TA report is supported by a UE, a smaller TA offset threshold leads to more power consumption of UE and increases the signaling overhead of the system, and the maximum TA mismatch caused by the limited value of the TA threshold can be controlled by the configuration of gNB. The benefit of smaller granularity is limited. The TA mismatch caused by TA drifting can be covered by CP.

Observation 4: If the TA report is not supported by the UE, the maximum TA variation can be estimated by the gNB based on the maximum differential delay and the timing drift. The maximum differential delay in the cell can be calculated by the gNB. The gNB can perform scheduling based on the estimated maximum TA variation to avoid UL and DL collision.

Proposal 1: The handling of collision with dedicated PDSCH scheduled by PDCCH with DCI scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI by type 0/0A PDCCH CSS is left to UE implementation.

Proposal 2: Handling of collision with PDCCH order triggered PRACH can follow the priority rule and controlled by the network indication.

Proposal 3: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all cases

    - This is signaled by one UE specific RRC parameter

  - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note2: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Proposal 4: Delete the condition "operating on a non-NTN serving cell" to include UEs operating in NTN that do not support these new capabilities, and add ‘subject to UE capability’ or ‘UE may determine’ to properly reflect that the behavior is dependent on whether the UE supports the new capability.

Proposal 5: The handling collision case of PDSCH scheduled by a PDCCH in type 0/0A/1/2 CSS should be excluded in the ‘other use cases’ for case 4 (i.e., It should not be controlled by the RRC parameter ntnRedCapCollisionCase4UlPriority).

Proposal 6: Adopt TP#1.

Proposal 7: When the collision happens at the UE side for collision case 3 and case 4, it is up to UE implementation to prioritize UL transmission or DL reception.

Proposal 8: No enhancement on TA report in Rel-19.

- Reference

- RP- 243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.

- R1-2503179 Introduction of Non-Terrestrial Networks (NTN) for NR Phase 3, Samsung.

- R1-2503392, Discussion on UE features for NTN for NR Phase 3, vivo.