3GPP TSG RAN WG1 Meeting #120-bis	R1- 2503453

Wuhan, China, April 7th – 11st, 2025

Source: 	Moderator (Intel Corporation)

Title:	Editor’s summary on draft CR 38.215 for NR_LPWUS

Agenda item:	9

Document for:	Discussion

# Introduction

In this contribution, editor of TS38.215 summarizes email discussions for introduction of Rel-19 LP-WUS features for TS38.215.

# Summary of Discussion

### Round #1 Discussion

Editor has provided initial version of the draft CR for TS38.215 containing introduction of Rel-19 LP-WUS features in

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_LPWUS/R1-250xxxx%20TS38215_Rel19_CRdraft_LPWUS_v00.docx

Editor has asked companies to provide inputs and comments on initial version.

##### Company Comments for Initial version of draft CR

| Company | Comments |
| --- | --- |
| Xiaomi | Generally OK with the CR, and just a editorial change: Low power received signal strength indicator (LP-RSSI), comprises the linear average of the total received power (in [W]) observed only both in ON and OFF Low power synchronization signal (LP-SS) on-off keying (OOK) symbols over the resource elements, that carry LP-SS OOK symbols, from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. |
| Qualcomm | For idle/inactive, the LP-WUR RRM measurement is only for the serving cell. LP-SS is configured within the same carrier as SSB. In this case, we wonder why it is applicable to “RRC_IDLE inter-frequency” and “RRC_INACTIVE inter-frequency” Agreement For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB |
| vivo | 1. Similar view with Qualcomm, LP-WUR RRM measurement is only for serving cell per WID. Therefore, it is not applicable to inter-frequency case, therefore, RRC_IDLE inter-frequency & RRC_INACTIVE inter-frequency is to be deleted. Regarding intra-frequency, to avoid wrong impression of intra-frequency for neighboring cell, better to add ‘for serving cell only’. From WID: Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2). 2. For LP-RSSI in section 5.1.54 as copied below, the meaning of ‘only’ is unclear. We suggest to delete ‘only’. Low power received signal strength indicator (LP-RSSI), comprises the linear average of the total received power (in [W]) observed only in ON and OFF Low power synchronization signal (LP-SS) on-off keying (OOK) symbols over the resource elements, that carry LP-SS OOK symbols, from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. 3. For LP-SSS-RSRQ and LP-SSS-RSRP, RAN1 agreed no separate metrics to be introduced in the spec, it does not mean, no separate measurement quantity for LP-SSS-RSRQ and LP-SSS-RSRP. SSS-RSRP/SSS-RSRQ and LP-SSS-RSRP/LP-SSS-RSRQ is different measurement quantity, one for MR and one for LP-WUR. We suggest to separately captured in the spec to avoid any confusion as shown below. Besides, RAN1 agreed SMTC window is not applicable, which is not reflected in the CR. Agreement For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable. Send an LS to RAN4 to confirm the above agreement. Final LS in R1-2503103.. Suggested separate section for LP-SSS-RSRP and LP-SSS-RSRQ. ### 5.1.55 Low power Secondary Synchronization Signal reference signal received power (LP-SSS-RSRP) Definition; Low power secondary Synchronization Signal reference signal received power (LP-SSS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. LP-SSS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. For frequency range 1, the reference point for the LP-SSS-RSRP shall be the antenna connector of the UE. For frequency range 2, LP-SSS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported LP-SSS-RSRP value shall not be lower than the corresponding LP-SSS-RSRP of any of the individual receiver branches.. Applicable for; RRC_IDLE intra-frequency for serving cell only RRC_INACTIVE intra-frequency for serving cell only. NOTE 1: The number of resource elements within the measurement period that are used by the UE to determine LP-SSS-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled. NOTE 2: The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP. ### 5.1.56 Low power Secondary Synchronization Signal reference signal received quality (LP-SSS-RSRQ) Definition; Low power secondary synchronization signal reference signal received quality (LP-SSS-RSRQ) is defined as the ratio of N×LP-SSS-RSRP / LP-RSSI, where N is the number of resource blocks in the NR carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks. Low power Received Signal Strength Indicator (LP-RSSI), comprises the linear average of the total received power (in [W]) observed only in certain OFDM symbols of measurement time resource(s), in the measurement bandwidth, over N number of resource blocks from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. For frequency range 1, the reference point for the LP-SSS-RSRQ shall be the antenna connector of the UE. For frequency range 2, LP-RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch, where the combining for LP-RSSI shall be the same as the one used for LP-SSS-RSRP measurements. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported LP-SSS-RSRQ value shall not be lower than the corresponding LP-SSS-RSRQ of any of the individual receiver branches.. Applicable for; RRC_IDLE intra-frequency for serving cell only RRC_INACTIVE intra-frequency for serving cell only. |
| Huawei, HiSilicon | Thank you for the draft CR. Comment#1 As commented by other companies, LP-RSRP and LP-RSRQ performed by LP-WUR are not applicable to measurements of neighboring cell according to WID. According to the definitions below, both intra-frequency and inter-frequency are used for neighboring cell measurements. Therefore, both should be deleted. S5.1 of TS 38.215 Intra-frequency appended to the RRC state: Shall be possible to perform in the corresponding RRC state on an intra-frequency cell; Inter-frequency appended to the RRC state: Shall be possible to perform in the corresponding RRC state on an inter-frequency cell. Suggested changes: For both LP-RSRP and LP-RSRQ RRC_IDLE, RRC_IDLE, RRC_INACTIVE, RRC_INACTIVE Comment#2 The value of legacy SS-RSRP is average power over REs instead of the bandwidth of SSS (127 REs), i.e. it is a per-RE value. Although LP-WUR does not perform frequency domain measurement, but it is important to confirm the value unit of LP-RSRP is also per-RE, i.e. the measured value will be divided by a factor 132 in LP-WUR, which can facilitate to have similar value ranges between SS-RSRP based and LP-RSRP-based thresholds that are used for entry&exit procedures. For this purpose, suggest to take the exact wording as SS-RSRP with regard to what being average over. If the intention is to confirm that the unit is power of per-11-RBs, then please take an explicit note or any explicit wording that could be quite different from the legacy wording of SS-RSRP. For example, “LP-RSRP is defined as the linear average of the total received power (in [W]) observed only in on-off keying (OOK) ON of LP-SS duration, in the measurement bandwidth of 11 resource blocks of LP-SS.” SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS. Suggested changes: S5.1.53: Low power reference signal received power (LP-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry Low power synchronization signals (LP-SS) in on-off keying (OOK) ON symbols. Or Low power reference signal received power (LP-RSRP) is defined as the linear average of the total received power (in [W]) observed only in on-off keying (OOK) ON of LP-SS duration, in the measurement bandwidth of 11 resource blocks of LP-SS. |
| Nokia1 | As commented by other companies, we think also that in minimum some updates are needed to SS-RSRP and SS-RSRQ are needed to account e.g. the agreements on the applicability of the existing metrics for the OFDM-based LR, i.e. limit the applicability (for LR’s) only for IDLE and Inactive mode (intra-frequency). |
| Editor | @vivo: For issue 3 on LP-SSS-RSRQ and LP-SSS-RSRP, Editor is confused to what is being suggested. What is the difference between metric and quantity? LP-SSS-RSRQ and LP-SSS-RSRQ was not captured to 215 as agreed in RAN1. Adding the definition seems to go against what was agreed in RAN1. We could update the SS-RSRP and SS-RSRQ that SMTC is not applicable when WUR is performing the measurements. This seems to be the most reasonable approach given the agreement in RAN1. The following are agreements related to LP-SSS-RSRQ and LP-SSS-RSRP. Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Agreement Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. Agreement For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable. - Send an LS to RAN4 to confirm the above agreement. Final LS in R1-2503103.. |

### Round #2 Discussion

Editor has asked companies to provide inputs and comments on revised draft CR.

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_LPWUS/R1-250xxxx%20TS38215_Rel19_CRdraft_LPWUS_v01.docx

Editor has implemented the following changed based on comments received:

- Removal of “only” for LP-RSRP

- Limit measurement for IDLE/INACTIVE for intra-frequency (remove inter-frequency). Note addition of serving cell may not be appropriate as in IDLE mode there is no “serving cell”. The UE is in camped cell state. So intra-frequency limitation should be sufficient.

- Added exception for SMTC for WUR for SS-RSRQ and SS-RSRQ.

- Reformulated LP-RSRP/RSRQ to mimic the definition for SS-RSRP definition over resource elements.

##### Company Comments for v01 version of draft CR.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Thanks for the updated draft CR. Comment#1 As a reminder, inter-frequency is not removed yet in the latest draft CR. Intra-frequency should be also removed because: - The prefix “Intra” is not defined for single cell but for the relationship of two cells, i.e. the frequency relationship between serving cell/camping cell and neighboring cell. Since LR measurement is applicable only to the single camping cell, “intra-frequency” is misleading and imply measurement of neighboring cell having the same center frequency as the camping cell. Since only single cell measurement, there is no “intra”. - The definition of “intra-frequency measurement” can be found in TS 38.133, which definitely involves neighboring cells. TS 38.133 ## 9.2 NR intra-frequency measurements ### 9.2.1 Introduction A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same.. Suggested changes: Remove both words “intra-frequency” and “inter-frequency” Or If “intra-frequency” has to be kept, then please add a note to clarify no neighboring cell by “ NOTE: Refer to TS38.133 [18] for cell applicability. ” Comment#1 For S5.1.1 and S5.1.3, in TR 38.869, the word “R” in LP-WUR represents receiver instead of radio. Better to reuse the same term or a term used by RAN4. TR 38.869 LP-WUR Low Power-Wake Up Receiver LR LP-WUR Suggested changes: For S5.1.1 and S5.1.3 except when measurement is being performed by wake-up receiver (WUR). |
| ZTE, Sanechips | “intra-frequency” and “inter-frequency” are based on neighbor cell measurement, which is not applicable for WUR. Therefore, we can wait for RAN2 progress and check whether there is a new scenario, like “intra-frequency” or “inter-frequency” is defined. Additionally, regarding ‘except when measurement is being performed by wake-up radio (WUR)’, we do not think we should mention WUR in TS 38.215 since definition of WUR is unclear and it is kind of implementation description. Instead, we can try the following: except when measurement is being performed based on FG 62-1a. |
| vivo | Thanks for the updated CR and your kind response to our issue 3. 1. For issue 3, our intention to have separate section for LP-SSS-RSRP/LP-SSS-RSRQ is to avoid complication of RAN4 specification, e.g., it is very hard to distinguish in TS 38.133 which SS-RSRP requirements are applicable for MR and which are applicable to LP-WUR. Having said that, we understand and respect that editors can coordinate if needed. If we reuse SS-RSRP/RSRQ section, texts which are not applicable to LP-WUR (copied as below) needs to be clearly spelt out, e.g., adding ‘Not applicable when the measurement is being performed by wake-up radio (WUR)’ at beginning of each corresponding paragraph. For SS-RSRP: For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For SS-RSRQ: If indicated by higher-layers, if measurement gap is not used, the NR Carrier RSSI is measured in slots within the SMTC window duration that are indicated by the higher layer parameter measurementSlots and in OFDM symbols given by Table 5.1.3-1 and, if measurement gap is used, the NR Carrier RSSI is measured in slots within the SMTC window duration that are indicated by the higher layer parameter measurementSlots and in OFDM symbols given by Table 5.1.3-1 that are overlapped with the measurement gap, which is defined in TS38.133 [12]. - For intra-frequency measurements, NR Carrier RSSI is measured with timing reference corresponding to the serving cell in the frequency layer - For inter-frequency measurements, NR Carrier RSSI is measured with timing reference corresponding to any cell in the target frequency layer Otherwise not indicated by higher-layers, if measurement gap is not used, NR Carrier RSSI is measured from OFDM symbols within SMTC window duration and, if measurement gap is used, NR Carrier RSSI is measured from OFDM symbols corresponding to overlapped time span between SMTC window duration and the measurement gap. If higher-layers indicate certain SS/PBCH blocks for performing SS-RSRQ measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s).. 2. Regarding inter/intra-frequency measurement, it seems editor forget to delete inter-frequency. Regarding intra-frequency, the comment ‘Note addition of serving cell may not be appropriate as in IDLE mode there is no “serving cell”. The UE is in camped cell state.’, we share different view. As captured in TS 38.304 for RRC idle/inactive sate, serving cell is the cell on which the UE is camped in section 3.1 ‘Serving cell: The cell on which the UE is camped’. Therefore, we still prefer ‘RRC_IDLE intra-frequency for serving cell only & RRC_INACTIVE intra-frequency for serving cell only’, for section 5.1.1, 5.1.3, 5.1.53 and 5.1.54, or also remove intra-frequency and only keep for serving cell. |
| Editor | @Huawei: Will update to measurement applicability to serving cell Will update radio to receiver. @ZTE: The problem is FG62-1a is only label that is used internally between RAN WGs and not part of any normative text. We would need to reference this by the RAN2 RRC IE for the capability in question. However, I do not believe the RAN2 CR is yet available, so Editor does not know what this capability parameter name is. According to Huawei, the RAN4 specification seems to have defined the term, so Editor suggests to keep this term for now. RAN1 can update this further in next meeting. @vivo: The motivation of the comment is well understood. However, given the RAN1 agreements, Editor simply can’t start adding new metrics when there is an explicit agreement not to do so. If the intent of the RAN1 agreement was something different, Editor would like to encourage companies to update and clarify the RAN1 agreement on “Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.” As for not applying PBCH DMRS, and CSI-RS for RSRP/RSRQ for WUR, while the proposal may seem reasonable, there is no RAN1 agreement to support the changes. Editor would suggest interest companies to bring the proposal to the next meeting and get agreements to update the relevant text in 215. See response to Huawei regarding intra-frequency applicability changes. |

### Round #3 Discussion

Editor has asked companies to provide inputs and comments on revised draft CR.

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_LPWUS/R1-250xxxx%20TS38215_Rel19_CRdraft_LPWUS_v02.docx

Editor has implemented the following changed based on comments received:

- Removed inter-frequency applicability for LP-RSRP/RSRQ

- Updated intra-frequency applicability to serving cell

##### Company Comments for v02 version of draft CR.

| Company | Comments |
| --- | --- |
| vivo | Thank editor for your suggestion and updated CR. Just one more comment for the measurement applicability. Measurement only for serving cell per WID is applicable to both the measurement based on LP-SS and SSB. Therefore, we suggest to add ‘RRC_IDLE for serving cell and RRC_INACTIVE for serving cell’ also in section 5.1.1 and 5.1.3 as shown below. For section 5.1.1, Applicable for; If SS-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency. If SS-RSRP is being performed by wake-up receiver (WUR), RRC_IDLE for serving cell, RRC_INACTIVE for serving cell. Otherwise, RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. For section 5.1.3 Applicable for; If SS-RSRQ is being performed by wake-up receiver (WUR), RRC_IDLE for serving cell, RRC_INACTIVE for serving cell. Otherwise, RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. |
| Apple | Agree with the comments from vivo: it should be clarified that SS-RSRP/SS-RSRQ performed by WUR is for idle/inactive for serving cell only. Typo in definition of SS-RSRP (Clause 5.1.1): reciver -> receiver |
| Editor | @vivo: Will update SS-RSRP and SS-RSRQ as suggested. @Apple: Will correct the typo. |

### Round #4 Discussion

Editor has asked companies to provide inputs and comments on revised draft CR.

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_LPWUS/R1-250xxxx%20TS38215_Rel19_CRdraft_LPWUS_v03.docx

Editor has implemented the following changed based on comments received:

- Added serving cell applicability for SS-RSRP and SS-RSRQ for WUR.

- Corrected typo for receiver.

##### Company Comments for v03 version of draft CR.

| Company | Comments |
| --- | --- |
| vivo | Thanks editor for the updated CR addressing our comment. One more comment for a typo. In section 5.1.3 for SS-RSRQ, ‘If SS-RSRP is being performed by wake-up receiver (WUR),’ -> ‘If SS-RSRQ is being performed by wake-up receiver (WUR)’. |

### Round #5 Discussion

Editor has asked companies to provide inputs and comments on revised draft CR.

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_120b/Inbox/drafts/9(NR_R19)/38.215%20draft%20CRs/NR_LPWUS/R1-250xxxx%20TS38215_Rel19_CRdraft_LPWUS_v04.docx

Editor has implemented the following changed based on comments received:

- Corrected typo for RSRQ.

##### Company Comments for v04 version of draft CR.

| Company | Comments |
| --- | --- |

### Summary of Discussion

Draft CR v04 has been endorsed by the Chairman.