3GPP TSG-RAN WG1 Meeting #120bis	R1-2503473

# Wuhan, China, 7-11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-19 NR MIMO Phase 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-4-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of Rel-19 NR MIMO Phase 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancements for UE-initiated/event-driven beam management - Clause 3.3: Add abbreviation for UEIRI. - Clause 6.3.1: Add operation of the new UCI type UEIRI - Clause 6.3.2.1.2: Add Table 6.3.1.1.2-8C as the UCI format of UE-initiated/event-driven beam reporting CSI enhancements - Clause 6.3.1.1.2: Add Table 6.3.1.1.2-1A/1B/2A/3C and update Table 6.3.1.1.2-4/10/11/11C to reflect the agreements on CSI enhancements for up to 128 ports; - Clause 6.3.2.1.2: Update Table 6.3.2.1.2-1A/1C/2B/3/4/5/5H/8/8B/9 to reflect the agreements on CSI enhancements for up to 128 ports; - Clause 6.3.1.1.2: Add Table 6.3.1.1.2-7B/9C/10C/11D and update Table 6.3.1.1.2-12/13/14 to reflect the agreements on CRI-based CSI reporting; - Clause 6.3.2.1.2: Add Table 6.3.2.1.2-3D/4C/5I/5J/8C and update Table 6.3.2.1.2-6/7 to reflect the agreements on CRI-based CSI reporting; - Clause 6.3.2.1.2: Add Table 6.3.2.1.2-3E/3F/3G/3H/11/12/13 and update Table 6.3.2.1.2-6 to reflect the agreements on reporting for UE assisted calibration; Support for 3-antenna-port codebook-based transmissions - Clause 7.3.1.1.2: Update Precoding information and number of layers field and PTRS-DMRS association field in DCI format 0_1 to reflect the agreements on 3Tx; - Clause 7.3.1.1.3: Update Precoding information and number of layers field and PTRS-DMRS association field in DCI format 0_2 to reflect the agreements on 3Tx; Enhancement for asymmetric DL sTRP/UL mTRP scenarios - Clause 7.3.1.2.1: Add Pathloss offset indicator filed and update PRACH association indicator field - Clause 7.3.1.2.2: Add TPC command field for SRS and Closed loop indicator field for SRS - Clause 7.3.1.3.4: Add Closed loop indicator field |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 MIMO Evolution for Downlink and Uplink will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.3.1, 6.3.1.1.2, 6.3.2.1.2, 7.3.1.1.2, 7.3.1.1.3, 7.3.1.2.1, 7.3.1.2.2, 7.3.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BCH	Broadcast Channel

CAPC	Channel Access Priority Class

CBG	Code Block Group

CBGTI	Code Block Group Transmission Information

CG	Configured Grant

CG-DFI	CG - Downlink Feedback Information

CG-UCI	CG - Uplink Control Information

CORESET	Control Resource Ret

COT	Channel Occupancy Time

CP	Cyclic Prefix

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRI	CSI-RS Resource Indicator

CSI	Channel State Information

CSI-RS	CSI - Reference Signal

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink - Shared Channel

DMRS	Demodulation Reference Signal

HARQ	Hybrid Automatic repeat Request

HARQ-ACK	Hybrid Automatic repeat Request - Acknowledgement

LDPC	Low Density Parity Check

LI	Layer Indicator

MBS	Multicast Broadcast Services

MCS	Modulation and Coding Scheme

NCR	Network-controlled repeater

OFDM	Orthogonal Frequency Division Multiplex

PBCH	Physical Broadcast Channel

PCH	Paging Channel

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PMI	Precoding Matrix Indicator

PRB	Physical Resource Block

PRACH	Physical Random Access Channel

PSBCH	Physical Sidelink Broadcast Channel

PSCCH	Physical Sidelink Control Channel

PSFCH	Physical Sidelink Feedback Channel

PSSCH	Physical Sidelink Shared Channel

PTRS	Phase-Tracking Reference Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RACH	Random Access Channel

RI	Rank Indicator

RSRP	Reference Signal Received Power

SCI	Sidelink Control Information

SFCI	Sidelink Feedback Control Information

SFN	System Frame Number

SL	Sidelink

SL-BCH	Sidelink - Broadcast Channel

SL PRS	Sidelink Positioning Reference Signal

SL-SCH	Sidelink - Shared Channel

SR	Scheduling Request

SRS	Sounding Reference Signal

SS	Synchronisation Signal

SUL	Supplementary Uplink

TCI	Transmission Configuration Indicator

TPC	Transmit Power Control

TrCH	Transport Channel

TRS	Tracking Reference Signal

UCI	Uplink Control Information

UE	User Equipment

UEIRI	UE Initiated Report Indicator

UL	Uplink

UL-SCH	Uplink Shared Channel

UTO-UCI	Unused Transmission Occasion - Uplink Control Information

VRB	Virtual Resource Block

ZP CSI-RS	Zero power CSI-RS

< Unchanged parts are omitted >

    - 6.3.1	Uplink control information on PUCCH

The procedure in this clause applies to PUCCH formats 2/3/4.

The following clauses 6.3.1.2, 6.3.1.3 and 6.3.1.5 apply regardless of whether the higher layer parameter uci-MuxWithDiffPrio is configured or not. The following clauses 6.3.1.1, 6.3.1.4 and 6.3.1.6 apply by assuming uci-MuxWithDiffPrio is not configured, or uci-MuxWithDiffPrio is configured and the UCIs for transmission on a PUCCH are of the same priority index, unless stated otherwise.

If the UE is configured with a PUCCH-SCell, uci-MuxWithDiffPrio is replaced by uci-MuxWithDiffPrioSecondaryPUCCHgroup for the secondary PUCCH group in this clause.

If UEIRI is transmitted on a PUCCH, the procedure in this clause 6.3.1 applies by replacing SR with UEIRI in all the notations and texts, when applicable.

      - 6.3.1.1	UCI bit sequence generation

##### 6.3.1.1.1	HARQ-ACK/SR only

If only HARQ-ACK bits are transmitted on a PUCCH, the UCI bit sequence  is determined by setting  for  and , where the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213].

If only HARQ-ACK and SR bits are transmitted on a PUCCH, the UCI bit sequence  is determined by setting  for , ai=oi-OACKSR for , and , where the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS 38.213], and the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213].

        - 6.3.1.1.2	CSI only

If cqi-BitsPerSubband is configured, this Clause 6.3.1.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the bitwidth of a CSI field of the CSI sub-report is determined following the procedure in this clause 6.3.1.1.2 by taking configurations in CSI-ReportSubConfig when applicable. If CSI-ReportSubConfig configures a list of CSI-RS resource IDs, for the determination of the bitwidth of a CRI field, the value of KsCSI-RS is the number of CSI-RS resources configured in the corresponding CSI-ReportSubConfig.

The bitwidth for PMI of codebookType=typeI-SinglePanel with 2 CSI-RS ports is 2 for Rank=1 and 1 for Rank=2, according to Clause 5.2.2.2.1 in [6, TS 38.214].

The bitwidth for PMI of codebookType=typeI-SinglePanel with more than 2 CSI-RS ports is provided in Table 6.3.1.1.2-1, where the values of and  are given by Clause 5.2.2.2.1 in [6, TS 38.214].

Table 6.3.1.1.2-1: PMI of codebookType=typeI-SinglePanel

|  | Information field for wideband PMI |  |  | Information field for wideband PMI or per subband PMI |  |
| --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |
|  | codebookMode=1 | codebookMode=2 |  | codebookMode=1 | codebookMode=2 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (,) | N/A | 2 | 4 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (, 0) | N/A | 2 | 4 |
| Rank=2 with 4 CSI-RS ports, | (,) | (, 0) | 1 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (,) | 2 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (, 0) | 2 | 1 | 3 |
| Rank=3 or 4, with 4 CSI-RS ports | (,) |  | 0 | 1 |  |
| Rank=3 or 4, with 8 or 12 CSI-RS ports | (,) |  | 2 | 1 |  |
| Rank=3 or 4 , with >=16 CSI-RS ports | (, ) |  | 2 | 1 |  |
| Rank=5 or 6 | (,) |  | N/A | 1 |  |
| Rank=7 or 8, | (, ) |  | N/A | 1 |  |
| Rank=7 or 8, | (,) |  | N/A | 1 |  |
| Rank=7 or 8, with or or | (,) |  | N/A | 1 |  |

The bitwidth for PMI of codebookType=typeI-SinglePanel-r19 is provided in Table 6.3.1.1.2-1A/1B for typeI-codebookMode=ModeA and typeI-codebookMode=ModeB respectively, where the values of (N1, N2) and (O1, O2) are given by Clause 5.2.2.2.1a in [6, TS 38.214].

Table 6.3.1.1.2-1A: PMI of codebookType=typeI-SinglePanel-r19 and typeI-codebookMode=ModeA

|  | Information field X1 for wideband PMI |  |  |  |  | Information field X2 for wideband PMI or per subband PMI |
| --- | --- | --- | --- | --- | --- | --- |
|  | (i1,1, i1,2) | i1,3 | (i1,1,2, i1,2,2) | (i1,1,3, i1,2,3) | (i1,1,4, i1,2,4) | i2 |
| Rank = 1 | (log2N1O1, log2N2O2) | N/A | N/A | N/A | N/A | 2 |
| Rank=2, 3 or 4 | (log2N1O1, log2N2O2) | 2 | N/A | N/A | N/A | 1 |
| Rank=5 or 6, | (log2N1O1, log2N2O2) | 1 | If i1,3=0, (log2N1, log2N2O2); If i1,3=1, (log2N1O1, log2N2). | If i1,3=0, (log2N1, log2N2O2); If i1,3=1, (log2N1O1, log2N2). | N/A | 1 |
| Rank=7 or 8 | (log2N1O1, log2N2O2) | 1 | If i1,3=0, (log2N1, log2N2O2); If i1,3=1, (log2N1O1, log2N2). | If i1,3=0, (log2N1, log2N2O2); If i1,3=1, (log2N1O1, log2N2). | If i1,3=0, (log2N1, log2N2O2); If i1,3=1, (log2N1O1, log2N2). | 1 |

Table 6.3.1.1.2-1B: PMI of codebookType=typeI-SinglePanel-r19 and typeI-codebookMode=ModeB

|  | Information field X1 for wideband PMI |  |  |  |  |  | Information field X2 for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,1 | i1,2,1 | i1,2,2 | i1,2,3 | i1,2,4 | i1,2 | i2,1 | i2,2 | i2,3 | i2,4 |
| Rank=1 | log2(O1O2) | log2N1N2 | N/A | N/A | N/A | N/A | 2 | N/A | N/A | N/A |
| Rank=2 | log2(O1O2) | log2N1N2 | log2N1N2 | N/A | N/A | N/A | 2 | 2 | N/A | N/A |
| Rank=3 | log2(O1O2) | log2N1N2 | log2N1N2 | log2N1N2 | N/A | N/A | 2 | 2 | 2 | N/A |
| Rank= 4 | log2(O1O2) | log2N1N2 | log2N1N2 | log2N1N2 | log2N1N2 | N/A | 2 | 2 | 2 | 2 |
| Rank=5 | log2(O1O2) | N/A | N/A | N/A | N/A | log2N1N23 | 1 | 1 | 2 | N/A |
| Rank=6 | log2(O1O2) | N/A | N/A | N/A | N/A | log2N1N23 | 1 | 1 | 1 | N/A |
| Rank=7 | log2(O1O2) | N/A | N/A | N/A | N/A | log2N1N24 | 1 | 2 | 1 | 1 |
| Rank=8 | log2(O1O2) | N/A | N/A | N/A | N/A | log2N1N24 | 1 | 1 | 1 | 1 |

The bitwidth for PMI of codebookType= typeI-MultiPanel is provided in Table 6.3.1.1.2-2, where the values of and  are given by Clause 5.2.2.2.2 in [6, TS 38.214].

Table 6.3.1.1.2-2: PMI of codebookType= typeI-MultiPanel

|  | Information fields for wideband |  |  |  |  | Information fields for wideband or per subband |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |  |  |  |  |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | N/A | N/A | 2 | N/A | N/A | N/A |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | 2 | 2 | 2 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=1 with codebookMode=2 | (,) | N/A | 2 | 2 | N/A | N/A | 2 | 1 | 1 |
| Rank=2 with , codebookMode=2 | (,) | 1 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=3 or 4 with , codebookMode=2 | (,) | 0 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=2 or 3 or 4 with , codebookMode=2 | (,) | 2 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |

The bitwidth for PMI of codebookType= typeI-MultiPanel-r19 is provided in Table 6.3.1.1.2-2A, where the values of   (Ng, N1, N2) and (O1, O2) are given by Clause 5.2.2.2.2a in [6, TS 38.214].

Table 6.3.1.1.2-2A: PMI of codebookType= typeI-MultiPanel-r19

|  | Information fields X1 for wideband |  |  |  |  |  |  |  |  |  |  | Information fields X2 for wideband or per subband |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (i1,1,1, i1,2,1) | i1,3,1 | (i1,1,2, i1,2,2) | i1,3,2 | (i1,1,3, i1,2,3) | i1,3,3 | (i1,1,4, i1,2,4) | i1,3,4 | i1,4,1 | i1,4,2 | i1,4,3 | i2,1 | i2,2 | i2,3 | i2,4 |
| Rank=1 with Ng=2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | N/A | N/A | N/A | N/A | 2 | N/A | N/A | 2 | 2 | N/A | N/A |
| Rank=1 with Ng=4 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Rank=2 with Ng=2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | N/A | N/A | N/A | N/A | 2 | N/A | N/A | 1 | 1 | N/A | N/A |
| Rank=2 with Ng=4 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 |
| Rank=3 with Ng=2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | N/A | N/A | N/A | N/A | 2 | N/A | N/A | 1 | 1 | N/A | N/A |
| Rank=3 with Ng=4 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 |
| Rank=4 with Ng=2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | N/A | N/A | N/A | N/A | 2 | N/A | N/A | 1 | 1 | N/A | N/A |
| Rank=4 with Ng=4 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | (log2N1O1, log2N2O2) | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 |

The bitwidth for PMI with 1 CSI-RS port is 0.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel or reportQuantity set to 'cri-RI-CQI' or 1 CSI-RS port is provided in Table 6.3.1.1.2-3.

Table 6.3.1.1.2-3: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel, or
reportQuantity set to 'cri-RI-CQI', or 1 CSI-RS port

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator when codebookType=typeI-SinglePanel | 0 |  |  |  |  |
| Rank Indicator when reportQuantity set to 'cri-RI-CQI' | 0 | 1 | 2 | 3 | 3 |
| Layer Indicator | 0 |  |  |  |  |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI |  |  |  |  |  |

in Table 6.3.1.1.2-3 is the number of allowed rank indicator values according to Clause 5.2.2.2.1 [6, TS 38.214].  is the value of the rank. The value of  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. For higher layer parameter reportQuantity set to 'cri-RI-CQI', the values of the rank indicator field are mapped to rank indicator values with increasing order, where '0' is mapped to rank-1.

Table 6.3.1.1.2-3A: RI, LI, CQI, and CRI associated with one CSI-RS resource pair and
csi-ReportMode= Mode 1 or Mode 2

| Field | Bitwidth |  |
| --- | --- | --- |
|  | 1 antenna port per Resource | > 1 antenna ports per Resource |
| Rank Combination Indicator | 0 | min(2,log2 nRI,NCJT) |
| The first Layer Indicator | 0 | log2v1 |
| The second Layer Indicator | 0 | log2v2 |
| Wide-band CQI for the first TB | 4 | 4 |
| Subband differential CQI for the first TB | 2 | 2 |
| CRI if csi-ReportMode= Mode 1 | log2N | log2N |
| CRI if csi-ReportMode= Mode 2 | log2M1+M2+N | log2M1+M2+N |

Table 6.3.1.1.2-3B: RI, LI, CQI, and CRI associated with one CSI-RS resource and
csi-ReportMode= Mode 1 or Mode 2

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator | 0 | min(1,log2 nRI,sTRP) | min(2,log2 nRI,sTRP) | log2 nRI,sTRP | log2 nRI,sTRP |
| Layer Indicator | 0 | log2v | min⁡(2, log2v) | min⁡(2, log2v) | min⁡(2, log2v) |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI if csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1 = 1 | log2M1+M2 | log2M1+M2 | log2M1+M2 | log2M1+M2 | log2M1+M2 |
| CRI if csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1 = 2 | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI |
| CRI if csi-ReportMode= Mode 2 | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N |

nRI,NCJT in Table 6.3.1.1.2-3A is the number of allowed rank combination indicator values associated with one CSI-RS resource pair according to Clause 5.2.1.4.2 [6, TS 38.214]. The values of the rank combination indicator field are mapped to allowed rank combinations in the following order: {1,1}, {1,2}, {2,1},{2,2}, where '0' is mapped to the first allowed rank combination. v1 and v2 are the values of the first and the second rank associated with two CSI-RS resources of the CSI-RS resource pair respectively.

nRI, sTRP in Table 6.3.1.1.2-3B is the number of allowed rank indicator values associated with one CSI-RS resource according to Clause 5.2.1.4.2 [6, TS 38.214]. v is the value of the rank associated with the CSI-RS resource. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The value of N in Table 6.3.1.1.2-3A and Table 6.3.1.1.2-3B is the number of CSI-RS resource pairs configured within a CSI-RS resource set. The values of M1 and M2 in Table 6.3.1.1.2-3A and Table 6.3.1.1.2-3B are given by

-	If sharedCMR = "Enabled", M1 = K1 and M2 = K2

-	If sharedCMR is absent and N = 1, M1 = K1 - 1 and M2 = K2 - 1

-	If sharedCMR is absent and N = 2,

-	M1 = K1 - 2 and M2 = K2 - 2, if the two resource pairs do not share any CSI-RS resource

-	M1 = K1 - 1 and M2 = K2 - 2, if the two resource pairs share the same CSI-RS resource from the first CSI-RS resource group

-	M1 = K1 - 2 and M2 = K2 - 1, if the two resource pairs share the same CSI-RS resource from the second CSI-RS resource group

where the values of K1 and K2 are the numbers of CSI-RS resources in the first and second CSI-RS resource groups within the CSI-RS resource set respectively.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel-r19 is provided in Table 6.3.1.1.2-3C.

Table 6.3.1.1.2-3C: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel-r19

| Field | Bitwidth |  |
| --- | --- | --- |
|  | Rank1~4 | Rank5~8 |
| Rank Indicator | log2nRI | log2nRI |
| Layer Indicator | min(2,log2υ) | min(2,log2υ) |
| Wide-band CQI for the first TB | 4 | 4 |
| Wideband CQI for the second TB | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 2 |
| CRI | 0 | 0 |
| NOTE: nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.1a in [6, TS 38.214]. υ is the value of the rank. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. |  |  |

The bitwidth for RI/LI/CQI/CRI of codebookType= typeI-MultiPanel or codebookType=typeI-MultiPanel-r19 is provided in Table 6.3.1.1.2-4.

Table 6.3.1.1.2-4: RI, LI, CQI, and CRI of codebookType=typeI-MultiPanel or codebookType=typeI-MultiPanel-r19

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| CRI if codebookType=typeI-MultiPanel |  |
| CRI if codebookType=typeI-MultiPanel-r19 | 0 |
| NOTE: nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.2 and 5.2.2.2.2a in [6, TS 38.214], υ is the value of the rank, and KsCSI-RS is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. |  |

The bitwidth for RI/LI/CQI of codebookType= typeII or codebookType=typeII-PortSelection is provided in Table 6.3.1.1.2-5.

Table 6.3.1.1.2-5: RI, LI, and CQI of codebookType=typeII or typeII-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the number of non-zero wideband amplitude coefficients for layer |  |
| NOTE: nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and υ is the value of the rank. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. |  |

The bitwidth for CRI, SSBRI, RSRP, differential RSRP, and CapabilityIndex are provided in Table 6.3.1.1.2-6.

Table 6.3.1.1.2-6: CRI, SSBRI, RSRP, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI |  |
| SSBRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |
| CapabilityIndex | 2 |
| NOTE: KsCSI-RS is the number of CSI-RS resources in the corresponding resource set, KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Index'. |  |

The bitwidth for CRI, SSBRI, SINR, differential SINR, and CapabilityIndex are provided in Table 6.3.1.1.2-6A.

Table 6.3.1.1.2-6A: CRI, SSBRI, SINR, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI | log2KsCSI-RS |
| SSBRI | log2KsSSB |
| SINR | 7 |
| Differential SINR | 4 |
| CapabilityIndex | 2 |
| NOTE: KsCSI-RS is the number of CSI-RS resources in the corresponding resource set, and KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-SINR' or 'ssb-Index-SINR-Index'. |  |

KsCSI-RSKsSSB

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the mapping order of CSI fields of one CSI sub-report is determined following the procedure in this clause 6.3.1.1.2, by replacing CSI report #n in the following Tables 6.3.1.1.2-7, 6.3.1.1.2-9 and 6.3.1.1.2-10 with CSI sub-report #n, and taking only Tables 6.3.1.1.2-1/1A/1B/2/2A /3/4 for the determination of the bitwidth of a CSI field.

Table 6.3.1.1.2-7: Mapping order of CSI fields of one CSI report,
pmi-FormatIndicator=widebandPMI and cqi-FormatIndicator=widebandCQI or reportQuantity
set to 'cri-RI-CQI' and cqi-FormatIndicator=widebandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/3C/4, if reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/3C/4, if reported |
|  | Zero padding bits , if needed |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/3C/4, if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/3C/4, if reported |

The number of zero padding bits  in Table 6.3.1.1.2-7 is 0 for 1 CSI-RS port and  for more than 1 CSI-RS port, where:

-	 and  is the set of rank values  that are allowed to be reported;

-	, where  is the reported rank;

-	For 2 CSI-RS ports, ;

-	For more than 2 CSI-RS ports, ;

-	if PMI is reported,  and ; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/1A/1B/2/2A; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/1A/1B/2/2A; otherwise, ;

-	if CQI is reported,  is obtained according to Tables 6.3.1.1.2-3/3C/4; otherwise, ;

-	if LI is reported,  is obtained according to Tables 6.3.1.1.2-3/3C/4; otherwise, .

Table 6.3.1.1.2-7A: Mapping order of CSI fields of one CSI report, pmi-FormatIndicator=widebandPMI, cqi-FormatIndicator=widebandCQI, csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1=0

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3A, if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | Zero padding bits OP, if needed |
|  | PMI wideband information fields X1, from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X2, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X1, from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X2, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if reported |

The number of zero padding bits OP in Table 6.3.1.1.2-7A is 0 for 1 CSI-RS port and OP=Nmax-Nreported for more than 1 CSI-RS port, where:

-	Nmax=maxr∈SRankB(r)  and SRank is the set of rank combination values of r={r1,r2} that are allowed to be reported;

-	Nreported=B(R) where R is the reported rank combination;

-	For 2 CSI-RS ports, Br=NPMI(r1)+NPMI(r2)+NCQIr+NLI(r1)+NLI(r2);

-	For more than 2 CSI-RS ports, Br=NPMI,i1(r1)+NPMI,i1(r2)+NPMI,i2(r1)+NPMI,i2(r2)+NCQIr+NLI(r1)+NLI(r2) ;

-	if PMI is reported, NPMI1=2 and  NPMI2=1; otherwise, NPMI=0;

-	if PMI i1 is reported, NPMI,i1(r1) and NPMI,i1(r2) are obtained according to Tables 6.3.1.1.2-1; otherwise, NPMI,i1=0;

-	if PMI  i2 is reported, NPMI,i2(r1) and NPMI,i2(r2) are obtained according to Tables 6.3.1.1.2-1; otherwise,  NPMI,i2=0;

-	if CQI is reported, NCQIr is obtained according to Tables 6.3.1.1.2-3A; otherwise, NCQIr=0;

-	if LI is reported, NLI(r1) and NLI(r2) are obtained according to Tables 6.3.1.1.2-3A; otherwise , NLI=0.

Table 6.3.1.1.2-7B: Mapping order of CSI fields of one CSI report configured with valueOfM,
pmi-FormatIndicator=widebandPMI and cqi-FormatIndicator=widebandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | The first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Rank Indicator for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Layer Indicator for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Zero padding bits OP,1 for the first reported CRI, if needed |
|  | PMI wideband information fields X1 for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the first reported CRI according to Clause 5.2.2.2.1 in [6, TS38.214], if reported |
|  | Wideband CQI for the first TB for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Wideband CQI for the second TB for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | … |
|  | The M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Rank Indicator for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Layer Indicator for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Zero padding bits OP,M for the first reported CRI, if needed |
|  | PMI wideband information fields X1 for the M-th reported CRI, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the M-th reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the M-th reported CRI according to Clause 5.2.2.2.1 in [6, TS38.214], if reported |
|  | Wideband CQI for the first TB for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Wideband CQI for the second TB for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |

The number of zero padding bits OP,k for k-th reported CRI in Table 6.3.1.1.2-7B is 0 for 1 CSI-RS port and OP,k=Nmax-Nreported,k for more than 1 CSI-RS port, where:

-	Nmax= maxj∈Q  (Nmax,j), where Q is the set of CRIs corresponding to the Ks resources and Nmax,j is the maximum payload size of associated CSI fields for the j-th CRI;

-	Nmax,j=NRI(j)+maxrj∈Srank,jB(rj), where NRI(j) is the payload size of RI field for the j-th CRI,  and Srank,j is the set of rank values rj that are allowed to be reported for the j-th CRI obtained according to Table 6.3.1.1.2-3;

-	Nreported,k= NRI(k)+B(Rk), where Rk is the reported rank for the k-th CRI;

-	For 2 CSI-RS ports, Br=NPMIr+NCQIr+NLIr;

-	For more than 2 CSI-RS ports, Br=NPMI,i1r+NPMI,i2r+NCQIr+NLIr;

-	if PMI is reported, NPMI1=2 and NPMI2=1; otherwise, NPMIr=0;

-	if PMI i1 is reported, NPMI,i1r is obtained according to Table 6.3.1.1.2-1; otherwise, NPMI,i1r=0;

-	if PMI i2 is reported, NPMI,i2r is obtained according to Table 6.3.1.1.2-1; otherwise, NPMI,i2r=0;

-	if CQI is reported, NCQIr is obtained according to Table 6.3.1.1.2-3; otherwise, NCQIr=0;

-	if LI is reported, NLIr is obtained according to Table 6.3.1.1.2-3; otherwise, NLIr=0.

Table 6.3.1.1.2-8: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting, or mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #3 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #4 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #1 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #2 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #3 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #4 as in Table 6.3.1.1.2-6, if reported |

Table 6.3.1.1.2-8A: Mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.1.1.2-6A, if reported |
|  | SINR #1 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #2 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #3 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #4 as in Table 6.3.1.1.2-6A, if reported |
|  | CapabilityIndex #1 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #2 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #3 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #4 as in Table 6.3.1.1.2-6, if reported |

Table 6.3.1.1.2-8B: Mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Resource set indicator |
|  | CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | RSRP of CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6 |
|  | Differential RSRP of CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6 |
|  | Differential RSRP of CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported |

where the 1-bit resource set indicator, with value of 0 or 1, indicates the 1st or the 2nd channel measurement resource set respectively, from which CRI or SSBRI #1 of 1st resource group is reported from; and all remaining resource groups, if reported, follow the same mapping order as the 1st resource group where CRI or SSBRI #1 of all remaining resource groups is reported from the indicated channel measurement resource set. For all reported resource groups, CRI or SSBRI #1 and CRI or SSBRI #2 are reported from different channel measurement resource sets.

Table 6.3.1.1.2-8C: Mapping order of CSI fields of one report for SSBRI/RSRP reporting for L1/L2-triggered mobility

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | SSBRI # L×M as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | Differential RSRP # L×M as in Table 6.3.1.1.2-6, if reported |
| NOTE: L is the number of reported cells provided by higher layer parameter nrOfReportedCells and M is the number of reported SSBRI/RSRP pairs per cell and equal to the value provided by higher layer parameter nrofReportedRS-PerCell. |  |

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/3C/4/5, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/3C/4/5, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/3C/4/5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

Table 6.3.1.1.2-9A: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if reported |
|  | CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; First CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Rank Indicator associated with the first CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI associated with the first CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 if reported; Subband differential CQI associated with the first CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Second CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with the second CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with the second CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with the second CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

Table 6.3.1.1.2-9B: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Rank Indicator as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported; Zero padding bits OP, if needed |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Wideband CQI for the first TB as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

The number of zero padding bits OP in Table 6.3.1.1.2-9B is 0 for 1 CSI-RS port and OP=Nmax-Nreported(R) for more than 1 CSI-RS port, where:

-	Nmax=maxr∈SRankN(r) . SRank is the set of rank and rank combination values r that are allowed to be reported. Nr is obtained according to Tables 6.3.1.1.2-3A/3B for rank combination indicator and rank indicator respectively.

-	Nreported R is obtained according to Tables 6.3.1.1.2-3A for rank combination indicator and R is the reported rank combination.

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3B for rank indicator and R is the reported rank.

Table 6.3.1.1.2-9C: Mapping order of CSI fields of one CSI report configured with valueOfM, CSI part 1, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI for the first reported CRI as in Tables 6.3.1.1.2-3 |
|  | Rank Indicator for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Zero padding bits OP,1 for the first reported CRI, if needed |
|  | Wideband CQI for the first TB for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | … |
|  | CRI for the M-th reported CRI as in Tables 6.3.1.1.2-3 |
|  | Rank Indicator for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Zero padding bits OP,M for the M-th reported CRI, if needed |
|  | Wideband CQI for the first TB for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

The number of zero padding bits OP,k in Table 6.3.1.1.2-9C is 0 for 1 CSI-RS port and OP,k=Nmax-Nreported,k for more than 1 CSI-RS port, where:

-	Nmax=maxi∈SNi, where S is the set of CRIs corresponding to the Ks resources and Ni is the payload size of RI field for the i-th CRI obtained according to Table 6.3.1.1.2-3;

-	Nreported,k is the payload size of RI field for the k-th CRI obtained according to Table 6.3.1.1.2-3.

Table 6.3.1.1.2-10: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/3C/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/3C/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.1.1.2-10A: Mapping order of CSI fields of one CSI report,
CSI part 2 wideband, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the first CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the second CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.1.1.2-10B: Mapping order of CSI fields of one CSI report,
CSI part 2 wideband, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, if reported part 1 is associated with one CSI-RS resource pair, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second CSI-RS resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.1.1.2-10C: Mapping order of CSI fields of one CSI report configured with valueOfM, CSI part 2 wideband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB for the first reported CRI as in Tables 6.3.1.1.2-3, if present and reported |
|  | Layer Indicator for the first reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | PMI wideband information fields X1 for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the first reported CRI according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |
|  | … |
|  | Wideband CQI for the second TB for the M-th reported CRI as in Tables 6.3.1.1.2-3, if present and reported |
|  | Layer Indicator for the M-th reported CRI as in Tables 6.3.1.1.2-3, if reported |
|  | PMI wideband information fields X1 for the M-th reported CRI, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the M-th reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the first reported CRI according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.1.1.2-11: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

Table 6.3.1.1.2-11A: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
csi-ReportMode= Mode 1

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.1.1.2-11B: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
csi-ReportMode= Mode 2

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.1.1.2-11C: Mapping order of CSI fields of one CSI report containing Nnsub CSI sub-report(s), CSI part 2 subband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI of CSI sub-report #1 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields X2 of CSI sub-report #1 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports of CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #1 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #1 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index of for 2 antenna ports CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nnsub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to.

Table 6.3.1.1.2-11D: Mapping order of CSI fields of one CSI report configured with valueOfM, CSI part 2 subband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number for the first reported CRI, as in Table 6.3.1.1.2-3, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields X2 of all even subbands with increasing order of subband number for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number for the first reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number for the first reported CRI, as in Table 6.3.1.1.2-3, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all odd subbands with increasing order of subband number for the first reported CRI, from left to right as in Table 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number for the first reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number for the M-th reported CRI, as in Table 6.3.1.1.2-3, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all even subbands with increasing order of subband number for the M-th reported CRI, from left to right as in Table 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number for the M-th reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number for the M-th reported CRI, as in Table 6.3.1.1.2-3, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all odd subbands with increasing order of subband number for the M-th reported CRI, from left to right as in Table 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number for the M-th reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. |  |

If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to.

Table 6.3.1.1.2-12: Mapping order of CSI reports to UCI bit sequence ,
without two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 as in Table 6.3.1.1.2-7/7A/7B/8/8B |
|  | CSI report #2 as in Table 6.3.1.1.2-7/7A/7B/8/8B |
|  | … |
|  | CSI report #n as in Table 6.3.1.1.2-7/7A/7B/8/8B |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, all CSI sub-reports within the CSI report #i are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the length of UCI bit sequence  is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3.

Table 6.3.1.1.2-13: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 if CSI report #1 is not of two parts, or CSI report #1, CSI part 1, if CSI report #1 is of two parts, as in Table 6.3.1.1.2-7/7A/7B/8/8B/9/9A/9B/9C |
|  | CSI report #2 if CSI report #2 is not of two parts, or CSI report #2, CSI part 1, if CSI report #2 is of two parts, as in Table 6.3.1.1.2-7/7A/7B/8/8B/9/9A/9B/9C |
|  | … |
|  | CSI report #n if CSI report #n is not of two parts, or CSI report #n, CSI part 1, if CSI report #n is of two parts, as in Table 6.3.1.1.2-7/7A/7B/8/8B/9/9A/9B/9C |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, either all CSI sub-reports not of two parts or CSI part 1 of all CSI sub-reports of two parts, are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-13 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.1.1.2-14: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B/10C if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B/10C if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B/10C if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B/11C/11D if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B/11C/11D if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B/11C/11D if CSI part 2 exists for CSI report #n |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}: - CSI part 2 widebands of all CSI sub-reports are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number; - CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-14 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

        - 6.3.1.1.3	HARQ-ACK/SR and CSI

If none of the CSI reports for transmission on a PUCCH is of two parts, the UCI bit sequence  is generated according to the following, where :

-	if there is HARQ-ACK for transmission on the PUCCH, the HARQ-ACK bits are mapped to the UCI bit sequence , where  for , the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213], and  is number of HARQ-ACK bits; if there is no HARQ-ACK for transmission on the PUCCH, set ;

-	if there is SR for transmission on the PUCCH, set ai=oi-OACKSR for , where the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR for transmission on the PUCCH, set ;

-	the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with , where  is the number of CSI bits.

If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and , according to the following, where  and :

-	if there is HARQ-ACK for transmission on the PUCCH, the HARQ-ACK bits are mapped to the UCI bit sequence , where  for , the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213], and  is number of HARQ-ACK bits; if there is no HARQ-ACK for transmission on the PUCCH, set ;

-	if there is SR for transmission on the PUCCH, set ai=oi-OACKSR for , where the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR for transmission on the PUCCH, set ;

-	the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence  starting with , where  is the number of CSI bits in CSI part 1 of all CSI reports;

-	the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence  starting with , where  is the number of CSI bits in CSI part 2 of all CSI reports. If the length of UCI bit sequence  is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3.

        - 6.3.1.1.4	UCI with different priority indexes

If uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1, and SR associated with priority index 1 if any are transmitted on a PUCCH, two UCI bit sequences are generated, a0(1), a1(1), a2(1), a3(1), …,aA(1)-11 and a0(2), a1(2), a2(2), a3(2), …,aA(2)-12, according to the following, where A(1)=OACK-HP+OSR-HP and A(2)=OACK-LP:

-	the HARQ-ACK bits associated with priority index 1 are mapped to the UCI bit sequence a0(1), a1(1), a2(1), a3(1), …,aOACK-HP-11, where ai(1)=oiACK-HP  for i=0,1,…,OACK-HP-1, the HARQ-ACK bit sequence o0ACK-HP, o1ACK-HP, …, oOACK-HP-1ACK-HP is given by Clause 9.1 of [5, TS 38.213], and OACK-HP is the number of HARQ-ACK bits associated with priority index 1;

-	if there is SR associated with priority index 1 for transmission on the PUCCH, set ai(1)=oi-OACK-HPSR-HP for  i=OACK-HP,OACK-HP+1,…,OACK-HP+OSR-HP-1, where the SR bit sequence o0SR-HP, o1SR-HP, …,oOSR-HP-1SR-HP  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR associated with priority index 1 for transmission on the PUCCH, set OSR-HP=0;

-	the HARQ-ACK bits associated with priority index 0 are mapped to the UCI bit sequence a0(2), a1(2), a2(2), a3(2), …,aOACK-LP-12, where ai(2)=oiACK-LP  for i=0,1,…,OACK-LP-1, the HARQ-ACK bit sequence o0ACK-LP, o1ACK-LP, …, oOACK-LP-1ACK-LP is given by Clause 9.1 of [5, TS 38.213], and OACK-LP is the number of HARQ-ACK bits associated with priority index 0.

      - 6.3.1.2	Code block segmentation and CRC attachment

The UCI bit sequence from clause 6.3.1.1 is denoted by , where  is the payload size. The procedure in Clause 6.3.1.2.1 applies for  and the procedure in Clause 6.3.1.2.2 applies for .

        - 6.3.1.2.1	UCI encoded by Polar code

If the payload size , code block segmentation and CRC attachment is performed according to Clause 5.2.1. If ( and ) or if , ; otherwise , where  is the rate matching output sequence length as given in Clauses 6.3.1.4.1 and 6.3.1.4.3.

If , the parity bits  in Clause 5.2.1 are computed by setting  to 6 bits and using the generator polynomial  in Clause 5.1, resulting in the sequence  where  is the code block number and  is the number of bits for code block number .

If , the parity bits  in Clause 5.2.1 are computed by setting  to 11 bits and using the generator polynomial  in Clause 5.1, resulting in the sequence  where  is the code block number and  is the number of bits for code block number .

        - 6.3.1.2.2	UCI encoded by channel coding of small block lengths

If the payload size , CRC bits are not attached.

The output bit sequence is denoted by , where  for  and .

      - 6.3.1.3	Channel coding of UCI

        - 6.3.1.3.1	UCI encoded by Polar code

Information bits are delivered to the channel coding block. They are denoted by  , where  is the code block number, and  is the number of bits in code block number . The total number of code blocks is denoted by  and each code block is individually encoded by the following:

If , the information bits are encoded via Polar coding according to Clause 5.3.1, by setting , , ,  if  and  if , where  is the rate matching output sequence length as given in Clauses 6.3.1.4.1 and 6.3.1.4.3.

If , the information bits are encoded via Polar coding according to Clause 5.3.1, by setting , , , and .

After encoding the bits are denoted by , where  is the number of coded bits in code block number .

        - 6.3.1.3.2	UCI encoded by channel coding of small block lengths

Information bits are delivered to the channel coding block. They are denoted by , where  is the number of bits.

The information bits are encoded according to Clause 5.3.3.

After encoding the bits are denoted by , where  is the number of coded bits.

      - 6.3.1.4	Rate matching

For PUCCH formats 2/3/4, the total rate matching output sequence length  is given by Table 6.3.1.4-1, where  , , and  are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively; ,  and NPRBPUCCH,4 are the number of PRBs that are determined by the UE for PUCCH formats 2/3/4 transmission respectively according to Clause 9.2 of [5, TS38.213]; and NSFPUCCH,2, NSFPUCCH,3, and  are the spreading factors for PUCCH format 2, PUCCH format 3, and PUCCH format 4, respectively.

Table 6.3.1.4-1: Total rate matching output sequence length

| PUCCH format | Modulation order |  |
| --- | --- | --- |
|  | QPSK | π/2-BPSK |
| PUCCH format 2 | 16∙Nsymb,UCIPUCCH,2∙NPRBPUCCH,2/NSFPUCCH,2 | N/A |
| PUCCH format 3 | 24∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3/NSFPUCCH,3 | 12∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3/NSFPUCCH,3 |
| PUCCH format 4 | 24∙Nsymb,UCIPUCCH,4∙NPRBPUCCH,4NSFPUCCH,4 | 12∙Nsymb,UCIPUCCH,4∙NPRBPUCCH,4NSFPUCCH,4 |

        - 6.3.1.4.1	UCI encoded by Polar code

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

Table 6.3.1.4.1-1: Rate matching output sequence length

| UCI(s) for transmission on a PUCCH | UCI for encoding | Value of |
| --- | --- | --- |
| HARQ-ACK | HARQ-ACK |  |
| HARQ-ACK, SR | HARQ-ACK, SR |  |
| CSI (CSI not of two parts) | CSI |  |
| HARQ-ACK, CSI (CSI not of two parts) | HARQ-ACK, CSI |  |
| HARQ-ACK, SR, CSI (CSI not of two parts) | HARQ-ACK, SR, CSI |  |
| CSI (CSI of two parts) | CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, CSI (CSI of two parts) | HARQ-ACK, CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, SR, CSI (CSI of two parts) | HARQ-ACK, SR, CSI part 1 |  |
|  | CSI part 2 |  |

Rate matching is performed according to Clause 5.4.1 by setting  and the rate matching output sequence length to , where  is the number of code blocks for UCI determined according to Clause 6.3.1.2.1 and the value of  is given by Table 6.3.1.4.1-1:

-	 is the number of bits for HARQ-ACK for transmission on the current PUCCH;

-	 is the number of bits for SR for transmission on the current PUCCH;

-	 is the number of bits for CSI part 1 for transmission on the current PUCCH;

-	 is the number of bits for CSI part 2 for transmission on the current PUCCH;

-	if ,  ; otherwise,  is the number of CRC bits determined according to Clause 6.3.1.2.1, where  equals  for "CSI (CSI of two parts)", equals  for "HARQ-ACK, CSI (CSI of two parts)", and equals  for "HARQ-ACK, SR, CSI (CSI of two parts)" respectively in Table 6.3.1.4.1-1;

-	 is the configured maximum PUCCH coding rate;

-	 is given by Table 6.3.1.4-1.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

        - 6.3.1.4.2	UCI encoded by channel coding of small block lengths

The input bit sequence to rate matching is .

The value of  is determined according to Table 6.3.1.4.1-1 by setting .

Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length .

The output bit sequence after rate matching is denoted as .

        - 6.3.1.4.3	UCI with different priority indexes encoded by Polar code

The following procedure in this clause 6.3.1.4.3 applies if uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and SR associated with priority index 1 if any are transmitted on a PUCCH.

The input bit sequence to rate matching is dr0,dr1,dr2,dr3,…,dr(Nr-1) where r is the code block number, and Nr is the number of coded bits in code block number r.

Table 6.3.1.4.3-1: Rate matching output sequence length EUCI for UCIs with different priority indexes

| UCIs for transmission on a PUCCH | UCI for encoding | Value of EUCI |
| --- | --- | --- |
| HARQ-ACK of priority index 1, HARQ-ACK of priority index 0 | HARQ-ACK of priority index 1 | EUCI=minEtot,OACK-HP+L/RUCImax-HP/Qm∙Qm |
|  | HARQ-ACK of priority index 0 | EUCI=Etot-minEtot,OACK-HP+L/RUCImax-HP/Qm∙Qm |
| HARQ-ACK of priority index 1, SR of priority index 1, HARQ-ACK of priority index 0 | HARQ-ACK of priority index 1, SR of priority index 1 | EUCI=minEtot,OACK-HP+OSR-HP+L/RUCImax-HP/Qm∙Qm |
|  | HARQ-ACK of priority index 0 | EUCI=Etot-minEtot,OACK-HP+OSR-HP+L/RUCImax-HP/Qm∙Qm |

Rate matching is performed according to Clause 5.4.1 by setting IBIL=1 and the rate matching output sequence length to Er=EUCICUCI, where CUCI is the number of code blocks for UCI determined according to Clause 6.3.1.2.1 and the value of EUCI is given by Table 6.3.1.4.3-1:

-	OACK-HP is the number of bits for HARQ-ACK associated with priority index 1 for transmission on the current PUCCH;

-	OSR-HP is the number of bits for SR associated with priority index 1 for transmission on the current PUCCH;

-	if A≥360, L=11; otherwise, L is the number of CRC bits determined according to clause 6.3.1.2.1, where A equals OACK-HP for the case of "HARQ-ACK of priority index 1, HARQ-ACK of priority index 0", and equals OACK-HP+OSR-HP for the case of "HARQ-ACK of priority index 1, SR of priority index 1, HARQ-ACK of priority index 0" respectively in Table 6.3.1.4.3-1;

-	RUCImax-HP is the configured maximum PUCCH coding rate of priority index 1;

-	Etot is given by Table 6.3.1.4-1.

The output bit sequence after rate matching is denoted as fr0,fr1,fr2,…,fr(Er-1) where Er is the length of rate matching output sequence in code block number r.

        - 6.3.1.4.4	UCI with different priority indexes encoded by channel coding of small block lengths

The following procedure in this clause 6.3.1.4.4 applies if uci-MuxWithDiffPrio is configured, and HARQ-ACK bits associated with priority index 0, HARQ-ACK bits associated with priority index 1 and SR associated with priority index 1 if any are transmitted on a PUCCH.

The input bit sequence to rate matching is d0,d1,d2,…,dN-1.

The value of EUCI is determined according to Table 6.3.1.4.3-1 by setting L=0.

Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length E=EUCI.

The output bit sequence after rate matching is denoted as f0,f1,f2,…,fE-1.

      - 6.3.1.5	Code block concatenation

The input bit sequence for the code block concatenation block are the sequences, for  and where  is the number of rate matched bits for the -th code block.

Code block concatenation is performed according to Clause 5.5.

The bits after code block concatenation are denoted by, where  with the values of  and  given in Clauses 6.3.1.4.1 and 6.3.1.4.3. Let  be the total number of coded bits for transmission and . Set  for .

      - 6.3.1.6	Multiplexing of coded UCI bits to PUCCH

If CSI of two parts or UCIs with different priority indexes are transmitted on a PUCCH, the coded bits corresponding to UCI bit sequence  is denoted by and the coded bits corresponding to UCI bit sequence  is denoted by .

For PUCCH format 2 when uci-MuxWithDiffPrio is configured, the coded bit sequence g0,g1,g2,g3,…,gG-1 is generated for UCIs with different priority indexes by setting gi=gi(1) for i=0,1,…,G1-1, and setting gi=gi-G1(2) for i=G1,G1+1,…,G1+G2-1.

For PUCCH format 3/4, the coded bit sequence , where , is generated according to the following.

Table 6.3.1.6-1: PUCCH DMRS and UCI symbols

| PUCCH duration (symbols) | PUCCH DMRS symbol indices | Number of UCI symbol indices sets | 1st UCI symbol indices set | 2nd UCI symbol indices set | 3rd UCI symbol indices set |
| --- | --- | --- | --- | --- | --- |
| 4 | {1} | 2 | {0,2} | {3} | - |
| 4 | {0,2} | 1 | {1,3} | - | - |
| 5 | {0, 3} | 1 | {1, 2, 4} | - | - |
| 6 | {1, 4} | 1 | {0, 2, 3, 5} | - | - |
| 7 | {1, 4} | 2 | {0, 2, 3, 5} | {6} | - |
| 8 | {1, 5} | 2 | {0, 2, 4, 6} | {3, 7} | - |
| 9 | {1, 6} | 2 | {0, 2, 5, 7} | {3, 4, 8} | - |
| 10 | {2, 7} | 2 | {1, 3, 6, 8} | {0, 4, 5, 9} | - |
| 10 | {1, 3, 6, 8} | 1 | {0,2,4,5,7,9} | - | - |
| 11 | {2, 7} | 3 | {1,3,6,8} | {0,4,5,9} | {10} |
| 11 | {1,3,6,9} | 1 | {0,2,4,5,7,8,10} | - | - |
| 12 | {2, 8} | 3 | {1,3,7,9} | {0,4,6,10} | {5, 11} |
| 12 | {1,4,7,10} | 1 | {0,2,3,5,6,8,9,11} | - | - |
| 13 | {2, 9} | 3 | {1,3,8,10} | {0,4,7,11} | {5,6,12} |
| 13 | {1,4,7,11} | 2 | {0,2,3,5,6,8,10,12} | {9} | - |
| 14 | {3, 10} | 3 | {2,4,9,11} | {1,5,8,12} | {0,6,7,13} |
| 14 | {1,5,8,12} | 2 | {0,2,4,6,7,9,11,13} | {3, 10} | - |

Denote  as UCI OFDM symbol index. Denote  as the number of elements in UCI symbol indices set  for , where  and  are given by Table 6.3.1.6-1 according to the PUCCH duration and the PUCCH DMRS configuration. Denote  as the number of OFDM symbols carrying UCI in the PUCCH. Denote  as the modulation order of the PUCCH.

For PUCCH formats 3/4, set NUCIsymbol=12⋅NPRBPUCCH,s/NSFPUCCH,s , where NPRBPUCCH,s is the number of PRBs that is determined by the UE for the corresponding PUCCH format transmission according to Clause 9.2 of [5, TS 38.213], and NSFPUCCH,s is the spreading factor for the corresponding PUCCH format [4, TS 38.211], where s∈3,4.

Find the smallest such that .

Set ;

Set ;

Set ;

Set ;

for  to

if

for  to

for  to

;

;

end for

end for

elseif

if

;

else

;

end if

;

for  to

for  to

;

;

end for

end for

for  to

for  to

;

;

end for

end for

else

for  to

for  to

;

;

end for

end for

end if

end for

Set

for  to

for  to

for  to

;

;

end for

end for

end for

    - 6.3.2	Uplink control information on PUSCH

The following clauses 6.3.2.2, 6.3.2.3, and 6.3.2.5 apply regardless of whether the higher layer parameter uci-MuxWithDiffPrio is configured or not. The following clauses 6.3.2.1, 6.3.2.4, and 6.3.2.6 apply by assuming uci-MuxWithDiffPrio is not configured, or uci-MuxWithDiffPrio is configured and the UCIs for transmission on a PUSCH are of the same priority index, unless stated otherwise.

If the UE is configured with a PUCCH-SCell, uci-MuxWithDiffPrio is replaced by uci-MuxWithDiffPrioSecondaryPUCCHgroup for the secondary PUCCH group in this clause.

      - 6.3.2.1	UCI bit sequence generation

< Unchanged parts are omitted >

        - 6.3.2.1.2	CSI

If cqi-BitsPerSubband is configured, this Clause 6.3.2.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the bitwidth of a CSI field of the CSI sub-report is determined following the procedure in this clause 6.3.2.1.2 by taking configurations in CSI-ReportSubConfig when applicable. If CSI-ReportSubConfig configures a list of CSI-RS resource IDs, for the determination of the bitwidth of a CRI field, the value of KsCSI-RS is the number of CSI-RS resources configured in the corresponding CSI-ReportSubConfig.

The bitwidth for PMI of codebookType=typeI-SinglePanel, codebookType=typeI-SinglePanel-r19, codebookType=typeI-MultiPanel and codebookType=typeI-MultiPanel-r19 is specified in Clause 6.3.1.1.2.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel, codebookType=typeI-SinglePanel-r19, codebookType=typeI-MultiPanel and codebookType=typeI-MultiPanel-r19 is specified in Clause 6.3.1.1.2.

The bitwidth for PMI/RI/LI/CQI/CRI with 1 CSI-RS port is specified in Clause 6.3.1.1.2.

The bitwidth for PMI of codebookType=typeII is provided in Tables 6.3.2.1.2-1, where the values of , , , , , , and  are given by Clause 5.2.2.2.3 in [6, TS 38.214].

Table 6.3.2.1.2-1: PMI of codebookType= typeII

|  | Information fields for wideband PMI |  |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-r16 and codebookType=etypeII-r19 is provided in Tables 6.3.2.1.2-1A, where the values of (N1, N2), (O1, O2), L, KNZ, N3, and Mll=1,…, υ are given by Clause 5.2.2.2.5 and 5.2.2.2.5a in [6, TS 38.214].

Table 6.3.2.1.2-1A: PMI of codebookType= typeII-r16 or codebookType=etypeII-r19

|  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | i1,1 |  |  |  | i1,2 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
| Rank=1 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 | i1,5 |  | i1,6,1 |  | i1,6,2 | i1,6,3 |  | i1,6,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A |  | N/A | N/A | N/A |  | log2N3-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 |  | N/A | N/A | N/A |  | log2N3-1M2-1 |  | log2N3-1M2-1 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 | N/A | N/A |  | log2N3-1M3-1 |  | log2N3-1M3-1 | log2N3-1M3-1 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 | 4 | N/A |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | log2N3-1M4-1 |  | 3(KNZ-4) | 4(KNZ-4) |  | 8LM4 |
| Rank=1 N3>19 | 4 | N/A |  | N/A | N/A | log2(2M1) |  | log22M1-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | 2LM1 |
| Rank=2 N3>19 | 4 | 4 |  | N/A | N/A | log2(2M2) |  | log22M2-1M2-1 |  | log22M2-1M2-1 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | 4LM2 |
| Rank=3 N3>19 | 4 | 4 |  | 4 | N/A | log2(2M3) |  | log22M3-1M3-1 |  | log22M3-1M3-1 | log22M3-1M3-1 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 6LM3 |
| Rank=4 N3>19 | 4 | 4 |  | 4 | 4 | log2(2M4) |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | log22M4-1M4-1 |  | 3(KNZ-4) | 4(KNZ-4) |  | 8LM4 |
| Note: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.5 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-CJT is provided in Tables 6.3.2.1.2-1B, where the values of (N1, N2), (O1, O2), O3, KNZ, N3, N0, Ln, σ(n) and Mll=1,…, υ are given by Clause 5.2.2.2.8 in [6, TS 38.214].

Table 6.3.2.1.2-1B: PMI of codebookType= typeII-CJT

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,1 |  | i1,2 |  | i1,8,1 | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| Rank=1 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2KNZ | N/A |  | N/A |  | N/A |
| Rank=2 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | N/A |  | N/A |
| Rank=3 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | N/A |
| Rank=4 N3≤19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |
| Rank=1 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2KNZ | N/A |  | N/A |  | N/A |
| Rank=2 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | N/A |  | N/A |
| Rank=3 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | N/A |
| Rank=4 N3>19 | N0log2(O1O2) |  | n=1N0log2N1N2Lσ(n) |  | log2(n=1N02Lσ(n)) | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |  | log2(n=1N02Lσ(n)) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 |  | i1,5 |  | i1,6,1 | i1,6,2 |
| Rank=1 N3≤19 | 4 | N/A |  | N/A | N/A |  | N/A |  | log2N3-1M1-1 | N/A |
| Rank=2 N3≤19 | 4 | 4 |  | N/A | N/A |  | N/A |  | log2N3-1M2-1 | log2N3-1M2-1 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 | N/A |  | N/A |  | log2N3-1M3-1 | log2N3-1M3-1 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 | 4 |  | N/A |  | log2N3-1M4-1 | log2N3-1M4-1 |
| Rank=1 N3>19 | 4 | N/A |  | N/A | N/A |  | log2(2M1) |  | log22M1-1M1-1 | N/A |
| Rank=2 N3>19 | 4 | 4 |  | N/A | N/A |  | log2(2M2) |  | log22M2-1M2-1 | log22M2-1M2-1 |
| Rank=3 N3>19 | 4 | 4 |  | 4 | N/A |  | log2(2M3) |  | log22M3-1M3-1 | log22M3-1M3-1 |
| Rank=4 N3>19 | 4 | 4 |  | 4 | 4 |  | log2(2M4) |  | log22M4-1M4-1 | log22M4-1M4-1 |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |
|  | i1,6,3 |  |  | i1,6,4 | i1,9 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=1N0Lσ(n)M1 |
| Rank=2 N3≤19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=1N0Lσ(n)M2 |
| Rank=3 N3≤19 | log2N3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=1N0Lσ(n)M3 |
| Rank=4 N3≤19 | log2N3-1M4-1 |  |  | log2N3-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=1N0Lσ(n)M4 |
| Rank=1 N3>19 | N/A |  |  | N/A | N0-1log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-1) |  | 4(KNZ-1) | 2∙n=1N0Lσ(n)M1 |
| Rank=2 N3>19 | N/A |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-2) |  | 4(KNZ-2) | 4∙n=1N0Lσ(n)M2 |
| Rank=3 N3>19 | log22M3-1M3-1 |  |  | N/A | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-3) |  | 4(KNZ-3) | 6∙n=1N0Lσ(n)M3 |
| Rank=4 N3>19 | log22M4-1M4-1 |  |  | log22M4-1M4-1 | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3(KNZ-4) |  | 4(KNZ-4) | 8∙n=1N0Lσ(n)M4 |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2n=1N0Lσ(n)Mυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.8 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler and codebookType=typeII-Doppler-r19 is provided in Tables 6.3.2.1.2-1C, where the values of (N1, N2), (O1, O2), L, KNZ, N3, N4, Q and Mll=1,…, υ are given by Clause 5.2.2.2.10 and 5.2.2.2.11a in [6, TS 38.214].

Table 6.3.2.1.2-1C: PMI of codebookType=typeII-Doppler or codebookType=typeII-Doppler-r19

|  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | i1,1 |  |  |  | i1,2 |  |  |  | i1,8,1 |  |  | i1,8,2 |  |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3≤19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
| Rank=1 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2KNZ |  |  | N/A |  |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | N/A |  |
| Rank=4 N3>19 |  | log2(O1O2) |  |  |  | log2N1N2L |  |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |  | log2(2LQ) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | i1,5 |  | i1,6,1 |  |  | i1,6,2 |  |  | i1,6,3 |  |  | i1,6,4 |  |  |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A |  | N/A |  | log2N3-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A |  | N/A |  | log2N3-1M2-1 |  |  | log2N3-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A |  | N/A |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | log2N3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 |  | N/A |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  | log2N3-1M4-1 |  |  |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A |  | log2(2M1) |  | log22M1-1M1-1 |  |  | N/A |  |  | N/A |  |  | N/A |  |  |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A |  | log2(2M2) |  | log22M2-1M2-1 |  |  | log22M2-1M2-1 |  |  | N/A |  |  | N/A |  |  |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A |  | log2(2M3) |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | log22M3-1M3-1 |  |  | N/A |  |  |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 |  | log2(2M4) |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  | log22M4-1M4-1 |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i1,10,1 |  |  | i1,10,2 |  |  |  | i1,10,3 |  |  | i1,10,4 |  |  | {i2,4,l}l=1,…,υ |  |  | {i2,5,l}l=1,…,υ |  |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3≤19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| Rank=1 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  |  | NA |  |  | NA |  |  | 3(KNZ-1) |  |  | 4(KNZ-1) |  |  | 2LM1Q |
| Rank=2 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | NA |  |  | NA |  |  | 3(KNZ-2) |  |  | 4(KNZ-2) |  |  | 4LM2Q |
| Rank=3 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | NA |  |  | 3(KNZ-3) |  |  | 4(KNZ-3) |  |  | 6LM3Q |
| Rank=4 N3>19 | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | log2N4-1 if N4>2 and Q=2, NA otherwise |  |  | 3(KNZ-4) |  |  | 4(KNZ-4) |  |  | 8LM4Q |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυQ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,τ(3), kl,i,f,τ(2), and cl,i,f,τ, respectively), where KlNZ as defined in Clause 5.2.2.2.10 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType= typeII-PortSelection is provided in Tables 6.3.2.1.2-2, where the values of , , , , , , and  are given by Clause 5.2.2.2.4 in [6, TS 38.214].

Table 6.3.2.1.2-2: PMI of codebookType= typeII-PortSelection

|  | Information fields for wideband PMI |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-PortSelection-r16 is provided in Tables 6.3.2.1.2-2A, where the values of PCSI-RS, d, L, KNZ, N3, and  Mll=1,…, υ are given by Clause 5.2.2.2.6 in [6, TS 38.214].

Table 6.3.2.1.2-2A: PMI of codebookType= typeII-PortSelection-r16

|  |  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | i1,1 |  |  |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |  |  |
| Rank=1 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3≤19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
| Rank=1 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2KNZ |  | N/A |  |  | N/A |  | N/A |  |  |
| Rank=2 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  | N/A |  |  |
| Rank=3 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  |
| Rank=4 N3>19 |  |  |  | log2PCSI-RS2d |  |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 | i1,5 | i1,6,1 |  | i1,6,2 |  | i1,6,3 | i1,6,4 |  | {i2,4,l}l=1,…,υ |  | {i2,5,l}l=1,…,υ | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A | N/A |  | N/A | N/A | log2N3-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 | N/A |  | N/A | N/A | log2N3-1M2-1 |  | log2N3-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 | 4 |  | N/A | N/A | log2N3-1M3-1 |  | log2N3-1M3-1 |  | log2N3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 | 4 |  | 4 | N/A | log2N3-1M4-1 |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |
| Rank=1 N3>19 | 4 | N/A | N/A |  | N/A | log2(2M1) | log22M1-1M1-1 |  | N/A |  | N/A | N/A |  | 3(KNZ-1) |  | 4(KNZ-1) | 2LM1 |
| Rank=2 N3>19 | 4 | 4 | N/A |  | N/A | log2(2M2) | log22M2-1M2-1 |  | log22M2-1M2-1 |  | N/A | N/A |  | 3(KNZ-2) |  | 4(KNZ-2) | 4LM2 |
| Rank=3 N3>19 | 4 | 4 | 4 |  | N/A | log2(2M3) | log22M3-1M3-1 |  | log22M3-1M3-1 |  | log22M3-1M3-1 | N/A |  | 3(KNZ-3) |  | 4(KNZ-3) | 6LM3 |
| Rank=4 N3>19 | 4 | 4 | 4 |  | 4 | log2(2M4) | log22M4-1M4-1 |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | 3(KNZ-4) |  | 4(KNZ-4) | 8LM4 |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.5 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-PortSelection-r17 and codebookType=typeII-FePortSelection-r19 is provided in Tables 6.3.2.1.2-2B, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.7 and 5.2.2.2.9a in [6, TS 38.214].

Table 6.3.2.1.2-2B: PMI of codebookType= typeII-PortSelection-r17 and typeII-FePortSelection-r19

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2B is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.7 in TS 38.214 [6] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the mapping order of CSI fields of one CSI sub-report is determined following the procedure in this clause 6.3.2.1.2, by replacing CSI report #n in the following Tables 6.3.2.1.2-3 and 6.3.2.1.2-4 with CSI sub-report #n, and taking only Tables 6.3.1.1.2-1/1A/1B/2/2A /3/4 for the determination of the bitwidth of a CSI field.

The bitwidth for PMI of codebookType=typeII-CJT-PortSelection is provided in Tables 6.3.2.1.2-2C, where the values of PCSI-RS, K1,n, KNZ, N3, N, M and σn are given by Clause 5.2.2.2.9 in [6, TS 38.214].

Table 6.3.2.1.2-2C: PMI of codebookType= typeII-CJT-PortSelection

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  |  | i1,6 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | N/A |  | N/A |  | N/A |
| Rank=2 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | N/A |  | N/A |
| Rank=3 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |  | N/A |
| Rank=4 | n=1N0log2PCSI-RS/2K1,σ(n)/2 |  |  | log2(N-1) if N > M=2, N/A otherwise |  | log2n=1N0K1,σ(n)M |  |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |  | log2n=1N0K1,σ(n)M |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | i1,9 |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=n=1N0K1,σ(n)M; n=1N0K1,σ(n)M otherwise |  |
| Rank=2 | 4 | 4 | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | N/A if KNZ=2n=1N0K1,σ(n)M; 2n=1N0K1,σ(n)M otherwise |  |
| Rank=3 | 4 | 4 | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 3∙n=1N0K1,σ(n)M |  |
| Rank=4 | 4 | 4 | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | (N0-1)log2O3N3 if Mode1 is configured, NA otherwise |  | 4∙n=1N0K1,σ(n)M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2C is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are n=1N0K1,σ(n)M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f,j(3), kl,i,f,j(2), and cl,i,f,j, respectively), where KlNZ as defined in Clause 5.2.2.2.9 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType=typeII-Doppler-PortSelection is provided in Tables 6.3.2.1.2-2D, where the values of PCSI-RS, K1, KNZ, N3, N and M are given by Clause 5.2.2.2.11 in [6, TS 38.214].

Table 6.3.2.1.2-2D: PMI of codebookType= typeII-Doppler-PortSelection

|  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i1,2 |  | i1,6 |  | i1,8,1 |  | i1,8,2 |  |  | i1,8,3 |  | i1,8,4 |
| Rank=1 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | N/A |  |  | N/A |  | N/A |
| Rank=2 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | N/A |  | N/A |
| Rank=3 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | N/A |
| Rank=4 | log2PCSI-RS/2K1/2 |  | log2(N-1) if N > M=2, N/A otherwise |  | log2(K1M) |  | log2(K1M) |  |  | log2(K1M) |  | log2(K1M) |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 |  | i2,3,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |  |
| Rank=1 | 4 | N/A |  | N/A |  | N/A |  | 3(KNZ-1) | 4(KNZ-1) |  | N/A if KNZ=K1M; K1M otherwise |  |
| Rank=2 | 4 | 4 |  | N/A |  | N/A |  | 3(KNZ-2) | 4(KNZ-2) |  | N/A if KNZ=2K1M; 2K1M otherwise |  |
| Rank=3 | 4 | 4 |  | 4 |  | N/A |  | 3(KNZ-3) | 4(KNZ-3) |  | 3K1M |  |
| Rank=4 | 4 | 4 |  | 4 |  | 4 |  | 3(KNZ-4) | 4(KNZ-4) |  | 4K1M |  |
| NOTE: the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-2D is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively, and the corresponding per layer bitwidths are K1M, 3KlNZ-1, and 4KlNZ-1, (i.e., 1, 3, and 4 bits for each respective indicator elements kl,i,f(3), kl,i,f(2), and cl,i,f, respectively), where KlNZ as defined in Clause 5.2.2.2.11 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ=l=1υKlNZ. |  |  |  |  |  |  |  |  |  |  |  |  |

For CSI on PUSCH, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence  starting with . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-7, are mapped to the UCI bit sequence  starting with .

The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting is provided in Table 6.3.1.1.2-8A. The mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8B. The mapping order of CSI fields of one report for TDCP reporting is provided in Table 6.3.2.1.2-3C. The mapping order of CSI fields of one report for SSBRI/RSRP reporting for L1/L2-triggered mobility is provided in Table 6.3.1.1.2-8C. The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP, SSBRI/RSRP, CRI/SINR, SSBRI/SINR reporting, TDCP reporting, delay offset reporting, frequency offset reporting, both delay offset and frequency offset reporting, and phase offset reporting.

Table 6.3.2.1.2-3: Mapping order of CSI fields of one CSI report, CSI part 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4/6, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/3C/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/3C/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/3C/4/5 or 6.3.2.1.2-8/8A/8B/9/9A, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
|  | Indicator of the N0 selected CSI-RS resources by a bitmap with NTRP bits, this field is present only if NTRP>1 and restrictedCMR-Selection is configured to OFF |
|  | Indicator of selected Ln value combination or αn value combination with bitwidth of log2(NL), this field is present only if NL>1 |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ as in Tables 6.3.2.1.2-8/8A/8B/9/9A, if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3A: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported |
|  | Rank Combination Indicator as in Table 6.3.1.1.2-3A, if reported |
|  | Wideband CQI for the first TB as in Table 6.3.1.1.2-3A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3A, if reported |
|  | CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; First CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Rank Indicator associated with the first CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI associated with the first CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 if reported; Subband differential CQI associated with the first CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Second CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with the second CRI as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with the second CRI for the first TB as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with the second CRI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3B: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; CRI as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Rank Combination Indicator as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Rank Indicator as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported; Zero padding bits OP, if needed |
|  | Wideband CQI for the first TB as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Wideband CQI for the first TB as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Subband differential CQI for the first TB with increasing order of subband number as in Table 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-3C: Mapping order of CSI fields of one CSI report for reportQuantity=tdcp

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Amplitude value for the configured delay values as in Table 6.3.2.1.2-10 based on the order from the first configured delay to the last configured delay |
|  | Phase value for the configured delay values as in Table 6.3.2.1.2-10 based on the order from the first configured delay to the last configured delay, if reported |

The number of zero padding bits OP in Table 6.3.1.1.2-9B is 0 for 1 CSI-RS port and OP=Nmax-Nreported(R) for more than 1 CSI-RS port, where:

-	Nmax=maxr∈SRankN(r) . SRank is the set of rank and rank combination values r that are allowed to be reported. Nr is obtained according to Tables 6.3.1.1.2-3A/3B for rank combination indicator and rank indicator respectively.

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3A for rank combination indicator and R is the reported rank combination

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3B for rank indicator and R is the reported rank

Table 6.3.2.1.2-3D: Mapping order of CSI fields of one CSI report configured with valueOfM, CSI part 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | Rank Indicator for the first CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Wideband CQI for the first TB for the first CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number for the first CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ for the first CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8D, if reported |
|  | … |
|  | Rank Indicator for the MR-th CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Wideband CQI for the first TB for the MR-th CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number for the MR-th CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ for the MR-th CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8D, if reported |
|  | … |
|  | First reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Rank Indicator for the first reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Zero padding bits OP,1 for the first reported CRI, if needed |
|  | Wideband CQI for the first TB for the first reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number for the first reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ for the first reported CRI as in Tables 6.3.2.1.2-8D, if reported |
|  | … |
|  | The (M-MR)-th reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Rank Indicator for the (M-MR)-th reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Zero padding bits OP,M-MR for the (M-MR)-th reported CRI, if needed |
|  | Wideband CQI for the first TB for the (M-MR)-th reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number for the (M-MR)-th reported CRI as in Tables 6.3.2.1.2-8C or 6.3.2.1.2-8D, if reported |
|  | Indicator of the total number of non-zero coefficients summed across all layers KNZ for the (M-MR)-th reported CRI as in Tables 6.3.2.1.2-8D, if reported |
| NOTE1: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. NOTE2: MR is set to 0 if higher layer parameter mrSelectedResources is not configured for the CSI report. |  |

The number of zero padding bits OP,k in Table 6.3.1.1.2-3D is 0 for 1 CSI-RS port and OP,k=Nmax-Nreported,k for more than 1 CSI-RS port, where:

-	Nmax=maxi∈SNi, where S is the set of CRIs corresponding to the Ks-MR resources and Ni is the payload size of RI field for the i-th CRI obtained according to Table 6.3.2.1.2-8C or 6.3.2.1.2-8D;

-	Nreported,k is the payload size of RI field for the k-th CRI obtained according to Table 6.3.2.1.2-8C or 6.3.2.1.2-8D.

Table 6.3.2.1.2-3E: Mapping order of CSI fields of one CSI report for reportQuantity=cjtc-Dd

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | The reference TRS resource set index as in Table 6.3.2.1.2-11. |
|  | The delay offset for each TRS resource set configured for the CSI report as in Table 6.3.2.1.2-11, except that indicated by reference TRS resource set, ordered from the lowest to highest CSI-RS resource set ID. |
|  | The inside/outside CP indicator for each TRS resource set configured for the CSI report as in Table 6.3.2.1.2-11, except that indicated by reference TRS resource set, ordered from the lowest to highest CSI-RS resource set ID. |

Table 6.3.2.1.2-3F: Mapping order of CSI fields of one CSI report for reportQuantity=cjtc-F

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | The reference TRS resource set index as in Table 6.3.2.1.2-12. |
|  | The frequency offset for each TRS resource set configured for the CSI report as in Table 6.3.2.1.2-12, except that indicated by reference TRS resource set, ordered from the lowest to highest CSI-RS resource set ID. |

Table 6.3.2.1.2-3G: Mapping order of CSI fields of one CSI report for reportQuantity=cjtc-Dd-F

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | The reference TRS resource set index for delay offset as in Table 6.3.2.1.2-11. |
|  | The reference TRS resource set index for frequency offset as in Table 6.3.2.1.2-12. |
|  | The delay offset for each TRS resource set configured for the CSI report as in Table 6.3.2.1.2-11, except that indicated by reference TRS resource set, ordered from the lowest to highest CSI-RS resource set ID. |
|  | The inside/outside CP indicator for each TRS resource set configured for the CSI report as in Table 6.3.2.1.2-11, except that indicated by reference TRS resource set, ordered from the lowest to highest CSI-RS resource set ID. |
|  | The frequency offset for each TRS resource set configured for the CSI report as in Table 6.3.2.1.2-12, except that indicated by reference TRS resource set, ordered from the lowest to highest CSI-RS resource set ID. |

Table 6.3.2.1.2-3H: Mapping order of CSI fields of one CSI report for reportQuantity=cjtc-P

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | The reference CSI-RS resource index as in Table 6.3.2.1.2-13. |
|  | Subband phase offsets for CSI-RS resource #0 as in Table 6.3.2.1.2-13 if subband reporting is configured, with increasing order of subband number; otherwise, wideband phase offset for CSI-RS resource #0 as in Table 6.3.2.1.2-13. |
|  | Subband phase offsets for CSI-RS resource #1 as in Table 6.3.2.1.2-13 if subband reporting is configured, with increasing order of subband number; otherwise, wideband phase offset for CSI-RS resource #1 as in Table 6.3.2.1.2-13. |
|  | … |
|  | Subband phase offsets for CSI-RS resource #(NTRP-1) as in Table 6.3.2.1.2-13 if subband reporting is configured, with increasing order of subband number; otherwise, wideband phase offset for CSI-RS resource #(NTRP-1) as in Table 6.3.2.1.2-13. |
| NOTE: The CSI-RS resources are ordered from the lowest to highest CSI-RS resource ID, except that indicated by reference CSI-RS resource index. NTRP is the number of CSI-RS resources configured for the CSI report. |  |

Table 6.3.2.1.2-3I: Mapping order of CSI fields of one report for CSI report configuration indicator/CRI/RSRP/Condition met indicator, or CSI report configuration indicator/SSBRI/RSRP/Condition met indicator

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CSI report configuration indicator as in Table 6.3.2.1.2-14, if reported |
|  | CRI or SSBRI #1 as in Table 6.3.2.1.2-14 |
|  | CRI or SSBRI #2 as in Table 6.3.2.1.2-14, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.2.1.2-14, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.2.1.2-14, if reported |
|  | RSRP #1 as in Table 6.3.2.1.2-14 |
|  | Differential RSRP #2 as in Table 6.3.2.1.2-14, if reported |
|  | Differential RSRP #3 as in Table 6.3.2.1.2-14, if reported |
|  | Differential RSRP #4 as in Table 6.3.2.1.2-14, if reported |
|  | Condition met indicator #1 in Table 6.3.2.1.2-14, if reported |
|  | Condition met indicator #2 in Table 6.3.2.1.2-14, if reported |
|  | Condition met indicator #3 in Table 6.3.2.1.2-14, if reported |
|  | Condition met indicator #4 in Table 6.3.2.1.2-14, if reported |
|  | RSRP or Differential RSRP as in Table 6.3.2.1.2-14 for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state, if reported |
|  | Zero padding bits OP, if needed |
| NOTE: The CSI report configuration indicator with value i indicates the (i+1)-th CSI report configuration among the CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR, which are ordered in ascending order of CSI-ReportConfigID. The condition met indicators are reported only when the UE is configured with higher layer parameter PresenceOfConditionMetIndicator. If the L1-RSRP of the reference signal associated with CRI or SSBRI #j is eventThreshold greater than the L1-RSRP of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state by at least eventInstanceCount times before a timer provided by eventDetectionTimeWindowLength for the reference signal associated with CRI or SSBRI #j is expired, the value of the condition met indicator #j is set to 1; otherwise set to 0. RSRP or Differential RSRP is reported for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state only when the UE is configured with higher layer parameter enabledCurrentBeamReport. RSRP is reported if the CSI report is associated with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event1’; otherwise differential RSRP is reported. |  |

The number of zero padding bits OP in Table 6.3.2.1.2-3I is 0 if there is only one CSI report configuration associated with the PUCCH carrying UEIRI; otherwise OP=Nmax-Nreported, where:

-	Nmax=maxi∈SNi, where S is the set of CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR, and Ni is the payload size of the report for the i-th CSI report configuration, obtained according to Table 6.3.2.1.2-3I prior to padding;

-	Nreported is the payload size of the current report, obtained according to Table 6.3.2.1.2-3I prior to padding.

Table 6.3.2.1.2-4: Mapping order of CSI fields of one CSI report, CSI part 2 wideband

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/3C/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/3C/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A or 6.3.2.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.2.1.2-4A: Mapping order of CSI fields of one CSI report, CSI part 2 wideband,
csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the first CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the second CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.2.1.2-4B: Mapping order of CSI fields of one CSI report, CSI part 2 wideband,
csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, if reported part 1 is associated with one CSI-RS resource pair, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second CSI-RS resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.2.1.2-4C: Mapping order of CSI fields of one CSI report configured with valueOfM, CSI part 2 wideband

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB for the first CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C, if present and reported |
|  | Layer Indicator for the first CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C, if reported |
|  | PMI wideband information fields X1 for the first CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the first CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the first CRI among the non-reported MR CRIs according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |
|  | … |
|  | Wideband CQI for the second TB for the MR-th CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C, if present and reported |
|  | Layer Indicator for the MR-th CRI among the non-reported MR CRIs as in Tables 6.3.2.1.2-8C, if reported |
|  | PMI wideband information fields X1 for the MR-th CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the MR-th CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the MR-th CRI among the non-reported MR CRIs according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |
|  | … |
|  | Wideband CQI for the second TB for the first reported CRI as in Tables 6.3.2.1.2-8C, if present and reported |
|  | Layer Indicator for the first reported CRI as in Tables 6.3.2.1.2-8C, if reported |
|  | PMI wideband information fields X1 for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the first reported CRI according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |
|  | … |
|  | Wideband CQI for the second TB for the (M-MR)-th reported CRI as in Tables 6.3.2.1.2-8C, if present and reported |
|  | Layer Indicator for the (M-MR)-th reported CRI as in Tables 6.3.2.1.2-8C, if reported |
|  | PMI wideband information fields X1 for the (M-MR)-th reported CRI, from left to right as in Tables 6.3.1.1.2-1, if reported |
|  | PMI wideband information fields X2 for the (M-MR)-th reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports for the (M-MR)-th reported CRI according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |
| NOTE: MR is set to 0 if higher layer parameter [mrSelectedResources] is not configured for the CSI report. |  |

Table 6.3.2.1.2-5: Mapping order of CSI fields of one CSI report, CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B/2/2A or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-5A: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-r16 or typeII-PortSelection-r16

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Tables 6.3.2.1.2-1A/2A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Tables 6.3.2.1.2-1A/2A: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |

Table 6.3.2.1.2-5B: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-PortSelection-r17 or typeII-Doppler-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-2B/2D, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2B/2D: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*K1M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2B/2D: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |

Table 6.3.2.1.2-5C: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
ReportMode= Mode 1

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.2.1.2-5D: Mapping order of CSI fields of one CSI report, CSI part 2 subband,
ReportMode= Mode 2

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.2.1.2-5E: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-CJT

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1B, if reported; |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,9, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2∙n=1N0Lσ(n)Mυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if present and if reported; |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if reported; |

Table 6.3.2.1.2-5F: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType= typeII-Doppler

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1C, if reported; The second time-domain wideband CQI as in Table 6.3.2.1.2-8B, if present and reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,10,l:l=1,…,υ, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυQ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f for N4=1 or Pril,i,f,j for N4>1 defined in clause 5.2.3 of TS 38.214 [6], if present and if reported; The second time-domain subband differential CQI of all even subbands with increasing order of subband number, as in Table 6.3.2.1.2-8B, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1C: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f for N4=1 or Pril,i,f,j for N4>1 defined in clause 5.2.3 of TS 38.214 [6], if reported; The second time-domain subband differential CQI of all odd subbands with increasing order of subband number, as in Table 6.3.2.1.2-8B, if present and if reported |

Table 6.3.2.1.2-5G: Mapping order of CSI fields of one CSI report, CSI part 2 of
codebookType=typeII-CJT-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-2C, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ}, i1,9 and ν*n=1N0K1,σ(n)M-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril, ij,f,j defined in clause 5.2.3 of TS 38.214 [6] where j is equivalent to n, if reported |

Table 6.3.2.1.2-5H: Mapping order of CSI fields of one CSI report containing Nnsub CSI sub-report(s), CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI of CSI sub-report #1 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C /4, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields X2 of CSI sub-report #1 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B /2/2A, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C /4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B /2/2A, or codebook index for 2 antenna ports of CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C /4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B /2/2A, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #1 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C /4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #1 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B /2/2A, or codebook index for 2 antenna ports of CSI sub-report #1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI of CSI sub-report #2 for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C /4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #2 of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B /2/2A, or codebook index for 2 antenna ports of CSI sub-report #2 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI of CSI sub-report #Nnsub for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/3C /4, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of CSI sub-report #Nnsub of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/1A/1B /2/2A, or codebook index for 2 antenna ports of CSI sub-report #Nnsub according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to '1' are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to '1' as subband 0. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nnsub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

Table 6.3.2.1.2-5I: Mapping order of CSI fields of one CSI report configured with valueOfM, CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number for the first CRI among the non-reported MR CRIs, as in Tables 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields X2 of all even subbands with increasing order of subband number for the first CRI among the non-reported MR CRIs, from left to right as in Table 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number for the first CRI among the non-reported MR CRIs, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number for the first CRI among the non-reported MR CRIs, as in Table 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all odd subbands with increasing order of subband number for the first CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number for the first CRI among the non-reported MR CRIs, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number for the MR-th CRI among the non-reported MR CRIs, as in Tables 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all even subbands with increasing order of subband number for the MR-th CRI among the non-reported MR CRIs, from left to right as in Table 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number for the MR-th CRI among the non-reported MR CRIs, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number for the MR-th CRI among the non-reported MR CRIs, as in Table 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all odd subbands with increasing order of subband number for the MR-th CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number for the MR-th CRI among the non-reported MR CRIs, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number for the first reported CRI, as in Tables 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all even subbands with increasing order of subband number for the first reported CRI, from left to right as in Table 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number for the first reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number for the first reported CRI, as in Table 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all odd subbands with increasing order of subband number for the first reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number for the first reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
|  | … |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number for the (M-MR)-th reported CRI, as in Tables 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all even subbands with increasing order of subband number for the (M-MR)-th reported CRI, from left to right as in Table 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number for the (M-MR)-th reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number for the (M-MR)-th reported CRI, as in Table 6.3.2.1.2-8C, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields X2 of all odd subbands with increasing order of subband number for the (M-MR)-th reported CRI, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number for the (M-MR)-th reported CRI, if pmi-FormatIndicator= subbandPMI and if reported |
| NOTE1: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. NOTE2: MR is set to 0 if higher layer parameter [mrSelectedResources] is not configured for the CSI report. |  |

Table 6.3.2.1.2-5J: Mapping order of CSI fields of one CSI report configured with valueOfM, CSI part 2 of codebookType=typeII-r16

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1 for the first CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.2.1.2-1A, if reported … PMI fields X1 for the MR-th CRI among the non-reported MR CRIs, from left to right as in Tables 6.3.2.1.2-1A, if reported PMI fields X1 for the first reported CRI, from left to right as in Tables 6.3.2.1.2-1A, if reported … PMI fields X1 for the (M-MR)-th reported CRI, from left to right as in Tables 6.3.2.1.2-1A, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2 for the first CRI among the non-reported MR CRIs, from left to right, as in Tables 6.3.2.1.2-1A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2 for the first CRI among the non-reported MR CRIs, from left to right, as in Tables 6.3.2.1.2-1A: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
|  | … |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2 for the MR-th CRI among the non-reported MR CRIs, from left to right, as in Tables 6.3.2.1.2-1A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2 for the MR-th CRI among the non-reported MR CRIs, from left to right, as in Tables 6.3.2.1.2-1A: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
|  | … |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2 for the first reported CRI, from left to right, as in Tables 6.3.2.1.2-1A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2 for the first reported CRI, from left to right, as in Tables 6.3.2.1.2-1A: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
|  | … |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2 for the (M-MR)-th reported CRI, from left to right, as in Tables 6.3.2.1.2-1A: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2LMυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2 for the (M-MR)-th reported CRI, from left to right, as in Tables 6.3.2.1.2-1A: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,i,f defined in clause 5.2.3 of TS 38.214 [6], if reported |
| NOTE: MR is set to 0 if higher layer parameter [mrSelectedResources] is not configured for the CSI report. |  |

Table 6.3.2.1.2-6: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI part 1 of CSI report #1 as in Table 6.3.2.1.2-3/3A/3B/3C/3D/3E/3F/3G/3H/3I or Table 6.3.1.1.2-8/8A/8B/8C |
|  | CSI part 1 of CSI report #2 as in Table 6.3.2.1.2-3/3A/3B/3C/3D/3E/3F/3G/3H/3I or Table 6.3.1.1.2-8/8A/8B/8C |
|  | … |
|  | CSI part 1 of CSI report #n as in Table 6.3.2.1.2-3/3A/3B/3C/3D/3E/3F/3G/3H/3I or Table 6.3.1.1.2-8/8A/8B/8C |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, CSI part 1 of all CSI sub-reports are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-6 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.2.1.2-7: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B/4C, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G/5J, if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B/4C, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G/5J, if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.2.1.2-4/4A/4B/4C, or CSI part 2 with group 0, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G/5J, if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D/5H/5I, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G/5J, if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D/5H/5I, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G/5J, if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.2.1.2-5/5C/5D/5H/5I, or CSI part 2 with group 1 and 2, as in Table 6.3.2.1.2-5A/5B/5E/5F/5G/5J, if CSI part 2 exists for CSI report #n |
| NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, - CSI part 2 wideband of all CSI sub-reports are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number; - CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId. |  |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-7 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

The bitwidth for RI/CQI of codebookType= typeII-r16, codebookType=typeII-PortSelection-r16 or codebookType= eTypeII-r19 is provided in Table 6.3.2.1.2-8.

Table 6.3.2.1.2-8: RI and CQI of codebookType=typeII-r16, typeII-PortSelection-r16 or codebookType= eTypeII-r19

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |
| NOTE: nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.5, 5.2.2.2.5a and 5.2.2.2.6 [6, TS 38.214], K0=2Lp1×N3Rβ, where p1, N3, R, and β are given by Clause 5.2.2.2.5, 5.2.2.2.5a and 5.2.2.2.6 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.5, 5.2.2.2.5a and 5.2.2.2.6 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1. |  |

nRI K0=2Lp1×N3Rβp1N3RβKNZKNZKNZ=1

The bitwidth for RI/CQI of codebookType= typeII-CJT is provided in Table 6.3.2.1.2-8A.

Table 6.3.2.1.2-8A: RI and CQI of codebookType= typeII-CJT

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers, and all CSI-RS resources if configured, KNZ | log22βM1Lmax if max allowed rank is 1; log222βM1Lmaxotherwise |
| NOTE: nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.8 TS 38.214 [6], Lmax is the maximum of n=1NTRPLn for all NL configured Ln combinations, where Lnn=1,…, NTRP, M1, and β are given by Clause 5.2.2.2.8 in TS 38.214 [6]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clause 5.2.2.2.8 TS 38.214 [6], with increasing order, where '0' is mapped to KNZ=1. |  |

nRI Lmaxn=1NTRPLnNLLn Lnn=1,…, NTRPM1βKNZKNZKNZ=1

The bitwidth for RI/CQI of codebookType= typeII-Doppler and codebookType= typeII-Doppler-r19 is provided in Table 6.3.2.1.2-8B.

Table 6.3.2.1.2-8B: RI and CQI of codebookType= typeII-Doppler or codebookType= typeII-Doppler-r19

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| The second time-domain wide-band CQI | 4 |
| The second time-domain subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |
| NOTE: nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.10 and 5.2.2.2.11a in [6, TS 38.214], K0=2Lp1×N3RβQ, where p1, N3, R, β and Q are given by Clause 5.2.2.2.10 and 5.2.2.2.11a in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clause 5.2.2.2.10 and 5.2.2.2.11a in [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1. |  |

nRI K0=2Lp1×N3RβQp1N3RβQKNZKNZKNZ=1

The bitwidth for RI/LI/CQI/CRI of a CSI report configured with valueOfM and codebookType=typeI-SinglePanel is provided in Table 6.3.2.1.2-8C.

Table 6.3.2.1.2-8C: RI, LI, CQI, and CRI of a CSI report configured with valueOfM and cocodebookType=typeI-SinglePanel

| Field | Bitwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator | min1,log2nRI | min2,log2nRI | log2nRI | log2nRI |
| Layer Indicator | log2υ | min2,log2υ | min2,log2υ | min2,log2υ |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 2 |
| CRI | log2KSCSI-RS-MR | log2KSCSI-RS-MR | log2KSCSI-RS-MR | log2KSCSI-RS-MR |
| NOTE: nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.1 [6, TS 38.214]. υ is the value of the rank. The value of KSCSI-RS is the number of CSI-RS resources in the corresponding resource set, MR is the number of CSI-RS resources that are configured to report their corresponding CSI (except CRI), and is set to 0 if higher layer parameter mrSelectedResources is not configured for the CSI report. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. |  |  |  |  |

The bitwidth for RI/CQI/CRI of a CSI report configured with valueOfM and codebookType= typeII-r16 is provided in Table 6.3.2.1.2-8D.

Table 6.3.2.1.2-8D: RI, CQI and CRI of a CSI report configured with valueOfM and codebookType=typeII-r16

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| CRI | log2KSCSI-RS-MR |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |
| NOTE: nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], KSCSI-RS is the number of CSI-RS resources in the corresponding resource set, MR is the number of CSI-RS resources that are configured to report their corresponding CSI (except CRI), and is set to 0 if higher layer parameter mrSelectedResources is not configured for the CSI report, K0=2Lp1×N3Rβ, where p1, N3, R, and β are given by Clause 5.2.2.2.5 and 5.2.2.2.6 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.5 and 5.2.2.2.6 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1. |  |

The bitwidth for RI/CQI of codebookType=typeII-PortSelection-r17, typeII-Doppler-PortSelection or codebookType= typeII-FePortSelection-r19 is provided in Table 6.3.2.1.2-9.

Table 6.3.2.1.2-9: RI and CQI of codebookType=typeII-PortSelection-r17, 
typeII-Doppler-PortSelection, or codebookType= typeII-FePortSelection-r19

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers KNZ | log2(K0) if max allowed rank is 1; log2(2K0) otherwise |
| NOTE: nRI is the number of allowed rank indicator values according to Clauses 5.2.2.2.7, 5.2.2.2.9a and Clauses 5.2.2.2.11 [6, TS 38.214], K0=K1Mβ, where K1, M, and β are given by Clause 5.2.2.2.7, 5.2.2.2.9a and Clauses 5.2.2.2.11 in [6, TS 38.214]. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clauses 5.2.2.2.7, 5.2.2.2.9a and Clauses 5.2.2.2.11 [6, TS 38.214], with increasing order, where '0' is mapped to KNZ=1. |  |

nRI K0=K1MβK1MβKNZKNZKNZ=1

The bitwidth for RI/CQI of codebookType=typeII-CJT-PortSelection is provided in Table 6.3.2.1.2-9A.

Table 6.3.2.1.2-9A: RI and CQI of codebookType= typeII-CJT-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator | min2,log2nRI |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the total number of non-zero coefficients summed across all layers, and all CSI-RS resources if configured, KNZ | log2βMK1,max if max allowed rank is 1; log22βMK1,max otherwise |
| NOTE: nRI is the number of allowed rank indicator values according to Clause 5.2.2.2.9 TS 38.214 [6], K1,max is the maximum of n=1NTRPK1, n for all NL configured αn combinations, where K1.nn=1,…, NTRP, M, and β are given by Clause 5.2.2.2.9 in TS 38.214 [6], and. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. The values of the KNZ indicator field are mapped to the allowed values of KNZ, according to Clause 5.2.2.2.9 TS 38.214 [6], with increasing order, where '0' is mapped to KNZ=1. |  |

nRI, K1,maxn=1NTRPK1, nNLαnK1.nn=1,…, NTRPMβKNZKNZKNZ=1

The bitwidth for reportQuantity=tdcp is provided in Table 6.3.2.1.2-10.

Table 6.3.2.1.2-10: Amplitude and phase values for reportQuantity=tdcp

| Field | Bitwidth |
| --- | --- |
| Amplitude value | 4 |
| Phase value | 4 |

The bitwidth for reportQuantity=cjtc-Dd is provided in Table 6.3.2.1.2-11.

Table 6.3.2.1.2-11: Reference TRS resource set index, delay offset and inside/outside indicator for reportQuantity=cjtc-Dd

| Field | Bitwidth |
| --- | --- |
| Reference TRS resource set index | log2NTRP, where NTRP is the number of TRS resource sets configured for the CSI report. |
| Delay offset | log2MD, where MD is configured by valueOfMD. |
| Inside/outside CP indicator | 1 |

The bitwidth for reportQuantity=cjtc-F is provided in Table 6.3.2.1.2-12.

Table 6.3.2.1.2-12: Reference TRS resource set index, and frequency offset for reportQuantity=cjtc-F

| Field | Bitwidth |
| --- | --- |
| Reference TRS resource set index | log2NTRP, where NTRP is the number of TRS resource sets configured for the CSI report. |
| Frequency offset | log2MFO, where MFO is configured by valueofMFO. |

The bitwidth for reportQuantity=cjtc-P is provided in Table 6.3.2.1.2-13.

Table 6.3.2.1.2-13: Reference CSI-RS resource set index, and phase offset for reportQuantity=cjtc-P

| Field | Bitwidth |
| --- | --- |
| Reference CSI-RS resource index | log2NTRP, where NTRP is the number of CSI-RS resources configured for the CSI report. |
| Wideband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |
| Subband Phase offset | log2MΦ, where MΦ is configured by valueOfMPhi. |

The bitwidth for CRI, SSBRI, RSRP, differential RSRP, CSI report configuration indicator and Condition met indicator are provided in Table 6.3.2.1.2-14.

Table 6.3.2.1.2-14: CRI, SSBRI, RSRP, CapabilityIndex, CSI report configuration indicator and Condition met indicator

| Field | Bitwidth |
| --- | --- |
| CRI | log2(KsCSI-RS) |
| SSBRI | log2(KsSSB) |
| RSRP | 7 |
| Differential RSRP | 4 |
| CSI report configuration indicator | log2(KsCSI_config) |
| Condition met indicator | 1 |
| NOTE: KsCSI-RS is the number of CSI-RS resources in the corresponding resource set, KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Index', KsCSI_config is the number of CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR. |  |

< Unchanged parts are omitted >

    - 7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

< Unchanged parts are omitted >

#### 7.3.1.1	DCI formats for scheduling of PUSCH

< Unchanged parts are omitted >

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

< Unchanged parts are omitted >

-	Precoding information and number of layers - number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} if multipanelSchemeSDM is configured, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 2 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 2 if multipanelSchemeSDM is configured, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 1 if multipanelSchemeSDM is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 1 if multipanelSchemeSDM is configured, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 2 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 2 if multipanelSchemeSDM is configured, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN}=1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM}=1 if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRank, maxRankSFN} = 1 if multipanelSchemeSFN is configured or max{maxRank, maxRankSDM} = 1 if multipanelSchemeSDM is configured, and the values of higher layer parameter codebookSubset;

-	7 bits according to Table 7.3.1.1.2-5B for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is disabled, maxRank = 8, and according to codebook1;

-	7 bits according to Table 7.3.1.1.2-5C for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is disabled, maxRank = 7, and according to codebook1;

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is disabled, maxRank = 4, 5 or 6, and according to maxRank;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports, if CodebookTypeUL= codebook1, transform precoder is enabled or maxRank = 1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRank;

-	8 bits according to Table 7.3.1.1.2-5F for 8 antenna ports, if CodebookTypeUL= codebook4, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports, if CodebookTypeUL= codebook4, transform precoder is disabled, maxRank = 2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports, if CodebookTypeUL= codebook4, transform precoder is enabled or maxRank = 1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	10 bits according to Table 7.3.1.1.2-5I for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	10 bits according to Table 7.3.1.1.2-5K for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRank;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRank;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRank = 2, 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is enabled or maxRank = 1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRank = 1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank;

-	5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRank = 1, 2, 3, or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRank;

-	8 or 9 bits according to Table 7.3.1.1.2-5Q for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	10 bits according to Table 7.3.1.1.2-5R for 8 antenna ports, if CodebookTypeUL=codebook2, transform precoder is disabled, maxRank = 5, 6, 7 or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	10 bits according to Table 7.3.1.1.2-5S for 8 antenna ports, if CodebookTypeUL=codebook3, transform precoder is disabled, maxRank = 5, 6, 7, or 8, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRank;

-	3 bits according to Table 7.3.1.1.2-5T for 3 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank;

-	2 bits according to Table 7.3.1.1.2-5U for 3 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports or 8 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 4, and at least one SRS resource with 8 antenna ports is configured in the SRS resource set with usage set to 'codebook', and an SRS resource with 4 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-2 is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel, and there are two SRS resource sets configured by srs-ResourceSetToAddModList and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_1.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Precoding information and number of layers field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Precoding information and number of layers field for the case with smaller bit width until the bit width of the Precoding information and number of layers field for the two cases are the same.

-	Second Precoding information - number of bits determined by the following:

-	0 bits if SRS resource set indicator field is not present;

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=2 if multipanelSchemeSFN is configured, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=1 if multipanelSchemeSFN is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRank=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=2 if multipanelSchemeSFN is configured, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN if multipanelSchemeSFN is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRank=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN=1 if multipanelSchemeSFN is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5V with the same number of layers indicated by Precoding information and number of layers field for 3 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRank;

-	2 bits according to Table 7.3.1.1.2-5U with the same number of layers indicated by Precoding information and number of layers field for 3 antenna ports, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSDM if multipanelSchemeSDM is configured, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSDM = 2 if multipanelSchemeSDM is configured, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankSDM if multipanelSchemeSDM is configured, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSDM = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankSDM if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, transform precoder is disabled, maxRankSDM = 2 if multipanelSchemeSDM is configured, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankSDM= 1 if multipanelSchemeSDM is configured, and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, maxRankSDM = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field, and an SRS resource with 2 antenna ports is indicated via Second SRS resource indicator field in the same SRS resource set, then Table 7.3.1.1.2-4B is used.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in the second SRS resource set with usage set to 'codebook' as defined in Table 7.3.1.1.2-36. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Second Precoding information field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Second Precoding information field for the case with smaller bit width until the bit width of the Second Precoding information field for the two cases are the same.

< Unchanged parts are omitted >

-	PTRS-DMRS association - number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxMIMO-Layers=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRank=1 and maxRankSFN=1, or if maxMIMO-Layers=1 and maxMIMO-LayersforSFN=1, or if maxRank=1 and maxRankSDM=1 when two PTRS ports are configured by maxNrofPorts-SDM, or if maxMIMO-Layers=1 and maxMIMO-LayersforSDM=1 when two PTRS ports are configured by maxNrofPorts-SDM;

-	1 or 2 or 4 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26A/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", and maxRank<=4 or maxMIMO-Layers<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or 4 or maxMIMO-Layers=3 or 4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=2 or maxMIMO-Layers=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRank<=3 or maxMIMO-Layers<=3, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25.

-	1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRank<=3 or maxMIMO-Layers<=3, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or maxMIMO-Layers=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank=2 or maxMIMO-Layers=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port and DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRank<=3 or maxMIMO-Layers<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B.

-	2 bits when two PTRS ports are configured by maxNrofPorts-SDM in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port(s) corresponding to the selected codeword according to Table 7.3.1.1.2-25B, where the selected codeword is the codeword with higher MCS for the initial PUSCH if the MCS indices of the two codewords are different for the initial PUSCH, or codeword 0 otherwise.

-	4 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is absent, maxRank>4 or maxMIMO-Layers>4, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-26A.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same.

-	Second PTRS-DMRS association

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank>2 or maxMIMO-Layers>2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured;

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank=3 or maxMIMO-Layers=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured;

-	1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRank<=3 or maxMIMO-Layers<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured;

-	0 bit otherwise.

Tables 7.3.1.1.2-25 and 7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

< Unchanged parts are omitted >

Table 7.3.1.1.2-5S: Precoding information and number of layers, for 8 antenna ports, if transform precoder is disabled, maxRank = 5, 6, 7, 8, CodebookTypeUL=codebook3, and ul-FullPowerTransmission is configured to fullpowerMode1

| Bit field mapped to index | maxRank = 5 | Bit field mapped to index | maxRank = 6 | Bit field mapped to index | maxRank = 7 | Bit field mapped to index | maxRank = 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 | 15 | 1 layer: TPMI=15 |
| 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 | 16 | 2 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 | 119 | 2 layers: TPMI=103 |
| 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 | 120 | 3 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 | 423 | 3 layers: TPMI=303 |
| 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 | 424 | 4 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 | 703 | 4 layers: TPMI=279 |
| 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 | 704 | 5 layers: TPMI=0 |
| … | … | … | … | … | … | … | … |
| 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 | 863 | 5 layers: TPMI=159 |
| 864 | 1 layer: TPMI=16 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 | 864 | 6 layers: TPMI=0 |
| 865 | 2 layers: TPMI=104 | … | … | … | … | … | … |
| 866 | 3 layers: TPMI=304 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 | 943 | 6 layers: TPMI=79 |
| 867-1023 | reserved | 944 | 1 layer: TPMI=16 | 944 | 7 layers: TPMI=0 | 944 | 7 layers: TPMI=0 |
|  |  | 945 | 2 layers: TPMI=104 | … | … | … | … |
|  |  | 946 | 3 layers: TPMI=304 | 975 | 7 layers: TPMI=31 | 975 | 7 layers: TPMI=31 |
|  |  | 944-1023 | reserved | 976 | 1 layer: TPMI=16 | 976 | 8 layers: TPMI=0 |
|  |  |  |  | 977 | 2 layers: TPMI=104 | … | … |
|  |  |  |  | 978 | 3 layers: TPMI=304 | 991 | 8 layers: TPMI=15 |
|  |  |  |  | 979-1023 | reserved | 992 | 1 layer: TPMI=16 |
|  |  |  |  |  |  | 993 | 2 layers: TPMI=104 |
|  |  |  |  |  |  | 994 | 3 layers: TPMI=304 |
|  |  |  |  |  |  | 995-1023 | reserved |

Table 7.3.1.1.2-5T: Precoding information and number of layers, for 3 antenna ports, if transform precoder is disabled, maxRank = 2 or 3, and ul-FullPowerTransmission is not configured or configured to fullpower

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 2 layers: TPMI=0 |
| 4 | 2 layers: TPMI=1 |
| 5 | 2 layers: TPMI=2 |
| 6 | 3 layers: TPMI=0 |
| 7 | reserved |

Table 7.3.1.1.2-5U: Precoding information and number of layers or Second Precoding information, for 3 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission is not configured or configured to fullpower, or if transform precoder is disabled, maxRank = 1, and ul-FullPowerTransmission is not configured or configured to fullpower

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | reserved |

Table 7.3.1.1.2-5V: Second precoding information, for 3 antenna ports, if transform precoder is disabled, maxRank = 2 or 3, and ul-FullPowerTransmission is not configured or configured to fullpower

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | reserved |
| 0 | 2 layers: TPMI=0 |
| 1 | 2 layers: TPMI=1 |
| 2 | 2 layers: TPMI=2 |
| 3 | reserved |
| 0 | 3 layers: TPMI=0 |
| 1-3 | reserved |

< Unchanged parts are omitted >

Table 7.3.1.1.2-26A: PTRS-DMRS association for UL PTRS ports 0 and 1, maxRank>4 or maxMIMO-Layers>4

| Value of 2 MSBs | DMRS port | Value of 2 LSBs | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

Table 7.3.1.1.2-26B: PTRS-DMRS association or Second PTRS-DMRS association for the actual UL PTRS port

| Value | DMRS port |
| --- | --- |
| 0 | 1st DMRS port which shares the actual PTRS port |
| 1 | 2nd DMRS port which shares the actual PTRS port |
| Note: - The actual PTRS port is PTRS port 0 if txConfig = codebook - The actual PTRS port is the PTRS port associated with more than one SRS resources indidcated by SRS resource indicator, if txConfig = nonCodeBook |  |

< Unchanged parts are omitted >

##### 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

< Unchanged parts are omitted >

-	Precoding information and number of layers - number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 2 if multipanelSchemeSFN is configured or max{maxRanDCI-0-2k, maxRankSDM-DCI-0-2} = 2 if multipanelSchemeSDM is configured, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 1 if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 2 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 2 if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2}=1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2}=1 if multipanelSchemeSDM is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports by replacing maxRank, maxRankSFN, maxRankSDM and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2, maxRankSDM-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSFN-DCI-0-2} = 1 if multipanelSchemeSFN is configured or max{maxRankDCI-0-2, maxRankSDM-DCI-0-2} = 1 if multipanelSchemeSDM is configured, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetDCI-0-2.

-	7 bits according to Table 7.3.1.1.2-5D for 8 antenna ports by replacing maxRank-n8 with maxRankDCI-0-2, if CodebookTypeUL=codebook1, transform precoder is disabled, maxRankDCI-0-2 =4, and according to maxRankDCI-0-2;

-	4, 6 or 7 bits according to Table 7.3.1.1.2-5E for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook1, transform precoder is enabled or maxRankDCI-0-2 =1, 2 or 3 if transform precoder is disabled, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5G for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRankDCI-0-2=2, 3 or 4, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to maxRankDCI-0-2;

-	3 bits according to Table 7.3.1.1.2-5H for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook4, transform precoder is enabled or maxRankDCI-0-2=1 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower.

-	5, 9 or 10 bits according to Table 7.3.1.1.2-5J for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	4, 7, 9 or 10 bits according to Table 7.3.1.1.2-5L for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to transform precoder and maxRankDCI-0-2;

-	6 or 7 or 8 bits according to Table 7.3.1.1.2-5M for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook4, transform precoder is disabled, maxRankDCI-0-2=2, 3 or 4, ul-FullPowerTransmission is configured to fullpowerMode1, and according to maxRankDCI-0-2;

-	4 bits according to Table 7.3.1.1.2-5N for 8 antenna ports, if CodebookTypeUL=codebook4, transform precoder is enabled or maxRankDCI-0-2=1 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1.

-	6, 9 or 10 bits according to Table 7.3.1.1.2-5O for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook2, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRankDCI-0-2;

-	5, 7, 9 or 10 bits according to Table 7.3.1.1.2-5P for 8 antenna ports by replacing maxRank with maxRankDCI-0-2, if CodebookTypeUL=codebook3, transform precoder is enabled or maxRankDCI-0-2 =1, 2, 3 or 4 if transform precoder is disabled, ul-FullPowerTransmission is configured to fullpowerMode1, and according to transform precoder and maxRankDCI-0-2;

-	3 bits according to Table 7.3.1.1.2-5T for 3 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5U for 3 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field if present, otherwise in an SRS resource set with usage set to 'codebook', and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in all SRS resource set(s) with usage set to 'codebook'. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Precoding information and number of layers field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Precoding information and number of layers field for the case with smaller bit width until the bit width of the Precoding information and number of layers field for the two cases are the same.

When the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs, and is provided sTx-2Panel, and there are two SRS resource sets configured by srs-ResourceSetToAddModListDCI-0-2 and associated with usage of value 'codebook' or 'nonCodeBook', the Precoding information and number of layers field is associated with the SRS resource set that is associated with the coresetPoolIndex value for the CORESET used for the PDCCH carrying the DCI format 0_2.

-	Second Precoding information - number of bits determined by the following:

-	0 bits if SRS resource set indicator field is not present;

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	3, 4, or 5 bits according to Table 7.3.1.1.2-2C with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-2D with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, the values of higher layer parameters maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=2 if multipanelSchemeSFN is configured, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-2E with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, maxRankDCI-0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetDCI-0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A with the same number of layers indicated by Precoding information and number of layers field for 4 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=1 if multipanelSchemeSFN is configured, and the value of higher layer parameter codebookSubsetDCI-0-2;

-	1 or 3 bits according to Table7.3.1.1.2-4B with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-4C with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, transform precoder is disabled, the maxRankDCI-0-2=2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2 if multipanelSchemeSFN is configured, and codebookSubsetDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5A with the same number of layers indicated by Precoding information and number of layers field for 2 antenna ports by replacing maxRank, maxRankSFN and codebookSubset with maxRankDCI-0-2, maxRankSFN-DCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission =fullpowerMode1, and according to whether transform precoder is enabled, or disabled and maxRankDCI-0-2=1 if neither multipanelSchemeSDM nor multipanelSchemeSFN is configured or maxRankSFN-DCI-0-2=1 if multipanelSchemeSFN is configured, and the value of higher layer parameter codebookSubsetDCI-0-2.

-	2 bits according to Table 7.3.1.1.2-5V with the same number of layers indicated by Precoding information and number of layers field for 3 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, transform precoder is disabled, and according to the values of higher layer parameters maxRankDCI-0-2;

-	2 bits according to Table 7.3.1.1.2-5U with the same number of layers indicated by Precoding information and number of layers field for 3 antenna ports by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2, respectively, if SRS resource set indicator field is present, txConfig = codebook, ul-FullPowerTransmission is not configured or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRankDCI-0-2;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission is configured to fullpowerMode2, the values of higher layer parameters maxRankDCI-0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in the SRS resource set indicated by SRS resource set indicator field, and an SRS resource with 2 antenna ports is indicated via Second SRS resource indicator field in the same SRS resource set, then Table 7.3.1.1.2-4B is used by replacing maxRank and codebookSubset with maxRankDCI-0-2 and codebookSubsetDCI-0-2 respectively.

For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in an SRS resource among the configured SRS resources in the second SRS resource set with usage set to 'codebook' as defined in Table 7.3.1.1.2-36. If the number of ports for a configured SRS resource in the set is less than the maximum number of ports in an SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

For the higher layer parameter txConfig = codebook, when the Transform precoder indicator field is present, if the bit width of the Second Precoding information field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the Second Precoding information field for the case with smaller bit width until the bit width of the Second Precoding information field for the two cases are the same.

< Unchanged parts are omitted >

-	PTRS-DMRS association - number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxMIMO-LayersDCI-0-2=1 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, or if maxRankDCI-0-2=1 and maxRankSFN-DCI-0-2=1, or if maxMIMO-LayersDCI-0-2=1 and maxMIMO-LayersforSFN-DCI-0-2=1, or if maxRankDCI-0-2=1 and maxRankSDM-DCI-0-2=1 when two PTRS ports are configured by maxNrofPorts-SDM, or if maxMIMO-LayersDCI-0-2=1 and maxMIMO-LayersforSDM-DCI-0-2=1 when two PTRS ports are configured by maxNrofPorts-SDM;

-	1 or 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A/7.3.1.1.2-25B/7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s), and the DMRS ports are indicated by the Antenna ports field.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or “01” and maxRankDCI-0-2<=4 or maxMIMO-LayersDCI-0-2<=4, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or “11”, maxRankDCI-0-2=3 or 4 or maxMIMO-LayersDCI-0-2=3 or 4 and neither multipanelSchemeSDM nor multipanelSchemeSFN is not configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 2, 4, or 8 antenna ports, the SRS resource set indicator field is present and equals "10" or "11" and maxRankDCI-0-2=2 or maxMIMO-LayersDCI-0-2=2 and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25.

-	1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, SRS resource set indicator field is absent or SRS resource set indicator field is present and equals "00" or "01", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=3 or maxMIMO-LayersDCI-0-2=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-25.

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=2 or maxMIMO-LayersDCI-0-2=2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, the MSB of this field indicates the association between PTRS port and DMRS port corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port and DMRS port corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, the SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Tables 7.3.1.1.2-26B.

-	2 bits when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, the SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, the MSB of this field indicates the association between PTRS port 0 and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port 1 and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

-	2 bits when one PTRS port is configured by maxNrofPorts-SDM in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10" and multipanelSchemeSDM is configured, this field indicates the association between PTRS port and DMRS ports corresponding to SRS resource indicator field and Second SRS resource indicator field and/or Precoding information and number of layers field and Second Precoding information field according to Table 7.3.1.1.2-25.

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, SRS resource set indicator field is present and equals "10", multipanelSchemeSFN is configured, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

When the Transform precoder indicator field is present, if the bit width of PTRS-DMRS association field for the case with transform precoder enabled is not equal to that for the case with transform precoder disabled, a number of most significant bits with value set to '0' are inserted to the PTRS-DMRS association field for the case with smaller bit width until the bit width of the PTRS-DMRS association field for the two cases are the same.

-	Second PTRS-DMRS association

-	2 bits when one PTRS port or two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 4 or 8 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2>2 or maxMIMO-LayersDCI-0-2>2, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured;

-	2 bits when one PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2=3 or maxMIMO-LayersDCI-0-2=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured;

-	1 bit when two PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig for 3 antenna ports, PTRS-DMRS association field is present, SRS resource set indicator field is present and equals "10" or "11", maxRankDCI-0-2<=3 or maxMIMO-LayersDCI-0-2<=3, and neither multipanelSchemeSDM nor multipanelSchemeSFN is configured;

-	0 bit otherwise.

Tables 7.3.1.1.2-25 and 7.3.1.1.2-26/7.3.1.1.2-26B are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

< Unchanged parts are omitted >

#### 7.3.1.2	DCI formats for scheduling of PDSCH

< Unchanged parts are omitted >

        - 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment -  bits where  is given by Clause 7.3.1.0

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator - 1 bit.

-	If the Cell indicator field is absent or the Cell indicator field indicates serving cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	If the Cell indicator field indicates a candidate cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with ltm-EarlyUL-SyncConfigSUL in LTM-Candidate for the candidate cell, this field indicates which UL carrier in the candidate cell to transmit the PRACH according to Table 7.3.1.1.1-1;

-	Otherwise, this field is reserved.

-	SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUL-SyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUL-SyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUL-SyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity.

-	PRACH association indicator - 0 or 1 bit

-	1bit if the UE is provided with tag2-Id, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. This field is reserved if the cell indicated by Cell indicator field is a candidate cell.

-	This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states.

-	This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format.

-	1bit if the UE is provided with tag2-Id and SSB-MTC-AddtionalPCI, and the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided, and the UE is provided with PrachAssociationIndicator_InDCI_format_1_0. This field is reserved if the cell indicated by Cell indicator field is a candidate cell.

-	This field indicates the PL-RS for the PRACH transmission. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format.

-	0 bit otherwise.

-	PRACH retransmission indicator - 0 or 1 bit

-	1bit if the UE is configured with higher layer parameter EarlyUL-SyncConfig. This field indicates initial transmission or retransmission of PRACH according to Table 7.3.1.2.1-3 if the cell indicated by Cell indicator field is a candidate cell, and this field is reserved if the cell indicated by Cell indicator field is a serving cell but not a candidate cell.

-	0 bit otherwise.

-	Pathloss offset indicator – 0 or 1 bit

-	1 bit if the UE is configured with higher layer parameter plOffsetInPrach_InDCI and at least one configured TCI state for the serving cell is configured with plOffset.

-	If there is only one indicated joint/UL TCI state, the bit field index 0 of this field indicates that no pathloss offset is applied for the PRACH transmission, and the bit field index 1 of this field indicates that the pathloss offset configured in the indicated joint/UL TCI state is applied for the PRACH transmission.

-	If there are two indicated joint/UL TCI states, the bit field index 0 of this field indicates that the pathloss offset configured in the first indicated joint/UL TCI state is applied for the PRACH transmission, and the bit field index 1 of this field indicates that the pathloss offset configured in the second indicated joint/UL TCI state is applied for the PRACH transmission.

-	0 bit otherwise.

-	Reserved bits - a number of bits as determined by the following:

-	(12 - Y1 - Y2 - Y3) bits for operation in a cell with shared spectrum channel access in frequency range 1 or when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2;

-	(10 - Y1 - Y2 - Y3) bits otherwise;

where,

-	Y1=0 if the UE is not configured with higher layer parameter EarlyUL-SyncConfig; Y1=log2C+1+1 otherwise.

-	Y2=0 if the "PRACH association indicator" field is not present in this DCI format; Y2=1 otherwise.

-	Y3=0 if the "Pathloss offset indicator" field is not present in this DCI format; Y3=1 otherwise.

Otherwise, all remaining fields are set as follows:

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS 38.214]

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number - 4 bits

-	Downlink assignment index - 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; 0 bits otherwise

-	Reserved bits - 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Frequency domain resource assignment - bits. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig or trs-ResourceSetConfig-r18 is configured, are carried, this bit field is reserved.

-	TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured or the number of bits is equal to one plus the highest value of all the indBitID-r18(s) provided by the trs-ResourceSetConfig-r18 if configured; 0 bits otherwise.

-	Reserved bits - (8 - M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 - M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by SI-RNTI:

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	System information indicator - 1 bit as defined in Table 7.3.1.2.1-2

-	Reserved bits -  17 bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; otherwise 15 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by RA-RNTI or MsgB-RNTI:

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0 if CORESET 0 is configured for the cell and  is the size of initial DL bandwidth part if CORESET 0 is not configured for the cell

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]

-	LSBs of SFN - 2 bits for the DCI format 1_0 with CRC scrambled by MsgB-RNTI as defined in Clause 8.2A of [5, TS 38.213] if msgB-responseWindow is configured to be larger than 10 ms; or 2 bits for the DCI format 1_0 with CRC scrambled by RA-RNTI as defined in Clause 8.2 of [5, TS 38.213] for operation in a cell with shared spectrum channel access if ra-ResponseWindow or ra-ResponseWindow-v1610 is configured to be larger than 10 ms; 0 bit otherwise

-	Reserved bits - (16 - A) bits for operation in a cell without shared spectrum access in frequency range 1 and frequency range 2-1, (18 - A) for operation in a cell with shared spectrum access in frequency range 1 or for operation in a cell in frequency range 2-2, where the value of A is the number of bits for the field of 'LSBs of SFN' as defined above

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment - bits

-	 is the size of CORESET 0

-	Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	New data indicator - 1 bit

-	Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number - 4 bits

-	Downlink assignment index - 2 bits

-	2 bits indicating the number of repetitions for PUCCH as defined in clause 9.2.6 of [5, TS38.213] according to Table 7.3.1.2.1-4, if the higher layer parameter numberOfMsg4HARQ-ACK-Repetitions is configured with at least two repetition factors and the UE has indicated capability of PUCCH repetition on common PUCCH resource [8, TS38.321];

-	otherwise, reserved.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext - 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; otherwise 0 bit

-	Reserved bits - 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

Table 7.3.1.2.1-1: Short Message indicator

| Bit field | Short Message indicator |
| --- | --- |
| 00 | Reserved |
| 01 | Only scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI |
| 10 | Only short message, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI |
| 11 | Both scheduling information for Paging, TRS availability indication if trs-ResourceSetConfig is configured and short message are present in the DCI |

Table 7.3.1.2.1-2: System information indicator

| Bit field | System information indicator |
| --- | --- |
| 0 | SIB1 [9, TS38.331, Clause 5.2.1] |
| 1 | SI message [9, TS38.331, Clause 5.2.1] |

Table 7.3.1.2.1-3: PRACH retransmission indicator

| Bit field | PRACH retransmission indicator |
| --- | --- |
| 0 | Initial transmission of PRACH |
| 1 | Retransmission of PRACH |

Table 7.3.1.2.1-4: Number of repetitions NPUCCHrepeat as a function of 2 bits of Downlink assignment index field

| Bit field | NPUCCHrepeat |
| --- | --- |
| 00 | First repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions |
| 01 | Second repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions |
| 10 | Third repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions if provided, otherwise reserved |
| 11 | Fourth repetition factor configured by numberOfMsg4HARQ-ACK-Repetitions if provided, otherwise reserved |

        - 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator - 0 or 3 bits as defined in Clause 10.1 of [5, TS 38.213]. This field is reserved when this format is carried by PDCCH on the primary cell and the UE is configured for scheduling on the primary cell from an SCell, with the same number of bits as that in this format carried by PDCCH on the SCell for scheduling on the primary cell.

-	Bandwidth part indicator - 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment - number of bits determined by the following, where  is the size of the active DL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	bits if only resource allocation type 1 is configured, or

-	 bits if resourceAllocation is configured as 'dynamicSwitch'.

-	If resourceAllocation is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if resourceAllocation is configured as 'dynamicSwitch' for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment - 0, 1, 2, 3, 4, 5 or 6 bits

-	If the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH is not configured and if the higher layer parameter pdsch-TimeDomainAllocationList is configured, 0, 1, 2, 3 or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter is configured;

-	if the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH is configured, 0, 1, 2, 3, 4, 5 or 6 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH;

-	otherwise I is the number of entries in the default table.

-	VRB-to-PRB mapping - 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if interleaved VRB-to-PRB mapping is not configured by high layers;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6  of [4, TS 38.211].

-	PRB bundling size indicator - 0 bit if the higher layer parameter prb-BundlingType is not configured or is set to 'staticBundling', or 1 bit if the higher layer parameter prb-BundlingType is set to 'dynamicBundling' according to Clause 5.1.2.3 of [6, TS 38.214].

-	Rate matching indicator - 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger - 0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where  is the number of aperiodic ZP CSI-RS resource sets configured by higher layer.

For transport block 1:

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator - 1 bit if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	Redundancy version - number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2):

-	Modulation and coding scheme - 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator - 1 bit if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1; otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined based on the maximum number of schedulable PDSCH among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214].

-	Redundancy version - number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PDSCH indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PDSCHs among all entries in the higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, where each bit corresponds to one scheduled PDSCH as defined in clause 5.1.3 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number - 5 bits if higher layer parameter harq-ProcessNumberSizeDCI-1-1 is configured; otherwise 4 bits

-	Downlink assignment index - number of bits as defined in the following

-	6 bits if more than one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included is configured. The 4 MSB bits are the counter DAI and the total DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group.

-	4 bits if only one serving cell is configured in the DL and the higher layer parameter nfi-TotalDAI-Included is configured. The 2 MSB bits are the counter DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group;

-	4 bits if more than one serving cell are configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic, and nfi-TotalDAI-Included is not configured, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	4 bits if one serving cell is configured in the DL, and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16=enhancedDynamic, and nfi-TotalDAI-Included is not configured, when the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided or the UE is not configured with ackNackFeedbackMode = joint, where the 2 bits are the counter DAI;

-	0 bits otherwise.

If the UE is configured with a PUCCH-SCell, the number of serving cells is determined within a PUCCH group.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 if present for the secondary PUCCH group.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the Downlink assignment index in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	Second TPC command for scheduled PUCCH - 2 bits as defined in Clause 7.2.1 of [5, TS 38.213] if higher layer parameter SecondTPCFieldDCI-1-1 is configured; 0 bit otherwise.

-	PUCCH resource indicator - 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator - 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the other HARQ-ACK codebook on the same cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

If higher layer parameter pucch-sSCellDyn is configured, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with one cell for PUCCH transmission is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with the other cell for PUCCH transmission, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 1_1 associated with the two cells are the same.

If the UE is configured with a PUCCH-SCell, pucch-sSCellDyn is replaced by pucch-sSCellDynSecondaryPUCCHgroup for the secondary PUCCH group.

-	One-shot HARQ-ACK request - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-OneShotFeedback-r16 or pdsch-HARQ-ACK-EnhType3ToAddModList is configured;

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-EnhType3ToAddModList is replaced by pdsch-HARQ-ACK-EnhType3SecondaryToAddModList for the secondary PUCCH group.

-	Enhanced Type 3 codebook indicator - 0, 1, 2, or 3 bits.

-	0 bit if pdsch-HARQ-ACK-EnhType3DCI-Field is not configured;

-	log2(nCB) bits otherwise, where nCB is the number of entries in the higher layer parameter pdsch-HARQ-ACK-EnhType3ToAddModList.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-EnhType3DCI-Field is replaced by pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup for the secondary PUCCH group, and pdsch-HARQ-ACK-EnhType3ToAddModList is replaced by pdsch-HARQ-ACK-EnhType3SecondaryList for the secondary PUCCH group.

-	PDSCH group index - 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic;

-	0 bit otherwise.

-	New feedback indicator - 0, 1 or 2 bits.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic and the higher layer parameter nfi-TotalDAI-Included is not configured;

-	2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic and the higher layer parameter nfi-TotalDAI-Included=true; the MSB corresponds to the scheduled PDSCH group, and the LSB corresponds to the non-scheduled PDSCH group, as defined in [TS38.213] clause 9.1.3.3

-	0 bit otherwise.

-	Number of requested PDSCH group(s) - 0 or 1 bit.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic;

-	0 bit otherwise.

-	HARQ-ACK retransmission indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pdsch-HARQ-ACK-Retx is configured.

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pdsch-HARQ-ACK-Retx is replaced by pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup for the secondary PUCCH group.

-	Antenna port(s) - 4, 5, 6, 7 or 8 bits as defined by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 and Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4/7/8/9/10 or Tables 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A. When a UE not configured with dl-OrJointTCI-StateList receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, or when a UE configured with dl-OrJointTCI-StateList is having two indicated TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A/7A/8A/9A/10A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4/7/8/9/10. The UE can receive an entry with DMRS ports equals to 1000, 1002, 1003 when two the UE is not configured with dl-OrJointTCI-StateList and TCI states are indicated in a codepoint of DCI field 'Transmission Configuration Indication', or when the UE configured with dl-OrJointTCI-StateList is having two indicated TCI states to be applied to PDSCH.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication - 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_1,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	TCI selection - 0 bit if higher layer parameter tci-SelectionPresentInDCI is not configured; otherwise 2 bits according to Table 7.3.1.2.2-11.

-	SRS request - 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	SRS offset indicator - 0, 1 or 2 bits.

-	0 bit if higher layer parameter AvailableSlotOffset is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter AvailableSlotOffset is configured for at least one aperiodic SRS resource set in the scheduled cell  and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

-	CBG transmission information (CBGTI) - 0 bit if higher layer parameter PDSCH-CodeBlockGroupTransmission for PDSCH is not configured, otherwise, 2, 4, 6, or 8 bits as defined in Clause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and maxNrofCodeWordsScheduledByDCI for the PDSCH.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG transmission information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG transmission information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG transmission information until the bit width of the CBG transmission information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	CBG flushing out information (CBGFI) - 1 bit if higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE", 0 bit otherwise.

If higher layer parameter priorityIndicatorDCI-1-1 is configured, if the bit width of the CBG flushing out information in DCI format 1_1 for one HARQ-ACK codebook is not equal to that of the CBG flushing out information in DCI format 1_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller CBG flushing out information until the bit width of the CBG flushing out information in DCI format 1_1 for the two HARQ-ACK codebooks are the same.

-	DMRS sequence initialization - 1 bit.

-	Priority indicator - 0 bit if higher layer parameter priorityIndicatorDCI-1-1 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	ChannelAccess-CPext - 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if channelAccessMode-r16 = "semiStatic" is provided, for operation in a cell with shared spectrum channel access in frequency range 1, or for operation in frequency range 2-2 if ChannelAccessMode2-r17 is provided; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

-	Minimum applicable scheduling offset indicator - 0 or 1 bit

-	0 bit if higher layer parameter minimumSchedulingOffsetK0 is not configured;

-	1 bit if higher layer parameter minimumSchedulingOffsetK0 is configured. The 1 bit indication is used to determine the minimum applicable K0 for the active DL BWP and the minimum applicable K2 value for the active UL BWP, if configured respectively, according to Table 7.3.1.1.2-33. If the minimum applicable K0 is indicated, the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP shall be the same as the minimum applicable K0 value.

-	SCell dormancy indication - 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

If one-shot HARQ-ACK request is not present or set to '0', and all bits of frequency domain resource assignment are set to 0 for resource allocation type 0 or set to 1 for resource allocation type 1 or set to 0 or 1 for dynamic switch resource allocation type, this field is reserved and the following fields among the fields above are used for SCell dormancy indication, where each bit corresponds to one of the configured SCell(s), with MSB to LSB of the following fields concatenated in the order below corresponding to the SCell with lowest to highest SCell index

-	Modulation and coding scheme of transport block 1

-	New data indicator of transport block 1

-	Redundancy version of transport block 1

-	HARQ process number

-	Antenna port(s)

-	DMRS sequence initialization

-	PDCCH monitoring adaptation indication - 0, 1 or 2 bits

-	1 or 2 bits, if searchSpaceGroupIdList-r17 is not configured and if pdcch-SkippingDurationList is configured

-	1 bit if the UE is configured with only one duration by pdcch-SkippingDurationList;

-	2 bits if the UE is configured with more than one duration by pdcch-SkippingDurationList.

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

-	PUCCH Cell indicator - 0 or 1 bit.

-	1 bit if higher layer parameter pucch-sSCellDyn is configured.

-	0 bit otherwise.

If the UE is configured with a PUCCH-SCell, pucch-sSCellDyn is replaced by pucch-sSCellDynSecondaryPUCCHgroup for the secondary PUCCH group.

-	Co-scheduled UE information – 0 or 3 bits

-	3 bits as defined in Table 7.3.1.2.2-12 if higher layer parameter advReceiver-MU-MIMO-DCI-1-1 is configured. This field is reserved if two codewords are scheduled by this DCI format 1_1.

-	0 bit otherwise.

-	TPC command for SRS – 2 bits as defined in Clause 7.3.1 of [5, TS 38.213] if higher layer parameter tpcOfSrsClosedLoopIndex_InDCI_format_1_1 is configured; 0 bit otherwise.

-	Closed loop indicator for SRS – 1 bit if higher layer parameter srsClosedLoopIndexIndicator_InDCI_format_1_1 and higher layer parameter enableTwoSeparatePowerControlAdjustmentStatesForSRS are both configured; 0 bit otherwise.

If DCI formats 1_1 are monitored in multiple search spaces associated with multiple CORESETs in a BWP for scheduling the same serving cell, zeros shall be appended until the payload size of the DCI formats 1_1 monitored in the multiple search spaces equal to the maximum payload size of the DCI format 1_1 monitored in the multiple search spaces.

If the number of information bits in DCI format 1_1 scheduling a single PDSCH prior to padding is not equal to the number of information bits in DCI format 1_1 scheduling multiple PDSCHs for the same serving cell, zeros shall be appended to the DCI format 1_1 with smaller size until the payload size is the same for scheduling a single PDSCH and multiple PDSCHs.

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 1_1 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 1_1 with smaller size until the payload size is the same:

-	If application of step 4C in clause 7.3.1.0 results in additional zero padding for DCI format 1_1 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 1_1 monitored on the primary cell and DCI format 1_1 monitored on the SCell for scheduling on the primary cell.

-	If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

< Unchanged parts are omitted >

#### 7.3.1.3	DCI formats for other purposes

< Unchanged parts are omitted >

##### 7.3.1.3.4	Format 2_3

DCI format 2_3 is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs. Along with a TPC command, a SRS request may also be transmitted.

The following information is transmitted by means of the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI:

-	block number 1, block number 2, …, block number

where the starting position of a block is determined by the parameter startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one or two blocks are configured for the UE by higher layers where one block applies to non-SUL carriers and another block applies to SUL carriers, with the following fields defined for each block:

-	SRS request - 0 or 2 bits. The presence of this field is according to the definition in Clause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24.

-	TPC command number 1, TPC command number 2, ..., TPC command number N, where each TPC command applies to a respective UL carrier provided by higher layer parameter cc-IndexInOneCC-Set

-	Closed loop indicator 1, Closed loop indicator 2, ..., Closed loop indicator N, where each Closed loop indicator applies to a respective UL carrier provided by higher layer parameter cc-IndexInOneCC-Set. A Closed loop indicator is present only if the UE is configured with higher layer parameter enableTwoSeparatePowerControlAdjustmentStatesForSRS for the UL carrier.

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeB for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block or more blocks is configured for the UE by higher layers where each block applies to an UL carrier, with the following fields defined for each block:

-	SRS request - 0 or 2 bits. The presence of this field is according to the definition in Clause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24.

-	TPC command -2 bits

-	Closed loop indicator - 1 bit if the UE is configured with higher layer parameter enableTwoSeparatePowerControlAdjustmentStatesForSRS for the UL carrier; 0 bit otherwise.

The number of information bits in format 2_3 shall be equal to or less than the payload size of format 1_0 monitored in common search space in the same serving cell. If the number of information bits in format 2_3 is less than the payload size of format 1_0 monitored in common search space in the same serving cell, zeros shall be appended to format 2_3 until the payload size equals that of format 1_0 monitored in common search space in the same serving cell.

< Unchanged parts are omitted >