3GPP TSG-RAN WG1 Meeting # 120bis	R1-2503500

# Wuhan, China, April 7th – 11th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TEI19 Extension of SRS frequency hopping for positioning to non-RedCap UEs [Pos_SRSHop] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN1#120 agreement: Agreement - Extend Rel-18’s UL frequency hopping UL SRS for positioning transmission to non-RedCap UEs in a single carrier - UE capability for non-RedCap UEs for UL SRS frequency hopping for positioning transmission - Send LS to RAN2 to inform this agreement, whether new parameter is needed is up to RAN2 discussion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As agreed in RAN1#120bis, specifying that the SRS frequency hopping for positioning can be applied also for non-RedCap UEs. Below specification change reflects the agreed TP from RAN1#120bis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed TEI item is not supported by the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

##### 6.2.1.4.1	SRS frequency hopping for positioning

The reduced capability UE may be configured via SRS-PosTx-Hopping, subject to UE capability, to perform transmit frequency hopping separate from the UL BWP configuration via bwp and outside of the UL BWP, where the UE may be configured with subcarrier spacing, CP and bandwidth that are different from the UL active BWP. The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning in higher layer parameter srs-PosConfig, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE, in RRC_CONNECTED or RRC_INACTIVE mode. The reduced capability UE transmit frequency hopping, may be configured with overlapping or non-overlapping frequency hops in the frequency domain.  When the reduced capability UE is configured to perform transmit frequency hopping:

-	it expects to be configured with the following parameters:

-	starting PRB of the first hop in time domain in freqDomainShift

-	starting slot offset for the first hop in periodicityAndOffset for periodic and semi-persistent SRS and slotOffset for aperiodic SRS, starting slot offset for each hop following the first hop in periodicityAndOffset for periodic and semi-persistent SRS and slotOffset for aperiodic SRS in SlotOffsetForRemainingHops, and starting symbol for each hop in startPosition

-	number of symbols in each hop in nrofSymbols

-	hop bandwidth in c-SRS

-	number of overlapping resource block(s) between hops, if present, in overlapValue

-	number of hops in numberOfHops.

-	it does not expect to be configured with the sum of startPosition and nrofSymbols for a hop that exceeds a slot duration.

-	it expects to be configured with the same periodicity of each hop of an SRS resource with the transmit frequency hopping.

The reduced capability UE may be configured, via srs-PosUplinkTransmissionWindowConfig, subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is not expected to be configured with a SRS resource for positioning with transmit frequency hopping for which the transmission, which includes all the hops once and the switching time from/to active BWP required ahead of the first hop and after the last hop, is partially overlapped with an UL time window.

For aperiodic positioning SRS with Tx frequency hopping, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 symbols and an additional time duration corresponding to the switching time from the active uplink BWP.

The reduced capability UE is expected to switch back to the active BWP if the time between two consecutive hops exceeds twice the switching time from/to the active BWP.

In RRC_CONNECTED mode, for a transmission of a hop for an SRS resource for positioning with frequency hopping starting in symbol Nc1 and a colliding PUSCH or PUCCH transmission starting in symbol NS , the UE shall apply the dropping rules taking into account:

-	DCI(s) for which the time interval between the last symbol of PDCCH and the SRS symbol Nc1is at least N2 symbols and additional time duration TSRSh, where TSRSh is the switching time to/from the active BWP.

-	DCI(s) for which the time interval between the last symbol of PDCCH and the colliding PUSCH/PUCCH symbol NS is at least N2 symbols, where calculation of N2 is based on the smallest SCS between the SCS configured for positioning SRS with the frequency hopping, the SCS of the PUSCH/PUCCH, and the SCS of the PDCCH.

-	semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSh before Nc1, and considered active at least N2 symbols before NS.

When the reduced capability UE is configured by the higher layer parameter SRS-PosTx-Hopping, if the SRS symbol(s), including a switching time to and from the active bandwidth part, of the transmit frequency hopping collides with downlink physical channels/signals on flexible symbols configured by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, or with PUSCH or PUCCH, the UE shall use the same priority rules as defined in Clause 6.2.1 in this specification and Clauses 11.1, and 17.2 in [6, TS38.213].

For operation in the same carrier, the reduced capability UE is not expected to be configured on overlapping symbols with an SRS resource of the transmit frequency hopping configured by the higher layer parameter SRS-PosTx-Hopping including the switching time to or from the active bandwidth part and an SRS resource with resourceType of both SRS resources as 'periodic'.

For operation in the same carrier, the reduced capability UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with a SRS resource of the transmit frequency hopping configured by the higher layer parameter SRS-PosTx-Hopping including the switching time to or from the active bandwidth part and a SRS resource with resourceType of both SRS resources as 'semi-persistent' or 'aperiodic'.

A UE shall perform SRS frequency hopping for positioning according to clause 6.2.1.4.1, subject to UE capability, with the following modifications:

-	 “reduced capability UE” is replaced by “UE”

-	“The reduced capability UE transmit frequency hopping is configured within one SRS resource for positioning in higher layer parameter srs-PosConfig, that may be configured with a bandwidth larger than the maximum bandwidth of the reduced capability UE” is replaced by “The UE transmit frequency hopping is configured within one SRS resource for positioning in higher layer parameter srs-PosConfig”.

<omitted text>