3GPP TSG RAN WG1 #121			R1-2503512

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.3.1

Source: 	Spreadtrum, UNISOC

Title: 		Discussion on SBFD TX/RX/measurement procedures

Document for:	Discussion and decision

Introduction

In this contribution, we discuss remaining issues related to SBFD TX/RX/measurement procedures.

Discussion

SBFD subband indication

Frequency location indication

In RAN1#116 and 116bis meeting, the following agreement was achieved. Cell-specific configuration on time location of SBFD subbands is supported. Subband time domain locations can be same for all the SBFD-aware UEs in a same serving cell.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

Agreement A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

The other open issue includes the working assumption to support cell-specific configuration on frequency location of SBFD subbands, and FFS on additional support of UE-specific configuration on frequency domain of SBFD subbands.

From our perspective, we do not support  UE-specific configuration on frequency domain of SBFD subbands.

- It should be single one UL/DL subband locations within a TDD carrier from gNB perstpective to optimize the self-interference. Also RAN4 decide to specify DL/UL subband configuration in BS RF requirements. No matter which active BWP is applied by a UE in the cell, there should be one common resource allocated, in the TDD carrier. There is no need to configure UE dedicated  SBFD subband. The available resources for DL and UL are based on the overlapped frequency resources between UL/DL subbands and the UE active UL/DL BWP.

- The motivation of guard band is for gNB self-interference suppression, not for UE-UE CLI cancellation.  And the size is based on guardbands requirements. So no need to have UE specific subbands due to UL Rx filtering.  One reason to support UE-secific frequency location is to have per-BWP UE-dedicated UL SBFD frequency configurations since different guardband sizes for different SBFD aware UEs due to Rx filter at UE side and/or to mitigate impact on DL reception due to UE-UE CLI. gNB can configure a large guard band in a conservative way and consider the worst interference case.

- The specific guard band size for different Rx filtering capability SBFD aware UEs may improve resource utilization. However, different usable resources for different UEs can be achieved by not scheduling some RBs by gNB in a transparent manner, that is by gNB implementation.

Thus, we prefer to have cell-specific SBFD subband frequency location indication only.

- Only cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

Guard period

The remaing open issue is whether or not to explicitly configure guard periods between SBFD and non-SBFD symbols. According to the following agreements in RAN4, the existing TDD BS transmitter transient period is reused and the length of transient period is fixed. For Case A or Case-D, the location of transient periods is in TA_offset in SBFD symbol(s) or non-SBFD UL symbol(s). For case B and Case C, the location of guard symbol(s)  is up to gNB implementation. It can be ensured by gNB  implementation that no UL transmission or DL reception in  guard symbol(s). Thus, it does not need to explicitly configure transient period.

For invalid symbols determination for PUSCH repetition Type B, invalid symbols for DL(non-SBFD)->SBFD switching and SBFD->UL(non-SBFD) switching can be discussed separately.

Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided. Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-D: in TA_offset in non-SBFD UL symbol(s). Agreement: - For Case-B, guard symbol(s) should be reserved before UL symbols - The transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation. Agreement: - For the location of transient periods Case-C, - Both Option 1 (guard symbols reserved in non-SBFD DL symbol(s)) and Option 2 (blanked SBFD symbols in the beginning of SBFD UL transmission) shall be allowed, up to gNB implementation.

Figure 1: Illustration of four cases of transitions to/from SBFD symbols

- Explicit indication of transition periods between SBFD and non-SBFD symbols is not supported.

Transmission/Reception/measurement procedures

TX/RX/measurement behaviors

Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot

For PUSCH repetition type B in case a nominal repetition is mapped to SBFD and non-SBFD symbols within a slot, it was agreed that a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. As illustrated in figure 2, a nominal repetition#3 is segmented into actual repetition#5, actual repetition#6 and actual repetition#7. Each actual repetition includes only SBFD symbols or non-SBFD symbols.

Agreement For PUSCH repetition type B, adopt Option 1. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

In current specification, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols are considered as invalid symbol(s) for PUSCH repetition Type B transmission. Samilar mechanism can be reused to indicate additional invalid symbol for DL(non-SBFD)->SBFD switching and SBFD->UL(non-SBFD) switching.

Section 6.1.2 in TS 38.214 - For operation in unpaired spectrum, if numberOfInvalidSymbolsForDL-UL-Switching is configured, numberOfInvalidSymbolsForDL-UL-Switching symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberOfInvalidSymbolsForDL-UL-Switching are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

For SBFD->UL(non-SBFD) switching, RAN4 has agreed that transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation. If transient period is reserved in the whole channel bandwidth, UL transmission in UL subband of SBFD symbol(s) is not allowed. So UE needs to know invlaid SBFD symbols when UE perform segmentation around the boundary of SBFD symbols and non-SBFD symbols. For DL(non-SBFD)->SBFD switching, if guard symbols is blanked SBFD symbols in the beginning of SBFD UL transmission, UE also needs to know invlaid SBFD symbols when UE perform segmentation around the boundary of non-SBFD symbols and SBFD symbols. So two new RRC parameter can be introduced to indicate the invalid symbol for these two switching cases.

- If transient period is reserved in the whole channel bandwidth, invalid symbol for SBFD->UL(non-SBFD) switching needs to be indicated.

- Two new RRC parameter can be introduced to indicate the invalid symbol for SBFD->UL(non-SBFD) switchin and DL(non-SBFD)->SBFD switching.

Figure 2 : Configuration 2: transmissions in SBFD symbol and non-SBFD symbol

For PUSCH repetition Type B with Configuration 1, the valid symbol type determination was discussed and following agreement was made. For the FFS part, some company think that the invalid symbol type can be treated as invalid symbols for repetition type B, so there would be no actual repetition in the invalid symbol type. In this case, there is no need for the additional dropping process. However, to be more aligned with other PUSCH/PUCCH/SRS transmission, only transmit the repetition in valid symbol type.

UE should perform segmentation around boundary of SBFD symbols and non-SBFD symbols first and UE drops an actual repetition if the actual repetition is in the invalid symbol type

Agreement For PUSCH repetition Type B with Configuration 1: - Alt 2: - For type 2 CG PUSCH, the valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first actual repetition associated with activation DCI. - For dynamically scheduled PUSCH, the valid symbol type is determined based on the symbol type of the first actual repetition occasion indicated by scheduling DCI. - FFS: UE drops an actual repetition if the actual repetition is in the invalid symbol type.

- For PUSCH repetition type B, segmentation around boundary of SBFD symbols and non-SBFD symbols is supported with either Configuration 1/2.

- For PUSCH repetition Type B with Configuration 1, UE drops an actual repetition if the actual repetition is in the invalid symbol type.

Resource allocation in frequency domain in SBFD symbols

RBG in Resource allocation Type 1

For frequency resource allocation Type 1 for PDSCH or PUSCH in a single slot by DCI format 0_2/0_3/1_2/1_3, PDSCH/PUSCH can be scheduled by starting RBG and length of RBG. It is suffer form the non-contiguous DL subbands and disaligned granularity. It may be allocated with a RBG for PDSCH which is fully/partially outside DL usable PRBs due to RIV mechanism. And PUSCH may be disaligned granularity and allocated RBGs are partially inside UL usable PRBs.

Thus, we have the following proposal:

- For frequency resource allocation Type 1 with RBG for PDSCH or PUSCH in a single slot by DCI format 0_2/0_3/1_2/1_3 based scheduling (without repetition or TBoMS),

- When an allocated RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be assigned for PDSCH reception and only the PRBs within UL usable PRBs are considered to be assigned for PUSCH transmission.

- Allocated RBGs that fall outside DL usable PRBs are considered to be not assigned and should not be used for PDSCH resource mapping.

Other issues on non-contiguous DL subbands

CSI-RS

In current specification, the bandwidth of CSI-RS within a given DL BWP is equal or greater than the minimum between 24 PRBs and the DL BWP size to ensure measurement accuracy. In last meeting, the associated minimum bandwidth for CSI-RS across the two DL subbands was discussed and no consensus was reached. From our perspective, since the bandwidth of CSI-RS is across the two DL subband, so minimum bandwidth requirement should be applied per subband and equal or greater than the minimum between 24 PRBs and DL usable PRBs in each DL subband.

#### Proposal 2-15 Proposed Agreement: The number of PRBs of a CSI-RS resource within each DL subband should be ≥ min⁡(24,NDL-SBsize), where NDL-SBsize is the number of DL usable PRBs in one DL subband.

- The number of PRBs of a CSI-RS resource within each DL subband should be ≥ min⁡(24,NDL-SBsize), where  NDL-SBsize is the number of DL usable PRBs in one DL subband.

Wideband PRG

Wideband PRG is good for better channel estimation performance. The main concern is whether wideband PRG can be non-contiguous due to discontinuous DL resources. In RAN 1#118 meeting, the folloing conclusion was made. If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

Conclusion If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

The open issue is whether the exsting condition for determination of wideband PRG can be reused. The rule is size of scheduled PRBs is more than half of the size of active DL BWP.  It needs further discussion. Due to the existence of UL subband, the size of usable DL PRBs may be less than half of the size of active DL BWP, or the contiguous PRBs in SBFD symbols are less than half of the size of active DL BWP. So, the condition of wideband PRG is not true.

If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise is set to the remaining configured value of 2 or 4, respectively.

In RAN1#118b meeting, the following proposal was discussed.

#### Proposal 2-2 Proposed Agreement: For dynamic determination of the precoding bundling for a PDSCH, when DCI bitfield (bundling size indicator) indicates ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband where the PDSCH is located.

For SBFD Subband configuration with {DUD} pattern,  DL usable PRBs are two discontigious ones, the condition in the proposal may never be satisfied. According to above agreement, scheduled PRBs is limited to one DL subband if PRG is determined as wideband, so wideband PRG should be determined per DL subband. The condition for determining wideband or narrowband depends on whether the number of scheduled PRBs in a DL subband is larger than half of the number of PRBs in the DL subband.

- For dynamic determination of the precoding bundling for a PDSCH, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband where the PDSCH is located.

Fallback DCI

PDSCH scheduled by DCI format 1_0 in CSS

In current spec, if a PDSCH scheduled with a DCI format 1_0 in a CSS, RB numbering starts from the lowest RB of the CORESET in which the DCI was received. Only resource allocation of type 1 can be used. RBs can be allocated is within the size of CORESET 0 if it is configured or the size of initial DL BWP if CORESET 0 is not configured.

For a CORESET except CORESET0, its frequency resource is easy to be only within DL usable PRBs. Then the PDSCH scheduling is as same as non-SBFD symbols, e.g. no assigned PRBs outside DL usable PRBs.

For CORESET0 configured by MIB, due to it is configured in a continuous resources, it might be overlap with DL subband boundary. Generally, the PDSCH resource allocation by DCI format 1_0 in CSS should resue the legacy method as much as possible, including from the lowest PRB index of CORESET and maximum number of PRBs. Considering there might be SBFD non-aware UE share a same PDCCH and PDSCH, the exclusive of DL non-usable PRBs are not workable, and TBS re-calculation also cannot work. So we suggest the assigned PRBs in PDSCH scheduled by DCI format 1_0 in CSS are all inside DL usable PRBs, without extra handling for PRBs outside of DL usable PRBs.

- Assigned PRBs in PDSCH scheduled by DCI format 1_0 in CSS are all inside DL usable PRBs by scheduling.

Physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots

Resource allocation in frequency domain

In RAN1#120, it was agreed that the configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP.

Agreement The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP.

According to P/SP NZP CSI-RS resource, UE needs to aware whether it is configuration 1 or 2 due to impacts to PDSCH RE mapping according to the following specification.

38.213, 7.3.1.5 Mapping to virtual resource blocks … - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; …

For PDSCH resource mapping, there can be two alternatives for NZP CSI-RS. It can make a clarification.

- Alt 1: UE assume P/SP NZP CSI-RS receptions is configuration 2.

- Alt 2: The valid symbol type of CSI-RS resource is same as the valid symbol type for CSI derivation of associated CSI report.

There is no restriction that one CSI-RS only is associated with one symbol type of CSI reports. So if one CSI-RS resource is only associated with one symbol type of CSI report, only one symbol type is applied to CSI-RS. If one CSI-RS is associated with two CSI-RS report and the valid symbol type for these two CSI-RS report is different. In this case, the unit of all symbol types are applied to CSI-RS.

- For PDSCH resource mapping, there can be two alternatives for the symbol type of NZP CSI-RS:

- Alt 1: UE assume P/SP NZP CSI-RS receptions is configuration 2.

- Alt 2: The valid symbol type of CSI-RS resource is same as the valid symbol type for CSI derivation of associated CSI report.

In RAN1#119 meeting, it was agreed to support the following Equation 1-C2 for determining starting PRB for PUSCH transmissions in SBFD symbols in case PUSCH frequency hopping is not enabled. In case PUSCH frequency hopping is enabled, equation 1-C2 also should be applied before applying frequency hopping.

Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols, - Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - If RBoffsetSBFD is not configured, it is zero

- For PUSCH transmissions with Configuration 2, in case PUSCH frequency hopping is enabled, starting PRB of the first hop for PUSCH repetitions/transmissions in SBFD symbols is determined according to Equation 1-C2 as agreed for PUSCH repetitions/transmissions in SBFD symbols without frequency hopping.

CSI measurement and reporting

Option A/B for CSI report

For CSI reports, the following agreements were achieved in RAN1#117 meeting.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

For Option A and Option B, they are basically same, except separate CSI reports in Option A, but Option B using separate CSI sub-reports in one CSI report. From function supporting point of view, they have a same ability for CSI reporting. It is really not necessary to suppot both of them. However, from UE capability point of view, Option B require to use Rel-18 NES CSI reporting as prerequisisite. It may have some limits. And the specification impacts of Option A and B have a lot overlapping parts, such as to define CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols. So our suggestion is to support both Options in specification, and Option A/B can be UE capability.

For Option A, the only enhancement is to define occasions used for a CSI report, i.e. CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols.

In Rel 18 NES spatial domain enhancement, single CSI-ReportConfig with multiple sub-configurations is introduced. Each sub-configuration corresponds to a list of one or more CSI-RS resources or a CSI-RS antenna port subset, and/or a power offset for PDSCH relative to CSI-RS. For Option B, besides current two types of sub-configuration (CSI-RS resources and a CSI-RS antenna port subset), a new type of sub-configuration nees to be defined,  which corresponds to CSI-RS transmission occasions subset within SBFD symbols or non-SBFD symbols. Each sub-configuration corresponds to SBFD symbols or non-SBFD symbols in different time instance for one CSI-RS resource. Alternatively, each sub-configuration corresponds to two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively.

- For SBFD-aware UEs, for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols), support Option A and B.

- For Option A, it introduces occasions used for a CSI report, CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols

- For Option B, it defines a new type of sub-configuration, corresponding to CSI-RS transmission occasions subset within SBFD symbols or non-SBFD symbols

CSI reference resource

In CSI framework, UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise. And in the time domain, CSI reference resource for a CSI reporting is defined by a downlink slot offset with downlink slot n which is the corresponding downlink slot of CSI report in uplink slot n’ depicted in Figure 3. CSI reference resource is defined in downlink slot because it is used as the reference for CSI-RS which is a downlink signal. Similar as CSI reference resource, a reference resource should be defined to decide whether CLI measurement resource is valid for a CSI report named ‘CLI reference resource’.

So the validation rule of CLI reference resource should be enhanced. For now CSI reference resource is defined in a valid downlink slot which should comprise at least one higher layer configured downlink or flexible symbol and not fall within a configured measurement gap. However, the CLI measurement resource is used to measure the CLI caused by SBFD operation/dynamic TDD operation which should be in SBFD symbols or flexible symbols. The valid CLI reference resource slot should comprise at least one higher layer configured SBFD or flexible symbol and not fall within a configured measurement gap.

Figure 3. CSI reference resource determination in CSI framework

- The valid CLI reference resource slot should comprises at least one higher layer configured SBFD or flexible symbol and not fall within a configured measurement gap.

PDCCH

For a CORESET and search space configured that the monitoring occasions of the search space would occur in both SBFD and non-SBFD symbols. Whether the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols was discussed and following options was proposed.

#### Proposal 2-18 Proposed Agreement: For a CORESET and a search space configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols, down-select from the following options. - Option 1: No enhancements for SBFD-aware UEs in Rel-19. - It is up to gNB to avoid the configuration - Option 2: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL usable PRBs in SBFD symbols. - Applicable to at least USS

When PDCCH monitoring occasions crossing SBFD and non-SBFD symbols in different slots, it can be guaranteed by PDCCH monitoring occasion and CORESET configuration to never overlap with UL subband in SBFD symbols. Since if the frequency domain location of a CORESET in a SBFD symbol can also work for a non-SBFD symbol, CORESET location does not need to consider how to take full advantage of DL frequency resources, since PDSCH can use the other leftover parts. The location of CORESET only has to ensure the PDCCH capacity, and this part can be satisfied by enough RB number in CORESET.  SBFD is to improve UL delay and coverage, DL resource for is not bottleneck, so a clean CORESET and search space configuration has no problem. In addition, CORESET bitmap configuration has already sufficient flexibility to handle UL subband in frequency domain. Thus, we support Option 1.

In current specification, the frequency resource of CORESRT 0 consists of consecutive resource blocks, as specified in Tables 13-0 through 13-10 of TS38.213. The size of initial DL BWP size is equal to the size of CORESET 0 in RRC_IDLE/INACTIVE mode. Since the centre frequency between the initial UL BWP and initial DL BWP is aligned and  the size of initial UL BWP is no larger than the size of initial DL BWP. If CORESET 0 is confined within one DL subband, there is no usble UL PRBs and random access operation in SBFD symbols can not be achieved.  So the CORESET 0 would overlap the boundary of DL usable PRBs. However, even CORESET 0 overlaps with the boundary of DL usable PRBs in SBFD symbols, the CCE of PDCCH candicate can only be within DL usable PRBs. It can be ensured by the configuration of search space associated with CORESET 0.

In summary, PDCCH monitoring and CCE-REG mapping is the most complex part for UE implementation, existing tools in specifications on CORESET and search space configuration already have sufficient flexibility for PDCCH, no enhancement is required.

- For a CORESET and a search space configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols

- No enhancements for SBFD-aware UEs in Rel-19.

  - It is up to gNB to avoid the PDCCH candidate overlap with the boundary of DL usable PRBs in SBFD symbols

Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols

SRS

In RAN1#119, it was agreed to support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

In RAN1#120, whether the number of values configured in availableSlotOffsetList for SBFD symbols and number of values configured in availableSlotOffsetList for non-SBFD symbols is same or not was discussed.

- Proposal 2-7 Proposed Agreement: For separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage, the number of values configured in availableSlotOffsetList for SBFD symbols is same as the number of values configured in availableSlotOffsetList for non-SBFD symbols.

From our perspective, it is not necessary to restrict the number of values configured in availableSlotOffsetList for SBFD symbols and non-SBFD symbos. The available UL transmission occasions are different in SBFD symbols and non-SBFD symbols. We prefer independent availableSlotOffsetLists configurations for SRS Resource Sets in SBFD and non-SBFD symbols. The number of values configured in availableSlotOffsetList for SBFD symbols and non-SBFD symbos can be different.

- Separate configurations of availableSlotOffsetList for SRS Resource Sets in SBFD and non-SBFD symbols is supported.

UL power control parameters and spatial relation

Separate configurations of PCMAX

In RAN1#120bis meeting, whether to support separate configurations of p-Max for SBFD and non-SBFD symbols was discussed and no consensus was reached.

#### Proposal 2-12 Proposed Agreement: Support separate configurations of p-Max for SBFD and non-SBFD symbols. - Send an LS to RAN4 to inform the agreement and ask whether RAN4 sees any issue.

In previous meeting, it was agreed that separate P0AlphaSets for SBFD symbols and non-SBFD symbols, which already achieve separate power control for SBFD symbols and non-SBFD symbols. It is not necessary to support separate configurations of p-Max for SBFD and non-SBFD symbols. As commented by some companies, the motivation to configure a new lower p-Max for SBFD symbol/slot  is to limit the impact of UE-to-UE CLI. In our view, it is not benifical for UL coverage and object the motivation of SBFD operation.

In addition, RAN1 spec only see PCMAX, which is set by UE and within the following bounds defined in RAN4：

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

Thus, whether to support separate configurations of p-Max for SBFD and non-SBFD symbols is RAN4 issue, which should be determined by RAN4.

- Whether to support separate configurations of p-Max for SBFD and non-SBFD symbols is up to RAN4.

In addition, multiple parameters is related with maximum output power PCMAX. If we want to support separate for maximum output power PCMAX, except for p-Max, other parameters also needs to be discussed, such as UE power class, and UE maximum output power reduction (MPR/∆MPR). Whether one or multiple PCMAX related parameters needs further discussion, which also needs the involvement of RAN4.

- Whether to apply separate UE power class for SBFD and non-SBFD symbols needs further discussion.

PUSCH transmit power

In legacy, MRB,b,f,cPUSCH(i) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission occasion i. For RA type 0 for PUSCH, when an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. So only the PRBs within UL usable PRBs are used to calculate the  PUSCH transmit power.

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l   [dBm]

- Only the PRBs within UL usable PRBs are used to calculate the  PUSCH transmit power.

In the above formular, ΔTF  =10log102BPRE⋅Ks-1⋅βoffsetPUSCH corresponds to modulation scheme and channel coding rate used for PUSCH transmission, where  BPRE is computed as BPRE=r=0C-1KrNRE, and NRE is a number of resource elements determined as NRE=N⋅MRB,b,f,cPUSCH(i)⋅j=0Nsymb,b,f,cPUSCHi-1Nsc,dataRB(i,j), where Nsymb,b,f,cPUSCHi is a number of symbols for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c. For PUSCH repetition Type B with segmentation around boundary of SBFD symbols and non-SBFD symbols, it needs to discuss whether  Nsymb,b,f,cPUSCHi is a number of symbols for norminal repetition in transmission occasion i, or actual repetition in SBFD symbols or non-SBFD symbols.

- For PUSCH repetition type B, whether  Nsymb,b,f,cPUSCHi is a number of symbols for norminal repetition or actual repetition needs further discussion.

Separate UL spatial relation

In RAN1#119 meeting, it was agreed that same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols. It can be applied to the scenario where SBFD and non-SBFD share a same spatial relation.

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

In RAN1#119 meeting, whether to support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario was discussed but no consensus was reached.

#### Proposal 2-10 [HP] Proposed Agreement: Support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL. - It does not preclude using same beam/spatial relation for SBFD and non-SBFD symbols.

One reason to support separate spatial domain comes from the CLI handling, such as the preferred or non-preferred beam is different across SBFD slot and non-SBFD slots. Second reason is different antenna configurations are used by gNB,  so different beam may be selected. So in addition to separate power control, we are also supportive of separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL. If separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario is supported, separate power control parameters for SBFD symbol and non-SBFD symbol can be achieved by associated separate indicated TCI state.

In exsting R18 unified TCI MAC CE for M-TRP, one TCI codepoint can map to separate TCI states for two TRPs. Some companies support to reuse similar scheme for different symbol type, e.g, re-interpret TRP domain into time domain. However, SBFD and mTRP are different feature. There is no need to associate different TRP with different symbol type. New MAC CE can be introduced to indicate separate TCI state for SBFD symbol and non-SBFD symbol. It can use similar structure as R18 unified TCI MAC CE for M-TRP.

- Support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL.

- Introduce a new MAC CE to indicate separate TCI state for SBFD symbol and non-SBFD symbol

- No RRC impact

Figure 4: Separate TCI state and power control for different symbol type

UL power control is not associated within TCI state

In R17, one default power control parameters set for PUCCH, PUSCH and SRS is configured in BWP-UplinkDedicated when UL power control is not associated within TCI state. One set of default power control parameters would not applied to the case of different symbol type. So in the case of  UL power control is not associated within TCI state, two UL PC parameter settings for PUSCH/PUCCH/SRS can be configured in BWP-UplinkDedicated. Each set of default power control parameters is applied to SBFD symbol and non-SBFD symbol, respectively.

- For the case of UL power control is not associated within TCI state, support a first and a second UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated.

PHR

In RAN1#120bis, separate PHR reporting associated with UL transmissions in SBFD symbols and non-SBFD symbols were discussed by companies since separate power control for SBFD symbols and non-SBFD symbols is supported. It is beneficial for gNB to schedule appropriate MCS and frequency resource for UL transmissions in SBFD symbols and non-SBFD symbols. From our perspective, separate PHR reporting associated with UL transmissions in SBFD symbols and non-SBFD symbols can be supported. The structure of new MAC CE is up to RAN2. It can use similar structure as PHR for M-TRP.

- Support separate PHR reporting associated with UL transmissions in SBFD symbols and non-SBFD symbols.

Frequency Hopping for PUSCH repetition Type B

In current specification, frequency hopping of PUSCH repetition type B is determined by norminal repetition, thus all actual repetition belonging to one nominal repetition share a same frequency locations, without extra frequency hopping. E.g, in case of inter-repetition frequency hopping, the starting RB for an actual repetition within the n-th nominal repetition  is given by:

RBstart(n)=RBstart,                                           n  mod2=0(RBstart+RBoffset)mod NBWPsize, n mod2=1

In RAN1#120bis, it was agreed that a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.  For actual repetition in SBFD symbol, PUSCH frequency hopping range should within in the UL usable PRBs. Thus, frequency offset between two frequency hops  in SBFD symbols can be configured seperatly. In addition, FH calculation should be udated to only consider the UL usable PRBs.

For PUSCH repetition type B with inter-repetition frequency hopping and actual repetition in SBFD symbol, similar formular for PUSCH intra-slot frequency hopping and PUSCH inter-slot frequency hopping in SBFD symbols can be reused. Thus, the starting RB for an actual repetition in SBFD symbol within the n-th nominal repetition  is given by:

RBstartn=RBstart,                                            n  mod2=0RBUL SB start+(RBstart-RBUL SB start+RBoffset)mod NULSBsize, n mod2=1

Where

- For PUSCH transmissions with Configuration 2, RBstart is the starting PRB index with reference to the start of UL active BWP after applying RB offset between non-SBFD symbols and SBFD symbols, i.e., RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

- RB offset is the frequency hopping offset for actual PUSCH repetition in SBFD symbols

- Note: Definition of n is unchanged from existing specifications

- Support separate FH offsets for actual PUSCH repetition in SBFD symbols and non-SBFD symbols respectively.

- For determining starting RB for an actual repetition in SBFD symbol within the n-th nominal repetition  is given by:

RBstartn=RBstart,                                            n  mod2=0RBUL SB start+(RBstart-RBUL SB start+RBoffset)mod NULSBsize, n mod2=1

Where

- For PUSCH transmissions with Configuration 2, RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

- RB offset is the frequency hopping offset for actual PUSCH repetition in SBFD symbols

Multi-carrier scenario

In RAN1#120 meeting, it was agreed to support SBFD operation on one TDD carrier in multi-carrier scenario. FFS is whether/how to handle half-duplex CA case.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

In RAN1#120bis meeting, following proposal was discussed.

#### Proposal 3-2 Proposed Agreement: For SBFD operation in half-duplex CA scenario, the existing directional collision handling for half-duplex TDD CA is reused by treating SBFD symbols as flexible symbols.

For half-duplex CA case, the related specification is captured in TS 38.213 as follows.

If a UE is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the configured multiple serving cells, UE determine reference cell(s) first and then handle collision between  reference cell and another cell among the multiple cells.  In most cases, though transmissions or receptions on reference cells have higher priority than that of other cells for almost collision cases,  the legacy reference cell(s) determination rules can be reused. There is not necessary to introduce new reference cell(s) determination rules.

Based on previous agreement, SBFD symbols are configured in DL symbol/ flexible symbols configured in tdd-UL-DL-ConfigurationCommon. SBFD symbol may be used for DL reception or UL transmission, which is same as legacy flexible symbols. Thus, the existing directional collision handling for half-duplex TDD CA is reused by treating SBFD symbols as flexible symbols.

11.1 in TS 38.213 If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the configured multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among - the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells, where the symbol is configured by higher layers as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured by higher layers to receive PDCCH, PDSCH or CSI-RS on the symbol. And if another cell among the cells configured with directionalCollisionHandling-r16 operates in the same frequency band as the reference cell, the UE does not expect - a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, - tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and - to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, …

- For SBFD operation in half-duplex CA scenario, the existing directional collision handling for half-duplex TDD CA is reused by treating SBFD symbols as flexible symbols.

UE collision handling

For collision Case 5, it was agreed that re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission. The remaining open issue is whether SBFD-aware UEs are allowed to receive within the entire DL BWP in the SSB symbol, or still within the DL usable PRBs. In RAN1#118bis meeting, following three options were discussed.

- Proposal 3-3b Proposed Agreement: For SSB symbols configured with SBFD subbands, down select from the following options. - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Option 2: The SBFD configuration is not applicable to a slot consisting of SSB symbols. - Option 3: The SBFD configuration is not applicable to a TDD pattern period with slot(s) consisting of SSB symbols.

For Option 2, if SBFD configuration is not applicable to a slot consisting of SSB symbols, it means SBFD operation in SSB symbol is disabled and fallback to legacy DL symbol, which can make more transition points in a TDD pattern period. Such as there are two SSB in one slot in Figure 5 (b), there is additonal transition points caused by this fallback. It will increase the implementation complexity in gNB side. Thus, SSB reception within UL usable PRBs in Figure 5 (b) are not allowed. If DL reception only within the DL usable PRBs and left UL subband empty, it would waste some resources but not increase UE implementation complexity. From our perspective, we think SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. The following working assumption should be confirmed.

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

| (a) | (b) |
| --- | --- |

Figure 5: SBFD operation in SSB symbols

- Confirm the following working assumption:

- For SSB symbols configured with SBFD subbands,

-	Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

-	Note: The SSB block is assumed to be within DL subband

For collision handling in SBFD symbols for SBFD aware UEs, whether to firstly handle SBFD specific collision between transmissions/receptions and unusable resources (e.g. invalid symbol type for Configuration 1, Tx/Rx occasion mapped to SBFD and non-SBFD symbols etc.) was discussed.  Following there options for down selection.

#### Proposal 3-2a Proposed Agreement: For collision handling in SBFD symbols for SBFD aware UEs, down select one from the following options - Option 1 (From Round 1): - Step 1: Resolving SBFD specific collision between transmissions/receptions and unusable resources (e.g. invalid symbol type for Configuration 1, Tx/Rx occasion mapped to SBFD and non-SBFD symbols etc.) , if any. - Step 2: Resolving the collision as in legacy, if any. - Option 2 (From Samsung): - Step 1: Resolving SBFD specific collision between PUCCH with repetitions/PUSCH and unusable resources, if any. - Step 2: Resolving the collision as in legacy, if any. - Step 3: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any. - Option 3 (New): - Step 1: Resolving the collision as in legacy, if any. - Step 2: Resolving SBFD specific collision between transmissions/receptions and unusable resources, if any.

Case 1                                                                                                          Case 2

Figure 6: Collision handling order in SBFD operation

As illustrated in Case 1, the PUCCH in invalid symbol type would be cancelled if SBFD specific collision between transmissions/receptions and unusable resources is handled firstly. It would lead to UCI dropping in case of PUCCH without repetitions. So Option 2 is proposed by some company to ensure the performance for PUCCH transmission without repetitions. However, same issues exist in legacy collision handling and not be optimized. So it is necessary to introduce additional handling rule in SBFD operation. It would lead to additional specification impacts. We prefer other Options for simplicity.

As shown in following, legacy collison includes collisions between PUCCHs or between PUCCH and PUSCH, collison due to  cell DRX operation, UL transmission collide with the DL symbol or SSB symbol, or UL transmission collison with high priority transmissions, etc.

TS38.213 9 UE procedure for reporting control information When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority index other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission due to cell DRX operation [11, TS 38.321] or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 including repetitions if any, - first, the UE resolves the overlapping for PUCCHs with repetitions as described in clause 9.2.6, if any - second, the UE resolves the overlapping for PUCCHs without repetitions as described in clauses 9.2.5 - third, the UE resolves the overlapping for PUSCHs and PUCCHs with repetitions as described in clause 9.2.6 - fourth, the UE resolves the overlapping for PUSCHs and PUCCHs without repetitions as is subsequently described in this clause. After resolving the overlapping for PUCCH and/or PUSCH transmissions, and if cell DRX is activated for a serving cell and a PUCCH or PUSCH transmission would overlap with the non-active period of cell DRX of the serving cell, the UE does not transmit the PUCCH if HARQ-ACK information is not multiplexed in the PUCCH, or does not transmit the PUSCH if HARQ-ACK information is not multiplexed in the PUSCH and the PUSCH is not associated with a corresponding PDCCH, respectively.

In our view, UE should first handle SBFD specific collision between transmissions/receptions and unusable resources the overlapping PUCCH/PUSCH transmissions, and then handle the collision as in legacy. So we support Option 1. At last, the DL reception vs. UL transmission collisions within SBFD symbols can be handled.

- For collision handling in SBFD symbols for SBFD aware UEs,

  - Step 1: Resolving SBFD specific collision between transmissions/receptions and unusable resources (e.g. invalid symbol type for Configuration 1, Tx/Rx occasion mapped to SBFD and non-SBFD symbols etc.) , if any.

  - Step 2: Resolving the collision as in legacy, if any.

  - Step 3: Resolving the DL reception vs. UL transmission collisions within SBFD symbols

Conclusion

In this contribution, we made the following observations and proposal.

- Only cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

- Explicit indication of transition periods between SBFD and non-SBFD symbols is not supported.

- If transient period is reserved in the whole channel bandwidth, invalid symbol for SBFD->UL(non-SBFD) switching needs to be indicated.

- Two new RRC parameter can be introduced to indicate the invalid symbol for SBFD->UL(non-SBFD) switchin and DL(non-SBFD)->SBFD switching.

- For PUSCH repetition type B, segmentation around boundary of SBFD symbols and non-SBFD symbols is supported with either Configuration 1/2.

- For PUSCH repetition Type B with Configuration 1, UE drops an actual repetition if the actual repetition is in the invalid symbol type.

- For frequency resource allocation Type 1 with RBG for PDSCH or PUSCH in a single slot by DCI format 0_2/0_3/1_2/1_3 based scheduling (without repetition or TBoMS),

- When an allocated RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be assigned for PDSCH reception and only the PRBs within UL usable PRBs are considered to be assigned for PUSCH transmission.

- Allocated RBGs that fall outside DL usable PRBs are considered to be not assigned and should not be used for PDSCH resource mapping.

- The number of PRBs of a CSI-RS resource within each DL subband should be ≥ min⁡(24,NDL-SBsize), where  NDL-SBsize is the number of DL usable PRBs in one DL subband.

- For dynamic determination of the precoding bundling for a PDSCH, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband where the PDSCH is located.

- Assigned PRBs in PDSCH scheduled by DCI format 1_0 in CSS are all inside DL usable PRBs by scheduling.

- For PDSCH resource mapping, there can be two alternatives for the symbol type of NZP CSI-RS:

- Alt 1: UE assume P/SP NZP CSI-RS receptions is configuration 2.

- Alt 2: The valid symbol type of CSI-RS resource is same as the valid symbol type for CSI derivation of associated CSI report.

- For PUSCH transmissions with Configuration 2, in case PUSCH frequency hopping is enabled, starting PRB of the first hop for PUSCH repetitions/transmissions in SBFD symbols is determined according to Equation 1-C2 as agreed for PUSCH repetitions/transmissions in SBFD symbols without frequency hopping.

- For SBFD-aware UEs, for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols), support Option A and B.

- For Option A, it introduces occasions used for a CSI report, CSI-RS transmission occasions within SBFD symbols or non-SBFD symbols

- For Option B, it defines a new type of sub-configuration, corresponding to CSI-RS transmission occasions subset within SBFD symbols or non-SBFD symbols

- The valid CLI reference resource slot should comprises at least one higher layer configured SBFD or flexible symbol and not fall within a configured measurement gap.

- For a CORESET and a search space configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of DL usable PRBs in SBFD symbols

- No enhancements for SBFD-aware UEs in Rel-19.

  - It is up to gNB to avoid the PDCCH candidate overlap with the boundary of DL usable PRBs in SBFD symbols

- Separate configurations of availableSlotOffsetList for SRS Resource Sets in SBFD and non-SBFD symbols is supported.

- Whether to support separate configurations of p-Max for SBFD and non-SBFD symbols is up to RAN4.

- Whether to apply separate UE power class for SBFD and non-SBFD symbols needs further discussion.

- Only the PRBs within UL usable PRBs are used to calculate the  PUSCH transmit power.

- For PUSCH repetition type B, whether  Nsymb,b,f,cPUSCHi is a number of symbols for norminal repetition or actual repetition needs further discussion.

- Support separate beam/spatial relations for SBFD and non-SBFD symbols for single-TRP scenario for both UL and DL.

- Introduce a new MAC CE to indicate separate TCI state for SBFD symbol and non-SBFD symbol

- No RRC impact

- For the case of UL power control is not associated within TCI state, support a first and a second UL PC parameter settings for PUSCH/PUCCH/SRS configured in BWP-UplinkDedicated.

- Support separate PHR reporting associated with UL transmissions in SBFD symbols and non-SBFD symbols.

- Support separate FH offsets for actual PUSCH repetition in SBFD symbols and non-SBFD symbols respectively.

- For determining starting RB for an actual repetition in SBFD symbol within the n-th nominal repetition  is given by:

RBstartn=RBstart,                                            n  mod2=0RBUL SB start+(RBstart-RBUL SB start+RBoffset)mod NULSBsize, n mod2=1

Where

- For PUSCH transmissions with Configuration 2, RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

- RB offset is the frequency hopping offset for actual PUSCH repetition in SBFD symbols

- For SBFD operation in half-duplex CA scenario, the existing directional collision handling for half-duplex TDD CA is reused by treating SBFD symbols as flexible symbols.

- Confirm the following working assumption:

- For SSB symbols configured with SBFD subbands,

-	Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

-	Note: The SSB block is assumed to be within DL subband

- For collision handling in SBFD symbols for SBFD aware UEs,

  - Step 1: Resolving SBFD specific collision between transmissions/receptions and unusable resources (e.g. invalid symbol type for Configuration 1, Tx/Rx occasion mapped to SBFD and non-SBFD symbols etc.) , if any.

  - Step 2: Resolving the collision as in legacy, if any.

  - Step 3: Resolving the DL reception vs. UL transmission collisions within SBFD symbols

References

- RP-2401614	Revised WI on evolution of NR duplex operation

- TR38.858, V18.0.0, Study on Evolution of NR Duplex Operation, (Release 18)

- Draft_Minutes_report_RAN1#118bis

- R1-2409088  Summary #3 of 	SBFD TX/RX/measurement procedures, Moderator (CATT)

- R1-2502799, Summary #2 of SBFD TX/RX/measurement procedures, Moderator (Xiaomi)

- R4-2504767, LS on the transient period for SBFD operation,