3GPP TSG RAN WG1#121                                                                 R1-2503516

St Julian's, Malta, 19 - 23 May, 2025

Agenda Item:     9.4.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on line coding, FEC, CRC, repetition aspects for Ambient IoT

Document for:	Discussion and decision

Introduction

In RAN#107, a revised WID for Ambient IoT (Internet of Things) was approved with the following objectives [1].

- RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212. Applying or not applying the FEC to D2R is specified by ensuring it is under the reader control and applies to all devices targeted by the reader. - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this contribution, we present our opinions on physical channels design – line coding, FEC, CRC, repetition aspects. The following agreement was achieved in the last meeting.

Agreement For the initial values of the shift register of the CC encoder, the initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits defined in TS 36.212. - Note: K is the constraint length. - For discussion on timing relationships in 9.4.4, the entire processing time for encoding (including finishing CRC encoding) should be taken into account Agreement When CRC is attached to a PRDCH or PDRCH transmission, when the number of information bits is ≤ 24 bits, CRC-6 is used. Otherwise, when the number of information bits is > 24 bits, CRC-16 is used. Agreement There is no case in D2R or R2D where CRC is not attached. Conclusion RAN1 will not address the FFS in the agreement from RAN1#120: Agreement When CRC is attached to a PRDCH or PDRCH transmission, - When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error: - Alt. 1: 24 - Alt. 2: 56 - FFS impact of segmentation, if any - Note: impact may not be in RAN1 Agreement The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table: - FFS further down-selections of Tchip, Tb, and R to be supported - Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed.. Agreement For block-level repetition of a D2R transmission, the repeated blocks are transmitted in the single PDRCH of the D2R transmission.

Discussion

Small frequency shift (SFS)

R value and D2R chip length

An agreement was achieved in the last meeting as follows [2]:

Agreement The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table: - FFS further down-selections of Tchip, Tb, and R to be supported - Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed..

Take the time domain information bit length Tb with 266.67 μs as an exmple, the chip length of R = 1 with 133.33 μs and R = 2 with 66.67 μs is shown in Figure 2.1.1-1.

Figure 2.1.1-1. An example of MS option with R=1 and R=2.

Next step is discuss how to further down-selections of Tchip, Tb, and R to be supported. From our perspective, the potential values of R should also consider harmonic interference in D2R FDMA, according to the frequency spectrum summarized in [2], the selection of R = 2n can avoid the odd harmonics interference for the parallel transmission of FDMA. It is straightforward that defining R = 2n can avoid odd harmonics. For R = 2n within {1, 2, 4, 8, 16, 32, 64, 128, 256} in the table, the Tchip value with {11.11, 5.56, 2.78, 1.39, 0.69}μs in the column can be excluded. Correspondingly, Tb value with {22.22, 11.11, 5.56, 2.78, 1.39}μs in the row also needs to be deleted. The candidate Tb values is {266.67, 133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 2.08, 1.04}μs and Tchip values is Tb/(2R). To achieve the Rmax with 256, the sampling clock should be large as 3.84MHz.

In addition, to provide sufficient gap between frequency resources to avoid, R=2 can be precluded from the set. Then there are 8 candidate values of R can be adopt within {1, 4, 8, 16, 32, 64, 128, 256}. Reader can indicate each one of them to device.

After delete R=2 in the table, there are two bit duration 2.08μs and 1.04μs with only R=1, we can further down-select one of them. To achieve the competitive peak data rate larger than 640 kbps, Tb with 2.08μs can be deleted.

Observation 1: The number of 8 values with {1, 4, 8, 16, 32, 64, 128, 256} for R is sufficient to meet the system capacity requirements.

Proposal 1: The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

|  | Tchip (μs) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.52 |
| 266.67 | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 | R=256 |
| 133.33 |  | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 66.67 |  |  | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=4 | R=8 | R=16 | R=32 |
| 16.67 |  |  |  |  | R=1 |  | R=4 | R=8 | R=16 |
| 8.33 |  |  |  |  |  | R=1 |  | R=4 | R=8 |
| 4.17 |  |  |  |  |  |  | R=1 |  | R=4 |
| 1.04 |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |

The indication of frequency domain resource allocation

According to 9.4.4[3], the details of the frequency domain resource allocation will be handled under this agenda item. Reader needs to indicate Y, D2R chip length/D2R bit length, and R to the device, so that the device can obtain the frequency domain resource for the corresponding D2R transmission. When indicating these values to device, reader should be based on the table in Proposal 1.

As for which one should be indicated within chip length or bit length, from our perspective, indicating bit length is more suitable. As we all know, Tb is Time duration corresponding to a bit, where the bit is that after FEC if applied, without considering the repetition,

data rate kbps=bit rate kbps                                                (1)

At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions. For a given coding rate, the bit rate is same too for FDMed Msg1, while chip rate may different with multiple R values. The chip rate in Hz 1/Tc depends on SFS factor R and bit rate 1/Tb as

chip rate (Hz)=1Tc=2RTb=2R×bit rate (bps).                                         (2)

So under this case, only one D2R bit length is indicated to achieve the same data rate, which may save the bit overhead compared with indicate multiple chip lengths.

Observation 2: Reader indicate Y, D2R bit length, and R to the device to obtain the frequency domain resource for the corresponding D2R transmission.

Then we will discuss how to indicate details of the frequency domain resource allocation under the same data rate for FDMed transmissions.

As analyzed above, at least for FDMed Msg1 transmission in CBRA, only one Tb is needed to indicate to achieve the same data rate. There are 8 candidate values of Tb within the table, i.e., 266.67μs, 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 1.04μs, which may needs 3bits to indicate this value.

Given a specific Tb, a bitmap with 9 bit can be used to indicate which R can be used for device. Specifically, each one bit corresponds to a column, e.g., bit equal to 1 indicates the R value in the table can be used for device, device can also get Tc based on R and Tb values, while bit equal to 0 indicates the R value can not be used for device or no existence R in the table.

Take the following table as an example, if Tb with 266.67μs is indicated to device and the bitmap is 101100000, it means R=1, R=4, R=8 can be used for FDMed transmission.

Table 2.1.2-1. An example of bitmap

|  | Tchip (μs) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.52 |
| 266.67 | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 | R=256 |
| 133.33 |  | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 66.67 |  |  | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=4 | R=8 | R=16 | R=32 |
| 16.67 |  |  |  |  | R=1 |  | R=4 | R=8 | R=16 |
| 8.33 |  |  |  |  |  | R=1 |  | R=4 | R=8 |
| 4.17 |  |  |  |  |  |  | R=1 |  | R=4 |
| 1.04 |  |  |  |  |  |  |  |  | R=1 |
| bitmap | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |

Moreover, reader can indicate the number of FDMed device implicitly by the total of “1” bit.

In a word, 12 bit is needed to indicate the frequency domain resource for the corresponding D2R transmission. We have a proposal as follows:

Proposal 2: For the same data rate for FDMed transmissions, e.g., FDMed Msg1 transmission, 12 bit is needed to indicate the frequency domain resource for the corresponding D2R transmission based on the potential value in the table.

- 3 bit is needed for 8 candidate values of Tb within the table, i.e., 266.67μs, 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 1.04μs

- 9 bit with a bitmap is needed for R and Y, each one bit corresponds to a Tchip

D2R Code Rate

According to the discussion in previous meeting, D2R code rates is calculated by the formula r = (1/k) * (1/p), where 1/k is the code rate of the convolutional encoder and p is the number of repetitions.

For the code rate of the convolutional encoder 1/k, we have a proposal in the last meeting but not achieved an agreement yet.

[H] Proposal 4.2-1-v3 If FEC is applied for a D2R transmission, the code rate of FEC additionally supports values 1/2. - The 1/2 code rate is achieved by generating dk0,dk1 according to the polynomial G0 and G1, i.e. without using G2

From our perspective, we only support the code rate of FEC with 1/3 if FEC is applied for a D2R transmission. Other code rates would increase complexity, so we don’t see the necessity to support other code rate than the mother code rate 1/3. Furthermore, companies propose to support a higher code rate by puncturing, while higher code rates than 1/3 can be achieved through no FEC + repetition (e.g., repetition = 2), a flexible code rate can also be achieved by adopting the different repetition number. It may cause redundant to discuss puncturing or other alternative schemes to achieve the same goal.

Although as companies mentioned, under the same code rate 1/2, puncturing has better performance than no FEC + repetition (e.g., repetition = 2), the motivation of puncturing is not clear for us. If we focus on a higher code rate, no FEC (which is equivalent to code rate = 1) is more suitable to support a competitive D2R peak data rate, e.g., larger than 640 kHz. If we focus on a flexible code rate, as we mentioned before, no FEC + repetition (e.g., repetition = 2) can achieve the same goal. If we focus on a better performance, then the code rate of FEC with 1/3 and repetition with twice can have a better performance than code rate 1/2 by puncturing.

Proposal 3: Only support the code rate of FEC with 1/3 if FEC is applied for a D2R transmission.

Observation 3: To achieve the goals with supporting a higher performance, or a higher code rate, or a flexible code rate, it may cause redundant to discuss other alternative schemes like puncturing.

Regarding the repetition number p, we have a proposal in the last meeting but not achieved an agreement yet.

Proposal2 For the number of block-level repetitions, {1, 2, [4]} repetitions are supported. - Note: When the number of block-level repetition is 1, it indicates no repetition. - Note: no other candidate value than 4 is considered

As block level repetition from TR38.769 is supported for D2R repetition and the repeated blocks are transmitted via a single PDRCH, restricting to support the repetition number at 1 and 2 in order to minimize redundancy. Considering to reduce the signaling overhead, it is better not to support other candidate repetition number with 4. For the repetition number 2, only one bit is needed.

Proposal 4: For the number of block-level repetitions, {1, 2} repetitions are supported.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

Conclusion

The following observations and proposals are achieved:

Observation:

Observation 1: The number of 8 values with {1, 4, 8, 16, 32, 64, 128, 256} for R is sufficient to meet the system capacity requirements.

Observation 2: Reader indicate Y, D2R bit length, and R to the device to obtain the frequency domain resource for the corresponding D2R transmission.

Observation 3: To achieve the goals with supporting a higher performance, or a higher code rate, or a flexible code rate, it may cause redundant to discuss other alternative schemes like puncturing.

Proposal:

Proposal 1: The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

|  | Tchip (μs) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.52 |
| 266.67 | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 | R=256 |
| 133.33 |  | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |
| 66.67 |  |  | R=1 |  | R=4 | R=8 | R=16 | R=32 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=4 | R=8 | R=16 | R=32 |
| 16.67 |  |  |  |  | R=1 |  | R=4 | R=8 | R=16 |
| 8.33 |  |  |  |  |  | R=1 |  | R=4 | R=8 |
| 4.17 |  |  |  |  |  |  | R=1 |  | R=4 |
| 1.04 |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |

Proposal 2: For the same data rate for FDMed transmissions, e.g., FDMed Msg1 transmission, 12 bit is needed to indicate the frequency domain resource for the corresponding D2R transmission based on the potential value in the table.

- 3 bit is needed for 8 candidate values of Tb within the table, i.e., 266.67μs, 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 1.04μs

- 9 bit with a bitmap is needed for R and Y, each one bit corresponds to a Tchip

Proposal 3: Only support the code rate of FEC with 1/3 if FEC is applied for a D2R transmission.

Proposal 4: For the number of block-level repetitions, {1, 2} repetitions are supported.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

Reference

- RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#107, Incheon, Korea, March 12-14, 2025.

- R1-2503107, “Summary #6 for coding aspects of physical channel design”, RAN1#120b, Wuhan, China, April 7th – 11th, 2025

- R1-2503104, “FL summary#4 on other aspects for Rel-19 Ambient IoT”, RAN1#120b, Wuhan, China, April 7th – 11th, 2025