3GPP TSG-RAN WG1 Meeting #121         						 	R1-2503517

St Julian's, Malta, 19 - 23 May, 2025

Agenda Item:     9.4.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on timing acquisition and synchronization for Ambient IoT

Document for:	Discussion and decision

Introduction

In recent years, IoT has attracted much attention in the wireless communication world. More ‘things’ are expected to be interconnected for improving productivity efficiency and increasing comforts of life. In order to meet extreme demands of target use cases, e.g., batteryless, extremely low power consumption, etc, an Ambient-IoT(A-IoT) work item was approved [1] to open new markets within 3GPP systems.

RAN1 scope for A-IoT is as follows:

General Scope - … The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. …

In this contribution, we discuss timing acquisition and synchronization for Ambient IoT.

Discussion

R2D signal design

Preamble

R2D preamble immediately preceding the transmission of PRDCH was studied, which is included at least for timing acquisition and indicating the start of R2D transmission in the time domain. A start-indicator part provides the start of the R2D transmission, and immediately precedes a clock-acquisition part.

Start-indicator part

Start-indicator part was discussed in the last meeting with two alternatives [2].

Agreement For the pattern of SIP of R-TAS, only the following 2 alternatives are considered for further down-selection: - Alt 1-2: ON-OFF with a ratio of 1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration - Alt 2-4: ON-OFF-ON-OFF with a ratio of 1:1:1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration Agreement For R-TAS, SIP duration of 1 OFDM symbol is adopted with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH - Note: device cannot assume the presence/absence of RF transmission prior to the SIP.

For Alt1-2, a single ON-OFF transmission is considered. The purpose of the high-voltage transmission is that the device can detect the falling edge to determine the start indicator part. The duration of the high-voltage can be shorter than low-voltage. However, in order to ensure that the device can correctly determine whether it is a high-voltage, it is necessary to determine a reasonable duration of high-voltage.The M values of CAP and PRDCH are 2, 6, 12 and 24, the OFF duration of SIP should be different from that of CAP and PRDCH to avoid false detection. Alt 1-2 as shown in fig.1 (a) that ON followed by OFF with a duration ratio of 1: 3 can be considered. In this case, the duration of OFF in SIP is 3/4 OFDM symbol. When M=2, 6, 12, 24 in CAP and PRDCH, the possible maximum OFF duration in CAP and PRDCH correspond to 1, 1/3, 1/6, 1/12 OFDM symbol, which is different from that of SIP.

Two implementation methods for device detection can be considered.

Method 1: the device can use a fixed threshold or determine the threshold based on other implementation for SIP detection.

Method 2: the device can update the threshold in real-time based on historical maximum peak, and the ON part can achieve threshold training.

As shown in Fig.2, it is a simple detection implementation where the threshold is determined based on the value of Peak finder and can be updated in real-time based on the ON part.

For Alt2-4, Multi-ON-OFF transmission is considered. It mainly consists of two parts, the first part (Part 1) is used for training detection threshold, and the second part (Part 2) is used for SIP detection. As shown in Figure 1 (b), the duration ON and OFF is equal in Part 1, and the duration ratio of ON and OFF is 1:3 in Part 2. Alt2-4 requires the device to detect part 1 first and determine the threshold for part 2 detection. Compared to Alt1-2, the OFF duration is shorter in Alt2-4, resulting in poorer reception reliability.

- Alt1-2                                                        (b) Alt2-4

Fig.1 SIP design of Alt1 and Alt2

Fig.2 One candidate of detection method in device

In this contribution, the simulation verified the MDR and FAR of Alt1-2, Considering that a M value of PRDCH is 6, the SIP pattern is 1000, and there is no RF transmission prior to the SIP. Table 1 shows the simulation results of MDR and FAR at an SNR of 22 dB. In this simulation, it is assumed that the device uses the ON part to perform threshold training.

Table1 The MDR and FAR of Alt1-2

| SIP pattern | SNR | MDR | FAR |
| --- | --- | --- | --- |
| 1000 | 22dB | 0.06% | 0% |

From Table 1, it can be seen that the MDR of Alt1-2 is 0.06% and FAR is around 0%, indicating that only using ON part in Alt1-2 for threshold training is enough and the duration of OFF can be distinguished from PRDCH and CAP.

Obsevation1: For Alt1-2 that ON followed by OFF with a duration ratio of 1: 3, the MDR can achieve below 1%, and FAR can reach around 0%. Therefore, the reliability of Alt 1-2 is enough, and the “OFF” duration can be distinguished from subsequent CAP and PRDCH.

Proposal 1: For start indicator part in the R2D preamble:

- Alt 1-2 should supported that ON followed by OFF with a duration ratio of 1:3.

Clock-acquisition part

The following agreements for CAP were achieved in the last meeting [2].

Agreement For CAP of R-TAS, following is adopted: - Option 1 for CAP of R-TAS from TR 38.769 is adopted where the CAP duration becomes proportionally shorter with increasing value of M, i.e. if for M = X, duration is Y OFDM symbol long, then for M = aX, duration is Y/a OFDM symbol long - Note: Duration without CP insertion is considered above, with CP insertion, the total duration may not be exactly proportional - Only following two alternatives for CAP pattern are considered for further down-selection to one alternative: - Alt 1: ON-OFF-ON-OFF - Alt 2: ON-OFF-ON

According to the agreement of the last meeting, option 1 for CAP of R-TAS from TR 38.769 is supported.

Based on the agreement and analysis of the contribution in section 9.4.1 [3], the values of M are 2, 6, 12, and 24, and CP handling Alt M2-1-1 can be used for M=24.

- When M ≤ 12, CP handling is not needed or can be up to device implementation.

- When the M value is larger (e.g., M >12), the last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’.

As shown in figure 3, if M = 24, the last 2 out of M OOK chips at the end of the OFDM symbol are ‘ON’.

Based on the CP handling Alt M2-1-1, it can ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge. But the insertion of CP will make the first ON duration inaccurate. To improve reliability, the duration of OFF can be used to determine the chip duration. Therefore, the pattern of CAP should be ON-OFF-ON-OFF.

M = 24

Fig.3 CP handling in option 1 for CAP

Proposal 2: For CAP in R2D,

- The pattern of CAP is ON-OFF-ON-OFF.

- CP handling Alt M2-1-1 is used for M=24.

  - The last 2 out of M OOK chips at the end of the OFDM symbol are ‘ON’, the two OFF parts is used to determine the chip duration.

Postamble

How to determine or derive the end of PRDCH transmission was discussed in previous meetings, the two options are captured in TR38.769[2] and the latest proposal  is as following.

TR38.769 To determine or derive the end of PRDCH transmission, the following options are studied: - Option 1: TBS information via implicit/explicit L1 R2D control information. - Option 2: Postamble at the end of PRDCH. (High Priority) Proposal 4-1d Manchester coding rule violation corresponding to M value for PRDCH is applied by reader’s implementation that can be used by the device to determine the end of PRDCH transmission - Note: From timing perspective, end of last chip of PRDCH is the end of R2D transmission - There is no consensus to specify R2D postamble

For option 1, when the payload of PRDCH is relatively small, the overhead of postamble will be higher compared to PRDCH. For option 2, R2D postamble indicates the end of PRDCH with high efficiency for large PRDCH payload by avoiding a large overhead of R2D control information to indicate the TB size. But if postamble is very short, the false detection may be high.

Considering the advantages and disadvantages of the two options, the postamble pattern is left to the reader’s implementation is better. The device needs to assume that the reader transmitted the postamble, and when it detects Manchester coding rule violation corresponding to M value for PRDCH, it is considered the end of PRDCH. Manchester coding rule violation needs to exclude the effects of CP insertion.

Proposal 3: To determine or derive the end of PRDCH transmission:

- Postamble is supported, and the pattern is determined based on reader’s implementation.

- Device assumes that reader will definitely transmit postamble.

  - D2R amble(s) design

Preamble and Midamble are supported in D2R, potentially for SFO estimation/time tracking and channel estimation. The following agreements were achieved in the last meeting.

Agreement For D2R midamble, for determining the presence and location of midamble(s) at the device: - Reader explicitly indicates the same interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information - FFS: details of signalling - FFS: whether the reader can explicitly indicate with one bit whether a midamble is additionally present at the end - Note: This does not preclude the support of having no midamble present in the D2R transmission Agreement For indicating the interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information, following is adopted: - Unit of interval is number of bits after FEC (if FEC is applied) and repetition (if repetition is applied) - FFS: the candidate values in terms of the unit of interval Agreement - For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n-1 - Value(s) of n - Long preamble/midamble is generated based on n = 5 - Working assumption: Short preamble/midamble is generated based on n=3 - Only 1-part preamble/midamble are supported for D2R - Preamble immediately precedes the PDRCH without any gap - Both long and short preamble and midamble are supported based on the working assumption on n - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH: - Short preamble and short midamble - Long preamble and long midamble Note: the case of short preamble and long midamble will not be supported - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH

Length of amble(s)

When the payload size of D2R is small, such as only 16 bits, the preamble can use a short sequence to reduce overhead. When the payload size of D2R is large, long sequence can be considered for preamble to improve the reader's reception reliability. According to the last meeting, when both preamble and midamble are present, they need have same length. When midamble is not present the reader explicitly indicates short or long preamble for PDRCH. However, considering that when the payload size of PDRCH is small, the midamble is not present. In this case, the preamble should be a short sequence by default which doesn’t need to be indicated explicitly. For short preamble/midamble, the working assumption can be conformed that the length of the sequence is 23-1.

Proposal 4: For the length of D2R amble(s),

- When midamble is not present,  preamble should be a short sequence by default.

- For short preamble/midamble, the working assumption can be conformed that short preamble/midamble is generated based on n=3.

Midamble rated information indication

In the last meeting, it has been agreed that reader explicitly indicates the same interval between consecutive midambles, and between the preamble and the first midamble.For the midamble at the end, reader can implicitly indicate whether there is a midamble at the end through interval indication. For example, as shown in Figure 4 (a), when the reader considers that a midamble is needed at the end, it can indicate an interval of Y1 bits. As shown in Figure 4 (b), when the reader considers that there can be no midamble at the end, it can indicate an interval of Y2 bits. Therefore, there is no need to explicitly indicate with one bit whether a midamble is additionally present at the end.

- A midamble is present at the end of PDRCH

- A midamble is not present at the end of PDRCH

Fig.4 Midamble transmission examples

Proposal 5: It is not necessary to explicitly indicate with one bit whether a midamble is additionally present at the end of PDRCH.

Insertion of X-ambles

For PDRCH transmission, an implementation example of ambles insertion is given with the following assumptions.

- The number of coded data bit after line coding is 22 bits, and these bit stream are denoted as e1,e2, …e22

- One short preamble and one short midamble are used with n = 3, i.e., length = 7; and these two ambles’ bit sequence are denoted as r1,r2,…r7 and m1,m2,…m7, respectively.

- The interval for amble insertion is 6 bits.

- No midamble is inserted at the end of D2R transmission.

Based on the above assumption, PDRCH  coded data bit stream together with ambles bit sequence insertion are illustrated with the following figure 5 (a) ~ figure 5 (f).

(a)

(b)

(c)

(d)

(e)

(f)

Figure 5: Insertion of X-ambles

When the modulation starts, the flow of bit streams can be divided into the following two procedures for easily understanding.

Procedure for preamble insertion: As shown in Figure 5 (a) (b) (c), the first bit of preamble sequence will be modulated firstly. Because the data flows in a stream can not stop moving in all baseband procedures in Device 1, the first coded data bit output of line coding (i.e., the first bit of PDRCH after line coding) should move into soft buffer (registers region) to wait all preamble sequence bits insertion to bit stream. In other words, all preamble sequence bits will be modulated first, then followed by coded data bit stream. As the number of preamble sequence bits is 7, 7 registers are needed for temporarily buffer coded data bit stream to realize preamble insertion. In addition, to guarantee that the buffered coded data bit stream can flow out from registers region on time to consecutively follow to preamble sequence bits steam, additional counting registers is needed (3 registers can count the length of preamble sequence, which is 7.)

Furthermore, for midamble insertion procedure in next section, for determining the location to insert midamble, additional counting registers is needed (3 registers can count the interval for ambles insertion, which is 6).

Procedure for midamble insertion: As shown in Figure 5 (d) (e) (f), until the sixth coded data bit output of line coding is modulated, the first midamble should be inserted. At this point, all midamble sequence bits will be modulated, then followed by following coded data stream. During the modulation process of midamble, the coded bit output of line coding should move into soft buffer (register region) to wait all midamble sequence bits insertion to bit stream and no bit in register is read for modulation. As the number of midamble sequence bits is 7, 14 registers (7 bit due to preamble and 7 bit due to the first midamble) are needed for temporarily buffer coded data bit stream to realize the first midamble insertion. Similarly, each subsequent insertion of a midamble will increase 7 registers. The additional registers for counting the midamble sequence insertion and interval of midamble insertion is 3 and 3, respectively.

To sum up, the registers used for buffering PDRCH can be calculated as:

Register for buffering PDRCH  = length of preamble + number of midamble * length of midamble.

The additional register used for counting the X-ambles sequence insertion and interval of X-ambles insertion can be calculated as:

Register size for counting = log2(length of X-ambles) + log2(interval for X-amble insertion).

For example,

- With one short preamble and one short midamble for PDRCH transmission, totally additional 14 + X bits registers is needed for device 1.

  - 14 bit registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

- With one long preamble and one long midamble for PDRCH transmission, totally additional 62 + X bits registers is needed for device 1.

  - 62 bits registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

- With one long preamble and two long midamble for PDRCH transmission, totally additional 93+X bits registers is needed for device 1.

  - 93 registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

Observation 2:

- With one short preamble and one short midamble for PDRCH transmission, totally additional 14 + X bits registers is needed for device 1.

  - 14 bit registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

- With one long preamble and one long midamble for PDRCH transmission, totally additional 62 + X bits registers is needed for device 1.

  - 62 bits registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

- With one long preamble and two long midamble for PDRCH transmission, totally additional 93+X bits registers is needed for device 1.

  - 93 registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

Obviously, if more than 1 midamble is inserted, too many registers will be needed, which is not realized for device 1. Hence, we suggest at most one midamble insertion within one PDRCH transmission to reduce device 1 cost/complexity.

Proposal 6: The maximum number of midamble should be constrained, and at least for long midamble case, no larger than 1.

- Conclusions

Obsevation1: For Alt1-2 that ON followed by OFF with a duration ratio of 1: 3, the MDR can achieve below 1%, and FAR can reach around 0%. Therefore, the reliability of Alt 1-2 is enough, and the “OFF” duration can be distinguished from subsequent CAP and PRDCH.

Proposal 1: For start indicator part in the R2D preamble:

- Alt 1-2 should supported that ON followed by OFF with a duration ratio of 1:3.

Proposal 2: For CAP in R2D,

- The pattern of CAP is ON-OFF-ON-OFF.

- CP handling Alt M2-1-1 is used for M=24.

  - The last 2 out of M OOK chips at the end of the OFDM symbol are ‘ON’, the two OFF parts is used to determine the chip duration.

Proposal 3: To determine or derive the end of PRDCH transmission:

- Postamble is supported, and the pattern is determined based on reader’s implementation.

- Device assumes that reader will definitely transmit postamble.

Proposal 4: For the length of D2R amble(s),

- When midamble is not present,  preamble should be a short sequence by default.

- For short preamble/midamble, the working assumption can be conformed that short preamble/midamble is generated based on n=3.

Proposal 5: It is not necessary to explicitly indicate with one bit whether a midamble is additionally present at the end of PDRCH.

Observation 2:

- With one short preamble and one short midamble for PDRCH transmission, totally additional 14 + X bits registers is needed for device 1.

  - 14 bit registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

- With one long preamble and one long midamble for PDRCH transmission, totally additional 62 + X bits registers is needed for device 1.

  - 62 bits registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

- With one long preamble and two long midamble for PDRCH transmission, totally additional 93+X bits registers is needed for device 1.

  - 93 registers for temporary buffer because of ambles insertion, X = log2(length of X-ambles) + log2(interval for X-amble insertion) bits registers for counting

Proposal 6: The maximum number of midamble should be constrained, and at least for long midamble case, no larger than 1.

- References

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR. RAN#106, Madrid, Spain, December 9-12, 2024.

- Chair notes RAN1#120bis eom

- R1-2503515  Discussion on modulation aspects of physical channels design for Ambient IoT