3GPP TSG RAN WG1 #121                                                R1-2503520

Saint Julian, Malta, May 19th – 23th, 2025

Agenda Item:	9.5.2

Source:	Spreadtrum, UNISOC

Title: 	Discussion on on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify OD-SIB1 for case 2 for UEs in idle/inactive state.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

We will share our view on OD-SIB1 for UEs in idle/inactive state.

OD-SIB1 Tx beams

In RAN1#120bis [2], it was agreed that whether PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the transmitted PRACH for UL-WUS or not depend on 1-bit indication in UL WUS configuration.

Agreement The following agreement is updated as follows: The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. - Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions - Alt1: The information is provided in UL-WUS config - Alt2: The information is provided in RAR in response to UL-WUS transmission - Alt3: No additional information is provided

A remaining issue is whether/how to indicate additional information related to SSBs associated with PDCCH monitoring occasions. In our view, transmit PDCCH monitoring occasions corresponding only to the SSB associated with the transmitted PRACH for UL-WUS has network energy saving gain. If there are additional beams to transmit OD-SIB1, we can follow legacy OD-OSI procedure, e.g., PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. It is unnecessary to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

- Support option 3 for indication of OD-SIB1 Tx beams.

Option 3: No additional information is provided.

OD-SIB1 transmission window

In RAN1#120bis [2], the value range for od-sib1-duration is undetermined. In our view, we can reuse legacy OD-OSI configuration, e.g., from 5 to 5120 slots (5, 10, 20, 40, 80, 160, 320, 640, 1280, 5160 slots). In some cases, option 1 can cover option 2. For example, 15kHz SCS, 160/320/640/1280/2560 slots equal to 1/2/4/8/16 legacy SIB1 periodicities.

Agreement RAN1 to discuss the value range for od-sib1-duration. For example, - Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point. - Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity). - Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported) - Other options are not precluded.

- Support option 1 for od-sib1-duration.

Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

Feedback from gNB

In RAN1 #119 [3], it was agreed that the contents of RAR in response to SI request are included in RAR in response to UL WUS, e.g., RAPID in RAR. We think only RAPID in RAR is enough, there is no need to provide other information in RAR.

Agreement At least the contents of RAR in response to SI request are included in RAR in response to UL WUS

- Don’t support additional contents in RAR for OD-SIB1 other than contents of RAR for SI request.

Conclusion

We have the following observations and proposals.

- Support option 3 for indication of OD-SIB1 Tx beams.

Option 3: No additional information is provided.

- Support option 1 for od-sib1-duration.

Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

- Don’t support additional contents in RAR for OD-SIB1 other than contents of RAR for SI request.

Reference

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, RAN#105, Sep 9-12, 2024.

- 3GPP RAN1#120bis Chair’s Notes.

- 3GPP RAN1#119 Chair’s Notes.