3GPP TSG RAN WG1 #121                                                       R1-2503521

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.5.3

Source:	Spreadtrum, UNISOC

Title: 	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify adaptation of common signal/channel transmission.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we share our views on adaptation of PRACH in time domain.

Adaptation of SSB in time-domain

Regarding whether new RRC signaling is needed for indication of offset of SSB within the periodicity for the additional SSB periodicity, we prefer that the same configuration parameters can configured for SSB before adaptation and after adaptation.

Proposal 1: New RRC signaling is not needed for indication of offset of SSB within the periodicity for the additional SSB periodicity and the same configuration parameters can configured for SSB before adaptation and after adaptation.

In RAN1#120b, the following working assumption on application time of adapted SSB had been achieved [2].

Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst.

In our view, the above working assumption can be confirmed.

Proposal 2: The following working assumption can be confirmed.

Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst.

Adaptation of PRACH in time domain

In RAN1#120b, the following agreements on adaptation of PRACH in time domain had been achieved [2].

Agreement (Bit in DCI P-RNTI) For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers). - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs.

For the normal cell, there is only one initial UL/DL BWP where the UE camp in idle/inactive state. But for the cell configured with RedCap specific initial UL BWP, there are two different initial UL BWPs. Both normal UE and RedCap UE camp on the initial DL BWP which contains CORESET0. Both normal UE and RedCap UE receive system information and paging message through the initial DL BWP. When RedCap UE receives its paging, it initiates random access procedure on RedCap specific initial UL BWP.

According to the current agreements, DCI 1_0 with P-RNTI uses one bit to indicate whether the additional PRACH resources provided by semi-static signalling are available or not. For the cell with RedCap specific initial UL BWP, it is necessary to deal with which initial UL BWP is applicable for this one bit indicating additional PRACH resources. If this one bit is applicable for these two initial UL BWPs simultaneously, it may result in PRACH resources waste because there may be different PRACH requirements for these two initial BWPs. In a certain time period, only additional PRACH resources are required in initial UL BWP, and in another certain time period, only additional PRACH resources are required in RedCap Specific initial UL BWP. We think that it is necessary to have independent indications for additional PRACH resources for these two initial UL BWPs.

Proposal 3: For the cell with RedCap specific initial UL BWP, which initial UL BWP is applicable for the one bit indicating additional PRACH resources need to be clarified.

If the UE is configured with PEI, most of the time, the UE will not monitoring PO to receive paging DCI. Therefore, it is reasonable to use both paging DCI and DCI format 2_7 for the adaptation. If only supporting DCI format 1_0 scrambled by P-RNTI for indicating PRACH adaptation and the UE configured with DCI format 2_7, the UE have to monitor both PEI and paging DCI to monitor the adaptation indication.

Proposal 4: For PRACH adaptation indication, support the use of DCI 2_7 for idle/inactive UEs.

In RAN1#120b, the following agreements on the validity time duration of DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource had been achieved [2].

Agreement (Validity duration) For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI: - the validity duration is configured via higher layer signalling. Agreement - For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI. - The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..}).

Regarding whether DCI can be used to explicitly deactivate the additional PRACH resources, we think the same mechanism as indication of TRS resources in R17 can be reused, DCI explicitly deactivate the additional PRACH resources is not needed.

Proposal 5: DCI cannot be used to explicitly deactivate the additional PRACH resources.

Conclusion

We have the following proposals.

Proposal 1: New RRC signaling is not needed for indication of offset of SSB within the periodicity for the additional SSB periodicity and the same configuration parameters can configured for SSB before adaptation and after adaptation.

Proposal 2: The following working assumption can be confirmed.

Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst.

Proposal 3: For the cell with RedCap specific initial UL BWP, which initial UL BWP is applicable for the one bit indicating additional PRACH resources need to be clarified.

Proposal 4: For PRACH adaptation indication, support the use of DCI 2_7 for idle/inactive UEs.

Proposal 5: DCI cannot be used to explicitly deactivate the additional PRACH resources.

Reference

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson, RAN#102, Dec.  11th - 15th, 2023.

- 3GPP RAN1#120b Chairman’s Notes, April 07th – 11th, 2025.