3GPP TSG RAN WG1 #121                                                       R1-2503523

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.6.2

Source:	Spreadtrum, UNISOC

Title: 	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify LP-WUS procedures and configurations in idle/inactive state.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In this contribution, we will discuss procedure and configurations of LP-WUS

LP-WUS configuration

In RAN1#120b, the following agreement on the definition of nominal MO duration and actual LP-WUS duration had been achieved [2].

Agreement Terminology definition - Nominal MO duration: this includes both the available and unavailable symbols - Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives: - Alt A: Nominal MO duration is configured. (e.g. in unit of slots) - Actual LP-WUS duration is the number of available symbols within the MO. - Actual LP-WUS duration can vary from one MO to another MO. - Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values) - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration. - FFS: Additional termination condition such as time window. - Alt C: Both nominal MO duration and actual LP-WUS duration are configured. - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred. - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors. - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs. - Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

On the definition of nominal MO duration and actual LP-WUS duration, we prefer Alt A.

Proposal 1: Nominal MO duration is configured. (e.g. in unit of slots)

- Actual LP-WUS duration is the number of available symbols within the MO.

- Actual LP-WUS duration can vary from one MO to another MO.

In RAN1#120b, the following agreement on the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO had been achieved.

Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO. - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration. - FFS restriction on MO locations, e.g. only on DL slots. - FFS minimum gap is needed between two MOs to ensure LR processing time.

It was agreed that the starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. The offset is slot level or symbol level is FFS. In current specification, the starting time location of the first PEI monitoring occasion in a PEI-O is indicated by an offset w.r.t. the reference point using symbol-level offset. We think the same design as PEI should be adopted. In addition, only with slot-level offset may not be able to indicate the exact starting location of a MO.

Proposal 2: The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point using symbol level offset should be supported.

For the starting time locations of the subsequent LP-WUS MOs in a LO, we prefer that the start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. Specifically, a single gap between the end of the previous MO and the start of the next MO can be configured.

Proposal 3: the start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

LP-SS configuration

For configuration of the starting time location of the first LP-SS occasion, the following two options had been discussed in last meeting.

- Option 1: a slot-level offset

- Option 2: a symbol-level offset

In our view, the same design as LP-WUS should be adopted for LP-SS (i.e., Option 2).

Proposal 4: The starting time location of the first LP-SS occasion is configured w.r.t. the reference point provided by as LP-SS_periodicityoffset, using a symbol-level offset

For the starting time location of the subsequent LP-SS occasion(s),  the following two Options had been discussed in last meeting.

- Option 1: implicitly derived based on the time location of the first LP-SS occasion

- Option 2: explicitly configured using a slot-level offset or a symbol-level offset

Compared with Option 1, the signaling overhead of Option 2 is greater. We prefer Option 1 which is similar as SSB structure.

Proposal 5: The starting time location of the subsequent LP-SS occasion(s) is implicitly derived based on the time location of the first LP-SS occasion (i.e., Option 1)

Wake-up delay

In RAN1#120b, the following agreement on how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms had been achieved.

Agreement On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121. - Alt 1: Do not report for SSB periodicities other than 20ms. - Note: LP-WUS is not supported for SSB periodicities larger than 20ms. - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns): SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. [5]; [x]; [x]; [x]. [10]; [x]; [x]; [x]. 20; [70]; [500]; [900]. [40]; [x]; [x]; [x]. [80]; [x]; [x]; [x]. [160]; [x]; [x]; [x]. - Alt 3: It is up to RAN4.

On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, we support that do not report for SSB periodicities other than 20ms (i.e., Alt 1).

Proposal 6: Do not report for SSB periodicities other than 20ms (i.e., Alt 1).

Multi-beam operation

In previous RAN1 meeting, the following agreements on multi-beam operation of LP-WUS/LP-SS had been achieved.

Agreement At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams. Conclusion Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams. Conclusion No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams. Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. Agreement For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam. - The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second). Note: Above does not change the previous agreement on association between LP-WUS and SSB beams.

In our view, allowing gNB to transmit LP-WUS/LP-SS via a subset of SSB beams can reduce the overhead. In this case, gNB needs to inform UE which SSB beams are used to transmit LP-WUS/LP-SS.

Proposal 7: gNB needs to inform UE which SSB beams are used to transmit LP-WUS/LP-SS.

EPRE ratio between LP-WUS/LP-SS and SSB

In RAN1#120b, the following agreement on the EPRE ratio between LP-WUS/LP-SS and SSB had been achieved.

Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

Before transmitting LP-WUS, the UE periodically receive LP-SS for sync and measurement, the UE LR can do the AGC adjustment for LP-WUS based on the reception of LP-SS. Therefore, a common EPRE ratio configuration for LP-WUS and LP-SS to SSB should be supported.

Proposal 8: A common EPRE ratio configuration for LP-WUS and LP-SS to SSB should be supported.

Conclusion

We have the following proposals.

Proposal 1: Nominal MO duration is configured. (e.g. in unit of slots)

- Actual LP-WUS duration is the number of available symbols within the MO.

- Actual LP-WUS duration can vary from one MO to another MO.

Proposal 2: The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point using symbol level offset should be supported.

Proposal 3: the start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

Proposal 4: The starting time location of the first LP-SS occasion is configured w.r.t. the reference point provided by as LP-SS_periodicityoffset, using a symbol-level offset

Proposal 5: The starting time location of the subsequent LP-SS occasion(s) is implicitly derived based on the time location of the first LP-SS occasion (i.e., Option 1)

Proposal 6: Do not report for SSB periodicities other than 20ms (i.e., Alt 1).

Proposal 7: gNB needs to inform UE which SSB beams are used to transmit LP-WUS/LP-SS.

Proposal 8: A common EPRE ratio configuration for LP-WUS and LP-SS to SSB should be supported.

Reference

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC, RAN#102, Dec. 11th - 15th, 2023.

- 3GPP RAN1#120b Chairman’s Notes, April 07th – 11th, 2025.