3GPP TSG RAN WG1 #121                                                      R1-2503524

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.6.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on LP-WUS operation in CONNECTED modes

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

Activation/deactivation of LP-WUS monitoring

In RAN1 previous meeting, the following agreement on activation/deactivation of LP-WUS monitoring had been achieved.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission. Conclusion From RAN1 perspective, when LP-WUS monitoring Option 1-2 is enabled for RRC CONNECTED mode, it is supported that SR, PRACH and CG-PUSCH triggers PDCCH monitoring according to the legacy UE behaviour.

In last meeting, the following proposal had been discussed.

#### [Open]Proposal 4-1b: - When LP-WUS monitoring Option 1-1 is enabled for RRC CONNECTED mode, UE can fallback to PDCCH monitoring when UE monitors LP-WUS - FFS whether/how the condition is considered for the fallback - Alt1: When UE does not detect LP-WUS tagetting the UE for a certain time configured by gNB (i.e., timer1): - FFS: UE needs to send feedback - FFS: For the next timer, UE monitors LP-WUS - UE reset timer1 when UE receives LP-WUS tagetting the UE - Alt2: Without specified condition (by UE implementation): - FFS UE needs to send feedback - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

In RRC CONNECTED mode, gNB can control the activation/deactivation of LP-WUS monitoring. If gNB find out that a measurement report from a UE is not that good, it can deactivate the LP-WUS monitoring via RRC signaling and vise versa. In our view, RRC signaling for activation/deactivation of LP-WUS monitoring is enough. The condition of fall backing to PDCCH monitoring when UE monitors LP-WUS is not needed.

Proposal 1: The condition of fall backing to PDCCH monitoring when UE monitors LP-WUS is not needed.

Wake-up delay

In RAN1#120b, the following agreement on the UE capability report on the minimum time gap is between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS had been achieved [2].

Agreement For the UE capability report on the minimum time gap is between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3. - FFS: Whether to support different set of values for different receiver types.

Regarding whether to support different set of values for different receiver types, consider that the minimum time gap is mainly related to the sleep state of MR, the same value can be selected for OOK-based WUR and OFDM-based WUR.

Proposal 2: The same value can be selected for OOK-based WUR and OFDM-based WUR.

Regarding whether/how to consider the case of CA, the minimum time gap is related to the sleep state of MR, no matter CA case or non-CA case, the value of the minimum time gap should keep same.

Proposal 3: No matter CA case or non-CA case, the value of the minimum time gap should keep same.

Regarding the candidate values that can be indicated by UAI, in our view, the value reported by UAI depends on the sleep state and/or the synchronization time, therefore, candidate values for UAI should be same as candidate values for UE capability.

Proposal 4: Candidate values for UAI should be same as candidate values for UE capability.

QCL/TCI-state for LP-WUS

In previous RAN1 meeting, the following agreements on QCL/TCI-state for LP-WUS had been achieved.

Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement - For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s) Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable. Agreement For LP-WUS monitoring in RRC CONNECTED mode, select one of the options on how to define/configure/indicate QCL source of LP-WUS - Opt1: Separate indication/configuration for QCL source of LP-WUS - Opt2: Association of QCL source between LP-WUS and PDCCH/CORESET Agreement The case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, select one of the following in RAN1#121 for this case: - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS. - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them. The case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, - When Rel-17 unified TCI framework is configured, LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals (no change to DCI format). - When Rel-17 unified TCI framework is NOT configured, select one of the following in RAN1#121 for this case: - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS. - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them. Same alternative to be chosen for both cases.

For the case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19, we prefer that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them(i.e., Alt2).

Proposal 5: For the case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19, we prefer that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them (i.e., Alt2).

For the case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19 and when Rel-17 unified TCI framework is NOT configured, we prefer that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them (i.e., Alt2).

Proposal 6: For the case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19 and when Rel-17 unified TCI framework is NOT configured, we prefer that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them (i.e., Alt2).

Conclusion

In summary, we have following proposals:

Proposal 1: The condition of fall backing to PDCCH monitoring when UE monitors LP-WUS is not needed.

Proposal 2: The same value can be selected for OOK-based WUR and OFDM-based WUR.

Proposal 3: No matter CA case or non-CA case, the value of the minimum time gap should keep same.

Proposal 4: Candidate values for UAI should be same as candidate values for UE capability.

Proposal 5: For the case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19, we prefer that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them (i.e., Alt2).

Proposal 6: For the case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19 and when Rel-17 unified TCI framework is NOT configured, we prefer that RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them (i.e., Alt2).

Reference

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC, RAN#102, Dec. 11th - 15th, 2023.

- 3GPP RAN1#120b Chairman’s Notes, April 07th – 11th, 2025.