3GPP TSG RAN WG1 #121		                                        R1-2503525

Saint Julian, Malta, May 19th – 23th, 2025

Agenda Item:     9.7.2

Source:	Spreadtrum, UNISOC

Title:	             Discussion on ISAC channel modelling

Document for:	Discussion and decision

Introduction

In RAN#102 meeting, the new SID of ‘Study on channel modeling for Integrated Sensing And Communication (ISAC) for NR’ was approved [1]. The corresponding objective is copied below.

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In this contribution, we provide our views on the details of the ISAC channel modelling.

Channel model for sensing

LOS condition

In some scenario, EO type-2 will be modelled in the target channel and impact LOS/NLOS condition of Tx-target link and target-Rx link. RAN1 #120bis discuss the options about the impact of EO type-2 on LOS state as follows [2]:

Down-select in RAN1#120bis one option from the following options to determine the LOS condition of the Tx-target link and target-Rx link? - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modelled by a blockage model - Option C: Use the LOS probability equation to determine the LOS/NLOS condition of one link,

On the one hand, even if type-2 EO has a large size, it will also lead to power reduction because of the scattering in Tx-target link or target-Rx link like NLOS cluster. On the other hand, although EO is dropped randomly, EO is a non-target object with known location [3], so EO modelling is not suitable for stochastic LOS probability modelling. Therefore, if type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition.

Proposal 1: When the EO type-2 is used to model the Tx-target link and target-Rx link in the target channel, option A is preferred.

Spatial consistency modelling

Agreement When spatial consistency is enabled, the 1-by-1 random coupling generated by concatenation Option 3 is not updated per simulation drop even if Tx, target, Rx positions change during simulation. [FL1] Proposal 7.1-2 When spatial consistency is enabled, the set of paths generated by concatenation Option 3 (1-by-1 random mapping) is not updated during movement of Tx, target and/or Rx. - Note: the values of delay, angle and power could be changed based on spatial consistency. [FL1] Proposal 7.1-2 When spatial consistency is enabled, the 1-by-1 random coupling generated by concatenation Option 3 is not updated during movement of Tx, target and/or Rx.

The study on spatial consistency of multiple scattering points for a single target has not been thoroughly examined. From our perspective, the multiple scattering points of the same target do not always share the same LOS/NLOS condition. As an example, when applying the model with five scattering points for vehicle targets, if a BS has a LOS condition with the right side of a vehicle, the left side typically has an NLOS condition with the same BS. This situation also applies to the front and back of the vehicle.

Under this assumption, i.e., different LOS/NLOS conditions can be applied to different points—the multiple scattering points should not have spatial consistency. This is supported by an agreement reached in RAN1 #120, which states that:

“Spatial consistency is not modelled at least for the following links

……

- Case 2: the two links with different LOS/NLOS condition

……”

Proposal 2: Spatial consistency should not be applied for the multiple scattering points of the same target.

In Legacy TR 38.901 [4], two alternative spatial consistency procedures – Procedure A and Procedure B – are presented for spatially-consistent UT/BS mobility modelling. The procedures can be reused for TRP-target or target-TRP link by replacing UTs with targets for ISAC channel.

For Procedure A, the changes in the propagation distance of the electromagnetic wave or the equivalent velocity of the Tx/Rx in ∆t require consideration of the speed of the sensing target in addition to the speed of Tx/Rx.

Taking the change of propagation distance in ∆t as an example, before considering the sensing target speed, it can be expressed as:

rtxTtk-1⋅vtxtk-1+rrxT⋅vrxtk-1∙∆t

When the sensing target speed is taken into account, the change of distance can be expressed as:

rα,txTtk-1⋅vtxtk-1+rα,rxTtk-1∙vtk-1+rβ,txTtk-1∙vtk-1+rβ,rxT⋅vrxtk-1∙∆t

For Procedure B, the LSPs and SSPs are interpolated within the coherence length. In the simulation, we can draw the grid as shown in Figure 1 based on the coherence length, and the parameters value of the points inside a small square are interpolated by the parameters values of the four end points of the small square.

Figure 1 The grid based on coherence length

When the trajectory of sensing target is taken into account, the coherence grid will become complicated. A simplified approach is to build a unified grid global grid, and then obtain the UT/ST specific grid by shifting based UT/ST locations.

Proposal 3: For spatially-consistent UT/BS/Targets mobility modelling, enhancements on Procedure A is preferred.

Proposal 4: Global grid (aka. Unified grid) + shifting UT/ST locations is preferred to model the correlation of links between two nodes of STs and UTs.

Conclusion

In this contribution, we provided our views on the details of the ISAC channel modelling. The following proposals are made:

Proposal 1: When the EO type-2 is used to model the Tx-target link and target-Rx link in the target channel, option A is preferred.

Proposal 2: Spatial consistency should not be applied for the multiple scattering points of the same target.

Proposal 3: For spatially-consistent UT/BS/Targets mobility modelling, enhancements on Procedure A is preferred.

Proposal 4: Global grid (aka. Unified grid) + shifting UT/ST locations is preferred to model the correlation of links between two nodes of STs and UTs.

References

- RP-234069, New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR.

- R1-2503146, Summary #5 on ISAC channel modelling.

- 3GPP, RAN1 #116b Chairman’s Notes.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.