3GPP TSG RAN WG1 #121			R1-250XXXX

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.12.2

Source: 	Spreadtrum, UNISOC

Title: 		Discussion on low band carrier aggregation via switching

Document for:	Discussion and decision

Introduction

In RAN#107, revised WI on Low NR band carrier aggregation via switching was approved [1]. One objective is to specify only a semi-static switching pattern based on RRC configuration to enable low band carrier aggregation via switching.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

This contribution provides our views for semi-static switching pattern based on RRC configuration.

Discussion

Semi-static switching pattern

Number of switching pattern

RAN1 decided semi-static switching pattern based on RRC configuration to enable physical layer procedures to allow switching between {Case 1, Case 2}.

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carrier

An open issue is how many patterns can be configured. One argument for support more than one pattern is when SCell is in activation procedure, a different pattern is applied. However, according to the LS, RAN4 has decided the switching pattern for SDL SCell is not applied, but using potential scheduling restriction/interruption on PCell. So from this perspective, there is no need for RAN1 to configure the second switching pattern. We can directly agree with one switching pattern.

- For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation, e.g., in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - RAN4 will also discuss potential scheduling restriction/interruption on PCell during the SCell activation procedure for receiving the above mentioned RS occasions for the SCell activation - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - RAN4 will discuss and decide whether/how UE to perform the deactivated SDL SCell measurement

- For Rel-19 low NR band carrier aggregation via switching, support only one semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between PCell and SCell.

Periodicity and configuration Method

A slot level based bitmap was agreed for switching pattern configuration. Periodicity supports P={40} ms.

Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms

For the restriction of maximum X switches within Y slots, the intention is to avoid frequent switching. So at most one switching from PCell to SCell in 5ms is a reasonable combination. It means there can be both only one switching from PCell to SCell, or one switching from PCell to SCell and one switching from SCell to PCell. But it cannot more one usable period of SCell in 5 ms.

There is a restriction of K1 value of HARQ-ACK feedback. Therefore, another restriction is the contiguous slots of SCell cannot be exceed 8 slots in a period.

RAN4 also provides the RRM requirement; at least one SSB is available at the UE during every 160 ms for both of PCell and SCell if applicable.

- At most one switching from PCell to SCell exits in 5ms.

- Contiguous slots of SCell cannot be exceed 8 slots in a period.

- At least one SSB is available at the UE during every 160 ms for both of PCell and SCell if applicable

Switching gap location

It was agreed switching gap located at “switch from” carrier when SDL->FDD, and it was open for FDD->SDL.

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

For switching from FDD carrier to SDL carrier, switching gap can locate at the “switch from” carrier, i.e., FDD carrier, due to provide full PDCCH monitoring at SDL carrier and a full first slot schedulable.

The second open issue is when switching gap cover switching period and UL TA for SDL switching to FDD carrier, whether there is any specification impact. We believe it would be good to capture that UE does not expect switching gap cannot cover switching period and UL TA if needed. In another word, UE does not expect the UL dropping due to overlapping with the switching period to make UE behaviour clearly.

- For switching from FDD carrier to SDL carrier, switching gap can locate at the “switch from” carrier, i.e., FDD carrier.

- For the specification impact of UL TA, UE does not expect the UL dropping due to overlapping with the switching period.

Impact on PHY procedure

SCell dormancy

Similar as SCell deactivation, SCell dormancy is also facing the same issue. The switching pattern for SDL SCell is not applied if SCell in dormancy BWP. In addition, due to the DCI with dormancy indication only on PCell, when UE receive the DCI it is always within the switching pattern of PCell. Therefore, UE only needs to know switching pattern for SDL SCell is not applied for next SCell chance.

- The switching pattern for SDL SCell is not applied if SCell is in dormancy BWP

HARQ-ACK codebook

Type-1 HARQ-ACK codebook is impacted by switching pattern and switching gap. There was a discussion during the last meeting. The following proposal was provided by FL and updated by one company. We agree with the intention and support the proposal 2-5 and 2-6.

(Open) Proposal 2-5-update For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for Type-I HARQ-ACK codebook generation - A row of the TDRA table overlapping with the invalid symbol(s) is excluded from the determination of the set of occasions for candidate PDSCH receptions if the PDSCH associated with the row is not to be received by the UE due to LB-CA switching. (Open) Proposal 2-6-update For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for HARQ-ACK codebook generation - HARQ-ACK bit for a SPS PDSCH is not included in the HARQ-ACK codebook if the SPS PDSCH is not to be received by the UE due to LB-CA switching.

- For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for Type-I HARQ-ACK codebook generation

- A row of the TDRA table is excluded from the determination of the set of occasions for candidate PDSCH receptions if the PDSCH associated with the row is not to be received by the UE due to LB-CA switching.

- For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for HARQ-ACK codebook generation

- HARQ-ACK bit for a SPS PDSCH is not included in the HARQ-ACK codebook if the SPS PDSCH is not to be received by the UE due to LB-CA switching.

Collision handling

There was two related proposals during the last RAN1 meeting.

[Closed][R2] Proposal 3-1 For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, RAN1 does not consider the UE is not expected to perform additional switching not following the semi-static switching pattern - FFS: whether and, if needed, how to support the only candidate exceptional scenario, i.e., RACH [Closed][R2] Proposal 3-2 For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, the collision is expected to be handled/avoided by the NW implementation, except for none, or one, or multiple or all of the following scenarios, in which UE is expected to drop the corresponding reception and/or transmission (FFS: detailed dropping behavior) - Periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI' - P/SP-CSI-RS, including TRS - P/SP-SRS - SPS-PDSCH - CG-PUSCH (Type-1/2) - SR on PUCCH - Other scenarios

We support Proposal 3-1. For PRACH as exceptional, we think it can be only for CBRA. In RRC-connected state, the UE can perform random access procedures triggered by different events. When the UE is out-of-sync and there is downlink data arrival, the gNB may send a PDCCH order with CFRA/CBRA to resynchronize the UE.  Uplink data arrival while UE is out of sync, or scheduling request failure, or uplink data arrival for UE without PUSCH resource allocation, UE may trigger a CBRA with PRACH. During CBRA, gNB does not aware UE who send PRACH. Thus, gNB cannot know there is a switching pattern. The latter MSG2 in RAR window, MSG3 and MSG4 are all challenging. However, we also notice it would be very complex if this dynamic switching pattern disable/enable depend on PRACH.  We are open for this issue, but separate UE capability can be defined.

For Proposal 3-2, we prefer the formulation of Samsung, which is dynamic transmissions/receptions overlapping with unusable should be avoided by network and semi-static channels/signals can be canceled by UE if collision happens. And there is clear definition of dynamic and semi-static. Such as in 38.213.

Section 11.1 For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.

- For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, RAN1 does not consider the UE perform additional switching not following the semi-static switching pattern

- FFS: whether and, if needed, how to support the only candidate exceptional scenario, i.e., CB RACH

- For low band carrier aggregation via switching, semi-static pattern indicates UE to operate (receive or transmit) on only one serving cell.

- The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell.

- The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell.

Conclusion

In this contribution, we made the following observations and proposals.

- For Rel-19 low NR band carrier aggregation via switching, support only one semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between PCell and SCell.

- At most one switching from PCell to SCell exits in 5ms.

- Contiguous slots of SCell cannot be exceed 8 slots in a period.

- At least one SSB is available at the UE during every 160 ms for both of PCell and SCell if applicable

- For switching from FDD carrier to SDL carrier, switching gap can locate at the “switch from” carrier, i.e., FDD carrier.

- For the specification impact of UL TA, UE does not expect the UL dropping due to overlapping with the switching period.

- The switching pattern for SDL SCell is not applied if SCell is in dormancy BWP

- For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for Type-I HARQ-ACK codebook generation

- A row of the TDRA table is excluded from the determination of the set of occasions for candidate PDSCH receptions if the PDSCH associated with the row is not to be received by the UE due to LB-CA switching.

- For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for HARQ-ACK codebook generation

- HARQ-ACK bit for a SPS PDSCH is not included in the HARQ-ACK codebook if the SPS PDSCH is not to be received by the UE due to LB-CA switching.

- For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, RAN1 does not consider the UE perform additional switching not following the semi-static switching pattern

- FFS: whether and, if needed, how to support the only candidate exceptional scenario, i.e., CB RACH

- For low band carrier aggregation via switching, semi-static pattern indicates UE to operate (receive or transmit) on only one serving cell.

- The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell.

- The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell.

References

RP-250247	Revised WID on Low NR band carrier aggregation via switching

R1-2501700	LS of RAN4 RRM agreements on LB CA via switching	RAN4, Apple

R1-2501702	LS on switching periods for low-low band switching	RAN4, AT&T

R1-2502637	FL summary #3 of Low band carrier aggregation via switching		Moderator (Apple)

R1-2503215	LS of RAN4 RRM agreements on LB CA via switching	RAN4, Apple