3GPP TSG RAN WG1 #121			  R1-2503533

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on UE features for evolution of NR duplex operation SBFD

Document for:	Discussion and decision

Introduction

In this contribution, we provide our views on UE features agreed in RAN1#120bis for

- SBFD TX/RX/measurement procedures

- SBFD random access operation

- CLI handling

Discussion

SBFD TX/RX/measurement procedures

Basic FG for SBFD

The following agreements were made in RAN1#120bis.

Agreement: Introduce following basic FG for SBFD, and FFS on some potential components (that may be separate FGs). 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per Band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. Add the following components in FG 60-1 (basic FG for SBFD operation) - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols - [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols]

For component 2, support of DL reception in two DL subband in SBFD symbols should be separate FG.

For separate SRS resource set configuration in SBFD symbols and non-SBFD symbols, it should be part of basic FG. It is the basic parameter for UL transmission in SBFD symbols.

For separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols, it should be included in separate FG.

For the note column, we are fine to keep it.

FGs for DL reception in two non-contiguous DL subbands

For DL reception in two non-contiguous DL subbands, it should be multiple separate FGs for PDSCH reception in two non-contiguous DL subbands and for CSI-RS reception in two non-contiguous DL subbands.

RA Type 0 for PDSCH is included in FG 5-1(Basic scheduling/HARQ operation), which is mandatory UE feature. So some companies think PDSCH reception in two non-contiguous DL subbands should be part of basic FG. However, for RA Type 1 for PDSCH, only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. And the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only. So the resource mapping and TBS determination is different with legacy.

In SBFD operation, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware UE. Non-contiguous CSI-RS resource is derived by excluding frequency resources outside DL usable PRBs, which is different  legacy UE.

Thus, we prefer separate FG (60-1a) for DL reception in two non-contiguous DL subbands.

PDSCH reception in two non-contiguous DL subbands and CSI-RS reception in two non-contiguous DL subbands can be defined in one FG. For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, there are two candidate value . One value is CSI processing timeline and CPU counting are the same as legacy, and the other value is scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

In last meeting, some companies argued that gNB cannot know whether UE support one/two DL subband and gNB cannot do appropriate DUD/DU/UD configuration. From our perspective, the configuration of subband location is independent with capability of DL reception. As long as the basic SBFD FG is supported, any subband configuration can be supported. The location of the UL subband will depends on the SBFD carrier location within the NR band and the number of adjacent carriers. For example, if SBFD carrier has two adjacent carriers, gNB would configure the UL subband in the middle of carrier (DUD configuration) to reduce inter-gNB interference.

After random access is completed, gNB can schedule DL resource in one DL subband or across two DL subband based on capability of DL reception. Thus, support of DL reception in two non-contiguous DL subbands should be separate FG.

FG for DL partial PRG

Support of DL partial PRG should be separate FG (60-1b), which is aligned with RAN1 agreement.

The candidate value can be 2 and 4.

The consequence of not reporting this UE FG is partial PRGs in the subband boundary is not supported.

FG for Separate UL power control

In RAN1#120bis, whether to add the separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols in basic FG was discussed.

If it added in basic FG, FG 23-1-1h (Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS)  should be the prerequisite FG of basic SBFD FG. It means if FG 23-1-1h is not supported, the basic SBFD operation cannot be supported. So separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols should be included in separate FG 60-1c.

FG 23-1-1h (Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS) should be the prerequisite FG of FG 60-1c.

FG for Configuration 2

The following agreements were made in RAN1#120b.

Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs 60-2; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG; YES; n/a; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

From our perspective, it should be two separate FGs for UL transmission and for DL reception, respectively.

Based on RAN1 agreement, SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. So UE can only support Configuration 2 for UL transmission or DL reception. The UE behaviour is different for DL and UL. For example, TBS determination is same as legacy for UL transmission. But there are some new behaviour for TBS determination for DL reception.

Thus, define separate FG from FG for reception across SBFD symbols and non-SBFD symbols in different slots:

Support of configuration 2 for UL transmission should be single FG (60-2a).

Proposal 1. Support FGs in Table 1 for SBFD-aware UE TX/RX/measurement behaviour and procedures.

Table 1: Layer-1 feature list for SBFD-aware UE TX/RX/measurement behaviour and procedures

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols |  | Yes | N/A | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | No | N/A | Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s) Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-1a | DL reception in two non-contiguous DL subbands | - Support of PDSCH reception in two non-contiguous DL subbands - Support of CSI-RS reception in two non-contiguous DL subbands - CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands candidate value set {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy} | 60-1 | Yes | N/A | UE is not able to support DL reception in two non-contiguous DL subbands | Per band | No | No | N/A |  | Optional with capability signaling |
| 60-1b | Partial PRGs in SBFD symbols | Maximum number of supported partial PRGs in SBFD symbols: candidate value set {2, 4} | 60-1 | Yes | N/A | UE is not able to support partial PRGs in the subband boundary | Per band | No | No | N/A |  | Optional with capability signaling |
| 60-1c | Separate UL power control for SBFD symbols and non-SBFD symbols | Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols | 60-1, 23-1-1h | Yes | N/A | UE is not able to support separate UL power control based on unified TCI framework in SBFD symbols and non-SBFD symbols | Per band | No | No | N/A |  | Optional with capability signaling |
| 60-2 | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) | 60-1 |  | N/A | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per band | No | No | N/A |  | Optional with capability signalling |
| 60-2a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | - Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) | 60-1 |  | N/A | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per band | No | No | N/A |  |  |

SBFD random access operation

    - FG 60-3

Type of the FG: Per band

Other components:

- Support of reinterpretation of frequency offset of lowest RO

- Support of separate power control parameters in additional-ROs.

- Support of separate mapping of SSB and additional ROs, separate determination of the association period and association pattern period

    - FG 60-4

Prerequisite of the FG: 60-1. For FFS on whether UE supporting SBFD random access needs to support at least Option 1, Alt 3 is preferred. UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option).

Type of the FG: Per band

Other components:

- Support of separate power control parameters in additional-ROs.

- Support of separate mapping of SSB and additional ROs, separate determination of the association period and association pattern period

    - Other FG: RO across SBFD/non-SBFD

A separate FG for support of valid RO across SBFD and non-SBFD symbols

Type of the FG(s): Per band

Prerequisite of the FG: 60-1, 60-4

    - Other FG: PRACH repetitions

Two FGs: Separate FGs for PRACH repetition in additional RO for RACH configuration option 1 and option 2

Type of the FGs: Per band

Prerequisite of the FG of RACH configuration Option 1: 60-3 and 54-1

Prerequisite of the FG of RACH configuration Option 2: 60-4 and 54-1

    - Other FG: MSG3 repetitions in SBFD symbols

One FG for Msg.3 repetition in SBFD symbols

Type of the FG: Per band

Prerequisite of the basic FG: 60-3/60-4 and 30-6

Proposal 2. Support FGs in Table 2 for SBFD random access operation.

Table 2: Layer-1 feature list for SBFD random access operation

| X-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | - Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) - Support of reinterpretation of frequency offset of lowest RO - Support of separate power control parameters in additional-ROs - Support of separate mapping of SSB and additional ROs, separate determination of the association period and association pattern period |  | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | - Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) - Support of separate power control parameters in additional-ROs. - Support of separate mapping of SSB and additional ROs, separate determination of the association period and association pattern period | 60-1 | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-4a | RO across SBFD/non-SBFD | Support of valid RO across SBFD and non-SBFD symbols | 60-1,60-4 |  |  |  | Per band |  |  |  |  |  |
| 60-5a | PRACH repetition in additional RO for RACH configuration option 1 | Support of PRACH repetition in additional RO for RACH configuration option 1 | 60-3 , 54-1 |  |  |  | Per band |  |  |  |  |  |
| 60-5b | PRACH repetition in additional RO for RACH configuration option 2 | Support of PRACH repetition in additional RO for RACH configuration option 2 | 60-4 , 54-1 |  |  |  | Per band |  |  |  |  |  |
| 60-6 | MSG3 repetitions in SBFD symbols | Support of MSG3 repetitions in SBFD symbols | 60-3/60-4, 30-6 |  |  |  | Per band |  |  |  |  |  |

CLI handling

FGs for UL resource muting

In RAN1#120bis, following agreement was achieved.

Agreement: Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

For basic FG for UL resource muting, single UL muting pattern should be configured for DG PUSCH, Type 1 CG PUSCH and Type 2 CG PUSCH. There is no need to support separate configurations of up to two UL muting symbols for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH. Separate configurations should be separate FG. So FFS part in component 1 should be confirmed.

From our perspective, UL muting resource can not be applied to non-SBFD symbols. So the prerequisite FG of 60-7/60-7a is FG 60-1.

The granularity of the FG 60-7/60-7a is per Band.

FGs for L1-CLI-RSSI measurement

For FG for L1 CLI-RSSI measurement/reporting, potential component as following:

In RAN1#120bis, following agreement was achieved.

Agreement: Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling.

Periodic and aperiodic CLI-RSSI measurement resource should be part of basic FG for L1 CLI-RSSI measurement.  It reuses same principle in R16 L1-RSRP/SINR UE feature. So the FFS part in component 3 should be confirmed.

For semi-persistent CLI-RSSI measurement resource and aperiodic reporting, it should be included in separate FG.

For periodic L1 CLI-RSSI reporting on PUCCH, it can be included in basic FG for L1 CLI-RSSI measurement/reporting. So FFS part in component 1 can be confirmed.

The maximum number of CLI-RSSI measurement resources should be separate FG, which is similar as L3-CLI-RSSI capability. Maximum number of CLI resource per resource set in Rel-16 can be reused, i.e. maxNrofCLI-RSSI-Resources-r16 = 64. In legacy CLI-RSSI measurement, only total number of the measurement resources that the UE can support needs to be reported. There is no need to report maximum number of L1 CLI-RSSI measurement resources per CC. So maximum number of  L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) across all CCs configured to measure L1-CLI-RSSI should be separate FG.

The prerequisite FG of L1 CLI-RSSI measurement/reporting is FG 2-32 (Basic CSI feedback).

The granularity of the FG 60-8 is per Band.

FGs for L1-SRS-RSRP measurement

In RAN1#120bis, following agreement was achieved.

Agreement: Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Periodic and aperiodic SRS-RSRP measurement resource should be part of basic FG for L1 CLI-RSSI measurement. So the FFS part in component 1 of FG 60-9 can be confirmed.

Max number of L1 SRS-RSRP measurement resource (sum of aperiodic/semi-persistent/periodic) across all CCs should be separate FG. Max number of CLI resource per resource set in Rel-16 can be reused, i.e. maxNrofCLI-SRS-Resources-r16 = 32.

The prerequisite FG of FG 6-9 is FG 2-32 (Basic CSI feedback).

The granularity of the FG 60-9/60-9a/60-9b is per Band.

Proposal 3. Support FGs in Table 3 for UL resource muting, L1-CLI-RSSI measurement and L1-SRS-RSRP measurement.

Table 3: Layer-1 feature list for CLI handling

| 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  | Yes | N/A | UL resource muting for CP-OFDM waveform is not supported | Per band | TDD only | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH |  | Yes | N/A | UL resource muting for DFT-S-OFDM waveform is not supported | Per band | TDD only | No | N/A |  | Optional with capability signaling |
| 60-8 | Aperiodic and periodic L1 CLI-RSSI measurement resource and aperiodic/ periodic reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource | 2-32 | YES | n/a | Aperiodic and periodic L1 CLI-RSSI measurement resource and aperiodic/ periodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8a | Semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting | 2-32 | YES |  | Semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-8b | CLI-RSSI measurement resources for L1 based CLI measurement | Max number of L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) across all CCs | 60-8, 60-8a | YES |  |  |  |  |  |  |  | Optional with capability signaling |
| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource |  | YES | n/a | Aperiodic L1 SRS-RSRP measurement and reporting is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| X-9c | SRS-RSRP measurement resources for L1 based CLI measurement | - Max number of SRS-RSRP measurement resource (sum of aperiodic/semi-persistent/periodic) across all CCs | 60-9, | YES | n/a |  |  | TDD only | No | N/A |  | Optional with capability signaling |

Conclusion

In this contribution, we made the following observation and proposals.

Proposal 1. Support FGs in Table 1 for SBFD-aware UE TX/RX/measurement behaviour and procedures.

Proposal 2. Support FGs in Table 2 for SBFD random access operation.

Proposal 3. Support FGs in Table 3 for UL resource muting, L1-CLI-RSSI measurement and L1-SRS-RSRP measurement.

- References

- RP-2401614	Revised WI on evolution of NR duplex operation

- TR38.858, V18.0.0, Study on Evolution of NR Duplex Operation, (Release 18)

- 38.822, “NR; User Equipment (UE) feature list (Release 17)”, V17.1.0

- R1-2502965	Session Notes of AI 9.15, Ad-Hoc Chair (NTT DOCOMO, INC.)