3GPP TSG RAN WG1 #121			R1-2503534

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.8

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on UE features for NTN

Document for:	Discussion and decision

Introduction

In this contribution, we provide our views on the open issues of UE features for NR NTN.

FGs for DL coverage

FG 65-1-1

FG 65-1-1: 160 ms SSB periodicity assumed during initial access

- Component: confirm cell selection and access in the component, and no other component.

  - 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection and access

- Prerequisite: N/A

- Need for the gNB to know if the feature is supported: NO

- Consequence if the feature is not supported by the UE: UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial access

- Note: it can confirm Table 5.2.3-1 also applicable.

  - As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5

- Mandatory/Optional: Optional without capability signalling

FG 65-1-2

FG 65-1-2: PDCCH repetition for Type0 PDCCH CSS

- Component: confirm only searchSpaceZero in this FG, since other search space may not support PDCCH repetition or even support but in a different way. Furthermore, if other CSS is configured with searchSpaceZero, same PDCCH repetition is also applicable. We do not think it is necessary to merge with 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection and access.

  - 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1

  - 2. PDCCH repetition for Type0A/1/2 PDCCH CSS of searchSpaceZero

  - 3. SIB1 PDSCH repetition scheduled by Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1.

- Prerequisite: N/A

- Need for the gNB to know if the feature is supported: Yes

- Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5, not applicable to Table 5.2.3-1 in TS 38.101-5 nor TN.

- Mandatory/Optional: Optional with signalling

FG 65-1-3

It was agreed intra-slot PDCCH repetition is supported for PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero. Therefore, it can support separate FG 65-1-3 for other types of CSS.

FG 65-1-2: PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS

- Component: this FG only applicable to other type of CSS which is not searchSpaceZero, since they are intra-slot PDCCH repetition, while searchSpaceZero is inter-slot PDCCH repetition.

  - 1. Support reception of PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS other than SearchSpaceZero

- Prerequisite: N/A

- Need for the gNB to know if the feature is supported: Yes

- Consequence if the feature is not supported by the UE: PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS is not supported

- Type: per Band

- Need of FDD/TDD differentiation: N/A

- Need of FR1/FR2 differentiation: N/A

- Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5, not applicable to Table 5.2.3-1 in TS 38.101-5 nor TN.

- Mandatory/Optional: Optional with signalling

SIB1 PDSCH repetition

Depends on main session progress, if only SIB1 PDSCH repetition supported by searchSpaceZero, no additional FG here. If SIB1 PDSCH repetition supported by other search space except searchSpaceZero, a separate FG such as below can consider.

MSG4 PDSCH repetition

FG 65-1-5: Msg4 PDSCH repetition

- Component:

  - 1. Support reception of Msg4 PDSCH repetition

  - 2. Support receiving one repetition factor in system information

  - 3. Support Msg3 to report capability/request information for Msg4 PDSCH repetition

  - 4. Support of RSRP threshold for Msg4 PDSCH repetition

- Prerequisite: N/A

- Need for the gNB to know if the feature is supported: Yes

- Consequence if the feature is not supported by the UE: Msg4 PDSCH repetition is not supported

- Type: per Band

- Need of FDD/TDD differentiation: N/A

- Need of FR1/FR2 differentiation: N/A

- Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5, not applicable to Table 5.2.3-1 in TS 38.101-5 nor TN.

- Mandatory/Optional: Optional with signalling

FGs for RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

65-2-1: Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE

- Component: one FG for both Case 3 and 4. No separate FGs for RRC INACTIVE state, and no need to mention it in this feature. Because it is as same as legacy behaviour.

  - 1. Support handling collision Case 3

  - 2. Support handling collision Case 4

- Prerequisite: 28-3

- Mandatory/Optional: Optional with signalling.

  - For FFS of a UE that supports FG 28-3 in NTN band must support this FG, it can be removed. Hence this FG can be a separate FG with optional capability.

FGs for UL capacity

Two FGs for OCC length 2 and OCC length 4.

FG 65-3-1: PUSCH with inter-slot OCC of length 2

- Component:

  - 1. Support inter-slot OCC with OCC length 2 for DFT-s-OFDM PUSCH transmissions with repetition Type A

  - RV cycling across OCC groups

  - UCI multiplexing or drop when PUSCH with inter-slot OCC is overlapped with PUCCH at the {first, any} slot of OCC group. UE can additional report a value in {first, any}

- Prerequisite: One of {5-14, 5-16, 11-6}

- Need for the gNB to know if the feature is supported: Yes

- Consequence if the feature is not supported by the UE: UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A

- Type: per Band

- Need of FDD/TDD differentiation: N/A

- Need of FR1/FR2 differentiation: N/A

- Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5

- Mandatory/Optional: Optional with signalling

FG 65-3-2: PUSCH with inter-slot OCC of length 4

- Component:

  - 1. Support inter-slot OCC with OCC length 4 for DFT-s-OFDM PUSCH transmissions with repetition Type A

  - RV cycling across OCC groups

  - UCI multiplexing or drop when PUSCH with inter-slot OCC is overlapped with PUCCH at the {first, any} slot of OCC group. UE can additional report a value in {first, any}

- Prerequisite: One of {5-14, 5-16, 11-6}

- Need for the gNB to know if the feature is supported: Yes

- Consequence if the feature is not supported by the UE: UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A

- Type: per Band

- Need of FDD/TDD differentiation: N/A

- Need of FR1/FR2 differentiation: N/A

- Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5

- Mandatory/Optional: Optional with signalling

References

- RP-243300, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024.

- R1-2502967	Session Notes of AI 9.15.8	Ad-Hoc Chair (NTT DOCOMO, INC.)

- R1-2502977	Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis	Moderators (AT&T, NTT DOCOMO, INC.)