3GPP TSG RAN WG1 #121                                                                                R1-2503539

St Julian's, Malta, May 19th-23rd, 2025

Source:	TCL

Title:	Discussion on on-demand SSB Scell operation for NES

Agenda Item:	9.5.1

Document for:	Discussion and Decision

Introduction

In this contribution, we give the discussion on on-demand SSB SCell operation for NES based on agreement in last several meeting [1][2], including signaling of on-demand SSB  operation, transmission timing of indicated SSB, relationship between on-demand SSB and always-on SSB, and relation ship between on-demand SSB and other NR resources.

Signaling of on-demand SSB operation

Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

In last meeting, what the indication of deactivation for OD-SSB transmission do not use RRC/MAC-CE has been agreed for Case#1.

In RAN 1 #117, DCI based signaling to indicate OD-SSB transmission on Scell as one potential method has been discussed. We want to further discuss this option here.

For a cell supporting OD-SSB SCell operation, last meeting discussed whether DCI could be used for indicating OD-SSB transmission. From previous meetings’ agreement, it is clear that DCI will not be used to provide SCell  activation or deactivation. Even if most cases consider semi static-based MAC CE to indicate OD-SSB transmission,  but specific DCI could be used for reducing signaling overhead if multiple UE configured with same SCell, e.g., group common DCI. Thus, we are open to discuss this topic.

Observation 1: Even if most cases consider semi static-based MAC CE to indicate OD-SSB transmission,  but specific DCI could be used for reducing signaling overhead if multiple UE configured with same SCell, e.g., group common DCI.

In RAN  1#120 meeting, DCI format 2_9 has been agreed for the indication of periodicity adaptation of  SSB. In addition, DCI 2_9  is used for cell DTX/DRX activation/deactivation. From our perspective, it is easier to be introduced for indicating OD-SSB transmission if DCI supported, including activation/deactivation.

Observation 2: In RAN  1#120 meeting, DCI format 2_9 as group common DCI has been agreed for the indication of periodicity adaptation of  SSB.

Proposal 1: Support at least group common DCI used for indicating OD-SSB transmission.

Relationship between on-demand SSB and always-on SSB.

Contents of on-demand SSB configuration/indication

One agreement has been achieved for multiple candidate configuration/indication of OD-SSB in last meeting.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

- As we know, Scell needs to at least be configured before OD-SSB transmission configured, but how many multiple candidate values being configured is up to the configured Scell’s capability. One restriction is whether overlapping happened between two different OD-SSB transmission. As shown figure below, there is no any overlap between 1st OD-SSB transmission and 2nd OD-SSB transmission and with same center frequency.

Below figure gives overlap scenario between two different OD-SSB transmission with same center frequency.

One method to avoid overlap between different OD-SSB transmission is to regulate multiple OD-SSB transmission with different center frequency. Otherwise, following OD-SSB transmission indicated by MAC CE needs to wait until previous OD-SSB transmission is deactivated. Additional implicitly or explicitly indicating like deactivation or time offset needs to be considered.

Observation 3: One restriction is whether overlapping happened between two different OD-SSB transmission if same center frequency is assumed for these two OD-SSB.

Proposal 2: Consider how to avoid overlap of time domain or frequency domain between multiple OD-SSB transmissions.

Also, one agreement from RAN 1 #120 has given for providing time domain location of OD-SSB for case #2.

Agreement - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists

However, how to indicate time offset between OD-SSB and AO-SSB is still open issue. If AO-SSB existing, reference point could defined based on OD-SSB location or AO-SSB location, which is up to periodicity of AO-SSB.

Proposal 3: Support define reference point of OD-SSB for case #2 based on periodicity of AO-SSB.

How to multiplex on-demand SSB and always-on SSB

For case #2, collision may happen when resources overlapping between AO-SSB and OD-SSB.

- For some special cases, AO-SSB should be prioritized than OD-SSB to receive, e.g.,  AO-SSB could be received if configured special periodicity and downlink transmission power.

- For some special cases, OD-SSB could be prioritized to receive, e.g., when different center frequency from AO-SSB configured for OD-SSB, and the periodicity of OD-SSB less than that of AO-SSB, OD-SSB could be firstly received for synchronization or other functionalities.

- UE receives same type of SSB like previous SSB if collision happened. For example, UE received OD-SSB before collision, and still receives OD-SSB when collision happened. It is going to be potential benefits for UE to reduce switching complexity or signaling overhead.

- Proposal 4: Consider how to handle collision between OD-SSB and AO-SSB, including priority definition or following previous type of SSB to receive SSB when collision happened.

L1/L3 measurement based on OD-SSB

In RAN 1#118b meeting, one agreement has been given for considering L1 measurement base on OD-SSB, we copy it as below

Agreement (RAN1#118bis) For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not

For each BWP, UE is configured two sets to use for RLF or BFR,  and one of two sets is used for detecting beam failure and another one used for indicating candidate beam. If BFR happens during SCell operation, it could be triggered by BFR MAC CE and UE will send it to gNB based on new beam. For case #1 and case #2, OD-SSB transmission could impact BFD reporting periodicity or detection timer, which needs to further discuss.

Proposal 5: Discussion on the impact of OD-SSB transmission including case #1 and case #2 on BFD reporting periodicity or detection timer.

Relationship between on-demand SSB and NR resources

When OD-SSB transmission overlaps with other NR resources like PDSCH, how to handle collision needs to be discussed according to different requirement of scenario #2, #2A and #3B. As mentioned FL summary in last meeting, rate-matching could be used for handling collision between OD-SSB and NR resources and interference from neighboring cell. In our understanding, how to handle collision between OD-SSB resource and NR resource and how to handle rate-matching are needed to be considered. For rate-matching, it could be impacted by TBS related parameters and OD-SSB transmission parameter like downlink transmission power.

Proposal 6: Discuss how to handle collision between OD-SSB resource and NR resource and how to handle rate-matching.

Conclusion

This paper discussed how to design on-demand SSB Scell operation, with the following observation/proposals:

Observation 1: Even if most cases consider semi static-based MAC CE to indicate OD-SSB transmission,  but specific DCI could be used for reducing signaling overhead if multiple UE configured with same SCell, e.g., group common DCI.

Observation 2: In RAN  1#120 meeting, DCI format 2_9 as group common DCI has been agreed for the indication of periodicity adaptation of  SSB.

Observation 3: One restriction is whether overlapping happened between two different OD-SSB transmission if same center frequency is assumed for these two OD-SSB.

Proposal 1: Support at least group common DCI used for indicating OD-SSB transmission.

Proposal 2: Consider how to avoid overlap of time domain or frequency domain between multiple OD-SSB transmissions.

Proposal 3: Support define reference point of OD-SSB for case #2 based on periodicity of AO-SSB.

- Proposal 4: Consider how to handle collision between OD-SSB and AO-SSB, including priority definition or following previous type of SSB to receive SSB when collision happened.

Proposal 5: Discussion on the impact of OD-SSB transmission including case #1 and case #2 on BFD reporting periodicity or detection timer.

Proposal 6: Discuss how to handle collision between OD-SSB resource and NR resource and how to handle rate-matching.

# References

- Chair notes, 3GPP TSG RAN WG1 #120b, April,  2025.

- 9.5.1 FL summary, 3GPP TSG RAN WG1 #120b, April, 2025.