3GPP TSG RAN WG1 #121			R1-2503550

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.1.1

Source:	Samsung

Title:	Discussion for supporting AI/ML based beam management

Document for:	Discussion and Decision

- Introduction

In RAN#102, Rel-19 work item on “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. The objective of the work item is as follows.

Provide specification support for the following aspects: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we will provide our view on the specification support for AI/ML beam management, including the aspects for NW-side AI/ML model and UE-side AI/ML model, respectively.

- NW-side AI/ML model

In this section, specification impact with respect to NW-side AI/ML model for beam management is discussed. In particular, different LCM operations (e.g., data collection, model inference, model monitoring, etc) associated with the NW-side AI/ML and the corresponding signalling/mechanism to facilitate the LCM operations are further discussed as follows.

2.0 Typical procedure

Beam management specific LCM procedure for NW-side AI/ML model is illustrated in Figure 1.

Figure 1. LCM procedure for NW-side AI/ML model.

For data collection, NW configures RS resources (e.g., SSB resources or CSI-RS resources) for L1-RSRP measurement. For example, RS resources for Set A and/or Set B can be configured for transmission. UE performs measurements based on the configured RS resources and report the corresponding collected data based on NW control. From NW perspective, the collected data can be used for model training, model inference or performance monitoring. For model training, the collected data can be further delivered or process for offline training or fine tuning. For model inference, NW can use the collected data as model input and generate the corresponding output, where the output (e.g., predicted beam) can be further used for beam indication. The collected data can also be used to monitor the performance of the model, and NW can decide whether to activate/deactivate/fallback the AI/ML operation based on the performance monitoring.

2.1 Data collection for Model training

For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: - For gNB-side models, training data can be generated by the gNB or UE, while the termination point for training data may include the gNB, or OAM. - Note: RAN2 identified the case in which OTT server and Core Network may be used for gNB-side model training. However, no study was conducted since this is beyond the scope of this Working Group.

The procedure of data collection (e.g., data collection for model training) for NW-side AI/ML model was studied and the corresponding outcome are captured in TR 38.843 as above. From RAN1 perspective, the training data generated by UE needs to be looked into. The contents of collected data and the corresponding container/signalling to convey the collected data will be further discussed as follows.

2.1.1 Training Data collection content

In RAN2, the transmission of collected data for NW-side model was agreed via RRC signaling in MDT. However, the corresponding content is not finalized. Based on the previous discussion in RAN1, three types of contents are proposed as follows.

Proposal 1. From RAN 1 perspective, for the content of data collection for training for NW-sided model via higher layer signaling (i.e., MDT in RRC), support the following three types of content for each transmission occasion of configured RS resources

- Type 1: L1-RSRP of each of the configured RS resources

  - The resource index corresponding to the L1-RSRP is derived implicitly by the order of configured resources

- Type 2: L1-RSRP of each of configured RS resources in a first set and K1 resource indexes with K1 highest L1-RSRP for configured RS resources in a second set

  - The resource index corresponding to the L1-RSRP for the first set is derived implicitly by the order of configured resources

  - K1 is configured by RRC

- Type 3: L1-RSRP of each of configured RS resources in a first set and K2 resource indexes with K1 highest L1-RSRP and the corresponding L1-RSRPs for configured RS resources in a second set

  - The resource index corresponding to the L1-RSRP for the first set is derived implicitly by the order of configured resources

  - K2 is configured by RRC

Note: “for content for data collection for training for NW-sided model via higher layer signaling (i.e., MDT in RRC)” is only for discission purpose.

2.1.2 Signalling/Container for training data collection

Apart from the content of data collection, another aspect is the signalling/container to convey the content of collected data for training. It was agreed in RAN2 that data collected by UE can be conveyed by higher layer signaling via MDT. Apart from higher layer signaling, L1 signaling can also be leveraged for this purpose.

One issue for data collection is that the maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP (i.e., maxNumberCSI-RS-Resource) is 64. Also, the maximum of resource can be configured by a CMR set is 64 (maxNrofNZP-CSI-RS-ResourcesPerSet = 64, maxNrofCSI-SSB-ResourcePerSet = 64).

TS38.306 beamManagementSSB-CSI-RS Defines support of SS/PBCH and CSI-RS based RSRP measurements. The capability comprises signalling of - maxNumberSSB-CSI-RS-ResourceOneTx indicates maximum total number of configured one port NZP CSI-RS resources and SS/PBCH blocks that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). On FR2, it is mandatory to report >=8; On FR1, it is mandatory with capability signalling to report >=8. - maxNumberCSI-RS-Resource indicates maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] across all serving cells (see NOTE). It is mandated to report at least n8 for FR1. - maxNumberCSI-RS-ResourceTwoTx indicates maximum total number of two ports NZP CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). - supportedCSI-RS-Density indicates density of one RE per PRB for one port NZP CSI-RS resource for RSRP reporting, if supported. On FR2, it is mandatory to report either "three" or "oneAndThree"; On FR1, it is mandatory with capability signalling to report either "three" or "oneAndThree". - maxNumberAperiodicCSI-RS-Resource indicates maximum number of configured aperiodic CSI-RS resources across all serving cells (see NOTE). For FR1 and FR2, the UE is mandated to report at least n4..

TS38.331 BeamManagementSSB-CSI-RS ::= SEQUENCE { maxNumberSSB-CSI-RS-ResourceOneTx ENUMERATED {n0, n8, n16, n32, n64}, maxNumberCSI-RS-Resource ENUMERATED {n0, n4, n8, n16, n32, n64}, maxNumberCSI-RS-ResourceTwoTx ENUMERATED {n0, n4, n8, n16, n32, n64}, supportedCSI-RS-Density ENUMERATED {one, three, oneAndThree} OPTIONAL, maxNumberAperiodicCSI-RS-Resource ENUMERATED {n0, n1, n4, n8, n16, n32, n64} }

To enable data collection of beam set with considerable size (e.g., 256), these constrains need to be relaxed. For example, advanced UE capability is required to update the measurement of more than 64 CSI-RS/SSB resources. Also, the constrain for the size of CMR set can be relaxed or multiple CMR sets can be used for beam measurement/reporting. Hence, the following is proposed.

Proposal 2. For the measurement of more than 64 beams, support the following:

- Introduce extend the existing UE capability (i.e., maxNumberCSI-RS-Resource) to indicate the maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP to [256]

- For the configuration of measurement set, down-select one of the following

  - Alt-1. extend the maxNrofNZP-CSI-RS-ResourcesPerSet (i.e., maximum number of NZP CSI-RS resources per resource set) to [256].

  - Alt-2. Up to [4] resource sets can be configured for Set A in a CSI-ReportConfig, where a reported beam index is corresponding to a resource from one of the [4] resource sets.

2.2 Data collection for model inference

In general, there are two aspects for beam report enhancement: spatial domain (e.g., to support one reporting instance with measurements of more than 4 beams) and time domain (e.g., to support one reporting instance with beam measurements for multiple time instances). Details are discussed as follows. Note that the methods for data collection for model inference can be reused for performance monitoring.

2.2.1 Spatial domain related enhancement (> 4 beams)

Agreement (RAN1#116) For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement (RAN1#116bis) For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point - support using existing CSI framework for configuration of Set B as the starting point - Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications Agreement (RAN1#117) At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range Agreement (RAN1#117) For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information

An FFS point is the beam information for the reporting. Basically, there are two options: 1) CRI/SSBRI based indication as legacy; 2) bitmap based indication and a beam index to indicate the reference resource for differential L1-RSRP.

Proposal 3. For NW-side AI/ML model inference, at least for BM-Case 1, regarding the content in a beam report in L1 signalling, if M < the size of the measurement resource set, support the configurability between the following options:

- Option 1. M CRI/SSBRI and M corresponding L1-RSRP are reported

- Option 2. K-bit bitmap, a resource indicator and M corresponding L1-RSRP are reported, where

  - The number of nonzero bits in the bitmap is M, and K is the size of the measurement resource set

  - The k-th MSB of the bitmap corresponds to the k-th resource in the resource set by the increasing order of resource ID, and the m-th nonzero bit of the bitmap corresponds to the m-th reported resources, 1≤m≤M

  - The resource indicator indicates the resource with the largest measured value of L1-RSRP from the M reported resources in the resource set, and the size of the corresponding CSI field is log2M.

2.2.2 Time domain related enhancement (multiple past time instances)

For BM-Case2 with NW AI/ML model inference, NW requires L1-RSRPs for multiple past time instances as model input. For L1 signaling, there are two existing methods to obtain L1-RSRP measurement for a particular time instance: 1) CSI-ReportConfig configured with timeRestrictionForChannelMeasurements; 2) aperiodic CSI reporting using aperiodic CSI-RS. However, those methods can only provide measurement for a single time instance in the corresponding CSI reporting and the time instance is couple with the timing of the CSI reporting (e.g., the latest measurement no later than the CSI reference for the CSI reporting). This is very restrictive if more than one measurement results are desired for model inference from NW perspective. Hence, it is preferable to support L1 beam reporting (i.e., CSI-ReportConfig) with measurements for each of multiple past time instances in one reporting instance. NW can directly use the measurement results as model input for future beam prediction.

Proposal 4. For NW-side AI/ML model inference, support a CSI report with the measurement results for each of the P (P>1) latest transmission occasions no later than CSI reference resource corresponding to the CSI report

- P is configured in the corresponding CSI-ReportConfig via RRC subject to UE capability

- Reuse the same report format (i.e., CSI field mapping order) as for BM-Case2 for UE-sided model inference in case of Opt 2 by replacing “time instance” to “transmission occasion”

2.3 On the P2 beam update/indication after beam prediction

In RAN1#120bis, the following proposal was formulated with two different options.

Proposal (RAN1#120bis) For both UE/NW sided model, for enabling a P2 sweep based on predicted Top-K beams, further study the following solution: - Option A: NW configures K CSI-RS resources, and dynamically indicate the QCL information (TCI-stateID) as part of the MAC CE, to the resources of a CMR for at least for AP CSI-RS for AP CSI report - Option B: for UE-sided model only, NW indicates that the corresponding TCI states for the P2 ResourceSet follows the UE predicted Top-K beams, - FFS on details including, - linkage of the inference report and the P2 (prediction result measurements) CSI-Report configuration, including BM-Case 2 when UE predicts N time instances - transmission order of the beams in the subsequent P2 (prediction result measurements)

The intention of the two options is to update the beam information of K CSI-RS resources after the prediction of Top-K beams. For Option A, similar behavior can be done by using SP CSI-RS with AP CSI reporting. For example, SP CSI-RS can be (re-)activated by MAC-CE based on the existing TS 38.214 as follows (in yellow highlighted part).

Also, it is not necessary to additionally support TCI state update of AP CSI-RS resources in a given CSI triggering state using MAC-CE. In such case, SP CSI-RS resources can be configured and the existing CSI-RS triggering mechanism can be reused.

TS 38.214-i60 Clause 5.2.1.5.2 Semi-persistent CSI/Semi-persistent CSI-RS For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'. - when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and for FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

For Option B, similar TCI state update via MAC-CE can be done for one or more resource sets with SP CSI-RS resources without specification enhancement. Option B may save some signalling overhead but requires a lot of further discussion on the container (e.g., MAC-CE or DCI) of NW indication and what exact information is in the container.

Proposal 5. Agree the following as conclusion:

- For UE/NW-sided model, P2 beam sweeping based on predicted beams is supported by existing specification.

- UE-side AI/ML model

In this section, specification impact with respect to UE-side AI/ML model for beam management is discussed. In particular, different LCM operations (e.g., data collection, model inference, model monitoring, etc) associated with the UE-side AI/ML and the corresponding signalling/mechanism(s) to facilitate the LCM operations are further discussed as follows.

3.0 Typical procedure

Beam management specific LCM procedure for UE-side AI/ML model is illustrated in Figure 2.

Figure 2. LCM procedure for UE-side AI/ML model.

For data collection, NW can configure or trigger the CSI-RS/SSB transmission for L1-RSRP measurement. The RS configuration may depend on the request from UE. For example, the RS resources for Set A and/or Set B can be configured by NW based on the request from UE. UE performs measurements based on the configured/triggered RS resources and performs data collection correspondingly. From UE perspective, the collected data can be used for model training, model inference or performance monitoring. For model training, the collected data can be further delivered or process for offline training or fine tuning. For model inference, UE can use the collected data as model input and generate the corresponding output, where the output (e.g., predicted beam and/or predicted L1-RSRP) should be reported to NW for predictive beam indication. Moreover, the collected data can also be used to monitor the performance of the model. For type 1 performance monitoring, UE can provide measurement result or associated performance metric to NW and NW can further indicate the activation/deactivation/fallback of AI/ML operation via DL signalling. For type 2 performance monitoring, UE can decide whether to activate/deactivate/fallback the AI/ML operation based on the collected data.

3.1 Data collection for model training

Agreement (RAN1#120) For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model. Agreement (RAN1#120bis) For UE-sided model, regarding the resource type for data collection purpose, only always-on SSB and P/SP CSI-RS are supported.

In case of SP CSI-RS resources are used for both Set A and Set B, MAC-CE enhancement is needed for the activation of SP CSI-RS. The existing MAC-CE can only indicate one CSI-RS resource set while two resource sets are used for UE-side data collection. One solution is to support joint activation/deactivation of Set A and Set B using the existing MAC-CE. For example, if Set A is activated by a MAC-CE, UE perform measurement on both Set A and Set B. For example, if Set A is deactivated by a MAC-CE, UE perform measurement on both Set A and Set B. Also, in case of Set B is a subset of Set A, the associated TCI state indication for the resources in Set A can be applicable to the resources in Set B.

Proposal 6. For UE-sided model, regarding the SP CSI-RS resource for data collection purpose, support joint activation/deactivation of Set A and Set B using the existing MAC-CE.

- If Set A is activated by a MAC-CE, UE perform measurement on both Set A and Set B

  - If Set B is a subset of Set A, TCI state for a resource in Set B is the same as the TCI state of same resource in Set A indicated by the MAC-CE

- If Set A is deactivated by a MAC-CE, UE stop perform measurement on both Set A and Set B

3.2 Model inference

3.2.1 Procedure for model inference

Agreement (RAN1#116) For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction. Agreement (RAN1#117) Following Working Assumption is confirmed. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Agreement (RAN1#119) For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A. Agreement (RAN1#120) For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value. Agreement (RAN1#120) For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion. Agreement (RAN1#120bis) For BM-Case1, for the Top K beam(s) report as the inference results - K is configured in inference report configuration to the UE. Conclusion (RAN1#120bis) For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

AI/ML model output is dependent on the type of the model. For classification-based model, each of the Set A beams corresponds to a probability. Typically, top-K beams are the beams with the highest probability values. For regression-based model, each of the Set A beams corresponds to a predicted L1-RSRP. Typically, top-K beams are determined based on the value of the corresponding predicted L1-RSRPs. For regression-based model, one remaining issue from the RAN1#116bis is whether probability information is reported. In our view, the probability information could also be useful since the probability can reflect beam prediction accuracy in some extend. Hence, apart from the report of K predicted beam, it is also preferable to report: the probabilities correspond to the K predicted beams.

Proposal 7. For UE-sided model, at least for BM-Case1, for content in the report of inference results, support:

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

In case of K>1, one issue is the convey of the ranking information of the reported K beam. In legacy, for L1-RSRP based beam reporting, the ranking information is conveyed implicitly based on the reported L1-RSRP. For example, NW can assume that the beam corresponding to the highest L1-RSRP is the beam that has the highest ranking. For beam reporting for UE-side model inference, since new reporting contents/combinations are introduced, the method to convey the ranking information of the predicted Top K beams should be supported.

Proposal 8. For UE-sided model, at least for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams, if K>1, the reported CRI or SSBRI #k corresponding to the k-th strongest resource.

For BM-Case1, a remaining issue for inference report is the report format for option 1. A report format similar to option 2 is proposed as follows.

Proposal 9. For UE-sided model inference for BM-Case1, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on only predicted Top K beam(s) among a set of beams.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #K

- CRI or SSBRI #k corresponding to the k-th strongest predicted resource, where k = 1, 2…, K

- Note: existing payload sizes for CRI/SSBRI is reused.

Agreement (RAN1#116bis) For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N (N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement (RAN1#118) For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted. Agreement (RAN1#118bis) For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource Agreement (RAN1#120) For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8] - Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report. Agreement (RAN1#120bis) For BM-Case 2 of UE-side model, one RRC parameter represents the time gap configured for between two consecutive future time instances, and also represents the time gap between the reference time and the first future time instance for prediction.

A remaining issue for BM-Case2 inference report is the definition of a future time instance. In Rel-18, the time instance for doppler related CSI prediction is defined in slot level. Similarly, for BM-Case2, future time instance should be defined in slot level. As baseline, a future time instance is defined as a single slot.

Proposal 10. For BM-Case 2 of UE-side model, future time instance is defined as a single slot.

Agreement (RAN1#118) For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details Agreement (RAN1#120) For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

A remaining issue for BM-Case2 inference report is report format. Basically, for both option 1 and option 2, the following report format is proposed.

Proposal 11. For UE-sided model inference for BM-Case2, support the following report format (i.e., CSI field mapping order) for beam information on only predicted Top K beam(s) among a set of beams:

CRI or SSBRI #1 of time instance #1

CRI or SSBRI #2 of time instance #1

…

CRI or SSBRI #K of time instance #1

CRI or SSBRI #1 of time instance #2

CRI or SSBRI #2 of time instance #2

…

CRI or SSBRI #K of time instance #2

…

CRI or SSBRI #1 of time instance #N

CRI or SSBRI #2 of time instance #N

…

CRI or SSBRI #K of time instance #N

- Time instance #1~#N are mapped to the N time instance(s) based on the time domain order of the time instances

  - where time instance #1 is mapped to the earliest time instance from the N time instance(s)

- CRI or SSBRI #k corresponding to the k-th strongest predicted resource with the same time instance, where k = 1, 2…, K

- Note: existing payload sizes for CRI/SSBRI is reused.

3.2.2 On the consistency between training and inference

Agreement (RAN1#116bis) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. Agreement (RAN1#118) For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list Agreement (RAN1#120) For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

A remaining issue is whether aperiodic CSI RS can be configured for Set B for inference reporting. In our understanding, this configuration should be supported unless there are some critical issues.

Proposal 12. For UE-sided model, in CSI-ReportConfig for inference, support to configure Set B with aperiodic CSI-RS resources.

3.3 Performance monitoring

Agreement (RAN1#117) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In RAN1#117, Type 1 performance monitoring is supported for BM-Case1 and BM-Case2 with a UE-side AI/ML model. Also, the supported Type 1 performance monitoring includes two options: Option 1 (NW-side performance monitoring) and Option 2 (UE-assisted performance monitoring). In the following, discussions are provided from the perspective of different types of performance monitoring.

3.3.1 Type 1 performance monitoring

Agreement (RAN1#119) At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring. Agreement (RAN1#120bis) For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB Agreement (RAN1#120bis) For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support. Agreement (RAN1#120bis) For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement (RAN1#120bis) For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A

A remaining issue is the method for resource mapping in case of the size of a set for monitoring is smaller than the size of Set A. In our view, one to multiple mapping should be supported to avoid the case that predicted beam is not in monitoring set for a long time. To support one to multiple mapping, two methods can be considered: 1) predefined mapping rule; 2) mapping configured by RRC. Predefined mapping rule can save signalling overhead while RRC based mapping can provide more flexibility from gNB perspective. For example, the resource set for monitoring can be regard as a down-sampling subset of Set A. The corresponding mapping can be done in predefined manner, where the i-th resource in Set A is mapped to the iD-th resource in the resource set for monitoring, D is the down-sampling factor. For another example, each resources within the resource set for monitoring is mapped to one or more resources in Set A via dedicated RRC signaling.

Proposal 13. At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the calculation of the performance metric for beam prediction accuracy, if the size of Set A is the lager than the size of resource set for monitoring down-select from one of the following:

- Alt-1. The i-th resource within Set A is linked to the iD-th resource in the resource set for monitoring, where D is configured by RRC

  - D = [1, 2, 4, 8]

- Alt-2. Each resource within the resource set for monitoring is linked to one or more resources within Set A based on dedicated RRC signaling

  - A list with Y entries is configured by the RRC, where Y is the size of the set for monitoring

  - The y-th entry in the list corresponds to the y-th resources in the set for monitoring by increasing order of resource ID, 1≤y≤Y

  - Each entry in the list indicates one or more resources in Set A

- Alt-3. Each resource within the resource set for monitoring is linked to one resource within Set A based on dedicated RRC signaling

  - X-bit bitmap with Y non-zero bits is configured by the RRC, where X is the size of Set A and Y is the size of the set for monitoring

  - The x-th MSB of the bitmap corresponds to x-th resource in Set A by increasing order of resource ID

  - The y-th nonzero bit of the bitmap corresponds to the y-th resources in the set for monitoring by increasing order of resource ID, 1≤y≤Y

Working Assumption (RAN1#120bis) At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. Working Assumption (RAN1#120bis) For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring. Working Assumption (RAN1#120bis) For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring. - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

For BM-Case1, the time domain linkage between monitoring occasion and inference report was agreed as working assumption. An FFS part is whether to support the configuration of threshold to limit the minimum slot offset. The threshold is optionally configured which is a way forward for BM-Case1.

Proposal 14. Confirm the working assumption in RAN1#120bis with the following update

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

For BM-Case1, the method to calculate beam prediction accuracy was agreed as a working assumption. An FFS part is whether to support additional rule for the counting of N. The introduction of the threshold above is enough for the determination of N linked pair no further enhancement is needed. Hence the working assumption can be confirmed without FFS part.

Proposal 15. Confirm the working assumption in RAN1#120bis with the following update

For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

For BM-Case2, the method to calculate beam prediction accuracy was agreed as a working assumption. Similar to BM-Case1, the working assumption can be confirmed without FFS part.

Proposal 16. Confirm the working assumption in RAN1#120bis with the following update

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring. - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

In RAN1#120bis, the reporting of a new quantity (i.e., beam accuracy indicator, BAI) was discussed. BAI should be calculated based on the number of Np transmission occasions which satisfies the corresponding performance metric within the N latest transmission occasions. One decision point is whether to perform quantization to Np. In our understanding, such quantization is redundant. Hence, we have the following proposal.

Proposal 17. Introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N) and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1)

- The size of CSI field associated with the BAI is log2(N+1)

- Where Np is the number of transmission occasion for monitoring that meet the metric within the N transmission occasion(s)

3.4 PU occupation of CSI reports related to AI/ML

Agreement (RAN1#118bis) For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2 Agreement (RAN1#120bis) - For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection: - Option 1: only dedicated AI/ML PU is occupied, OAPU=N1 is reported by UE. - And OCPU=0 - Option 2: only legacy CPU is occupied, OCPU=M it is reported by UE. - Option 3: both dedicated AI/ML PU and legacy CPU are occupied, OAPU=N2 is reported by UE. - And OCPU=1 Note: The supported option by UE is reported by UE capability, if multiple options are supported. - The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately. Agreement (RAN1#120bis) For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, OCPU=1. Note: The occupation duration is a separate discussion.

For UE-side model data collection, the corresponding measurement and processing is up to UE implementation and can be not relevant to the use of CPU. Hence, OCPU=0 is proposed for this case.

Proposal 18. For UE-sided model, regarding a CSI-ReportConfig for data collection, OCPU=0.

In RAN1#120bis, three options were formulated for the occupation of AI/ML PU and CPU. In our understanding, for processing of a CSI report corresponding to a CSI-ReportConfig for inference, the number of occupied CPU is either 0 or 1 subject to UE capability. Meanwhile, the number of occupied dedicated AI/ML PU is subject to UE capability.

Proposal 19. For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, regarding the processing of a CSI report corresponding to a CSI-ReportConfig for inference, support the following

- The number of occupied dedicated AI/ML PU is subject to UE capability

- The number of occupied CPU is either 0 or 1 subject to UE capability

For CPU and/or AI/ML PU occupation time of a CSI report corresponding to a CSI-ReportConfig for inference, legacy definition of CPU can be reused. For BM-Case1, the legacy definition of Rel-15 P/SP/AP CSI can be reused. For BM-Case2, the legacy definition of CSI for Rel-18 doppler domain CSI prediction can be reused.

Proposal 20. For UE-side model, for processing of a CSI report corresponding to a CSI-ReportConfig for inference, legacy definition of CPU occupation time is reused for the occupation time for both CPU and AI/ML PU for the CSI report

- For BM-Case1, the legacy definition of Rel-15 P/SP/AP CSI is reused

- For BM-Case2, the legacy definition of CSI for Rel-18 doppler domain CSI prediction is reused

As agreed in RAN1#120bis, 1 CPU is occupied for a CSI report corresponding to a CSI-ReportConfig for monitoring. For CPU occupation time of a CSI report corresponding to a CSI-ReportConfig for monitoring, legacy definition of CPU occupation time Rel-18 doppler domain CSI prediction can be reused in principle. For AP CSI report, the legacy definition of CPU occupation time is the same as that of the existing AP CSI report. For periodic or semi-persistent CSI report, the first symbol of the N-th latest transmission occasion(s) of monitoring resources no later than CSI reference resource should be the starting point since the performance metric calculation starts from the earliest transmission occasion from the N latest transmission occasion(s) of monitoring resources.

Proposal 21. For UE-side model, for processing of a CSI report corresponding to a CSI-ReportConfig for inference, the principle of CSI for Rel-18 doppler domain CSI prediction is reused:

- For AP CSI report, the legacy definition of CPU occupation time for existing AP CSI report is reused

- For P/SP CSI report, CPU occupies from the first symbol of the N-th latest transmission occasion(s) of monitoring resources no later than CSI reference resource, until the last symbol of the PUSCH/PUCCH carrying the report

3.5 Priority rules for CSI reports

In the existing TS38.214, the priority rules for CSI reports are defined in clause 5.2.5. With the introduction of AI related CSI reports, those priority rules need to be revisited. The simplest solution is to reuse the legacy equation as much as possible. Note that parameter k in the existing equation is determined by the reporting contents of CSI report. Basically, in case of beam report (i.e., CSI reports carrying L1-RSRP or L1-SINR) k = 0; and k = 1 otherwise (i.e., CSI reports not carrying L1-RSRP or L1-SINR). Similarly, when the CSI report with inference results for beam prediction, the value of k is equal to 0.

Proposal 22. For the determination of CSI report associated with AI/ML, the existing PriiCSIy,k,c,s is reused

- k =0 when the CSI report is with inference result for BM-Case1 and BM-Case2 for UE-sided model

3.6 The determination of updated CSI report

TS 38.214-i60 Clause 5.2.1.6 CSI processing criteria The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds.

The existing CSI processing criteria based on CPU supports the feature that UE only updates a subset of requested CSI reports based on CPU occupation and priority rules for CSI reports. Similarly, the CSI processing criteria based on AI/ML PU should support the similar feature that UE only updates a subset of requested CSI reports based on AI/ML PU  occupation and priority rules for CSI reports.

Proposal 23. Support UE only updates a subset of requested CSI reports based on AI/ML PU occupation and priority rules for CSI reports

- Follow the legacy principle for updating a subset of requested CSI reports based on CPU occupation and priority rules for CSI reports

Since a CSI report for AI/ML may occupy both CPU and AI/ML PU, it is necessary to clarify the interaction between the rules for updating CSI reports. A efficient way is to identified the non-updated CSI reports for AI/ML first and then exclude them out of the pool of the CSI reports for the determination of updated CSI report based on CPU occupation and priority rule.

Proposal 24. For the determination of updated CSI reports based on CPU occupation and priority rules for CSI reports subject to a given OFDM symbol, a CSI report is not considered if the following conditions are met:

- the CSI report occupies one CPU and at least one AI/ML PU

- the CSI report is not required to be updated subject to the same OFDM symbol based on AI/ML PU occupation and priority rules for CSI reports

3.7 Z timeline

For legacy CSI report for beam management, the Z timeline is Z3/Z3’. However, for the generation of inference results, more time may be needed. The extension of CSI timeline can be the scaling of Z3/Z3’. Similar to the discussion of OCPU, the complexity of model inference is UE-specific. Then, it is safe to assume that the scaling factor is at least depending on UE capability.

Proposal 25. For the CSI report for inference for UE-side model, support the scaling of legacy Z timeline (i.e., Z3/Z3’) for beam management with a scaling factor

- The scaling factor is subject to UE capability

- On the contents for Option B of applicability report

Agreement (RAN1#119) - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

In RAN1#119, the above agreement was made for applicability report. For Option-B, a container other than CSI-ReportConfig is used for applicability report. Some further consideration of the contents in this container are discussed as follows.

At least the following information are needed: 1) One or two associated ID(s); 2) the size of Set A; 3) the size of Set B; 4) mapping between Set A and Set B (if one single associated ID is provided); 3) Report quantity; 4) time gap information (for BM-Case2); 5) the number of predicted time instance(s) (for BM-Case2). Here, Set A information is the size of Set A and Set B information is the size of Set B. This is enough at least for the case of 2 associated IDs are provided. If a single associated ID is provided, the mapping between Set A and Set B need to be additionally provided to indicate subset information.

Proposal 26. For Option B of applicability report, consider the following as part of the contents

- One or two associated ID(s)

- The size of Set A

- The size of Set B

- Mapping between Set A and Set B (if one single associated ID is provided)

- Report quantity

- Time gap information (for BM-Case2)

- The number of predicted time instance(s) (for BM-Case2)

- Conclusion

The observations and proposals made in this contribution are summarized below.

Proposal 1. From RAN 1 perspective, for the content of data collection for training for NW-sided model via higher layer signaling (i.e., MDT in RRC), support the following three types of content for each transmission occasion of configured RS resources

- Type 1: L1-RSRP of each of the configured RS resources

  - The resource index corresponding to the L1-RSRP is derived implicitly by the order of configured resources

- Type 2: L1-RSRP of each of configured RS resources in a first set and K1 resource indexes with K1 highest L1-RSRP for configured RS resources in a second set

  - The resource index corresponding to the L1-RSRP for the first set is derived implicitly by the order of configured resources

  - K1 is configured by RRC

- Type 3: L1-RSRP of each of configured RS resources in a first set and K2 resource indexes with K1 highest L1-RSRP and the corresponding L1-RSRPs for configured RS resources in a second set

  - The resource index corresponding to the L1-RSRP for the first set is derived implicitly by the order of configured resources

  - K2 is configured by RRC

Note: “for content for data collection for training for NW-sided model via higher layer signaling (i.e., MDT in RRC)” is only for discission purpose.

Proposal 2. For the measurement of more than 64 beams, support the following:

- Introduce extend the existing UE capability (i.e., maxNumberCSI-RS-Resource) to indicate the maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP to [256]

- For the configuration of measurement set, down-select one of the following

  - Alt-1. extend the maxNrofNZP-CSI-RS-ResourcesPerSet (i.e., maximum number of NZP CSI-RS resources per resource set) to [256].

  - Alt-2. Up to [4] resource sets can be configured for Set A in a CSI-ReportConfig, where a reported beam index is corresponding to a resource from one of the [4] resource sets.

Proposal 3. For NW-side AI/ML model inference, at least for BM-Case 1, regarding the content in a beam report in L1 signalling, if M < the size of the measurement resource set, support the configurability between the following options:

- Option 1. M CRI/SSBRI and M corresponding L1-RSRP are reported

- Option 2. K-bit bitmap, a resource indicator and M corresponding L1-RSRP are reported, where

  - The number of nonzero bits in the bitmap is M, and K is the size of the measurement resource set

  - The k-th MSB of the bitmap corresponds to the k-th resource in the resource set by the increasing order of resource ID, and the m-th nonzero bit of the bitmap corresponds to the m-th reported resources, 1≤m≤M

  - The resource indicator indicates the resource with the largest measured value of L1-RSRP from the M reported resources in the resource set, and the size of the corresponding CSI field is log2M.

Proposal 4. For NW-side AI/ML model inference, support a CSI report with the measurement results for each of the P (P>1) latest transmission occasions no later than CSI reference resource corresponding to the CSI report

- P is configured in the corresponding CSI-ReportConfig via RRC subject to UE capability

- Reuse the same report format (i.e., CSI field mapping order) as for BM-Case2 for UE-sided model inference in case of Opt 2 by replacing “time instance” to “transmission occasion”

Proposal 5. Agree the following as conclusion:

- For UE/NW-sided model, P2 beam sweeping based on predicted beams is supported by existing specification.

Proposal 6. For UE-sided model, regarding the SP CSI-RS resource for data collection purpose, support joint activation/deactivation of Set A and Set B using the existing MAC-CE.

- If Set A is activated by a MAC-CE, UE perform measurement on both Set A and Set B

  - If Set B is a subset of Set A, TCI state for a resource in Set B is the same as the TCI state of same resource in Set A indicated by the MAC-CE

- If Set A is deactivated by a MAC-CE, UE stop perform measurement on both Set A and Set B

Proposal 7. For UE-sided model, at least for BM-Case1, for content in the report of inference results, support:

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

Proposal 8. For UE-sided model, at least for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams, if K>1, the reported CRI or SSBRI #k corresponding to the k-th strongest resource.

Proposal 9. For UE-sided model inference for BM-Case1, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on only predicted Top K beam(s) among a set of beams.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #K

- CRI or SSBRI #k corresponding to the k-th strongest predicted resource, where k = 1, 2…, K

- Note: existing payload sizes for CRI/SSBRI is reused.

Proposal 10. For BM-Case 2 of UE-side model, future time instance is defined as a single slot.

Proposal 11. For UE-sided model inference for BM-Case2, support the following report format (i.e., CSI field mapping order) for beam information on only predicted Top K beam(s) among a set of beams:

CRI or SSBRI #1 of time instance #1

CRI or SSBRI #2 of time instance #1

…

CRI or SSBRI #K of time instance #1

CRI or SSBRI #1 of time instance #2

CRI or SSBRI #2 of time instance #2

…

CRI or SSBRI #K of time instance #2

…

CRI or SSBRI #1 of time instance #N

CRI or SSBRI #2 of time instance #N

…

CRI or SSBRI #K of time instance #N

- Time instance #1~#N are mapped to the N time instance(s) based on the time domain order of the time instances

  - where time instance #1 is mapped to the earliest time instance from the N time instance(s)

- CRI or SSBRI #k corresponding to the k-th strongest predicted resource with the same time instance, where k = 1, 2…, K

- Note: existing payload sizes for CRI/SSBRI is reused.

Proposal 12. For UE-sided model, in CSI-ReportConfig for inference, support to configure Set B with aperiodic CSI-RS resources.

Proposal 13. At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the calculation of the performance metric for beam prediction accuracy, if the size of Set A is the lager than the size of resource set for monitoring down-select from one of the following:

- Alt-1. The i-th resource within Set A is linked to the iD-th resource in the resource set for monitoring, where D is configured by RRC

  - D = [1, 2, 4, 8]

- Alt-2. Each resource within the resource set for monitoring is linked to one or more resources within Set A based on dedicated RRC signaling

  - A list with Y entries is configured by the RRC, where Y is the size of the set for monitoring

  - The y-th entry in the list corresponds to the y-th resources in the set for monitoring by increasing order of resource ID, 1≤y≤Y

  - Each entry in the list indicates one or more resources in Set A

- Alt-3. Each resource within the resource set for monitoring is linked to one resource within Set A based on dedicated RRC signaling

  - X-bit bitmap with Y non-zero bits is configured by the RRC, where X is the size of Set A and Y is the size of the set for monitoring

  - The x-th MSB of the bitmap corresponds to x-th resource in Set A by increasing order of resource ID

  - The y-th nonzero bit of the bitmap corresponds to the y-th resources in the set for monitoring by increasing order of resource ID, 1≤y≤Y

Proposal 14. Confirm the working assumption in RAN1#120bis with the following update

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Proposal 15. Confirm the working assumption in RAN1#120bis with the following update

For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Proposal 16. Confirm the working assumption in RAN1#120bis with the following update

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring. - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

Proposal 17. Introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, where BAI is Np (0 ≤ Np≤ N) and the corresponding beam prediction accuracy is NpN, (0≤ NpN≤ 1)

- The size of CSI field associated with the BAI is log2(N+1)

- Where Np is the number of transmission occasion for monitoring that meet the metric within the N transmission occasion(s)

Proposal 18. For UE-sided model, regarding a CSI-ReportConfig for data collection, OCPU=0.

Proposal 19. For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, regarding the processing of a CSI report corresponding to a CSI-ReportConfig for inference, support the following

- The number of occupied dedicated AI/ML PU is subject to UE capability

- The number of occupied CPU is either 0 or 1 subject to UE capability

Proposal 20. For UE-side model, for processing of a CSI report corresponding to a CSI-ReportConfig for inference, legacy definition of CPU occupation time is reused for the occupation time for both CPU and AI/ML PU for the CSI report

- For BM-Case1, the legacy definition of Rel-15 P/SP/AP CSI is reused

- For BM-Case2, the legacy definition of CSI for Rel-18 doppler domain CSI prediction is reused

Proposal 21. For UE-side model, for processing of a CSI report corresponding to a CSI-ReportConfig for inference, the principle of CSI for Rel-18 doppler domain CSI prediction is reused:

- For AP CSI report, the legacy definition of CPU occupation time for existing AP CSI report is reused

- For P/SP CSI report, CPU occupies from the first symbol of the N-th latest transmission occasion(s) of monitoring resources no later than CSI reference resource, until the last symbol of the PUSCH/PUCCH carrying the report

Proposal 22. For the determination of CSI report associated with AI/ML, the existing PriiCSIy,k,c,s is reused

- k =0 when the CSI report is with inference result for BM-Case1 and BM-Case2 for UE-sided model

Proposal 23. Support UE only updates a subset of requested CSI reports based on AI/ML PU occupation and priority rules for CSI reports

- Follow the legacy principle for updating a subset of requested CSI reports based on CPU occupation and priority rules for CSI reports

Proposal 24. For the determination of updated CSI reports based on CPU occupation and priority rules for CSI reports subject to a given OFDM symbol, a CSI report is not considered if the following conditions are met:

- the CSI report occupies one CPU and at least one AI/ML PU

- the CSI report is not required to be updated subject to the same OFDM symbol based on AI/ML PU occupation and priority rules for CSI reports

Proposal 25. For the CSI report for inference for UE-side model, support the scaling of legacy Z timeline (i.e., Z3/Z3’) for beam management with a scaling factor

- The scaling factor is subject to UE capability

Proposal 26. For Option B of applicability report, consider the following as part of the contents

- One or two associated ID(s)

- The size of Set A

- The size of Set B

- Mapping between Set A and Set B (if one single associated ID is provided)

- Report quantity

- Time gap information (for BM-Case2)

- The number of predicted time instance(s) (for BM-Case2)

References

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm.