3GPP TSG RAN WG1 #121		 R1-2503551

St Julian's, Malta, 19th – 23rd May, 2025

Agenda Item:		9.1.2

Source:				Samsung

Title:					Discussion for supporting AI/ML based positioning accuracy enhancement

Document for:	     Discussion

# Introduction

In the approved WID [1]( RP-242399), the following objective are included:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Thus, in the contribution, the important specification support related issues are discussed to facilitate the AI/ML based positioning accuracy enhancement.

# Discussion for AI/ML based positioning

Based on the report of the study item outcome, many spec impact on supporting the AI/ML based positioning is identified, especially for the LCM operations. In the following sections, the important aspects as we can see for supporting such feature are discussed.

## General Procedure

For all five cases, although there could be difference on the entity where AI/ML model is deployed, the general procedure to facilitate the AI/ML based positioning method could be shared.

Table 1 – AI/ML based Pos on each side

| UE side model | TRP(gNB) side model | LMF side model |
| --- | --- | --- |
| Case 1 | Case 3a | Case 3b |

From our understanding, such general procedure should include following operations:

- Triggering of using AI/ML based positioning: by this step, the entity determines to use AI/ML based method for positioning, instead of using other methods

- Model Determination/Selection: by this, the entity determines which AI/ML model to be used for the positioning purpose

- Data collection: this could contribute to multiple purpose, including model training, inference, monitoring, and/or finetuning;

- Model inference: with obtained model input, the entity could generate the model output

- Performance/model monitoring: the entity needs to keep tracking whether the current model is working properly, if not, some necessary operation is needed.

Then in the following, the above operations will be discussed with potential spec impact to support them.

## AI/ML based Positioning Triggering

As studied during the SI phase, the attractive application of the AI/ML model is motivated by the NLoS heavy scenarios, where the legacy RAT-dependent positioning method cannot work well.  Although by method perspective, there is no restrictions on using AI/ML based method for positioning on a particular situation, there should be some division on when the AI/ML based method is more preferable or the legacy method is so. When facing the positioning request from service, there should be a decision being made for the method selection, also meaning, the trigger condition of using AI/ML based positioning is satisfied or not. On top of the applicable conditions, the requirement on the AI/ML model could be another factor to impact whether the AI/ML is actually able to be used, e.g., an AI/ML model may require certain air interface resource to obtain the measurement/assistance information to facilitate the model operation, or certain computation/storage/power/processing time requirement is needed.

In summary, there could be triggers to enable the usage of AI/ML method, such trigger event may include the AI/ML model applicable conditions or the environment(channel) applicable conditions.

- RAN1 considers the trigger conditions for enabling AI/ML based method for positioning, including potential model applicable conditions or the environment(channel) applicable conditions.

## Model Determination/Selection

Once the entity (no matter it is UE/TRP/LMF) is determined to use AI/ML based method, the next question is which AI/ML model is to be used. Note here, we are trying to distinguish the Model identification, in which how a model is defined from 3GPP perspective. While for this step, we are to achieve a purpose that the involving entity (especially the entity where the model is deployed) could determine which model it will use.

For this model establishment, there could be two types of questions to discuss:

- Which entity is to determine the model?

- What are the criteria to determine the model?

Regarding first question, it will be related to where the AI/ML model is located (note that we only consider the single-side model here)

Table 2- possible entities to determine which model to use

| Model located entity | Which entity is to determine the model? |
| --- | --- |
| UE side | - UE only - UE with assisted by network (TRP/LMF) - TRP/LMF decides and indicates to UE |
| TRP side | - TRP only - TRP with assisted by LMF/UE - LMF decides and indicates to TRP - UE decides and indicates to TRP |
| LMF side | - LMF only - LMF with assisted by TRP/UE - TRP/UE decides and indicates to LMF |

As we can see from table 2, for the entity where the model is located, it can determine/decide the model to be used. On top of that,

- For UE side model, in general the network may have some assistance or even control on the used AI/ML model. Such assistance or control could be explicit or implicit, in which the explicit manner is to decides the model and indicate to UE while the implicit manner is to configure certain criteria or feedback certain applicable conditions to UE. The UE can decide which model to use among the models which satisfy the criteria or applicable conditions.

- For TRP side model, it’s allowed the LMF to decide the model used at the TRP. And both LMF/UE can provide certain assistance information for TRP to choose on the utilized model. While UE is not allowed to decide for TRP since the UE may be lack of full picture of the whole network situation.

- For LMF side model, it could additionally use assistance information from TPR/UE to determine the model in use. The TRP and UE are not allowed to decide for LMF.

- In addition to the entity where the model is deployed, the network can decide or /assist to decide the model used at UE side, the LMF can decide and LMF/UE can assist to decide the model used at TRP side, the TRP/UE can assist to decide the model used at LMF side.

Regarding the second question, on what the criteria is to determine the AI/ML model. In case of the AI model deployed entity to decide only, such criteria could be implementation issue thus not specified. While on other case, two types of specification efforts could be pursued:

Table 3 – model determination

| Type | Necessary spec impacts |
| --- | --- |
| Type 1: entity A decides for entity B | - The indication of the decision on model (align with the model identification outcome if any) |
| Type 2: entity A assists to entity B | - The indication of criteria, which may include certain measurement related threshold, or model applicable conditions - The indication of assistance information, which may include certain measurement related report/feedback, model applicable condition |

- RAN1 supports the indication of determined model from one entity to another entity

- RAN1 supports the indication of measurement related threshold/report/feedback and model applicable condition to assist the model determination from one entity to another entity

## Data Collection

Data collection is one important operation which can facilitate many LCM functions, including model training, inference, monitoring and finetuning. There are several aspects on such operation to be discussed including the data content, data generation entity, generated data signalling and general data characteristics.

### Data content

In this section, the data content related aspects are discussed. One important principle is that the input data needed for a model could be transparent to 3GPP specification, more effort should be spent on the ones have real spec impact.

#### Input data related

During the SI phase of the AI/ML for positioning, many simulations from companies are using the CIR/PDP/DP as the model input data. A few certain post-processing methods are applied to further reduce the overhead for potential reporting, e.g., down sampling, feature extraction.

Last meeting, we have following agreement

Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information.

In the agreement, the timing information and/or the power information are at least supported. More interestingly, the type (b) provides the possibility to jointly process the power and timing information. There are a few sub-options for the pared time/power information, like following:

(b-1): separate timing and power values with jointly reporting

(b-2): joint time and power value

The sub option (b-1) is simple in building the relation between timing and pairing, but has no reduction on the overhead of reporting. As clearly shown in the SI simulation, if we only rely on down sampling for the overhead reduction, the performance will be highly dependent on the size. If the remaining size is large, like 128/64, the performance degradation is not much but the overhead is still a lot. But if the remaining size is small, like the 16/8, the performance degradation is huge even though the overhead is reduced a lot. In sub option (b-2), it will involve the calculation of such joint value, which we don’t need to specify/fixed the calculation method in RAN1 but need to associated the calculation in model definition/identification. For example, the calculation of the joint value based on the measurement could be carried in the assistant information and the necessary entity knows how to calculate and report it. Then related to effect of sub option (b-2), it can achieve the great reduction of overhead as well as keeping the positioning accuracy, the burden is on the value calculation. But as we just explained, such burden could be alleviated by carrying methods in the assistant information to facilitate the calculation.

- For paired timing information and power information, following two types are considered:

(b-1): separate timing and power values with jointly reporting

(b-2): joint time and power value and reporting

#### Measurement to derive data

##### Reference signal for measurement

Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Both options, which are already somehow supported by current spec, have their use cases. The option A can be applied to the case when UE has experienced environment change or self-configuration change, so that original PRS setting is no longer suitable. The later one, option B is the basic functionality of network controlling the PRS configuration. But as the note explained, the configured PRS needs to firstly be coordinated between LMF and gNB.

- Both option A and B can be supported in terms of DL PRS configuration for collecting training data

##### Measurement for model input and output

In last meeting, there are a few agreements/working assumptions are made for measurement for model input and output, especially for training data collection. More importantly, there are still several FFS points to be discussed in these agreements and working assumptions like following:

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

Detailed definition of channel measurement

During the discussion, some company mentioned that here the channel measurement has not specific limitation on what the measurement is, which is the exactly concern from us, from the whole SI, the most simulation work is using full-size channel measurement (e.g., 256 long) and the observation to claim the important gain comparing the legacy RAT-dependent positioning method is also based on such simulation assumptions. RAN1 did simulate on the truncated channel measurement (e.g., smaller size of CIR/PDP/DP), but when the size goes down, the performance loss will be much more serious. From our simulation, which we used feature extraction to keep the channel properties and also reduce the size of the input data size, we can achieve the overhead reduction and no performance loss.

Now come to “the collected data sample” in the above agreement, if the intention is to ask a node to collect or potential signalling or store full-size channel measurement, it will be unacceptable, which consumes large overhead for collection or potential signalling, and also large storage consumption will be needed. Let Ntrp, Nue, Nt, Nantenna, Nbit denote the number of TRPs, the number of UE, the number of samples per link and the number of bits for one sample, respectively. Then one dataset is (Nt*Nantenna*Nbit) and total cost of all dataset becomes (Ntrp*Nue*Nt*Nantenna*Nbit). For example, in typical simulation assumption from SI, the Ntrp=18, Nue=20000, Nt=256, Nantenna=4 and consider Nbit=8; then the one data set already consumes around 8K bits and total cost could be more than 29 billion bits!

In that sense, the processed the channel measurement instead of the raw channel measurement (or full-size channel measurement) should be considered for collected data sample. For now, we are open consider both truncated channel measurement and feature extracted channel measurement as the channel measurement in the collected data sample, even though the truncated channel measurement did not show very promising performance before.

- Full-size (or raw) channel measurement is not suitable for collected data sample.

- Both truncated channel measurement and feature extracted channel measurement can be considered as candidate for the channel measurement in the collected data sample.

### Data generation entity

Considering the 5 cases and several involved LCM functions, we summarized the following table on this aspect.

Table 4 – data generation entity for corresponding case and LCM functions

|  | Case 1 | Case 3a | Case 3b |
| --- | --- | --- | --- |
| Training | PRU, UE | TRP | TRP(input)/LMF(label) |
| Inference | UE | TRP | TRP |
| Monitoring | Input based :UE Output based: UE/PRU, Measurement based - DL measurement based: PRU/UE - UL measurement based: TRP | Input based: TRP Output based: TRP Measurement based - DL measurement based: PRU/UE - UL measurement based: TRP | Input based: TRP Output based: TRP(input)/LMF(label) Measurement based - DL measurement based: PRU/UE - UL measurement based: TRP |
| Finetuning | PRU, UE | TRP | TRP(input)/LMF(label) |

- For data collection in training/finetuning, the data generation entity can be PRU/UE for case 1, TRP for case 3a, and TRP(input)/LMF(label) for case 3b.

- For data collection in inference, the data generation entity is UE for case 1 and TRP for case 3a/3b.

- For data collection in monitoring, the data generation entity is dependent on the monitoring metric.

However, there is one discussion point about whether UE is always applicable to be a “data provider” or “PRU”. For PRU, it’s nature that such PRU is functioned as data provider. But when discussing the non-PRU UE, whether every non-PRU UE needs to be the data provider is a question. Since to support data collection, technically speaking, more UE to provide data will be better. However, for certain UE which doesn’t support AI model, doing measurement and reporting only for AI/ML functionality make no sense to it, e.g., such measurement is only used for data collection for other node operating AI/ML model. Thus, such UE should be allowed to notify gNB/LMF that it doesn’t want to be a data provider. More specifically, such UE should not be always burdened with consuming energy/time only for data collection for other node’s model training. On the other hand, if the UE reports that it can be a data provider, but for certain moment when network request for data collection, some other condition the UE did not satisfied, meaning such UE cannot provide qualified data to network, such UE should be allowed to notify network about this situation. For example, if network requires channel measurement with certain SNR range (or RSRP level), but the UE is not qualified to this certain SNR range or RSRP level, such UE can report or notify network that it is not applicable to provide data now.

- RAN1 supports UE to notify network whether such UE is a data provider or not.

- RAN1 supports UE to notify network that it is inapplicable to provide data sample when certain condition is not satisfied. FFS certain condition, e.g., data sample requirement related condition.

### Generated Data Signalling

In some cases, and/or LCM functions, the generated data at one entity needs to be signalled to another entity. In following, the signalling direction and signalling content are discussed.

Signalling direction

Considering the 5 cases and several involved LCM functions, we summarized the following table on this potential signalling directions.

Table 5 - Generated data signalling direction summary

|  | Case 1 | Case 3a | Case 3b |
| --- | --- | --- | --- |
| Training | PRU/(other) UE the UE (via TRP or even LMF) | NA or other TRP/LMF the TRP | Input: TRPLMF label: NA or PRU/other UE LMF |
| Inference | NA | NA | TRPLMF |
| Monitoring | Input based: NA Output based: PRU/(other) UEthe UE (via TRP or even LMF), Measurement based - DL: PRU/UE monitoring entity* - UL: TRP monitoring entity* | Input based: NA Output based: NA or other TRP/LMF the TRP Measurement based - DL: PRU/UE monitoring entity* - UL: TRP monitoring entity* | Input based: TRPLMF Output based: - Input: TRPLMF - label: NA or PRU/other UE LMF Measurement based - DL: PRU/UE monitoring entity* - UL: TRP monitoring entity* |
| Finetuning | PRU/(other) UE the UE (via TRP or even LMF) | NA or other TRP/LMF the TRP | Input: TRPLMF Label: NA or PRU/other UE LMF |

As noted by *, since the monitoring entity may not be directly the entity deployed with the AI model, which entity it be could be separately discussed.

- RAN1 supports the collected data signalled from PRU (other UE) to UE deployed with AI model.

Signalling content

From data collection perspective, there could be a few data type to be collected. The data related to model input and/or monitoring, e.g., the measurement related, could be collected. However, some processing might be applied to the original measurement and derive the processed measurement finally. Such process could be feature extraction, or truncation, or some statistic calculation etc. In that sense, the data collected type and size could be defined or exchanged, so that the entities involving the signalling could understand.

- RAN1 supports to specify data type and size, the data type includes the measurement results and processed measurement results.

### General data characteristics

For the different LCM operations, the data collected for them may be also showing different characteristics, we are discussing the data set size, time-related feature and the label necessity in following.

Regarding the data set size, we roughly compared the set size required for different LCM function, and shown in below:

Table 6 – data set size for LCM operations

| LCM operation | Training | Inference | Monitoring | Finetuning |
| --- | --- | --- | --- | --- |
| Required data set size | Large | Small | Small to medium | Medium |

As commonly known, the training operation requires relatively large data set size in order to training the model to be functional. Likely, the data collected for training could last longer, which may not be required in as very short time period and such training operation could be left implementation. For finetuning, it’s actually like somewhat retraining the model with limited amount of (latest) data sets, so we think smaller but overall medium size of data sets will be needed. This also apply to some monitoring method which requires relatively long-term estimation of certain metrics, so that a certain amount of data sets will be needed. For inference, since it’s only for one operation of the model to generate the output, it consumes the least data set size among the four operations.

- Training requires (relatively) large data set size, finetuning and some monitoring requires medium data set size and inference and some other monitoring requires small data set size.

Regarding the time related feature for the collected data, basically, there can be two types: one is a time duration and  the other is a one-shot result. For example, for training, finetuning and some monitoring method, the collected data is usually from one time point A to time point B, and the duration should last for a while. The collected data could reach the required size as well as cover enough diversity in time domain, which make the model is more capable. On the other hand, in inference or some other monitoring method, the data collected is from one short-like manner, which requires the input collected to be within a short time so that we can consider the location of concerned entity are not moving.

- The collected data sets for training, some monitoring, finetuning are for (relatively) long time period; while that for inference and some other monitoring are for one time.

Then for label necessity, it’s somehow easy but also difficult to tell. When saying easy, as from the SI, most of proposed results are coming from the model trained with data with label, so naturally, many LCM operation required data needs label. In terms of difficulty, the model in use is actually up to implementation. So, by the development of AI model itself, the unsupervised model could be more and more powerful, such that label may be not that dependent. Thus, our observation is only for current study. The training, some monitoring, finetuning will requires label in collected data while which is not necessary for inference and some other monitoring.

- The training, some monitoring, finetuning will requires label in collected data while which is not necessary for inference and some other monitoring.

## Inference

In the data collection in above, the input part has been discussed for the inference. In this section, we focus on the output part and also the inference related content and signalling.

For some cases, e.g., case 3a, the TRP based model but its TRP assisted positioning in which the LMF obtains the measurement results and generates the location results. In such case, the model output from the TRP should be signalled to LMF. While for other two cases, e.g., case 1 and 3b, there is no need for signalling the model output.

- RAN1 supports TRP to signal the model output to LMF in case 3a.

## Performance/Model monitoring

In this section, the aspects related performance (model) monitoring operation are discussed, which includes the monitoring metric, monitoring entity, monitoring outcome with potential signalling and finally the post monitoring behaviour.

### Monitoring metric

During the SI phase, there are many proposed monitoring metrics and evaluated by the proponent, which claims the reasonably good monitoring performance in certain cases. These metrics can be divided into three types:

- Model output based  this type is most straightforward one which is based on the comparison between model output and label. One thing noted that, by using this type, there are two ways:

  - Monitoring measurement/metric derived by the AI model operating entity. It will compare the model output and the label (coming from other methods). One debating point is that if such entity can already obtain the “label” from other methods, why not it’s simply use that for positioning purpose. While intuitively this might look like a reasonable argument, there could be some applicable situation for above comparison. For example, if we view the “label” as the “output from other methods”, it can see that actually all the outputs are somewhat “estimation”. Then by this monitoring comparison, we are trying to see the gap between AI model output and other method output. In addition, from procedure-wise, once the AI based method is triggered, it anyway will need to generate the output, before actually get the “accurate label”, one cannot tell whether the output is accurate or not.  We think eventually, the trustworthy of the AI model output will go through the integrity check like the legacy methods would.

  - Monitoring measurement/metric derived by/with PRU (or UE with known location). In this way, such PRU will be activated to do the AI based positioning similar to the operating UE, and based on the comparison from PRU’s model output and the label, one can decide the performance of the model is acceptable or not. One point for using such way is that, the PRU may use the same/similar AI model as the operating UE.

- Model input based this type is using the feature of input data matching or not to decide whether the AI model is still applicable to the current situation. This is more like stochastic way to monitor the model, which may require the AI model shows some certain connection with the input data characteristic, e.g., certain distribution etc. Such information may need to be included during model identification.

- Other (measurement) based  this type is usually consisting of side information which associates the AI model. Such side information could be reflected by other measurement instead of the measurement for data input. For example, if certain SNR level or RSRP level shows for an AI model characters, based on the measurement of the non-AI used reference signals can also represents the current SNR or RSRP level, which could be used for the monitoring.

Based on above analysis, the three types of monitoring metric are feasible thus we think all of them should be considered.

- RAN1 considers the model output based, model input based and other (measurement) based monitoring metrics.

#### label-based model monitoring for Case 1

In last meeting, the following agreement on the study for case 1 monitoring metric calculation is concluded.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

For the listing options in above,

- Option A-1, the problem is if it’s really NLoS condition, it seems using LMF based method to calculate the location estimation will be quite bad choice. That obtained location estimation could be likely not qualified for judging the performance of the UE side model. For spec impact. It will require to activate multiple positioning methods at the same time, e.g., AI based method at UE and also the UL based or DL based method for LMF based positioning to get the location estimation.

- Option A-2, it’s not very clear what the position calculation assistance data (e.g., existing information for UE-based positioning method) is. It’s not the exact location estimation. This option needs more clarification. One guess, it’s to trigger using the legacy method to estimate the location as label to compare with outcome of model. Then the similar doubt will be similar as A-1, whether it really works in NLoS heavy condition.

- Option A-3, the monitoring of the model is to decide whether current model is working well or not. So if network forwards the PRU’s measurement and location to target UE, if the PRU stays in the same/near location as UE, or the PRU’s measurement is within the measurement trained for that model, this could be feasible solution to use PRU’s measurement and location to monitor the model. Otherwise, it’s not feasible.

- Option A-4, in addition to the similar doubt on A-3, one more high-level concern for A-4, is it could request a PRU (which could be a UE) transmitting the measurement to OTT server. If this method is purely 3GPP transparent, and no spec impact, we don’t need to list it as one option to discuss, it’s any up to implementation. If it’s to the target UE, then it’s covered by A-3, without PRU’s location information.

- Option B-1, it seems requiring the LMF to calculate the location estimation as well, so it is also feasible conditioning on the channel measurement condition e.g., (NLoS), so that whether the LMF calculate results are qualified to compare with model output.

- Option B-2, by the described procedure it works but facing same issue as A-3, whether the PRU’s measurement condition is aligned with the collected data sample in that model being trained.

In summary for option A track, option A-1/2 are conditionally feasible on the condition of the actual measurement condition (e.g., NLoS situation). Option A-3/4 can be also conditionally feasible but on the condition of the PRU measurement is aligned with the collected data sample in the model being trained. For option B track, similar to option A track, it is conditional feasible situation on the actual measurement condition and/or whether the PRU’s measurement condition is aligned with the collected data sample in that model being trained. So it seems the measurement condition is necessary decide whether the above options are feasible or not.

- RAN1 considers to define the measurement conditions (e.g., SNR level, LoS indication etc) to facilitate the performance (model) monitoring.

### Monitoring entity

Regarding the monitoring entity, it depends on two aspects: one is the AI model deployed entity and the other is whether network needs have control on the AI based method for positioning. For the second aspects, we think the AI based method is like a positioning algorithm which is transparent to network, then it may not be controlled by gNB, otherwise, we think certain level control from network is allowed. Thus, for the concerned cases, the possible entity for monitoring is given in following table:

Table 7 – monitoring entity for each side model

| Model deployed entity | UE side model | TPR side model | LMF side model |
| --- | --- | --- | --- |
| Monitoring entity | UE, TRP/LMF | TRP, LMF | LMF |

Note that, although the monitoring entity could be UE/TRP/LMF, but the related metric/measurement derivation could be from PRU.

- RAN1 considers the monitoring entity as UE/TRP/LMF for UE side model, TRP, LMF for TRP side model, LMF for LMF side model.

### Monitoring outcome and its signalling

When the corresponding monitoring entity obtain the monitoring metrics, it may need decide the monitoring outcome based on some criteria, for example, if the monitoring metrics are model output based, e.g., the output gap/error (model output - label), then whether the output gap/error is smaller than a threshold, we may consider the monitoring outcome as pass, otherwise, the monitoring outcome is considered as fail. Then, we can see, the monitoring metric related threshold could be considered for monitoring entity. Especially for UE monitoring case, such information is considered as one type of network control on the AI model performance.

- RAN1 considers the monitoring metric related threshold to be configured, dependent on the monitoring metric content.

In addition, for monitoring outcome or it’s related information (like above threshold), some signalling could be supported to facilitate the indication or configuration. On the other hand, in order to obtain the monitoring outcome, the other entity may provide the monitoring assistance, e.g., the monitoring refence signal configuration, monitoring measurement and it’s report Thus, for each case, we have following:

Table 8 – monitoring signalling for each side model

|  | UE side model | TPR side model | LMF side model |
| --- | --- | --- | --- |
| Monitoring outcome | NA (UE monitors) or TRP/LMF UE | NA (TPR monitors) or LMFTRP | NA (LMF monitors) |
| Monitoring assistance | TRP/LMF UE | UE/LMFTRP | UE/TRPLMF |

- RAN1 supports the signalling for the monitoring refence signal configuration, monitoring measurement and its report.

Another thing for this monitoring outcome determination is that, whether the decision is made on one short manner or accumulated manner. one short manner means the decision is on one-time result of the monitoring metric, while the accumulated manner means a multiple-time results of the monitoring metric. For example, if the output gap/error is larger than the threshold for a certain number of times, then the monitoring outcome is determined as fail.

- Monitoring outcome determination could be based on one time metric or multiple time metrics.

### Post monitoring behaviour

Once the AI model deployed entity obtains the monitoring outcome, certain post monitoring behaviour should be considered. If monitoring outcome is pass or not fail, then we think it’s natural to continues the AI/ML based method or keep using the current AI model. However, if the monitoring outcome is failed, then some possible handling could be considered:

- Finetuning, in case current model did not work well, some new data could be collected for finetuning the model to see whether the performance could be improved;

- Terminating current model or AI/ML based method and fallback to other non-AI method (e.g., legacy RAT dependent method)

- Report the AI/ML based method error, and wait next indication.

- AI model re-selection or re-determination as discussed in section 3.3.

- Post monitoring behaviour should be specified.

## Consistency between training and inference

As indicated by the WID, and also the discussion from RAN2, the consistency between training and inference needs to be ensured. The purpose of such ensure is to give a promising expectation that the inference results of model can be comparable as the training data used for training such model.

### When to check

First of all, by understanding of such purpose, the consistence check should happen in the stage of deciding which model to use. No matter what exactly the entity is, such entity needs to fulfil the consistence check in order to decide which model to be applied. Since any available model(s) for such entity to select should be already trained by enough data samples. And the features/characteristics of these data samples should be already known. Then for the time duration that the AI based positioning adopted, what kind of the data samples the relevant entity (e.g., the inference input data generator) can provide should be known as well. Given these, such consistence check is feasible to do when decides which model to use. Moreover, if it’s done after model selection/confirmation, it may end up with inference input data cannot be aligned with training data for that selected model.

- Consistence check should be done when/before the entity deciding which model to use.

### Content to check

In previous meetings, the debating on the #7 has been a lot.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). In last RAN1#120bis meeting, the following proposal was discussed Proposal: For AI/ML based positioning Case 1, assistance information, info #7, from legacy UE-based DL-TDOA, can optionally be provided explicitly or implicitly from LMF to UE. - If info #7 is not provided, it is up to UE implementation to determine consistency between training and inference. No specification impact is expected from RAN1 perspective. - FFS implicit indication (e.g., signaling, content, mapping on per area vs per TRP, etc.). Note: the above on info#7 is independent of other assistance information

No matter its newly introduced association ID or the #7, from case 1 UE perspective, it needs to know whether this information has been changed or not from training phase to inference phase. This information is used by UE to determine whether the model can be utilized.

- RAN1 agrees the necessity to use explicit or implicit way to ensure consistence of #7 for case 1.

# Conclusion

This contribution discusses the design consideration to support AI/ML for positioning accuracy improvement, the corresponding proposal and observations are summarized as following:

- RAN1 considers the trigger conditions for enabling AI/ML based method for positioning, including potential model applicable conditions or the environment(channel) applicable conditions.

- In addition to the entity where the model is deployed, the network can decide or /assist to decide the model used at UE side, the LMF can decide and LMF/UE can assist to decide the model used at TRP side, the TRP/UE can assist to decide the model used at LMF side.

- RAN1 supports the indication of determined model from one entity to another entity

- RAN1 supports the indication of measurement related threshold/report/feedback and model applicable condition to assist the model determination from one entity to another entity

- For paired timing information and power information, following two types are considered:

(b-1): separate timing and power values with jointly reporting

(b-2): joint time and power value and reporting

- Both option A and B can be supported in terms of DL PRS configuration for collecting training data

- Full-size (or raw) channel measurement is not suitable for collected data sample.

- Both truncated channel measurement and feature extracted channel measurement can be considered as candidate for the channel measurement in the collected data sample.

- For data collection in training/finetuning, the data generation entity can be PRU/UE for case 1, TRP for case 3a, and TRP (input) / LMF (label) for case 3b.

- For data collection in inference, the data generation entity is UE for case 1 and TRP for case 3a/3b.

- For data collection in monitoring, the data generation entity is dependent on the monitoring metric.

- RAN1 supports UE to notify network whether such UE is a data provider or not.

- RAN1 supports UE to notify network that it is inapplicable to provide data sample when certain condition is not satisfied. FFS certain condition, e.g., data sample requirement related condition.

- RAN1 supports the collected data signalled from PRU (other UE) to UE deployed with AI model.

- RAN1 supports to specify data type and size, the data type includes the measurement results and processed measurement results.

- Training requires (relatively) large data set size, finetuning and some monitoring requires medium data set size and inference and some other monitoring requires small data set size.

- The collected data sets for training, some monitoring, finetuning are for (relatively) long time period; while that for inference and some other monitoring are for one time.

- The training, some monitoring, finetuning will requires label in collected data while which is not necessary for inference and some other monitoring.

- RAN1 supports TRP to signal the model output to LMF in case 3a.

- RAN1 considers the model output based, model input based and other (measurement) based monitoring metrics.

- RAN1 considers to define the measurement conditions (e.g., SNR level, LoS indication etc) to facilitate the performance (model) monitoring.

- RAN1 considers the monitoring entity as UE/TRP/LMF for UE side model, TRP, LMF for TRP side model, LMF for LMF side model.

- RAN1 considers the monitoring metric related threshold to be configured, dependent on the monitoring metric content.

- RAN1 supports the signaling for the monitoring refence signal configuration, monitoring measurement and its report.

- Monitoring outcome determination could be based on one time metric or multiple time metrics.

- Post monitoring behavior should be specified.

- Consistence check should be done when/before the entity deciding which model to use.

- RAN1 agrees the necessity to use explicit or implicit way to ensure consistence of #7 for case 1.

# Reference

- RP-242399, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN Meeting #105, Melbourne, Australia , September 9-12, 2024;

# Agreement in RAN1#116

Agreement

For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211.

- Note: The use of SRS for MIMO resource is transparent to UE.

Agreement

- For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information;

- paired timing information and power information.

Agreement

- For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF:

- timing information;

- paired timing information and power information.

Agreement

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

      - Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

      - Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

The issues to be studied include, but not limited to, the following:

      - Tradeoff of positioning accuracy and signaling overhead

      - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values.

      - Whether the same Alternative(s) applies to all cases or not

      - Applicability and necessity of specifying the Alternative(s) to different cases

      - Note: different sub-cases may have different issues.

Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

Agreement

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

Agreement

For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

Agreement

For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring.

- RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Agreement

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time.

- FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time

Agreement

For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include:

- Tradeoff of positioning accuracy and signaling overhead

- The impact of transmitter and receiver implementation

- Specification impact

- Other aspects are not precluded

Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

# Agreement in RAN1#116b

Agreement

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215.

FFS: whether it is applicable when Case 3b is used to support multi-RTT

Conclusion

- It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data.

Working Assumption

For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE.

Agreement

For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB.

Agreement

For training data collection of AI/ML based positioning, the collected data sample can include the following components:

Part A:

- channel measurement

- quality indicator of channel measurement

- time stamp of channel measurement

Part B:

- ground truth label (or its approximation)

- quality indicator of label

- time stamp of label

Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification.

Note: contents in Part A and Part B may or may not be generated by different entities.

Note: Part A and/or Part B, and their contents may or may not apply for each case

FFS: detailed definition of channel measurement

Working Assumption

For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE.

Working Assumption

For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of the label and its related data is out of RAN1 scope.

Working Assumption

For training data generation of AI/ML based positioning Case 2a, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of the label and its related data is out of RAN1 scope.

Working Assumption

For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Working Assumption

For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- FFS: Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Agreement

For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least:

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

Agreement

For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option.

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform the monitoring metric calculation is up to implementation

Agreement

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation.

  - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE.

    - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label.

  - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE.

  - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE.

  - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server).

    - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method.

- Option B. The LMF performs monitoring metric calculation.

  - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

  - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Note: exact method to perform the monitoring metric calculation is up to implementation.

Note: Other options are not precluded.

# Agreement in RAN1#117

Working Assumption

For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Note: It is assumed that user data privacy of non-PRU UE is preserved.

Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

Agreement

Sample-based measurement is defined as:

- The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples.

- Nt' and k can be signalled

  - FFS: the value range of Nt'; the value range of integer k for the timing granularity T.

- The timing information is defined relative to a reference time

Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples.

Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec.

Agreement

Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

Agreement

For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are:

- for a same UE (PRU or Non-PRU UE), and

- for a same location associated with Part B.

- Note: the association can be discussed

Agreement

Draft LS R1-2405577 is endorsed in principle by adding the latest agreements made in this meeting and adding “agreements” to “Note: the working assumptions above are based on RAN1 understanding for RAN work item (NR_AIML_air).”

Agreement

Final LS R1-2405578 is endorsed.

# Agreement in RAN1#118

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform monitoring metric calculation is up to implementation.

Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Conclusion

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring,

- Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Agreement

For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement,

- For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective

- Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

Agreement

For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples

- The Nt samples have timing granularity T.

- FFS: the starting time of the list of Nt samples

- FFS: the value range of Nt

For the sample-based measurement (if accepted in Rel-19),

- For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power.

- Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation.

Agreement

For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input,

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

Conclusion

From RAN1 perspective, for Case 3a measurements,

- The existing procedures can be reused in terms of SRS configuration.

  - Note: parameter values for SRS configuration can be further discussed

- These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures.

- Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Agreement

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective,

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

# Agreement in RAN1#118b

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

# Agreement in RAN1#119

Conclusion

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- LOS/NLOS indicator can’t be reported independently from other measurements

Agreement

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

  - starting time = first detected path rounded down with timing granularity T.

Note: UE-side measurement is a separate discussion

Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

# Agreement in RAN1#120

Conclusion

For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF,

- Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective.

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting:

- k = {0...5}

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting:

- Nt' = {8, 16, 24}

Agreement

For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

Agreement

For AI/ML based positioning,

- When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed.

- When label is to be reported/sent, time stamp of label is the time instance for which the label is valid.

Agreement

Send Reply LS R1-2501522 to SA2 about Case 2b with the content of Section 2 of R1-2501521 by revising

“RAN1 would continue work and may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.”

To

“RAN1 may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.”

Agreement

Send Reply LS R1-2501524 to SA2 about Case 3b with the content of Section 3 of R1-2501521 by adding all the agreement made in this meeting for Case 3b.

Agreement

Final LS R1-2501523 is endorsed.

Final LS R1-2501525 is endorsed.

Agreement

For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B):

(A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b).

- Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly.

- The descriptive info of the channel measurement includes at least:

  - measurement type (A) or type (B) and

  - measurement parameters.

- Regarding measurement parameters,

  - For (A) path-based measurement, the measurement parameters include: number of additional paths, k.

  - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k.

    - FFS: Nt

- Note: Type (A) and Type (B) are used for discuss purpose only.

Conclusion

For Rel-19 AI/ML based positioning Case 3b,

- If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type.

  - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested.

- It is up to RAN3 to decide the signaling details.

  - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

Agreement

RAN1 send an LS to RAN3 and RAN2 to inform them above agreements and conclusions.

Agreement

- For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed:

- The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted).

- FFS: other information is not precluded (e.g., if Part B is valid for a duration)

Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

RAN1 send an LS to RAN3 and RAN2 to inform the above.

Agreement

Draft LS R1-2501627 is endorsed in principle.

Agreement

Final R1-2501628 is endorsed.

Email approval by Feb 25

# Agreement in RAN1#120b

Conclusion

- The offset (used in the agreement made in RAN1#119) refers to the duration (in samples) between the reference time and the starting time of the list of Nt consecutive values.

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: whether transmit offset from gNB to LMF” in RAN1#119 agreement is resolved by:

- No offset is transmitted from gNB to LMF

Conclusion

For channel measurement type (A) (i.e., path-based measurement), it is not necessary to introduce enhancements on the number of reported paths in Rel-19.

Agreement

For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by:

- the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

Agreement

For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label,

- The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label.

Note: It is assumed that user data privacy of non-PRU UE is preserved.

Conclusion

For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator,

- The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.