3GPP TSG RAN WG1 #121			   R1-2503554

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.2.1

Source: 	Samsung

Title: 	Views on Rel-19 UE-initiated/event-driven beam management

Document for:	Discussion and Decision

# Background

In this contribution, we provide our views on various design aspects related to the UE-initiated beam management enhancements according to the WID and the agreements made in prior RAN1 meetings including RAN1#120bis [1]. We also provide our thoughts on how we could proceed with the corresponding discussions in RAN1.

# Discussions on UE-initiated (UEI) beam reporting

In this section, we focus on various design aspects for the UEI beam reporting including trigger event(s) – along with the measurement RS(s) configuration/determination, UL signaling content(s)/report format(s) and the UCI based UL signaling procedure(s) design, and present our proposals on the related subject matters.

# Event(s) and triggering condition(s)

A few open issues related to Event-2 evaluation(s) have been addressed in RAN1#120bis [1]. Below, we provide our views on conditions of transmitting the first PUCCH based on Event-2 evaluation(s) and remaining details of determining the time window(s).

[117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. [120b] Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. [120b] Agreement On UE-initiated/event-driven beam reporting, the procedure of triggering event determination is captured in RAN1 spec. [120b] Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

Though RAN1 has agreed that determination of Event-2 is based on Event-2 instance(s) counting within a time window in RAN1#117, and RAN1 has agreed to adopt Option-4 (provided above) in RAN1#120b, details of how to determine the time window and its association to the first PUCCH transmission are still unclear, which makes the Event-2 evaluation procedure incomplete. First of all, we would like provide our views on some basic assumptions for determining the time window, which are important for further discussions on the design details.

For the UE-initiated/event-driven beam reporting, the determination/evaluation of the event is to decide whether the UE would transmit the first PUCCH to request UL resource(s) for transmitting beam reports (Mode A) or to notify transmission of beam reports (Mode B). According to the agreed Option-4 and considering one new beam RS, a time window with a fixed length T_window can start with a transmission occasion (TO) of the new beam RS that determines an event instance; in addition, prior to a PUCCH transmission occasion, multiple (>1) time windows for the (same) new beam RS can exist, and they are not allowed to be overlapping in time. In Figure 1, conceptual examples illustrating determination of the time window(s) for a given new beam RS (i.e., RS #1) are presented. As shown in Figure 1(a), there can be multiple time windows before a PUCCH transmission occasion – we highlight in green a TO of RS #1 that determines an event instance, otherwise a TO of RS #1 does not determine an event instance.  As it is possible that the number of determined event instances within a time window can be smaller than M, for the case in Figure 1(a) with M=3, the UE cannot determine the event based on the first time window (i.e., Window #1 in Figure 1(a)) – the number of the determined event instances within Window #1 is 2, which is less than M=3. On the other hand, the second time window (i.e., Window #2 in Figure 1(a)) has 3 determined event instances, and the UE can determine the event based on Window #2. For another case depicted in Figure 1(b), following those described in Figure 1(a), the UE can determine the event for the first time window (i.e., Window #1 in Figure 1(b)) but not the second time window (i.e., Window #2 in Figure 1(b)) – in this case, the UE would store the event evaluation result(s) before determining whether a PUCCH can be transmitted to initiate beam reporting.

Figure 1

Observation 1: For a new beam RS of a CSI report configuration, there can be multiple determined time windows before a PUCCH transmission occasion, where

- The multiple determined time windows are not overlapping in time domain.

- The number of instances for one same new beam may or may not greater than or equal to a configurable number M within a time window

- The UE should store the evaluation results for the multiple determined time windows to decide whether to transmit a first PUCCH.

In addition, if the evaluation is done a long time before transmitting the first PUCCH, the UEI-BR may not be accurate or the event may not even be satisfied in the nearest time window before the first PUCCH transmission. This also applies to the basic feature for the case of M = 1.

Observation 2: The end of a time window with a number of instances for one same new beam being no less than M should not be far before the first PUCCH transmission to ensure real-time indication of UEI-BR.

Based on the above observation/basic assumption and the objective of the WID for latency reduction, as soon as a UE has determined a number of event instances for one same new beam no less than M within a time window, the UE can quickly identify a corresponding first PUCCH (only accounting for an RRC configured time interval T_proc for gNB’s processing/preparation time) to transmit the one-bit indication where the time gap between the time window and the PUCCH transmission should ensure enough time for the UE to prepare the UL transmission.

Proposal 1: For a CSI report configuration, if a UE determines that a number of event instances for one same new beam is no less than M within a time window, the UE transmits the first PUCCH in an earliest transmission occasion T_proc after the end of the time window, where T_proc is configured by higher layer signalling.

[119] Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4.

It was agreed that the current beam RS and new beam RS are configured with a same periodicity. One remaining issue that should be clarified is the association between measured occasions for the current beam RS and new beam RS(s). It is straightforward to restrict that the measurement(s) for determining an event instance for Event-1 or Event-2 are performed within a same periodicity, i.e., to determine an event instance, the time gap between the measured occasions for the current beam RS and the new beam RS(s) should be no larger than the periodicity.

Proposal 2: For determining an event instance for Event-1 and Event-2, the measured occasions for the current beam RS and the new beam RS(s) should be within the same periodicity.

[120] Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. [120b] Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary)

Case (a)

Case (b)

Figure 2

In RAN1#120b, it was agreed that the UE would reset the event instance counting for all new beams when a UE receives a TCI state update (candidate #2). The intention of candidate #2 is to ensure that the reported beam is based on the latest indicated TCI state instead of a previous and invalid one. One remaining issue is to further clarify details of such update. The basic principle should be that the first PUCCH is only allowed to be transmitted if the triggering event is based on the current/latest TCI state at the transmission time of the first PUCCH. One simple case is that the measurement is performed in the time window after the TCI state update and there is no other DCI indicating the TCI state is received during the time window for event evaluation as show in Case (a) of Figure 2. Another example is given in Case (b) where the later received DCI (DCI #3) indicates a same TCI state as that indicated in an earlier received DCI (DCI #2).

In the above case, if the TCI state indicated in a latest PDCCH/DCI before a corresponding first PUCCH is the same as the “indicated” TCI state that triggers the event hence the UEI-BR (meaning that the current beam RS that determines the event is not changed/updated prior to the corresponding first PUCCH transmission occasion), the corresponding first PUCCH can be transmitted. If we go with the original descriptions of candidate #2 – i.e., when the current beam RS/TCI state is updated/changed, (1) first of all, we have not defined or specified which RS/TCI state such update/change is with respect/reference to, and (2) second, it would be difficult to describe the corresponding procedure(s) if the TCI state (and therefore the corresponding measurement RS) keeps updating/changing (the whole procedure would become non-casual, and is hard to specify). For instance, as shown in the Figure 2, because the latest DCI (DCI #3) received prior to the corresponding first PUCCH transmission occasion indicates a same TCI state (i.e., TCI #2) as that indicated in DCI #2 which has been used for triggering the event and the corresponding UEI-BR (i.e., Case (b)), the UE can determine that the corresponding first PUCCH can be transmitted. If there is no DCI #3, DCI #2 would become the latest DCI and the UE can also transmit the first PUCCH (i.e., Case (a)).

Based on the above, we would like to present the following proposal for a single-cell case.

Proposal 3: For a single CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state indicated in a latest PDCCH before a first PUCCH associated with the second PUSCH.

For a multiple-cell case, the principle is the same as the single-cell case. The only difference for the multi-cell case is that the indicated TCI state can apply to multiple serving cells, and the serving cell configured with new beam RS(s) should be one of these multiple serving cells that apply the indicated TCI state. Based on the above analysis, we have the following proposal for a cell list with multiple cells.

Proposal 4: For a multi-CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state, indicated in a latest PDCCH, for the serving cell of the new beam RS, where the latest PDCCH is before a first PUCCH associated with the second PUSCH.

Regarding other candidate condition(s) than candidate #2, we do not see them as critical condition(s) that must be supported at current stage. Besides TCI state update (hence the corresponding current beam RS change), other candidates do not have immediate impact on event evaluation.

Observation 3: Regarding triggering event determination for Event 2, other candidates than Candidate #2 do not have immediate impact on event evaluation.

Proposal 5: Based on Observation 2, deprioritize selection/determination of other candidate(s) than Candidate #2 for resetting the counting.

Furthermore, retransmission of the first PUCCH cannot ensure the aforementioned “real-time” indication of UEI-BR because the evaluation time window associated with the initial transmission of the first PUCCH can be far before the retransmission of the same first PUCCH. This is quite different from retransmission of SR which is used to request UL-SCH data. Once the data arrives at the UE side, it is stored in the buffer until it is transmitted. Therefore, there is no need to follow the retransmission mechanism of SR for the first PUCCH.

Observation 4: Retransmission of the first PUCCH cannot ensure the real-time indication of the UEI-BR.

Proposal 6: Do not support retransmission of the first PUCCH for both Mode A and Mode B.

Agreement (120b) On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement.

The issue of prohibit timer is not essential and not supporting the feature can also be beneficial for overhead reduction. For example, after transmitting the first PUCCH and the channel conditions have become better, the UE can stop transmitting the next first PUCCH to inform the network avoid indicating a PUSCH resource for UEI-BR or indicating TCI state update.

Proposal 7: Do not support prohibit timer.

Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4.

It was agreed in RAN1#119 that at least the current beam RS and candidate/new beam RS(s) can be configured to have the same periodicity. Regarding other design alternatives, we do not see strong need or motivation to configure different periodicities for the measurement RSs. Then with the assumption of the same periodicity between the measurement RSs, the event evaluation can be largely simplified, and the details can be up to RAN4.

Similar to Event-2, same principle should be applied for Event-1 and Event-7 determination. For Event-1, only support the same periodicity of the current beam RS and the candidate/new beam RS(s). For Event-7, only support the same periodicity of measured occasions for the RS(s) associated with all activated TCI state(s) and the candidate/new beam RS(s).

Proposal 8: Regarding the RS periodicity for Event determination,

- For Event-2, do not support different periodicities of the current beam RS and the candidate/new beam RS(s).

- For Event-1, only support the same periodicity of the current beam RS and the candidate/new beam RS(s).

- For Event-7, only support the same periodicity of measured occasions for the RS(s) associated with all activated TCI state(s) and the candidate/new beam RS(s).

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation: - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality. - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality. - Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs Note: For Event-2, we have the following definition for scheme-1 and scheme-2 - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Furthermore, in RAN1#119, additional events – i.e., the Event-1 and Event-7 as described above – are confirmed to be supported for Rel-19 UEI/ED BM, which should reuse the same design as Event-2. One open issue for Event-7 is whether to support counting more than one event instance. If counting more than one event instance is supported for Event-7, we need to first clarify whether the RS derived from the activated TCI state with the Q-th best quality is the same for each instance with respect to a same new beam considering that the channel conditions can vary across different evaluation instances. If we restrict a same RS derived from the activated TCI state with the Q-th best quality for event evaluation(s), it will inevitably lead to longer latency and thus against the objective of the WID for latency reduction; otherwise, the event evaluation procedure(s) would become very complicated, which is not friendly to UE’s implementations.

Observation 5: Supporting counting more than one event instance for evaluating or determining Event-7 could (1) increase the measurement latency, which is not aligned with the objective of the WID for latency reduction, and (2) complicate UE’s implementations.

Proposal 9: Do not support counting more than one event instance for evaluating or determining Event-7.

# Measurement RS(s) configuration/determination

Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management - Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS. - FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused. - FFS: Whether/how to specify filtering operation for L1-RSRP.

It was agreed in RAN1#116 that L1-RSRP on SSB for inter-cell is supported for the UE-initiated/event-driven beam reporting. After RAN1#116, RAN1 has made significant progress on defining new events and specifying the corresponding measurement RSs, e.g., means of determining the current beam RS and candidate/new beam RS(s) for Event-2 evaluation. However, it is unclear from the above agreement (made prior to RAN1 further progressing on this agenda item, and has made subsequent agreements) whether the inter-cell SSB can be used as the current beam RS and the candidate/new beam RS(s) for event evaluation.

Proposal 10: On UE-initiated/event-driven beam reporting, support inter-cell SSBs (associated with the serving cell PCI or PCI(s) other than the serving cell PCI) as current beam RSand candidate beam RS(s) for event evaluations.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

In RAN1#118, for explicit configuration of candidate beam measurement RS(s) for Event-2, Option-1 as provided in the above agreement is supported. One open point regarding Option-1 is whether or not to support RS update by MAC CE. We do not support introducing a new MAC CE to update the RS(s) in the configured set, which is another source of latency and overhead – note that for small to medium configurations (which is the case here), MAC CE update or MAC CE signaling in general would not have significant advantages (if any) over RRC signaling in terms of latency and/or overhead reduction(s).

Proposal 11: Regarding explicit RS configuration for candidate beam measurement in evaluating Event 2 (Option-1 in the RAN1#118 agreement), do not support updating the RS in the RS resource set by a new MAC CE.

Instead, to reduce latency of beam report as required by the WID, one straightforward way is to reduce the number of RS(s) for measurement considering that the UE would perform RS measurement in a TDMed manner for FR2. If the UE would like to measure a smaller number of RSs for fast beam tracking, the RS(s) in the TCI state(s) activated in the unified TCI state(s) activation/deactivation MAC CE can be used. Since the RS(s) in the activated TCI state(s) can be different from the RS(s) in the RS resource set (depending on network’s configuration(s)), the UE can just evaluate the RS(s) in the RS resource set that has the same value(s) as the RS(s) in the activated TCI state(s) for fast beam tracking purposes.

Proposal 12: Regarding Event 2 evaluation, a UE can be enabled to evaluate the L1-RSRP(s) of the RS(s) in the RS resource set (as in Option-1) that has the same value(s) as the RS(s) in the activated TCI state(s) as the candidate beam measurement results.

Another solution for latency reduction is new beam RS grouping. In FR2, there can be up to 128 beams and the corresponding RS(s) should be measured on different symbols – as a result, it can take 128 symbols or longer for one event instance measurement. If the number of event instances M is large, the measurement time can be tens of milliseconds. In some cases, a UE does not need to do measurement for all the new beams, instead, a subset of new beams, for example, surrounding new beams for a given current beam, would be enough. To enable this feature, the following two options can be considered.

- Option 1: Multiple CSI report configurations can be provided to the UE where each configuration corresponds to a current beam RS.

- Option 2: Multiple sets of new beam RSs CSI report configurations can be configured in one CSI report configuration where each set corresponds to a current beam RS.

Observation 6: For a given current beam of a serving cell, measuring a subset of surrounding new beams can be beneficial for latency reduction.

Proposal 13: For a given current beam of a serving cell, support current beam specific measurement for UEI-BR. The following options can be further considered,

- Option 1: Multiple CSI report configurations where each configuration corresponds to a current beam RS

- Option 2: One CSI report configuration consists of multiple sets of new beam RSs where each set corresponds to a current beam RS

# Report content(s)/format(s)

For candidate value(s) of N (the number of configured beams to be reported), RAN1#118 has made the following agreement:

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4} - FFS: additional candidate value(s) of {5, 6, 7, 8} - FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

Regarding the second FFS in the agreement, we prefer to have a default behavior when the value of N is not provided by RRC. The same consideration/design was already in legacy system/specification – i.e., when nrofReportedRS is not configured. In this case, when N is not provided by RRC, only one L1-RSRP and CRI/SSBRI are reported by default.

Proposal 14: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 of Event-2, if N is not provided by RRC, only one L1-RSRP and CRI/SSBRI are reported.

# UCI based UL signaling procedure(s) design

In this section, we provide our views on the following issues for UCI based procedure(s) design. In particular, our discussions are based on the latest RAN1 agreements and some offline discussions/proposals.

Issue #1: Details on 1-bit first PUCCH in Mode-A and Mode-B

As agreed in RAN1#118, some design details related to the 1-bit first PUCCH channel in both Mode-A and Mode-B need to be addressed to make the feature complete. We provide the corresponding agreement below:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A and/or Mode-B, further study the following for first PUCCH transmission - UCI multiplexing/dropping/prioritization rule - Conditions for the transmission of the first PUCCH. - Whether the PUCCH resource in the first PUCCH channel can be associated with multiple CSI report configurations for UE-initiated/event-driven beam reporting from one or multiple CC(s). - Whether/how to re-transmit the first PUCCH channel. - Whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel.

To avoid too frequent and “unnecessary” triggering(s) and/or transmission(s) of the UEI beam report(s), we need to further specify condition(s) of transmitting the first PUCCH (or not) in both Mode A and Mode B. In particular, when the event condition(s) keeps satisfying, it is impractical to have the UE to always transmit the first PUCCH channel, which would be a significant source of overhead and power consumption. More specifically, for both Mode-A and Mode-B, when there has already been transmission(s) of first PUCCH(s) – e.g., the PUCCH A in Figure 1 – within a time window associated to a first PUCCH (e.g., the PUCCH B in Figure 1), the UE does not need to transmit the first PUCCH B. Hence, we propose:

Proposal 15: For a first PUCCH transmission occasion for both Mode A and Mode B, if there has already been transmission(s) of another first PUCCH(s) within a time window associated with the first PUCCH transmission occasion, the UE would not transmit the first PUCCH.

Issue #2: Remaining details specific to Mode B.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, the value of X symbols for determining available transmission occasion of the second UL channel is configured by RRC (FFS: subject to a corresponding UE capability)

In RAN1#118, we have agreed to rely on a minimum time gap (i.e., X symbols) to determine the earliest available UL resource of the second UL channel in Mode B to transmit the beam report – the corresponding agreement is provided above. In RAN1#120, we have also agreed that the value of X is RRC configured by the network. One remaining issue is that whether this X value should also be subject to a corresponding UE capability. To our understanding, the value of X also comprises or reflects components related to UE’s processing/preparation time of UL channel/resource, which is subject to a corresponding UE’s capability. One thing should be clarified is the reference SCS of X – it is straightforward to use the SCS of the PUCCH.

Proposal 16: Regarding Mode B, the RRC configured value X symbols for determining the first available transmission occasion of the second UL channel after sending the last symbol of report notification on the first PUCCH channel to carry the actual UEI beam report is subject to a corresponding UE capability.

- X is based on the SCS of PUCCH.

Issue #3: Remaining details specific to Mode A

For Mode A, a UE first transmits a first PUCCH and then expects to detect a DCI format to indicate a PUSCH for the UEI-BR. It was discussed in previous meetings for the case where the UE detects a DCI format to indicate a PUSCH for the UEI-BR before transmitting the first PUCCH. This case was not covered by the agreement for the 3-step procedure of Mode A and the following options can be considered to handle this case.

- Option 1: UE considers it as an error case.

- Option 2: UE does not transmit the PUSCH.

- Option 3: UE does not multiplex the UEI-BR in the PUSCH.

Considering the DCI can also indicate the transmission of UL-SCH data, Option 3 can help reduce the latency for UL data transmission and thus is preferred.

In addition, the time gap between the first PUCCH and the DCI format indicating the PUSCH for the UEI-BR should be specified considering there should be some processing time for gNB detecting the first PUCCH as well as making the scheduling decision as shown in Figure 3. If the timeline condition is not satisfied, it is possible that it is a false alarm at the gNB side and the UE may not have enough time to prepare the UEI-BR to multiplex in the scheduled PUSCH.

Figure 3

Proposal 17: For Mode A, the UE multiplexes the UEI-BR of a CSI report configuration in the PUSCH indicated by a DCI format in a PDCCH reception if the PDCCH reception starts no earlier than T after the end of the first PUCCH transmission associated with the same CSI report configuration.

In addition, we observe that due to the design structure of Mode-A, in order to ensure reliable and valid UEI reporting content, the measurement window and timeline requirement for report content in the second UL channel needs to be discussed. To illustrate, in high mobility scenarios, it is possible that from the time that UE transmits the first UL channel until the transmission time of second UL channel, the event is no longer valid and/or the content of UEI report has been changed. Hence, we think that it is necessary to discuss UE behavior and the measurement window for reporting in such scenarios.

Proposal 18: For Mode-A, further study the measurement window for UEI reporting content.

Regarding the timeline requirements for Mode A, although the legacy timeline requirements for A-CSI report can be considered as the baseline, it would unnecessarily increase the latency. The difference from the legacy operation is that UEI-BR can be generated before receiving the PDCCH indicating the PUSCH considering that the UE can only transmit the first PUCCH after performing RS measurement(s) – in this case, the preparation time for UEI-BR can be reduced to the same as UL-SCH preparation time.

Proposal 19: Regarding UEI-BR processing timeline for Mode A, the legacy A-CSI reporting timeline as defined in TS 38.214 clause 5.4 should be supported as baseline and the legacy UL-SCH preparation time as defined in TS 38.214 clause 6.4 should be supported based on additional UE capability.

[120] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19

# Collision handling for overlapping channels

Issue #1: Collision handling for the first PUCCH

[119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1. [119] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases: - Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH - Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain. [120] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, support the following option of dropping rule for the Case-1: the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Option-1: LRR > first PUCCH > normal SR Note: When the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR, only one of them is transmitted based on the above priority rule [120] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rule for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules, i.e., the first PUCCH is dropped. - FFS: whether/how to handle the case of different PHY priorities. [120b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for Type-1 CG PUSCH with UEI-BR for Mode B [120b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support the following rule for the Case-4: the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ. - Option-1: Follow legacy SR multiplexing rule. - Note: There is no further enhancement on additional phase offset/constellation point(s). [120b] Conclusion There is no RAN1 concensus on the following. No spec change needed. On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support one of the following rule for the Case-3: the 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain - Option-1: Per CSI report configuration. - Option-2: Up to implementation. - Option-3: UE transmits the first PUCCH with higher priority (e.g., PUCCH resource with lower PUCCH resource ID, or event-type) - Option-4: Multiplexing all 1-bit indications [120b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities.

Regarding Case-2 where the 1-bit first PUCCH is collided/overlapped with a PUSCH, we support Option-3, i.e., piggybacking/appending the 1-bit indication into the PUSCH, which can largely reuse the legacy UCI multiplexing design. For example, reuse the design of HARQ-ACK by appending the UEIRI to HARQ-ACK information bits. As for Option-1, dropping PUSCH would result in too much spectral efficiency/throughput performance degradation, which is unacceptable to us – same view for Option-4.

Based on the above, we provide the following proposal:

Proposal 20: On beam report transmission procedure for UE-initiated/event-driven beam reporting,

- For the case where the first PUCCH is collided/overlapped with a PUSCH, support Option-3, i.e., the 1-bit indication of first PUCCH is multiplexed into the PUSCH.

  - Support joint coding with HARQ-ACK by appending the UEIRI to HARQ-ACK information bits.

To minimize specification impact, it is preferred to reuse legacy rules for the PUCCH resource and the associated PUSCH should be the same priority of the PUCCH. The overlapping between UL channels with different priorities and do prioritization following Rel-16 rules for simplicity.

Proposal 21: On beam report transmission procedure for UE-initiated/event-driven beam reporting,

- The PHY priority of PUCCH is determined as the same rule for SR and the PUSCH and PUCCH resources are of the same PHY priority.

- Reuse Rel-16 rules for prioritization for UL overlapping channels with different PHY priorities.

Issue #2: Collision handling for the second channel for Mode A

For Mode A, the PUSCH with UEI-BR can also be considered as A-CSI reports, therefore, the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI can be reused for PUSCH with UEI-BR.

Observation 7: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI can be reused for PUSCH with UEI-BR for Mode A.

Proposal 22: RAN1 to conclude to reuse the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH with UEI-BR for Mode A.

# Cross-CC measurement/report

Agreement (120b) On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

It was agreed in RAN1#120bis that an RRC parameter would be introduced to indicate the CC in which the indicated TCI state is applied. From RAN1’s perspective, there would be case(s) where the RRC parameter is not needed:

- When the indicated TCI state is applied to the same CC in which the candidate beam RS(s) is configured

- When there is only one RS specific to the indicated TCI state is in the same CC in which the candidate beam RS(s) is configured

Proposal 23: On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, clarify condition(s) of not providing the RRC parameter to indicate the CC in which the indicated TCI state is applied.

Another remaining issue for cross-CC beam report is whether to support an indication of the cell that corresponds to the configured first PUCCH resource in CSI report config, the issue was discussed in previous RAN1 meetings and not supported with the understanding that the PUCCH resource ID is a global parameter, the PUCCH resource ID can be used to determine the corresponding serving cell. Even if some companies argue that PUCCH resource ID is a local ID, it can be up to network implementation to ensure that the IDs are different for PUCCH resources on different cells.

Observation 8: There is no clear motivation to support an indication of the cell that corresponds to the configured first PUCCH resource in CSI report config.

# Priority rules for CSI reports

In the existing TS 38.214 Clause 5.2.5, the priority rules for CSI reports are defined. With the introduction of UE-initiated (UEI) beam reporting, those priority rules need to be revisited on the top of legacy rules.

In case of a PUCCH resource for the first PUCCH is associated with only one CSI report configuration, legacy priority rule can be reused in principle. For UEI-BR for Mode A, same rule for AP CSI report can be reused. For UEI-BR for Mode B, same rule for SP CSI report on PUSCH can be reused.

Proposal 24: For the determination of a priority value of a UEI-BR, in case of a PUCCH resource is associated with only one CSI report configuration

- For UEI-BR for Mode A, reuse the same rule for AP CSI report

- For UEI-BR for Mode B, reuse the same rule for SP CSI report on PUSCH

In case of multiple CSI report configurations are configured with the same PUCCH resource, if the priority value of a UEI-BR is based on the reported CSI report configuration, NW does not know the UEI-BR is initiated by which CSI report configuration until the content of the report is decoded and cannot know the result for prioritization if the UEI-BR overlaps with another CSI report. As a consequence, NW needs to perform blind detection. To avoid blind detection for the network, it is preferred to use a fixed value regardless of which CSI report configuration is reported. For example, the priority value is determined by the CSI report configuration with highest priority.

Observation 9: In case of the same PUCCH resource is associated with multiple CSI report configurations, NW may not know which CSI report is transmitted if the UEI-BR overlaps with another CSI report if the priority value of a UEI-BR is based on the reported CSI report configuration.

Proposal 25: In case of the same PUCCH resource is associated with multiple CSI report configurations, a priority value of a UEI-BR is determined by the CSI report configuration with highest priority from the multiple CSI report configurations.

# Conclusions

In this contribution, we provide our views on various design aspects related to Rel-19 UE-initiated beam reporting. In particular, we present the following proposals and observations focusing on event(s) and the corresponding triggering condition(s), measurement RS(s) configuration/determination for the Event-2, UL signaling content(s)/report format(s) and UCI based UL signaling procedure(s) designs:

Observation 1: For a new beam RS of a CSI report configuration, there can be multiple determined time windows before a PUCCH transmission occasion, where

- The multiple determined time windows are not overlapping in time domain.

- The number of instances for one same new beam may or may not greater than or equal to a configurable number M within a time window

- The UE should store the evaluation results for the multiple determined time windows to decide whether to transmit a first PUCCH.

Observation 2: The end of a time window with a number of instances for one same new beam being no less than M should not be far before the first PUCCH transmission to ensure real-time indication of UEI-BR.

Proposal 1: For a CSI report configuration, if a UE determines that a number of event instances for one same new beam is no less than M within a time window, the UE transmits the first PUCCH in an earliest transmission occasion T_proc after the end of the time window, where T_proc is configured by higher layer signalling.

Proposal 2: For determining an event instance for Event-1 and Event-2, the measured occasions for the current beam RS and the new beam RS(s) should be within the same periodicity.

Proposal 3: For a single CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state indicated in a latest PDCCH before a first PUCCH associated with the second PUSCH.

Proposal 4: For a multi-CC case and for a UEI-BR carried in a second PUSCH of a CSI report configuration, the current beam RS is the same as the RS derived by the TCI state, indicated in a latest PDCCH, for the serving cell of the new beam RS, where the latest PDCCH is before a first PUCCH associated with the second PUSCH.

Observation 3: Regarding triggering event determination for Event 2, other candidates than Candidate #2 do not have immediate impact on event evaluation.

Proposal 5: Based on Observation 2, deprioritize selection/determination of other candidate(s) than Candidate #2 for resetting the counting.

Observation 4: Retransmission of the first PUCCH cannot ensure the real-time indication of the UEI-BR.

Proposal 6: Do not support retransmission of the first PUCCH for both Mode A and Mode B.

Proposal 7: Do not support prohibit timer.

Proposal 8: Regarding the RS periodicity for Event determination,

- For Event-2, do not support different periodicities of the current beam RS and the candidate/new beam RS(s).

- For Event-1, only support the same periodicity of the current beam RS and the candidate/new beam RS(s).

- For Event-7, only support the same periodicity of measured occasions for the RS(s) associated with all activated TCI state(s) and the candidate/new beam RS(s).

Observation 5: Supporting counting more than one event instance for evaluating or determining Event-7 could (1) increase the measurement latency, which is not aligned with the objective of the WID for latency reduction, and (2) complicate UE’s implementations.

Proposal 9: Do not support counting more than one event instance for evaluating or determining Event-7.

Proposal 10: On UE-initiated/event-driven beam reporting, support inter-cell SSBs (associated with the serving cell PCI or PCI(s) other than the serving cell PCI) as current beam RSand candidate beam RS(s) for event evaluations.

Proposal 11: Regarding explicit RS configuration for candidate beam measurement in evaluating Event 2 (Option-1 in the RAN1#118 agreement), do not support updating the RS in the RS resource set by a new MAC CE.

Proposal 12: Regarding Event 2 evaluation, a UE can be enabled to evaluate the L1-RSRP(s) of the RS(s) in the RS resource set (as in Option-1) that has the same value(s) as the RS(s) in the activated TCI state(s) as the candidate beam measurement results.

Observation 6: For a given current beam of a serving cell, measuring a subset of surrounding new beams can be beneficial for latency reduction.

Proposal 13: For a given current beam of a serving cell, support current beam specific measurement for UEI-BR. The following options can be further considered,

- Option 1: Multiple CSI report configurations where each configuration corresponds to a current beam RS

- Option 2: One CSI report configuration consists of multiple sets of new beam RSs where each set

corresponds to a current beam RS

Proposal 14: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 of Event-2, if N is not provided by RRC, only one L1-RSRP and CRI/SSBRI are reported.

Proposal 15: For a first PUCCH transmission occasion for both Mode A and Mode B, if there has already been transmission(s) of another first PUCCH(s) within a time window associated with the first PUCCH transmission occasion, the UE would not transmit the first PUCCH.

Proposal 16: Regarding Mode B, the RRC configured value X symbols for determining the first available transmission occasion of the second UL channel after sending the last symbol of report notification on the first PUCCH channel to carry the actual UEI beam report is subject to a corresponding UE capability.

- X is based on the SCS of PUCCH.

Proposal 17: For Mode A, the UE multiplexes the UEI-BR of a CSI report configuration in the PUSCH indicated by a DCI format in a PDCCH reception if the PDCCH reception starts no earlier than T after the end of the first PUCCH transmission associated with the same CSI report configuration.

Proposal 18: For Mode-A, further study the measurement window for UEI reporting content.

Proposal 19: Regarding UEI-BR processing timeline for Mode A, the legacy A-CSI reporting timeline as defined in TS 38.214 clause 5.4 should be supported as baseline and the legacy UL-SCH preparation time as defined in TS 38.214 clause 6.4 should be supported based on additional UE capability.

Proposal 20: On beam report transmission procedure for UE-initiated/event-driven beam reporting,

- For the case where the first PUCCH is collided/overlapped with a PUSCH, support Option-3, i.e., the 1-bit indication of first PUCCH is multiplexed into the PUSCH.

  - Support joint coding with HARQ-ACK by appending the UEIRI to HARQ-ACK information bits.

Proposal 21: On beam report transmission procedure for UE-initiated/event-driven beam reporting,

- The PHY priority of PUCCH is determined as the same rule for SR and the PUSCH and PUCCH resources are of the same PHY priority.

- Reuse Rel-16 rules for prioritization for UL overlapping channels with different PHY priorities.

Observation 7: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI can be reused for PUSCH with UEI-BR for Mode A.

Proposal 22: RAN1 to conclude to reuse the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH with UEI-BR for Mode A.

Proposal 23: On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, clarify condition(s) of not providing the RRC parameter to indicate the CC in which the indicated TCI state is applied.

Observation 8: There is no clear motivation to support an indication of the cell that corresponds to the configured first PUCCH resource in CSI report config.

Proposal 24: For the determination of a priority value of a UEI-BR, in case of a PUCCH resource is associated with only one CSI report configuration

- For UEI-BR for Mode A, reuse the same rule for AP CSI report

- For UEI-BR for Mode B, reuse the same rule for SP CSI report on PUSCH

Observation 9: In case of the same PUCCH resource is associated with multiple CSI report configurations, NW may not know which CSI report is transmitted if the UEI-BR overlaps with another CSI report if the priority value of a UEI-BR is based on the reported CSI report configuration.

Proposal 25: In case of the same PUCCH resource is associated with multiple CSI report configurations, a priority value of a UEI-BR is determined by the CSI report configuration with highest priority from the multiple CSI report configurations.

# References

[1] Chairman Notes, 3GPP RAN1#120bis