3GPP TSG RAN WG1#121			R1-2503556

Malta, MT, May 19th – May 23th, 2025

Agenda item:	9.2.2

Source: 	Samsung

Title: 	Views on Rel-19 CSI enhancements

Document for:	Discussion and Decision

# Introduction

This contribution provides Samsung’s view on remaining issues related to Rel.19 CSI enhancements.

# Type-I/II

## Other issues

Remaining issues on IMR slot offset

[116bis] Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, the legacy resource configuration for interference measurement is reused, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured where the one IM resource is associated with all the K CSI-RS resources in the CSI-RS resource set for channel measurement [116bis] Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, support the following aperiodic CMR configuration: - A UE can be configured with KDOPP = {4, 8, 12} CSI-RS resource groups for the purpose of aperiodic CMR as needed by Type-II Doppler CSI - The time separation between the first resources from two consecutive groups (=m) can be configured from {1, 2} - Each CSI-RS resource group comprises K NZP CSI-RS resources (K defined in previous agreements) for aggregation associated with a same CSI-RS resource set assuming the agreed resource set rules for Rel-19 Type-I/II codebooks - All the KDOPP CSI-RS resource groups are associated with a same CSI-RS resource set configuration [120bis] Agreement For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler

For aggregated NZP aperiodic CSI-RS resources for Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, it was agreed that the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot, which is corresponding to aperiodic slot offset configured with the triggered CSI-RS resource set.

One remaining issue is how to determine a slot offset for NZP CSI-RS resource for interference measurement, since there could be two possible slot offsets. Similar with the case of CSI-IM, it would be good to clarify the triggering offset of the associated aperiodic NZP CSI-RS resource for interference measurement is configured with the same value of the associated NZP CSI-RS resource(s) for CMR in the first slot, which is corresponding to aperiodic slot offset configured with the triggered CSI-RS resource set.

Proposal 1:

For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset configured with the NZP CSI-RS resource set including an associated aperiodic NZP CSI-RS resource for interference measurement is the same as that of the associated NZP CSI-RS resource(s) for CMR in the first slot.

For Rel-19 Type-II Doppler CSI, an aperiodic CMR can be configured with KDOPP= {4, 8, 12} CSI-RS resource groups where the time separation between two consecutive groups can be 1 or 2 slots and each group has K CSI-RS resources that are configured within two slots. Hence, depending on an AP-CMR configuration, up to 24 consecutive slots can be used to allocate KDOPP×K CSI-RS resources for Rel-19 Type-II Doppler CSI. Then, among the slots, we may have a question on which slot offset would be appropriate for the associated aperiodic (only one) CSI-IM resource or (only one) NZP CSI-RS resource for interference measurement.

In our view, three aspects can be considered to suggest an appropriate slot offset for the CSI-IM resource or NZP CSI-RS resource for interference measurement:

- There is no prior information for a best slot offset for estimating interference at the time that PDSCH transmission is to be performed. (i.e., it is unknown which particular slot offset would be better in terms of performance for estimating the interference of the PDSCH transmission)

- Putting the CSI-IM resource or NZP CSI-RS resource for interference measurement in the last slot can relax the real-time buffer status of the UE, rather than putting it in the first slot.

- There is a tendency (e.g., Markov process) that the latest slot reflects more accurate environment for the future PDSCH transmission than the first slot (outdated).

From the above aspects, we suggest to use the last slot for the associated aperiodic CSI-IM resource or NZP CSI-RS resource for interference measurement:

Proposal 2:

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP CSI-RS resource groups are configured for aperiodic CMR, where each CSI-RS resource group comprises K>1 aggregated NZP aperiodic CSI-RS resources, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the last slot.

Proposal 3:

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP CSI-RS resource groups are configured for aperiodic CMR, where each CSI-RS resource group comprises K>1 aggregated NZP aperiodic CSI-RS resources, the triggering offset configured with the NZP CSI-RS resource set including an associated aperiodic NZP CSI-RS resource for interference measurement is the same as that of the associated NZP CSI-RS resource(s) for CMR in the last slot.

# CRI-based CSI

## Other Issues

‘Priority rule for multiple CSI reports’

Proposal 2.B: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order of NRep CSI reports, - For reports with non-M CRI based CSI reports (Legacy CSI reports till Rel-18) should have higher priority over M CRIs based CSI reports. - For reports configured with MR resources should have higher priority over reports with non-MR based CSI reports. - For reports with lower M values should have higher priority over higher M CRIs based CSI reports. PriiCSIy,k,c,s,m,M, MR=2∙Ncells∙Ms∙Mm∙y+Ncells∙Ms∙Mm∙k +Ms∙Mm∙c+s+Ms∙(M-MR)∙m where - m = 0 for non-M CRI based CSI reports. Mm= 1 (legacy CSI reports up to Rel-18), - m = 1 for M CRI based CSI reports. Mm is the maximum number of CRIs configured for multi-CRI CSI reports not carrying L1-RSRP or L1-SINR Note: It is up to the editor how this is captured in the specification

In RAN1#120, Priority rule for multiple NRep CSI reports was discussed. The main use case of multi CRI feedback for HBF is providing better MU-MIMO scheduling opportunities for network. However, this type of CSI report can have very high payload and it can vary depending on reporting of second TB. With this it can lead to CSI omission if multiple CSI reports are configured to report on same PUSCH/PUCCH. So for such cases, it can be beneficial to have higher priority for non-M CRI based CSI reports than M CRIs based CSI reports because omission of subband information of one or multiple CRI is still better than omission of whole subband information of non-M CRI based CSI reports. Hence we think non-M CRI based CSI reports should have priority over M CRIs based CSI reports. Second and third rule is quite intuitive based on configuration of higher priority MR resources and value of M.

In legacy, the priority for similar type of CSI reports can be set up by RRC configuration of CSI reportconfigID. The Reports with lower CSI reportconfigID has higher priority. However, The RRC configuration of CSI reportconfig IDs based on M CRIs and keeping track of multiple CSI reportconfigID can be cumbersome task for network. Hence incorporating this rule in priority equation can be good solution which handles the priority of reports irrespective of their CSI reportconfig ID values.

Proposal 4: Support the priority rules and updating of priority equation in Proposal 2.B for Rel-19 CRI based CSI refinement for up to 128 ports

Conclusion

In this contribution, the following observations and proposals are made:

Type-I/II

Proposal 1:

For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset configured with the NZP CSI-RS resource set including an associated aperiodic NZP CSI-RS resource for interference measurement is the same as that of the associated NZP CSI-RS resource(s) for CMR in the first slot.

Proposal 2:

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP CSI-RS resource groups are configured for aperiodic CMR, where each CSI-RS resource group comprises K>1 aggregated NZP aperiodic CSI-RS resources, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the last slot.

Proposal 3:

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, when the KDOPP CSI-RS resource groups are configured for aperiodic CMR, where each CSI-RS resource group comprises K>1 aggregated NZP aperiodic CSI-RS resources, the triggering offset configured with the NZP CSI-RS resource set including an associated aperiodic NZP CSI-RS resource for interference measurement is the same as that of the associated NZP CSI-RS resource(s) for CMR in the last slot.

CRI-based CSI

Proposal 4: Support the priority rules and updating of priority equation in Proposal 2.B for Rel-19 CRI based CSI refinement for up to 128 ports

# References