3GPP TSG RAN WG1 #121		R1-2503568

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.4.3

Source:	Samsung

Title:	Views on timing acquisition and synchronization

Document for:	Discussion and decision

# 1	Introduction

In this contribution, we share our views on timing acquisition and synchronization signals according to the WID [1] to complete the WI in this meeting. The discussion involves remaining issues on R2D/D2R preamble design, D2R midamble design, and necessities of R2D/D2R postamble.

# R2D preamble

Figure 1 below illustrates a basic PRDCH/PDRCH structure, including preamble, header and payload.

Figure 1 General PRDCH/PDRCH structure with preamble

For R2D, a preamble immediately precedes the transmission of PRDCH, and includes a start-indicator part (SIP) providing the start of the R2D transmission and a clock-acquisition part (CAP) providing a functionality for clock synchronization including determining the OOK chip rate of the subsequent PRDCH transmission.

For D2R, a preamble immediately precedes the transmission of PDRCH, and provides a functionality for synchronization and time tracking, determination of OOK chip rate or BPSK symbol rate of the subsequent PDRCH transmission, and channel estimation for coherent detection of the PDRCH.

  - Start-indicator part (SIP)

In RAN1 120b, the following agreements were made related to R2D SIP design [2].

Agreement For the pattern of SIP of R-TAS, only the following 2 alternatives are considered for further down-selection: - Alt 1-2: ON-OFF with a ratio of 1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration - Alt 2-4: ON-OFF-ON-OFF with a ratio of 1:1:1:3 and with following total SIP duration to be further down-selected: - Option 1: 0.5 OFDM symbol duration - Option 2: 1 OFDM symbol duration Agreement For R-TAS, SIP duration of 1 OFDM symbol is adopted with CAP pattern ON-OFF-ON-OFF for all values of M corresponding to PRDCH - Note: device cannot assume the presence/absence of RF transmission prior to the SIP.

The remaining issues for R2D SIP design include a selection of SIP pattern from Alt 1-2 and Alt 2-4 as illustrated below.

Figure 2 Illustration of Alt 1-2 and Alt 2-4 of SIP design per agreement

In our view, Alt 1-2 is sufficient for SIP.

The main purpose of Alt 2-4 is for dynamic detection threshold setting using the first part of ON-OFF pattern. In our view, the threshold setting is within an implementation scope and it does not require a particular specification supporting mechanism. In other words, Alt 1-2 with single ON-OFF pattern will be sufficient and serve the purpose.

For a reader intending to communicate with a certain device, it is unreasonable to assume a scenario where the reader continuously transmits ON signal immediately preceding the SIP. In a reasonable reader implementation, the reader will pause a transmission for a certain amount of time prior to the SIP for a device to be able to detect the start of the SIP. There may be noise and interference signal from other neighboring readers, which will be treated as an ambient power level of OFF states. A device would have a current detection threshold value and the detection threshold can be continuously updated as the device receives R2D signal. When the ON signal of the SIP is detected and measured, the device can adjust the detector threshold based on a power level measured from the ON state.

On the other hand, A-IoT devices are expected to have a higher receiver sensitivity compared to RFID devices. It is unclear what’s the scenario when it is claimed that A-IoT devices would have a lower receiver sensitivity compared to RFID devices.

- SIP based on Alt 1-2 provides necessary functionalities for setting a detector threshold and start indication.

As discussed, it is our view that a single ON/OFF pattern shall be sufficient and there is no need to have a sperate threshold detection training part, and the threshold for envelop detector can be set during the single ON duration.

For the pattern of SIP of R-TAS, Alt 1-2 (ON-OFF with a ratio of 1:3) is supported.

  - Clock-acquisition part (CAP)

In RAN1 120b, the following agreements were made related to R2D CAP design [2].

Agreement For CAP of R-TAS, following is adopted: - Option 1 for CAP of R-TAS from TR 38.769 is adopted where the CAP duration becomes proportionally shorter with increasing value of M, i.e. if for M = X, duration is Y OFDM symbol long, then for M = aX, duration is Y/a OFDM symbol long - Note: Duration without CP insertion is considered above, with CP insertion, the total duration may not be exactly proportional - Only following two alternatives for CAP pattern are considered for further down-selection to one alternative: - Alt 1: ON-OFF-ON-OFF - Alt 2: ON-OFF-ON

The CAP pattern had been agreed to Alt 1, i.e., ON-OFF-ON-OFF, in a later agreement as captured in the previous section. There are several remaining issues for CAP design as discussed below.

In RAN1 120b, there was an FFS ‘whether/how CAP use same M values set as PRDCH’ as copied below [2].

Agreement For R2D, at least for PRDCH, the set of M values is {2, 6, 12, 24} - FFS: whether/how CAP use same M values set as PRDCH

In our view, the CAP should use the same M value corresponding to the M value used in the following PRDCH. First of all, there is no clear motivation to do otherwise. Secondly, using the same M value between CAP and the following PRDCH helps the device to acquire a precise chip synchronization without any unnecessary steps.

- CAP using the same M value as the following PRDCH is the most simple and precise method for a device to acquire chip length without any unnecessary steps.

If the CAP uses different M value other than the M value set for PRDCH, it implies an approach where L1 control of PRDCH indicates an M value corresponding to a chip length for the PRDCH data part, which can be potentially different from the chip length obtained from the preamble.

Without any foreseeable benefit, this approach will increase the device complexity for switching chip rates quickly between control and data parts, which can be challenging for devices. For the reception of data part, the device has to deduce the chip synchronization from that obtained from the CAP rather than it can assume that the chip synchronization is directly conveyed over. This will require additional complexity as well as losing a precision.

Figure 3 PRDCH chip length indication via L1 control information (NOT preferred)

CAP uses the same M value corresponding to the M value used in the following PRDCH.

Another relevant remaining issue, albeit it does not directly affect the CAP design itself, is CP handling. The figure below describes CAP design for a set of supported M values according to the previous agreements.

Figure 4 CAP design for a set of supported M values according to RAN1 120b [2]

In RAN1 120b [2], the CP handling for M=24 case is still open for further discussion and decision.

Agreement For the below agreement, further update on the followings Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

Despite that a precise synchronization is required for M=24, it was agreed to have only four chips comprising of ON-OFF-ON-OFF for CAP. If random states of chips corresponding to CP are places immediately preceding the CAP, it will make the device detection of chips and synchronization more difficult. Although we understand that CAP detection is not the only consideration for determining CP handling method, we suggest to adopt Option 1, i.e., ‘The last 2 out of M OOK chips at the end of an OFDM symbol are always ON’, as a CP handling method for M=24.

- If random states of chips corresponding to CP are places immediately preceding the CAP, it will make the device detection of chips and synchronization more challenging for M=24.

For CP handling for M=24, Option 1 (The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’) is supported.

# D2R preamble/midamble

In RAN1 120b, the following agreements were made related to D2R preamble/midamble design [2].

Agreement For D2R midamble, for determining the presence and location of midamble(s) at the device: - Reader explicitly indicates the same interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information - FFS: details of signalling - FFS: whether the reader can explicitly indicate with one bit whether a midamble is additionally present at the end - Note: This does not preclude the support of having no midamble present in the D2R transmission Agreement - For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n-1 - Value(s) of n - Long preamble/midamble is generated based on n = 5 - Working assumption: Short preamble/midamble is generated based on n=3 - Only 1-part preamble/midamble are supported for D2R - Preamble immediately precedes the PDRCH without any gap - Both long and short preamble and midamble are supported based on the working assumption on n - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH: - Short preamble and short midamble - Long preamble and long midamble Note: the case of short preamble and long midamble will not be supported - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH Agreement For indicating the interval between consecutive midambles, and between the preamble and the first midamble, via R2D control information, following is adopted: - Unit of interval is number of bits after FEC (if FEC is applied) and repetition (if repetition is applied) - FFS: the candidate values in terms of the unit of interval

For short D2R preamble/midamble format, we suggest to confirm the working assumption of n=3 unless there are clear motivations to define otherwise.

For the short D2R preamble/midamble format, confirm the working assumption of n=3.

There are several remaining issues related to D2R preamble/midamble.

First remaining issue is whether the reader would explicitly indicate whether a midamble is additionally present at the end of PDRCH or not. Although allowing a reader to have a controllability of attaching midamble at the end of PDRCH has clear benefits, there was a concern on additional signaling overhead in the previous RAN1 meeting, albeit it is a higher layer signaling.

- Allowing a reader to have a controllability of attaching midamble at the end of PDRCH has clear benefits, while there was a concern on additional signaling overhead, albeit it is a higher layer signaling.

In the following, we discuss an approach that does not require an explicit indication for attaching midamble at the end of PDRCH.

Figure 5 D2R midamble at the end of PDRCH

In case 3) as described in the above figure, after inserting one or more midambles according to the indicated interval, if the remaining PDRCH transmission in a number of bits is exactly the same as the indicated interval, it is understood from the previous RAN1 120 [3] agreement that a device is expected to attach midamble at the end of PDRCH.

Agreement - For D2R preamble design, the functionalities of timing acquisition, SFO estimation/time tracking and channel estimation should be supported - For D2R midamble design, the functionalities of SFO estimation/time tracking and channel estimation should be supported - D2R midamble can be transmitted at the end of the PDRCH transmission. If it is at the end, it is not designed for being used for indicating the end of PDRCH transmission - FFS: condition(s) and/or indication where the D2R midamble is present or not

If the remaining PDRCH transmission length after the last midamble is exactly the same as the indicated interval, a device attaches midamble at the end of PDRCH.

In case 1), after inserting one or more midambles according to the indicated interval, the remaining PDRCH transmission in a number of bits is relatively small compared to the interval. In this case, attaching the midamble at the end of PDRCH would not be beneficial, as there would be a minimal gain by attaching additional midamble.

In case 2), after inserting one or more midambles according to the indicated interval, the remaining PDRCH transmission in a number of bits is smaller but comparable to the indicated interval. In this case, attaching the midamble at the end of PDRCH would be necessary, as there would be a clear benefit of attaching additional midamble at the end of PDRCH. The determination of attaching an additional midamble at the end of PDRCH can be based on an implicit rule without requiring an explicit indication to address the concern in the previous meeting.

- After inserting one or more midambles according to the indicated interval, if the remaining PDRCH transmission length is relatively large with respect to the indicated interval, it is beneficial to attach additional midamble at the end of PDRCH based on an implicit rule without requiring an explicit indication.

If the remaining PDRCH transmission length after the last midamble is larger than a certain threshold with respect to the indicated interval, a device attaches midamble at the end of PDRCH.

Another remaining issue is signaling details for indicating the format of preamble and the presence of midamble. A reader can indicate the format of preamble between long and short formats using 1-bit indication.

The format of D2R preamble between long and short formats is indicated using 1 bit in the preceding PRDCH control information.

The presence of midamble does not need to be indicated using a separate dedicated bit; it can be indicated via interval indication. For instance, one codepoint of indicating an interval can be reserved for indicating no midamble. It can be interpreted as an infinite interval for inserting midambles. Similarly, if an indicated interval is longer than the total PDRCH transmission length, a device can interpret it to no midamble. Otherwise, if a valid interval is indicated, a device can insert one or more midambles according to the indicated interval.

The presence of D2R midamble is indicated via interval indication without a separate dedicated field, e.g., one codepoint of interval is reserved for indicating no midamble.

Another remaining issue is the candidate values in terms of the unit of intervals. Assume that L-bit field is used to indicate the interval. One codepoint is reserved for indicating no midamble, and remaining 2L -1 codepoints can indicate an interval from a set of predefined interval values. One straightforward way to avoid lengthy discussion in the last meeting is to indicate the interval in the unit of bytes such that 1, 2, …, 2L-1 bytes can be indicates as a possible interval value, while one codepoint is reserved for no midamble.

The interval for midamble is indicated in the preceding PRDCH using L-bit field, where one codepoint is reserved for no midamble and the rest of codepoints indicate the interval in bytes, i.e., 1, 2, …, 2L-1 bytes.

# Postamble

  - R2D

In the previous meeting, it was discussed that Manchester coding rule violation by reader’s implementation can be used by the device to determine the end of PRDCH transmission. In our view, specifying a pattern is a clearer solution for device to detect the postamble. Leaving it up to any violating pattern make the device behavior complicated as a violating pattern could happen due to errors such as distortion, channel propagation, or sampling errors. In this case, the device is unable to distinguish between erroneous violating pattern and an intended violating pattern for the purpose of postamble.

- Manchester coding rule violation by reader’s implementation to indicate the end of PRDCH transmission is not preferable due to device blind detection complexity and possibility of false alarm.

One simple way to specifying postamble pattern is an inverse of the SIP.

The R2D postamble is an inverse of R2D SIP.

Unless explicitly specifying the R2D postamble, there should be no postamble and accordingly no need to agree that Manchester coding rule violation by reader’s implementation can be used by the device to determine the end of PRDCH transmission. This will not only require device blind detection but also it cannot require to a device to detect any random violation patterns.

- Unless explicitly specifying the R2D postamble, there is no need to agree on Manchester coding rule violation by reader’s implementation to be used by the device to determine the end of PRDCH transmission.

  - D2R

For D2R, there is no clear motivation to support D2R postamble.

D2R postamble is not supported.

# Conclusion

In this contribution, we shared our views on timing acquisition and synchronization signals. More specifically, the discussion involved remaining issues on R2D/D2R preamble design, D2R midamble design, and necessities of R2D/D2R postamble. To this end, the following observations and proposals were made:

In the previous sections we made the following observation:

Observation 1	SIP based on Alt 1-2 provides necessary functionalities for setting a detector threshold and start indication.

Observation 2	CAP using the same M value as the following PRDCH is the most simple and precise method for a device to acquire chip length without any unnecessary steps.

Observation 3	If random states of chips corresponding to CP are places immediately preceding the CAP, it will make the device detection of chips and synchronization more challenging for M=24.

Observation 4	Allowing a reader to have a controllability of attaching midamble at the end of PDRCH has clear benefits, while there was a concern on additional signaling overhead, albeit it is a higher layer signaling.

Observation 5	After inserting one or more midambles according to the indicated interval, if the remaining PDRCH transmission length is relatively large with respect to the indicated interval, it is beneficial to attach additional midamble at the end of PDRCH based on an implicit rule without requiring an explicit indication.

Observation 6	Manchester coding rule violation by reader’s implementation to indicate the end of PRDCH transmission is not preferable due to device blind detection complexity and possibility of false alarm.

Observation 7	Unless explicitly specifying the R2D postamble, there is no need to agree on Manchester coding rule violation by reader’s implementation to be used by the device to determine the end of PRDCH transmission.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the pattern of SIP of R-TAS, Alt 1-2 (ON-OFF with a ratio of 1:3) is supported.

Proposal 2	CAP uses the same M value corresponding to the M value used in the following PRDCH.

Proposal 3	For CP handling for M=24, Option 1 (The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’) is supported.

Proposal 4	For the short D2R preamble/midamble format, confirm the working assumption of n=3.

Proposal 5	If the remaining PDRCH transmission length after the last midamble is exactly the same as the indicated interval, a device attaches midamble at the end of PDRCH.

Proposal 6	If the remaining PDRCH transmission length after the last midamble is larger than a certain threshold with respect to the indicated interval, a device attaches midamble at the end of PDRCH.

Proposal 7	The format of D2R preamble between long and short formats is indicated using 1 bit in the preceding PRDCH control information.

Proposal 8	The presence of D2R midamble is indicated via interval indication without a separate dedicated field, e.g., one codepoint of interval is reserved for indicating no midamble.

Proposal 9	The interval for midamble is indicated in the preceding PRDCH using L-bit field, where one codepoint is reserved for no midamble and the rest of codepoints indicate the interval in bytes, i.e., 1, 2, …, 2L-1 bytes.

Proposal 10	The R2D postamble is an inverse of R2D SIP.

Proposal 11	D2R postamble is not supported.

# References

RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120b, Wuhan, China, April 7th – 11th, 2025.

RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120, Athens, Greece, February 17th – 21st, 2025.