3GPP TSG RAN WG1 #121		R1-2503570

St Julian's, Malta, 19th – 23rd May, 2025

Agenda item:	9.5.1

Source:	Samsung

Title:	On-demand SSB SCell operation

Document for:	Discussion and Decision

# Introduction

In RAN1#120bis [1], the agreements made regarding the on-demand SSB for SCell operation are summarized in the Appendix. This contribution discusses the further details on supporting on-demand SSB on SCell, including configuration for the on-demand SSB, activation of on-demand SSB, adaptation of on-demand SSB, deactivation of on-demand SSB, relationship between on-demand SSB and always-on SSB, L1 measurement based on on-demand SSB, and resource allocation and validation based on on-demand SSB.

# Configuration of on-demand SSB

Up to RAN1#120bis, it has been agreed to support multiple candidate values for the following parameters configured by RRC:

- Periodicity for on-demand SSB burst;

- SFN offset for the on-demand SSB burst;

- Half frame index for the on-demand SSB burst;

- Transmitted SSB indexes within the on-demand SSB burst;

- Number of transmitted on-demand SSB bursts.

Meanwhile, RAN2 agreed to support a single configuration index per serving cell by MAC CE to activate one of the candidate configurations for on-demand SSB. Then, each candidate configuration shall include the above parameters with multiple candidate values agreed in RAN1 (some parameter can be absent according to existing RAN1 agreements). Also, to control the overhead in the MAC CE for the configuration index indication and to limit the size of configurations for on-demand SSB by RRC, the maximum of configurations can be limited to 16, such that at most 4 bits are used for indicating the configuration index per serving cell by the MAC CE.

Proposal 1: For on-demand SSB:

- Support a list of configurations by RRC, wherein each configuration includes the following parameters:

  - od-ssb-Periodicity;

  - od-ssb-sfn-Offset;

  - od-ssb-halfFrameIndex;

  - od-ssb-PositionsInBurst;

  - od-ssb-nrofBurst.

- Note: some parameter(s) can be optional subject to RAN1 agreements.

- The maximum number of configurations in the list is 16.

# Activation of on-demand SSB

RAN1 has agreed to support indicating activation of on-demand SSB either by a MAC CE or by a RRC. When activating by a MAC CE, the MAC CE will provide an indication on the configuration index for the on-demand SSB, such that the UE can apply the corresponding parameters in the indicated configuration to determine a timing for starting to receive the on-demand SSB. When activating by a RRC, it is not clear which configuration is used for the on-demand SSB, when there are multiple configurations in the RRC, and a corresponding clarification is needed as in the following proposal. Also, since the configuration can include a parameter of the number of transmitted on-demand SSB bursts, there needs an alignment between the gNB and UE on how to count the number of transmitted on-demand SSB bursts (which does not imply the UE has to receive all of them), hence, a timeline needs to be specified to determine the first on-demand SSB burst after activation by the RRC.

Proposal 2: When on-demand SSB transmission is indicated to be activated by the RRC, the UE applies the parameters associated with the first configuration within the list of configurations included the RRC for the on-demand SSB transmission.

- Need to specify the timeline at least when the number of transmitted on-demand SSB bursts is provided.

# Adaptation of on-demand SSB

In RAN1#120bis, it was agreed to support using a MAC CE to adapt parameter(s) for an on-going on-demand SSB transmission, and it should be clarified that the MAC CE based adaptation can be applicable to any activation mechanism agreed in RAN1, e.g., from the network point of view, there is no restriction on using the MAC CE for the adaptation of parameter(s) for on-demand SSB. To be more precise, we have the following proposal.

Proposal 3: For adaptation of parameter(s) for on-demand SSB, support all the following cases:

- Case A1: activation by a RRC and N is not provided + adaptation by a MAC CE;

- Case A2: activation by a RRC and N is provided + adaptation by a MAC CE;

- Case B1: activation by a MAC CE and N is not provided + adaptation by a MAC CE;

- Case B2: activation by a MAC CE and N is provided + adaptation by a MAC CE.

# Deactivation of on-demand SSB

From RAN1 perspective, deactivation of on-demand SSB transmission has been agreed to be either by an explicit indication by a MAC CE, or by an implicit way based on the number of transmitted on-demand SSB burst if provided. There is a remaining issue on how to apply the deactivation mechanism to the activation and/or adaptation mechanisms. In our view, MAC CE based activation, deactivation, and adaptation shall be served as the basic functionalities for on-demand SSB operation, and RRC based activation may be used only in a special scenario (e.g., when configuration and activation of the SCell and the activation of the on-demand SSB transmission occur at the same time). From this point of view, all the combinations of deactivation mechanism with activation and adaptation mechanisms shall be supported.

In particular, when a counter on the number of transmitted on-demand SSB bursts is running, a gNB can use a MAC CE to terminate the on-demand SSB transmission as an early termination (e.g., Case X3, Y3, and Z3 in Proposal 4). This would be needed in the use case that the on-demand SSB transmission is intended for multiple UEs, while at least one UE didn’t receive the MAC CE for indicating the activation of the on-demand SSB transmission at the first reception occasion but in a later reception occasion as a retransmission. In such use case, the first transmitted SSB burst could be counted differently at different UEs, and a gNB may need to send a MAC CE to early terminate the on-demand SSB transmission for the UE receiving the retransmitted MAC CE indicating the activation of the on-demand SSB transmission. An illustration of the use case is shown in Figure 1.

Figure 1 Illustration of an example use case for early termination of on-demand SSB transmission by a MAC CE.

Proposal 4: For deactivation for on-demand SSB, support all the following cases:

- Case X1: activation by a RRC and N is not provided + deactivation by a MAC CE;

- Case X2: activation by a RRC and N is provided + deactivation after N transmission bursts;

- Case X3: activation by a RRC and N is provided + deactivation by a MAC CE;

- Case Y1: activation by a MAC CE and N is not provided + deactivation by another MAC CE;

- Case Y2: activation by a MAC CE and N is provided + deactivation after N transmission bursts;

- Case Y3: activation by a MAC CE and N is provided + deactivation by another MAC CE;

- Case Z1: adaptation by a MAC CE and N is not provided + deactivation by another MAC CE;

- Case Z2: adaptation by a MAC CE and N is provided + deactivation after N transmission bursts;

- Case Z3: adaptation by a MAC CE and N is provided + deactivation by another MAC CE.

# Relationship between on-demand SSB and always-on SSB

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, which is only applicable when always-on SSB is a CD-SSB located on a sync raster, the on-demand SSB should be only motivated for measurement purpose, and restricting it as a NCD-SSB to save the transmission of SIB1 would be beneficial for network energy saving.

Also, it has been agreed to support multiple candidate values for transmitted SSB indexes within the on-demand SSB burst, and there should be a further restriction that the transmitted SSB indexes for the on-demand SSB are same as or a subset of the ones for the always-on SSB, since there is no motivation to support a new beam for the on-demand SSB only.

Proposal 5: For the case when the center frequency locations of always-on SSB and on-demand SSB are different:

- On-demand SSB is a NCD-SSB;

- Transmitted SSB indexes for on-demand SSB are same as or a subset of the ones for always-on SSB.

# L1 measurement based on on-demand SSB

In RAN1#120bis, RAN1 agreed for Case 2 (i.e., with always-on SSB), a CSI report configuration can be associated with both of always-on SSB and on-demand SSB, or associated with on-demand SSB only. There is a further study point that whether the association with on-demand SSB only is applicable for the case where always-on SSB and on-demand SSB have different center frequencies. It can be observed that when always-on SSB and on-demand SSB have the same center frequency, all parameters regarding the always-on SSB and on-demand SSB are the same, and the UE is not required to distinguish the two types of SSBs. In this sense, gNB may not have strong motivation to ask the UE to perform L1 measurement based on on-demand SSB only.

Proposal 6: For Case 2 (i.e., with always-on SSB), only when always-on SSB and on-demand SSB have different center frequencies, a CSI report configuration is associated with on-demand SSB only.

Moreover, L1 measurement and reporting based on on-demand SSB shall be performed only after the on-demand SSB is indicated to be transmitted, and the UE may not need to perform any L1 measurement or reporting before the on-demand SSB is indicated to be transmitted. For another aspect, in the legacy, a UE is not required to perform CSI reporting for a deactivated SCell before receiving the SCell activation command, when the L1 measurement is performed based on always-on SSB. Now with on-demand SSB, which can be indicated to be transmitted earlier than receiving the MAC CE triggering the SCell activation, the UE shall perform CSI reporting even for a deactivated SCell before receiving the SCell activation command (i.e., Scenario #2) in order to facilitate the SCell activation procedure.

Proposal 7: For L1 measurement based on on-demand SSB:

- Before on-demand SSB is indicated to be transmitted, the UE is not required to perform L1 measurement or CSI reporting even though it receives the configuration on L1 measurement and reporting;

- When on-demand SSB is indicated to be transmitted in Scenario #2, the UE can be required to perform CSI reporting in Scenario #2.

# Resource allocation and validation based on on-demand SSB

In legacy NR, always-on SSB has the highest priority in term of the occupied time and frequency resources, such that the resource allocation of other DL signals and channels shall perform truncation or rate matching based on the transmission of the always-on SSB, and UL signals and channels may also need validation and/or cancellation based on the transmission of the always-on SSB. Similar operation is expected for on-demand SSB, but the discussion for the on-demand SSB can be separate for the UEs knowing the time and frequency resources of on-demand SSB (e.g., the UEs receiving the RRC/MAC CE for configuration/activation/adaptation) and the UEs not knowing the time and frequency resources of on-demand SSB.

For UEs knowing the time and frequency resources of on-demand SSB (e.g., the UEs receiving the RRC/MAC CE for configuration/activation/adaptation), the same procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) based on always-on SSB can be reused for on-demand SSB, wherein the time and frequency resources of on-demand SSB replaces the ones of always-on SSB in the procedures. It is noted that for Case 2 (i.e., with always-on SSB), the procedures for resource allocation and validation may need to consider both on-demand SSB and always-on SSB. For instance, the rate matching of a unicast PDSCH may need to be around both on-demand SSB and always-on SSB in the same slot, when on-demand SSB and always-on SSB have different center frequencies, wherein the corresponding separate UE capability can be considered.

For UEs not knowing the time and frequency resources of on-demand SSB (e.g., the UEs not receiving the RRC/MAC CE for configuration/activation/adaptation), gNB can handle the procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) taking care of the on-demand SSB transmission by implementation. For instance, the following can be considered by the gNB in implementation:

- For the slot with on-demand SSB transmission, do not transmit other signal or channel;

- For the RBs with on-demand SSB transmission, do not transmit other signal or channel;

- Use RateMatchPattern to cover the time and frequency resources of the on-demand SSB.

In particular, when RateMatchPattern is used to cover the time and frequency resources of the on-demand SSB, it is noted that the existing RateMatchPattern can only cover at most 2 slots in one pattern, and the associated periodicity for applying the pattern cannot exceed 40 ms. Such limitation may not cover all the existing SSB pattern, e.g., the SSB pattern may occupy 4 slot when the SCS is 240 kHz, and the periodicity of SSB can be up to 160 ms. In this sense, RAN1 may consider enhancement to the rate matching pattern to cover the on-demand SSB transmission.

Proposal 8:

- For UEs knowing the time and frequency resources of on-demand SSB, the same procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) based on always-on SSB can be reused for on-demand SSB.

- For UEs not knowing the time and frequency resources of on-demand SSB, it is up to gNB implementation to handle resource allocation and validation based on always-on SSB.

  - RateMatchPattern needs enhancement to cover the time and frequency resources of on-demand SSB.

# Uplink power control based on on-demand SSB

For the SCell configured with on-demand SSB, the on-demand SSB should be able to be utilized as the RS for uplink power control (e.g., for calculating downlink pathloss estimate), including PUSCH, PUCCH, SRS, and PRACH. For Case 1 (i.e., without always-on SSB), the type of SSB for uplink power control should be on-demand SSB only; and for Case 2 (i.e., with always-on SSB), a similar approach for determining the type(s) of SSB for L1 measurement can be adopted.

Proposal 9: Support uplink power control based on on-demand SSB, and similar approach to L1 measurement based on on-demand SSB can be used.

# Conclusion

The proposals from this contribution are summarized below:

Proposal 1: For on-demand SSB:

- Support a list of configurations by RRC, wherein each configuration includes the following parameters:

  - od-ssb-Periodicity;

  - od-ssb-sfn-Offset;

  - od-ssb-halfFrameIndex;

  - od-ssb-PositionsInBurst;

  - od-ssb-nrofBurst.

- Note: some parameter(s) can be optional subject to RAN1 agreements.

- The maximum number of configurations in the list is 16.

Proposal 2: When on-demand SSB transmission is indicated to be activated by the RRC, the UE applies the parameters associated with the first configuration within the list of configurations included the RRC for the on-demand SSB transmission.

- Need to specify the timeline at least when the number of transmitted on-demand SSB bursts is provided.

Proposal 3: For adaptation of parameter(s) for on-demand SSB, support all the following cases:

- Case A1: activation by a RRC and N is not provided + adaptation by a MAC CE;

- Case A2: activation by a RRC and N is provided + adaptation by a MAC CE;

- Case B1: activation by a MAC CE and N is not provided + adaptation by a MAC CE;

- Case B2: activation by a MAC CE and N is provided + adaptation by a MAC CE.

Proposal 4: For deactivation for on-demand SSB, support all the following cases:

- Case X1: activation by a RRC and N is not provided + deactivation by a MAC CE;

- Case X2: activation by a RRC and N is provided + deactivation after N transmission bursts;

- Case X3: activation by a RRC and N is provided + deactivation by a MAC CE;

- Case Y1: activation by a MAC CE and N is not provided + deactivation by another MAC CE;

- Case Y2: activation by a MAC CE and N is provided + deactivation after N transmission bursts;

- Case Y3: activation by a MAC CE and N is provided + deactivation by another MAC CE;

- Case Z1: adaptation by a MAC CE and N is not provided + deactivation by another MAC CE;

- Case Z2: adaptation by a MAC CE and N is provided + deactivation after N transmission bursts;

- Case Z3: adaptation by a MAC CE and N is provided + deactivation by another MAC CE.

Proposal 5: For the case when the center frequency locations of always-on SSB and on-demand SSB are different:

- On-demand SSB is a NCD-SSB;

- Transmitted SSB indexes for on-demand SSB are same as or a subset of the ones for always-on SSB.

Proposal 6: For Case 2 (i.e., with always-on SSB), only when always-on SSB and on-demand SSB have different center frequencies, a CSI report configuration is associated with on-demand SSB only.

Proposal 7: For L1 measurement based on on-demand SSB:

- Before on-demand SSB is indicated to be transmitted, the UE is not required to perform L1 measurement or CSI reporting even though it receives the configuration on L1 measurement and reporting;

- When on-demand SSB is indicated to be transmitted in Scenario #2, the UE can be required to perform CSI reporting in Scenario #2.

Proposal 8:

- For UEs knowing the time and frequency resources of on-demand SSB, the same procedures for resource allocation (e.g., PDCCH, and PDSCH) and validation (e.g., PRACH, and PUSCH) based on always-on SSB can be reused for on-demand SSB.

- For UEs not knowing the time and frequency resources of on-demand SSB, it is up to gNB implementation to handle resource allocation and validation based on always-on SSB.

  - RateMatchPattern needs enhancement to cover the time and frequency resources of on-demand SSB.

Proposal 9: Support uplink power control based on on-demand SSB, and similar approach to L1 measurement based on on-demand SSB can be used.

# Reference

[1] 3GPP RAN1#120bis chairman notes, April, 2025.

# Appendix

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}

  - N2= [8]

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement

Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

- Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

  - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

    - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - (Working assumption) T=T_min

  - RAN4 to confirm the above two working assumptions

  - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

LS to RAN4 on time instance B for on-demand SSB SCell operation is agreed. Final LS in R1-2503108.

Agreement

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

- od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap.

- od-ssb-PositionsInBurst can be absent for Case #2

  - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- A CSI report configuration can be associated with

  - both of AO-SSB and OD-SSB

    - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met

    - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst

    - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

  - OD-SSB only

    - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies