3GPP TSG RAN WG1 #121	R1-2503571

St Julian's, Malta, 19 - 23 May, 2025

Agenda item:	9.5.2

Source:	Samsung

Title:	On-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

- Introduction

To reduce energy consumption of common signal and channel for UEs in idle/inactive mode, in the WID of Rel-19 network energy saving [1], the following objective has been specified:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In this contribution, we discuss details on supporting on-demand SIB1 operation for UEs in idle/inactive mode.

- Value of kSSB while broadcasting on-demand SIB1 at NES Cell

In the previous RAN2 meeting [3], the following agreement has been made to support that legacy UE camps on the cell if the UE acquires the SIB1.

Agreement When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

As stated in above agreement, it is straightforward to allow for other UEs (including legacy UEs without supporting Rel-19 NES features) to receive SIB1 and camp on the cell, since SIB1 is cell-specific channel. Therefore, when gNB starts to periodically broadcast SIB1 after receiving the UL WUS, gNB also will broadcast SSB where kSSB <= 23 for FR1 or kSSB <= 12 for FR2 to allow for legacy UEs to monitor PDCCH of SIB1. Otherwise, gNB will broadcast SSB where kSSB >= 24 for FR1 or kSSB >=13 for FR2. To further clarify it is feasible without any RAN1 spec. impact as well, we propose to make a conclusion in RAN1 that it’s up to gNB implementation to set the kSSB value to be <24 in FR1 or <12 in FR2, such that legacy UE can camp on the cell when on-demand SIB1 is broadcasted.

Proposal 1: Make a conclusion in RAN1 that it’s up to gNB implementation to set the kSSB value to be <24 in FR1 or <12 in FR2, such that legacy UE can camp on the cell when on-demand SIB1 is broadcasted.

- Value range of od-sib1-duration

In the previous meeting [2], the following agreement has been made to determine the value range of duration for monitoring on-demand SIB1.

Agreement RAN1 to discuss the value range for od-sib1-duration. For example, - Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point. - Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity). - Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported) - Other options are not precluded.

In terms of Option 3, the on-demand SIB1 repetition periodicity remains unclear, as the current specification does not explicitly define a configuration for it. Additionally, on-demand SIB1 repetition depends on gNB implementation. Therefore, we have concerns about Option 3, as it requires additional bits in the WUS configuration for both on-demand SIB1 repetition periodicity and od-sib1-duration. For Option 1 and 2, both are feasible. However, since on-demand SIB1 is intended for both Rel-19 NES-capable UEs and legacy UEs, it would be preferable to use a multiple of the legacy SIB1 periodicity for od-sib1-duration. Furthermore, the minimum value of od-sib1-duration should be 160 ms.

Proposal 2: Support Option 2 (The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity)) for both Rel-19 NES capable UE and legacy UE

- Minimum value of od-sib1-duration is 160 ms.

- Additional information related to SSBs associated with PDCCH monitoring occasions

In previous meeting, the following agreement has been made to indicate whether PDCCH monitoring occasions corresponds only to the SSB associated with UL-WUS or each transmitted SSB.

Agreement The following agreement is updated as follows: The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. - Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions - Alt1: The information is provided in UL-WUS config - Alt2: The information is provided in RAR in response to UL-WUS transmission - Alt3: No additional information is provided

The remaining issue is whether/how to indicate additional information related to SSBs associated with PDCCH monitoring occasions. For Alt 2, it may conflict with a previous RAN2 agreement, which states that the RAR in response to UL-WUS transmission aligns with legacy RAR for OSI. Regarding Alt 1, providing additional information can be beneficial for UE to monitor the subset of PDCCH monitoring occasions corresponding to SSB associated with UL WUS. However, while the benefits might be marginal, it will bring additional overhead in WUS configuration. Therefore, we support Alt 3 since further optimization is not needed.

Proposal 3: Support Alt 3 (No additional information is provided)

- Conclusion

In this contribution, we discussed details on supporting on-demand SIB1 operation for UEs in idle/inactive mode. The following proposals were made:

Proposal 1: Make a conclusion in RAN1 that it’s up to gNB implementation to set the kSSB value to be <24 in FR1 or <12 in FR2, such that legacy UE can camp on the cell when on-demand SIB1 is broadcasted.

Proposal 2: Support Option 2 (The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity)) for both Rel-19 NES capable UE and legacy UE

- Minimum value of od-sib1-duration is 160 ms.

Proposal 3: Support Alt 3 (No additional information is provided)

- References:

- RP-242354, “Revised WID: Enhancements of network energy savings for NR”

- Chairman’s notes, RAN1#120bis, April 2025.

- Chairman’s notes, RAN2#129bis, April 2025.