3GPP TSG RAN WG1 #121   		R1-2503574

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:    9.6.2

Source:	Samsung

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Document for:	Discussion and decision

- Introduction

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

In the approved new WID for low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1], one important direction is to specify solutions for LP-WUS operation in IDLE/INACTIVE modes.

This contribution discusses aspects related to LP-WUS operation in IDLE/INACTIVE modes.

- LP-WUS monitoring configuration

In this section, we will further discuss the configuration for MO and the configuration for the wake-up delay.

- 2.1 MO configuration

Regarding the available symbol(s) for LP-WUS in each MO, the following agreement is agreed in the last meeting:

Agreement UE determines whether a symbol is available for LP-WUS based on: - Alt 1: Time-domain pattern configured by the gNB - Alt 1A: Periodic time-domain pattern - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern - Alt 1B: Per-MO pattern, applicable for all MOs - Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc]. - Alt 3: Combination of Alt 1 and Alt 2 - Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s).

Based on the discussion in the last meeting, we assume that companies share a same view that LP-WUS is with low priority than broadcast signals/channels like SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc. Therefore, a UE can determine which symbols are available for LP-WUS transmission based on the configuration of other broadcast signals/channels in our understanding. Since the parameters for these broadcast signals/channels are already configured by SIB1, we do not see any need to indicate it again by a duplicated indication as mentioned in Alt 1. If UE rely on the time-domain pattern configured in SIB to determine the available symbol, the duration of the pattern should at least equal to the maximum period of all the configured signals/channels (e.g., 2560 slots), the signalling overhead will be very large.

In addition, another argument to support Alt 1 is that LP-WUS should be avoid to overlap with some unicast signals/channels configured for other UE. Since UE cannot know the configuration for these unicast signals/channels, it should be indicated by gNB. In this case, the key issue needs to be discussed is that whether LP-WUS is low priority than other unicast signals/channels or not. Based on our understanding, follow the legacy logic, the priority for LP-WUS should be higher than other unicast signals/channels. That is to say, when the resources for LP-WUS overlap with other unicast signals/channels, the resources should be used for LP-WUS transmission. Besides, we also think this case can be avoid by gNB scheduling and a reasonable MO configuration, which means Alt 1 is not needed in this case either.

Therefore, we support Alt 2 to determine the available symbol for LP-WUS in each MO.

Proposal 1: Support Alt 2 to determine the available symbol(s) for LP-WUS in each MO.

Referring to the available symbol(s), one issue needs to be clarified is that if the symbols for LP-WUS are not overlapping with other signals/channels in the frequency domain, whether these symbols are available for LP-WUS or not. Based on our understanding, if the RBs for LP-WUS including the guard RBs are overlapping with other signals/channels in the frequency domain, the symbols are not available for LP-WUS. Otherwise, the symbols can be determined as available for LP-WUS according to Alt 2 mentioned in the agreement above.

Proposal 2: If the RBs for LP-WUS including the guard RBs are overlapping with other signals/channels in frequency domain, the symbols are not available for LP-WUS. Otherwise, the symbols can be determined as available symbols for LP-WUS according to the information configured in SIB1.

Regarding the starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, the following agreement is made in the last meeting:

Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO. - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration. - FFS restriction on MO locations, e.g. only on DL slots. - FFS minimum gap is needed between two MOs to ensure LR processing time.

Since the available symbol(s) for LP-WUS can be determined by UE according to the configuration information in SIB1, a slot-level offset can be used to indicate the starting time location of the first LP-WUS MO in a LO. The specific symbol(s) for LP-WUS transmission in the indicated slot can be decided by UE based on the information from existing configurations.

Referring to the starting time locations of the subsequent LP-WUS MOs in a LO, like the determination method for the starting slot for one or more search space sets for PEI in one specific periodicity, one common offset (i.e., Alt 2A) can be configured between 2 consecutive MOs to ensure sufficient LO processing time and facilitate flexible gNB scheduling. For Alt 2B, the details for the configuration method are still not that clear, if we completely reuse the configuration for search space sets for PEI, a period and a slot-level offset should be introduced for MO configuration. Since only a slot-level offset is configured to determine the offset between the starting position of period and the starting position of MO, the offset between 2 consecutive MOs is still fixed. Considering the signaling overhead, Alt 2A is simpler to determine the starting time locations of the subsequent LP-WUS MOs in a LO.

Proposal 3: Slot-level offset can be used to determine the starting time location of the first LP-WUS MO since the available symbol(s) for LP-WUS can be determined by UE according to the information configured in SIB1. For the starting time locations of the subsequent LP-WUS MOs, Alt 2A can be supported considering the signaling overhead.

For the definition for nominal MO duration and actual LP-WUS duration, if defined, 4 alternatives have been identified in the last meeting, which can be shown as following.

Agreement Terminology definition - Nominal MO duration: this includes both the available and unavailable symbols - Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives: - Alt A: Nominal MO duration is configured. (e.g. in unit of slots) - Actual LP-WUS duration is the number of available symbols within the MO. - Actual LP-WUS duration can vary from one MO to another MO. - Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values) - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration. - FFS: Additional termination condition such as time window. - Alt C: Both nominal MO duration and actual LP-WUS duration are configured. - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred. - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors. - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs. - Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

For Alt A, since the actual LP-WUS duration can vary from one MO to another MO, as the number of unavailable resources in one MO increases, the coverage will be reduced accordingly, and the miss-detection rate for paging increases due to incorrect LP-WUS decoding. Considering the impact to the LP-WUS reception, we do not favor this alternative.

For Alt D, considering the duration for one MO can be as large as a few slots, a common MO pattern cannot cover the maximum period of all the configured signals/channels in SIB. So, it may not always be feasible in our understanding.

In fact, both Alt B and Alt C can work. However, the resource overhead for Alt C is larger than Alt B when the actual LP-WUS duration is less than the nominal MO duration. In addition, when the LP-WUS is dropped due to invalid MO in Alt C, the paging latency will be increased if the LP-WUS is transmitted in the next paging cycle, or the miss detection rate for paging will be increased if UE is not expected to monitor PO when there is no valid MO for the beam(s) that the UE monitors since the NW cannot determine whether LP-WUS is received or not. If the UE fallbacks to monitoring PO due to the invalid MO in case that there is no valid MO for the beam that the UE monitors, but the dropped LP-WUS does not indicate a UE subgroup that the UE belongs to, the power consumption will be increased. Besides, if the LP-WUS is deferred due to the invalid MO, the difference between Alt B and Alt C is that symbol-level postpone is used in Alt B while MO-level/slot-level postpone is used in Alt C. Considering the resource overhead issues in Alt C, we think Alt C is some kinds of over-design and support Alt B to determine the actual LP-WUS duration. For the additional termination condition in Alt B, we assume it can be solved by gNB implementation.

Proposal 4: Support Alt B to determine the actual LP-WUS duration.

- 2.2 Assumption for wake-up delay reporting

In RAN1 #120bis, the following agreement was made to determine the wake-up delay, which can be shown as following.

Agreement On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121. - Alt 1: Do not report for SSB periodicities other than 20ms. - Note: LP-WUS is not supported for SSB periodicities larger than 20ms. - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns): SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. [5]; [x]; [x]; [x]. [10]; [x]; [x]; [x]. 20; [70]; [500]; [900]. [40]; [x]; [x]; [x]. [80]; [x]; [x]; [x]. [160]; [x]; [x]; [x]. - Alt 3: It is up to RAN4.

In the last meeting, RAN4 has also discussed the number of SSBs for different SSB periodicities based on MR wake up delay related requirements for LP-WUR, but no consensus was achieved. Since RAN1 cannot capture any requirement and/or side condition in RAN1’s spec, we think RAN4 can handle the wake-up delay related issues, and we should avoid a duplicated discussion in RAN1.

Proposal 5: How to handle the UE capability report on the wake-up delay for SSB periodicity should be determined by RAN4 (Alt 3).

- The relationship between LP-WUS/LP-SS and SSB

In RAN1 #120, EPRE ratio between LP-WUS/LP-SS and SSB was agreed to configured by gNB, which can be shown as following.

Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

Then, in the last meeting, it has been agreed that the M values for LP-WUS and LP-SS can be configured to be same or different, which is:

Agreement For the M value for LP-WUS and LP-SS, supports: - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - At least for M values larger than 1 (applies to both LP-WUS and LP-SS). - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both.

One remaining issue is that whether there is a common EPRE ratio configuration or separate EPRE ratio configuration for LP-WUS and LP-SS. As illustrated in our companion paper [2], it may be beneficial to have a consistent average ON power between LP-WUS and LP-SS taking into account the low power consumed receiver. However, if a common EPRE is configured and LP-WUS uses M=1, there is no way to support the consistent average ON power other than forcing the M values of both LP-WUS and LP-SS to 1. Another approach to guarantee the consistent ON power while allowing the gNB the flexibility to configure the M value of LP-SS for the better synchronization with the reduced overhead is to support a separate EPRE ratio configuration as shown in Figure 1 in [2]. Therefore, we support a separate EPRE ratio configuration for LP-WUS and LP-SS.

Proposal 6: A separate EPRE ratio can be configured for LP-WUS and LP-SS.

- Conclusion

This contribution discusses the solutions for LP-WUS operation in RRC_IDLE/RRC_INACTIVE modes. Proposals are summarized as follows:

Proposal 1: Support Alt 2 to determine the available symbol(s) for LP-WUS in each MO.

Proposal 2: If the RBs for LP-WUS including the guard RBsare overlapping with other signals/channels in frequency domain, the symbols are not available for LP-WUS. Otherwise, the symbols can be determined as available symbols for LP-WUS according to the information configured in SIB1.

Proposal 3: Slot-level offset can be used to determine the starting time location of the first LP-WUS MO since the available symbol(s) for LP-WUS can be determined by UE according to the information configured in SIB1. For the starting time locations of the subsequent LP-WUS MOs, Alt 2A can be supported considering the signaling overhead.

Proposal 4: Support Alt B to determine the actual LP-WUS duration.

Proposal 5: How to handle the UE capability report on the wake-up delay for SSB periodicity should be determined by RAN4 (Alt 3).

Proposal 6: A separate EPRE ratio can be configured for LP-WUS and LP-SS.

References

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC, RAN#102, December 2023.

- R1-2503573, Discussion on LP-WUS and LP-SS design, Samsung, RAN1#121.

- R1-2503575, Discussion on LP-WUS operation in CONNECTED modes, Samsung, RAN1#121.