3GPP TSG-RAN WG1 Meeting #121		R1-2503578

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.8.1

Title:	Discussion on channel model validation of TR38.901 for 7-24 GHz

Source:	Samsung

Document for:	Discussion and Decision

- Introduction

The Rel-19 study items on “Study on channel modelling enhancements for 7 – 24 GHz for NR” is endorsed in RAN#102 [1]. The objectives for this SI are shown below:

The objectives of this study are: - Validate using measurements the channel model of TR38.901 at least for 7-24 GHz - Note: Only stochastic channel model is considered for the validation. - Note: The validation may consider all existing scenarios: UMi-street canyon, UMa, Indoor-Office, RMa and Indoor-Factory. - Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios: - Near-field propagation (with consideration being given to consistency between near-field and far-field) - Spatial non-stationarity Note 1: Continuity of the channel model in the frequency domain below 7 GHz and above 24 GHz shall be ensured. Note 2: Mathematical and/or theoretical aspects (if any) may be studied before results of measurement campaigns are available. While measurement results may be available and submitted at any time, the study of measurement results may start later (e.g., Q3 2024).

In this contribution, we discuss channel mode validation of TR 38.901 [2] related aspects for Rel-19 channel model SI.

- Remaining consideration

2.1 Pathloss on point beyond breakpoint

One point we would like to propose to discuss is the LOS pathloss equation. Taking the UMa pathloss equation as an example, there are two equations for before and after the breakpoint in LOS pathloss. Upon plotting the pathloss across distance per frequency bands utilizing these equations, it was observed that beyond a particular distance (i.e., breakpoint), the values tend to converge. As shown in Figure 6, as distance greater than about 1 km, the pathloss for the 3.5 GHz band converges with other band’s pathlosses.

Certainly, consideration of the LOS probability is crucial while using the current channel model in system level simulation, and this phenomenon might be regarded as insignificant beyond 1 km. However, when analyzing link budget or pure coverage, the pathloss equation can be used. For example, coverage can be analyzed under only LOS conditions. In this case, considering only the LOS condition, accurate analysis becomes difficult beyond a specific point due to the aforementioned problem. Therefore, we would like to suggest a clarification discussion on how RAN1 addresses such situations.

Figure 6. LOS UMa pathloss considering breakpoint

Upon plotting the pathloss across distance per frequency bands utilizing these equations, it was observed that beyond a particular distance (i.e., breakpoint), the values tend to converge

Agreement To potentially resolve the UMi/UMa pathloss convergence beyond breakpoint distance, RAN1 to further discuss following options: - Option 1) Set hE to 0 m - Option 2) Applying only PL1 when performing link analysis - Option 3) add note that provide information on pathloss convergence phenomena beyond the breakpoint distance across different frequencies. Other options are not precluded.

Based on previous agreement, we look into why this phenomenon occurs. Taking the pathloss formula in UMa as an example below:

- PL1=28.0+22log10(d3D)+20log10(fc)

- PL2=28.0+40log10(d3D)+20log10(fc)-9log10(dBP'2+hBS-hUT2)

- dBP'=4*hBS-hE*hUT-hE*fcc

In the case of a UE located at ground level, hE(effective environment height) is set to 1 m. As a result, the breakpoint distance becomes a dominant parameter for frequency rather than the height of the UE. In other words, as the frequency increases, dBP' also increase, and at the same time, the correction term for -9log10(dBP'2+hBS-hUT2) increases, so the absolute value change of PL2 decreases.

The best way to solve this convergence phenomenon would be define a new formula. However, this approach requires measurement and proof, so it takes a lot of efforts. Therefore, one approach to consider is to use different formula depending on the situation. Such pathloss comparison with long distance will mainly be used for link budget or coverage analysis over frequency band rather than system-level simulation.

Generally, link budget and coverage analysis assume an ideal channel environment such as UE located ground level with LOS condition. Therefore, one alternative is to set the hE (effective environment height) to 0 m. The meaning of hE being 0 m is to assume reflection at a completely flat ground level. So, Figure 7 (a) compares the cases where hE is 1 m and 0 m according to frequency band, and it can be confirmed that the convergence phenomenon is alleviated as the breakpoint distance increases when hE is set to 0 m.

Another alternative is to consider using only PL1, which does not take into account the breakpoint, in some cases such as link budget or coverage analysis as shown in Figure 7 (b).

| (a) | (b) |
| --- | --- |
| Figure 7. Alternatives for updates of applying pathloss formula |  |

In the case of a UE located at ground level, hE(effective environment height) is set to 1 m. As a result, the breakpoint distance becomes a dominant parameter for frequency rather than the height of the UE. In other words, as the frequency increases, dBP' also increase, and at the same time, the correction term for -9log10(dBP'2+hBS-hUT2) in UMa increases, so the absolute value change of PL2 decreases

Pathloss comparison with long distance will mainly be used for link budget calculation or coverage analysis over frequency band rather than system-level simulation.

One alternative is to set the hE (effective environment height) to 0 m. The meaning of hE being 0 m is to assume reflection at a completely flat ground level and another alternative is to consider using only PL1, which does not take into account the breakpoint

During last RAN1 meeting, 3 options are discussed to address this issue. Although options 1 can address these issues, they have the drawback of requiring separate formula validation. Therefore, as mentioned in option 2 and 3, it may be the right approach to clearly state these limitations and leave the decision to the reader.

RAN1 decide the way among option 2 or option 3 to address pathloss convergence issue after breakpoint

2.2 Scaling factor of ZOA and ZOD for # of clusters in SMa O2I condition

In the existing TR38.901, as outlined in Step 7 of Clause 7.5, the azimuth angle and zenith angle are scaled according to Table 7.5-2 and Table 7.5-4, respectively, under NLOS conditions, where the scaling factor depends on the number of clusters. The currently defined tables align the number of clusters and scaling factor values for NLOS and O2I (Outdoor-to-Indoor) conditions in scenarios such as UMa, UMi, and RMa. However, in Rel-19, it has been agreed that the number of clusters for O2I conditions in the SMa scenario is 14, and the zenith angle scaling factor is not supported. Therefore, RAN1 needs to either define separate values for the SMa scenario or reconsider the number of SMa O2I clusters.

The currently defined tables align the number of clusters and scaling factor values for NLOS and O2I (Outdoor-to-Indoor) conditions in scenarios such as UMa, UMi, and RMa. However, in Rel-19, it has been agreed that the number of clusters for O2I conditions in the SMa scenario is 14, and the zenith angle scaling factor is not supported

RAN1 discuss to either define separate values for the SMa scenario or reconsider the number of SMa O2I clusters.

- Other channel parameters

3.1 UE antenna modeling

Agreement - Amend the previous agreement (from RAN1 #118bis) on handheld UE antenna placement location modeling. - The four corners and the centres of each edge are identified as potential locations of the UE antenna. Centre of the device is also identified as potential location of the UE antenna. - Antennas (except the centre of device antenna) are assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location. centre of device antenna is assumed to be oriented in either forward facing or backward facing the plane of the device.

Mapping to antenna location candidates depend on the context of the evaluation and preference of readers of the channel model. For instance, someone might want to know the difference in performance between having the FR1 antenna at the edge versus the corner, which could also apply to the FR2 antenna array. Additionally, whether antenna for new frequency band become antenna elements or arrays remains unknown. Hence, instead of suggesting guidelines regarding antenna placement under specific frequency band, it can be allowed to determine how the antenna elements and modules are mapped to antenna location candidates.

Some readers of channel model might want to know the difference in performance between having the FR1 antenna at the edge versus the corner, which could also apply to the FR2 antenna array. Additionally, whether antenna for new frequency band become antenna elements or arrays remains unknown.

RAN1 do not consider the detailed guidelines for how antenna elements and antenna modules are mapped to antenna location candidates

Working Assumption - Reference orientation of the handheld UE as follows: - FFS how to capture the reference orientation of the handheld UE - For calibration of handheld UE, use the following UE rotation based on reference UE orientation - For calibration with blockage: - For one-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg, - For dual-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 90 deg, - For hand and head blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg, - For all other calibration cases: - ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg, - Note: UT array orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle). - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 90 deg

- In addition, it is necessary to define the orientation of radiation pattern for each antenna element candidate location. As explained in the agreement, the direction of the radiation pattern of each antenna element can be aligned so that the directionality of the element’s maximum gain is directed towards the direction of the antenna element candidate element location.

- On the other hand, UE antenna modelling in existing channel model sets a bearing angle for the rotation of the UE. It is necessary to confirm that each antenna elements for the UE applies the same bearing angle. Therefore, the order of overall UE antenna modelling may be as follows; UE distribution  assume the corresponding the location as reference for orientation of UE  set the radiation pattern orientation for each antenna element candidate location according to the relative direction of the UE’s center orientation  apply same bearing angle for each antenna element candidate location for rotation.

UE antenna modelling in existing channel model sets a bearing angle for the rotation of the UE. It is necessary to confirm that each antenna elements for the UE applies the same bearing angle

RAN1 confirm that maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location

RAN1 consider that same bearing angle is applied to all antenna element candidates

- As discussed in the previous RAN1 meeting, it is necessary to define a reference orientation to determine the orientation of each antenna element after terminal distribution, considering the antenna radiation pattern. In previous modeling that did not account for the actual antenna placement within the terminal, the impact of the reference orientation might not be visible depending on the implementation. However, for fixed antenna placement within the terminal and subsequent performance evaluation, it is essential that the reference orientation aligns. Therefore, as explained in the Working Assumption, the vector for the terminal's reference orientation can be defined based on the z-axis. The orientation for each antenna element's radiation pattern can then be determined according to this z-axis-based vector.

RAN1 consider that the orientation for each antenna element's radiation pattern can then be determined according to this z-axis-based vector

- Calibration results

- Full calibration has been performed for UMa/UMi at 6 GHz and UMa/UMi/SMa at 7 GHz. The same assumptions were applied for 6 GHz to compare results since the last Rel-14. In addition, channel implementation for UMa/UMi at 7 GHz with updated parameters, new UE antenna modeling and more BS antennas has been performed. Also, channel implementation and calibration for SMa deployment scenario has been performed. Detailed results are summarized in the attached excel sheet.

- Conclusion

- Upon plotting the pathloss across distance per frequency bands utilizing these equations, it was observed that beyond a particular distance (i.e., breakpoint), the values tend to converge

- In the case of a UE located at ground level, hE(effective environment height) is set to 1 m. As a result, the breakpoint distance becomes a dominant parameter for frequency rather than the height of the UE. In other words, as the frequency increases, dBP' also increase, and at the same time, the correction term for -9log10(dBP'2+hBS-hUT2) in UMa increases, so the absolute value change of PL2 decreases

- Pathloss comparison with long distance will mainly be used for link budget calculation or coverage analysis over frequency band rather than system-level simulation.

- One alternative is to set the hE (effective environment height) to 0 m. The meaning of hE being 0 m is to assume reflection at a completely flat ground level and another alternative is to consider using only PL1, which does not take into account the breakpoint

- RAN1 decide the way among option 2 or option 3 to address pathloss convergence issue after breakpoint

- The currently defined tables align the number of clusters and scaling factor values for NLOS and O2I (Outdoor-to-Indoor) conditions in scenarios such as UMa, UMi, and RMa. However, in Rel-19, it has been agreed that the number of clusters for O2I conditions in the SMa scenario is 14, and the zenith angle scaling factor is not supported

- RAN1 discuss to either define separate values for the SMa scenario or reconsider the number of SMa O2I clusters.

- Some readers of channel model might want to know the difference in performance between having the FR1 antenna at the edge versus the corner, which could also apply to the FR2 antenna array. Additionally, whether antenna for new frequency band become antenna elements or arrays remains unknown.

- RAN1 do not consider the detailed guidelines for how antenna elements and antenna modules are mapped to antenna location candidates

UE antenna modelling in existing channel model sets a bearing angle for the rotation of the UE. It is necessary to confirm that each antenna elements for the UE applies the same bearing angle

RAN1 confirm that maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location

RAN1 consider that same bearing angle is applied to all antenna element candidates

RAN1 consider that the orientation for each antenna element's radiation pattern can then be determined according to this z-axis-based vector

Appendix. Simulation assumptions for calibration

UMa/UMi at 6 GHz

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-street Canyon |
| Carrier Frequency | 6 GHz |
| Bandwidth | 20MHz for 6GHz |
| BS Tx power | 44 dBm for UMi-Street Canyon, 49 for UMa at 6GHz |
| BS antenna configurations | Config 1: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ … calibration metrics 1), 2), 3) are calibrated |
| BS port mapping | Config 1: all 16 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations |
| UT antenna configurations | Mg = Ng = 1, M = N = 1, P = 2 |
| UT distribution | Following TR36.873 for UMa and UMi, (3D dropping) uniform dropping for indoor with minimum distance (2D) of 0 m |
| UT attachment | Based on RSRP (formula) from CRS port 0 |
| Polarized antenna modelling | Model-2 in TR36.873 |
| UT array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UT antenna pattern | Isotropic |
| Metrics | 1) Coupling loss – serving cell |
|  | 2) Wideband SIR before receiver without noise |
|  | 3) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) from the serving cell (according to circular angle spread definition of TR 25.996) |

UMa/UMi/SMa at 7 GHz

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, SMa |
| Carrier Frequency | 7 GHz |
| BS antenna electrical downtilting | 95 degrees for SMa for ISD = 1299m |
| BS antenna configurations | Config 3 for 7 GHz: Mg = Ng = 1, M = 8, N = 16, P = 2, dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated |
| BS Polarized antenna modelling | Model-2 in Clause 7.3.2 |
| BS port mapping | Config 3 for 7 GHz: Mp = 8, Np = 16, each antenna element is mapped to one port Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Tx power | 49 dBm for SMa |
| Bandwidth | [20] MHz for 7 GHz |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type. |
| UT array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 45 degree, ΩUT, = 0 degree |
| UT antenna configurations | Config B for 7 GHz: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern | Config B for 7 GHz: Based on directional antenna for UT described in Clause 7.3 |
| UT Polarized antenna modelling | Config B for 7 GHz: Based on directional antenna for UT described in Clause 7.3 |
| O2I penetration loss | For SMa, low-loss A model |
| SCS assumption | 30 kHz |

References

RP-234018 “New SID: Study on channel modelling enhancements for 7-24GHz for NR,” RAN#102

TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”

R1-2402480 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2404129 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2406666 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2405659 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2409610 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2500862 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2502380 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

Report ITU-R M.2135-1, Guidelines for evaluation of radio interface technologies for IMT-Advanced, 2009.