3GPP TSG RAN WG1 #121    		                           R1-2503586

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.11.5

Source:			Samsung

Title:			Discussion on IoT-NTN TDD mode

Document for:		Discussion and decision

- Introduction

In the RAN1#120bis [1], it was discussed on remaining issues for IoT-NTN TDD mode, and following agreements were achieved. This contribution discusses on remaining details.

Agreement Confirm the following working assumption: - Uplink transmission gaps do not apply in NB-IoT NTN TDD. Agreement NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU. - NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be 50ms at the ULSRP. - NOTE: This roughly corresponds to the largest separation between DL and UL Iridium slot pairs. - Note: Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA. Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode, down-select among the following options: - Option 1: DL gaps do not apply for NB-IoT NTN TDD mode. - Option 2: DL gaps apply based on current specifications. - Option 3: DL gaps apply, where new periodicity(ies) are introduced (either implicity e.g. the periodicity is scaled by a fixed factor F, or explicitly) Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc. Working assumption DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is not present in a NB-IoT NTN TDD cell. - NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes. - NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact. Agreement For GNSS measurement gap, RAN1 to further discuss at least the following options: - Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap - Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe - Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD Working assumption For precompensation, from RAN1 perspective: - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed. - FFS: whether spec impact is in RAN1, RAN4 or both Inform RAN4 of the WA above and ask if there are any issues with the above WA.

- Discussion

DL gap handling

Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode, down-select among the following options: - Option 1: DL gaps do not apply for NB-IoT NTN TDD mode. - Option 2: DL gaps apply based on current specifications. - Option 3: DL gaps apply, where new periodicity(ies) are introduced (either implicity e.g. the periodicity is scaled by a fixed factor F, or explicitly)

DL transmission gap allows the UE to pause intentionally between downlink transmissions for certain cases. In RAN1#120bis [1], the discussion on DL gap in NB-IoT NTN TN TDD mode is ongoing, with differing opinions on its necessity and implementation. FL suggested not applying DL gap in this mode, while alternative approaches propose configuring DL transmission gaps using a new parameter with a scaling factor to align with TDD patterns. Option 3 requires significant changes for the configuration of DL gaps, which is against WID (i.e., Specify a new NB-IoT TDD NTN mode based on minimum necessary changes). In that sense, it is preferable to consider either Option 1 or Option 2.

Proposal 1: Consider either option 1 or option 2 for the issue of handling DL gaps.

Handling NPDCCH postponing

Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc.

There was a discussion on how to handle PDCCH postponing. Some companies believe that new periodicities are not needed because the current configuration can operate without any issues. In contrast, other companies argue that new periodicities are necessary since dropping/postponing may not work for all search space configurations. An unclear point is whether there exists a case where it can create non-overlapping mapping between two different search spaces. If no configuration provides non-overlapping between two search spaces to avoid allowing new changes regarding postponing or overlapping, Option 2 might be considered. Otherwise, it is preferable to consider Option 1, except for the two FFSs.

Proposal 2: For handling NPDCCH postponing, if no configuration provides non-overlapping between two search spaces to avoid allowing new changes regarding postponing or overlapping, Option 2 might be considered. Otherwise, it is preferable to consider Option 1, except for the two FFSs.

GNSS measurement gap

Agreement For GNSS measurement gap, RAN1 to further discuss at least the following options: - Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap - Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe - Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

A proponent argued that the starting time and duration should be non-D and non-U NB-IoT subframes, and the above agreement was captured for further discussion. A GNSS measurement gap has been introduced to let the eNB know when a UE would perform a position fix within GNSS measurement without allowing DL/UL transmissions. Regardless of FDD and TDD modes, it is understood that the existing GNSS measurement gap can be applied to IoT NTN TDD mode, as NB-IoT UE cannot transmit/receive simultaneously even in FDD mode, and it does not require additional restrictions such as Option 2.

Proposal 3: No enhancement is needed for GNSS measurement gap in IoT NTN TDD mode.

Conclusion

This contribution discusses remaining issues for IoT NTN TDD mode, and the followings are proposals.

Proposal 1: Consider either option 1 or option 2 for the issue of handling DL gaps.

Proposal 2: For handling NPDCCH postponing, if no configuration provides non-overlapping between two search spaces to avoid allowing new changes regarding postponing or overlapping, Option 2 might be considered. Otherwise, it is preferable to consider Option 1, except for the two FFSs.

Proposal 3: No enhancement is needed for GNSS measurement gap in IoT NTN TDD mode.

Reference

[1] RAN1#120bis Chair note