3GPP TSG RAN WG1 #121			R1-2503590

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.14

Source: 	Samsung

Title: 	Views on Rel-19 TEI

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s view on Rel-19 TEI issues.

Discussion

  - Scheduling request (SR) transmission occasion switching upon TCI update

In RAN1#120bis [1], this issue was discussed and the following is the latest version captured in the final FL summary.

Proposal: On UL-spatial relation and unified TCI framework, support that one spatial relation and joint/UL TCI state can be associated with a time-domain offset (in slot level [and symbol level]), respectively. - If a spatial relation or joint/UL TCI state is applied to a PUCCH resource used for an SR transmission, then the associated time-domain offset is applied for deriving the corresponding SR transmission occasion. - Introduce a new RRC parameter of the time-domain offset (in slot level [and symbol level]) in a joint/UL TCI state and corresponding UE capability signaling. - For unified TCI framework, introduce a new UE capability signaling for the minimum beam application time for DCI-based TCI state update, and beamAppTime is configured based on the new UE capability signaling if any joint/UL TCI state is associated with a time-domain offset [Besides for time-domain offset, the spatial relation and joint/UL TCI state can be additionally associated with frequency resource configuration (i.e., startingPRB) and code-resource configuration (i.e., initialCyclicShift or timeDomainOCC).] Note: There is NO RAN2 MAC impact from above feature.

While we understand the motivation of the issue, overall, we believe that the solution could be general on semi-static UL transmission. Especially, periodic PUCCH for CSI report is important even for FR2, in order to maintain stable and basic performance via periodically acquired channel information from gNB side, to do link adaptation properly.

Even though it only focuses on SR (scheduling request), we have several comments on the current proposal.

    - Considering multiple SR configurations:

- There could be multiple SR configurations to the UE, but seems that current proposal doesn’t reflect the point but only indicates a single time domain offset value.

- If a UE is configured with multiple SR configurations, then it is most probably based on different periodicity and slot offset.

- Then, if a single time domain offset is indicated, the value range of indication is bounded with the smaller periodicity.

- Hence, time domain offset value shall be indicated separately per SR configuration, or a single time domain offset value is applied to only a single SR configuration. Considering possibly different configuration on periodicity and slot offset, applying a single time domain offset to all SR configurations doesn’t make sense.

    - Considering symbol level time domain offset (Possible SR configuration in terms of periodicity)

- For SR configuration, periodicity can be configured with a single value which is of 1) smaller than slots (symbol level), 2) equal to slot, 3) more than 1 slots.

- The current proposal only addresses the case of 3). Now we can only adjust time domain offset, we think that it is important to address cases 1) and 2) as well, which means that symbol level adjustment is also needed.

    - Considering additional MAC-CE delay (similar with additional delay on unified TCI framework)

- The current proposal introduces a new value for minimum beam application time by a new UE capability.

- If additional processing time is needed to apply such time domain offset, regardless of unified TCI state framework or UL spatial relation info, similar timing relaxation is needed.

- Hence, additional delay for MAC-CE is also needed. This is because, MAC-CE delay, in terms of the number of OFDM symbols, can be 168 and 336 in 60kHz, 120kHz SCS.

- Since there are some larger candidate values for minimum beam application time reported by UE (one of {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}) rather than MAC-CE delay, we believe that additional delay for MAC-CE is needed at least for 60kHz SCS.

Proposal 1. For TEI proposal on Scheduling request (SR) transmission occasion switching upon TCI update, it would be better to consider the following aspects.

- Considering multiple SR configurations

- Considering symbol level time domain offset

- Considering additional MAC-CE delay

  - Counting of the same active CSI-RS resources

In RAN1#120bis, the following agreement has been made [2].

Agreement For simultaneous CSI-RS reception in UE features 2-33, 2-36, 2-40, 2-41, 2-43, 16-3a and 16-3b, define 1 new UE capability: - UE capability 1: To allow the UE to indicate that CSI-RS ports within one periodic/semi-persistent CSI-RS resource, as well as the periodic/semi-persistent CSI-RS resource, are counted as one resource, even if the periodic/semi-persistent CSI-RS resource is referred by N Report Settings.

Remaining issue is that whether we further introduce a new UE capability supporting new active CSI-RS resource/port counting rule associated with NES CSI (i.e., a CSI reporting setting includes one or more sub-configurations). Since we were even negative view on introducing UE capability 1, due to huge principle change on UE side as well as non-negligible impact on gNB scheduler (treating different UEs depending on UE capability). Hence, considering no deployment on NES CSI and unnecessity, we don’t think such UE capability is needed.

Proposal 2. For TEI proposal on Counting of the same active CSI-RS resources, don’t support to introduce a new UE capability supporting new active CSI-RS resource/port counting rule associated with NES CSI.

Additional problem would be, if a new UE capability for NES CSI can be agreed, it is not clear how to count the CSI-RS resource/port if a certain CSI-RS resource is referred by N CSI report setting without sub-configuration and referred by M CSI report setting with sub-configuration.

Proposal 3. For TEI proposal on Counting of the same active CSI-RS resources, if a new UE capability for NES CSI can be agreed, it would be better to clarify how to count the CSI-RS resource/port if a certain CSI-RS resource is referred by N CSI report setting without sub-configuration and referred by M CSI report setting with sub-configuration.

Conclusion

In this contribution, the following proposal is given:

Proposal 1. For TEI proposal on Scheduling request (SR) transmission occasion switching upon TCI update, it would be better to consider the following aspects.

- Considering multiple SR configurations

- Considering symbol level time domain offset

- Considering additional MAC-CE delay

Proposal 2. For TEI proposal on Counting of the same active CSI-RS resources, don’t support to introduce a new UE capability supporting new active CSI-RS resource/port counting rule associated with NES CSI.

Proposal 3. For TEI proposal on Counting of the same active CSI-RS resources, if a new UE capability for NES CSI can be agreed, it would be better to clarify how to count the CSI-RS resource/port if a certain CSI-RS resource is referred by N CSI report setting without sub-configuration and referred by M CSI report setting with sub-configuration.

References

- R1-2503095, FL Summary #3 on Rel-19 TEIs, Moderator (NTT DOCOMO, INC.), RAN1#120bis.

- Chair’s note, RAN1#120bis.