3GPP TSG RAN WG1 #121					           	                                                         R1-2503591

St Julian’s, Malta, May 19th –23rd, 2025

Agenda item:	9.15.1

Source:	Samsung

Title:	UE features for AI/ML for NR Air Interface

Document for:	Discussion and Decision

- Introduction

In this contribution, our views on capabilities for AI/ML for NR Air Interface are discussed.

- Discussion

2.1 AI/ML based beam management

For AI/ML based beam management, the following agreements are made [1].

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agree 58. NR_AIML_air; 58-1-1; Increased number of reported RSs for beam management [for NW-sided model inference]; 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 FFS: whether/how to have separate FG for some of above components or how to merge components; FFS; yes; n/a; Increased number of reported beams for beam management [for NW-sided model inference] is not supported; FFS; FFS; FFS; FFS; Component 3 candidate values: {FFS}; Optional with capability signalling. Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed) 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] [12. Supported maximum number of predicted beams in each reporting instance] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 [for inference] is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for component 12; Optional with capability signalling. 58. NR_AIML_air; 58-1-3; UE-side beam prediction for BM Case1 with predicted RSRP [for inference]; - Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] [2. Supported maximum number of predicted beams in each reporting instance] FFS: other components; 58-1-2; yes; n/a; UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported; FFS; FFS; FFS; FFS; FFS: candidate values for component 2; Optional with capability signalling. Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed) 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case2] [4. Maximum number of inference report(s) activated for BM-Case2] [5. Maximum number of inference report(s) triggered for BM-Case2] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] [11. Supported maximum number of predicted beams in each predicted time instance] [12. Supported maximum number of predicted time instances] [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [14. Supported value(s) of time gap between predicted time instances] [15. Supported value(s) of setB periodicity] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection FFS: whether/how to report each of supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) FFS: whether/how to merge some of above components to report combination(s) of supported values; FFS; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for components; Optional with capability signalling. 58. NR_AIML_air; 58-1-5; UE-side beam prediction for BM-Case2 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] [2. Maximum number of future time instance] FFS: other components; 58-1-4; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed) 58. NR_AIML_air; 58-1-7; Data collection for UE-side beam prediction [for BM case 1]; 1. Support of UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] FFS: components for maximum number of resources; FFS; yes; n/a; Data collection for UE-side beam prediction is not supported [for BM case 1]; FFS; FFS; FFS; FFS; FFS: separate rows/FGs for BM case 1 and BM case 2 Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2; Optional with capability signalling.

For FG 58-1-1, the description on NW-sided model inference and suggest to be removed. Also, the value range of M should be determined which should be consecutive number from 5 to 64.

Proposal 1: Support the following update to FG 58-1-1 with red highlighted.

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management [for NW-sided model inference] | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 FFS: whether/how to have separate FG for some of above components or how to merge components | FFS | yes | n/a | Increased number of reported beams for beam management [for NW-sided model inference] is not supported | FFS | FFS | FFS | FFS | Component 3 candidate values: {5,6…64} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 58-1-2, mapping pattern is undefined in any RAN1 agreement and should be removed. Also, we suggest to keep the description “of inference” and split semi-persistent CSI report into two sub-types. Moreover, supported maximum number of predicted beams in each reporting instance should be reported and the associated values can be 1, 2, 3, 4.

Proposal 2: Support the following update to FG 58-1-2 with red highlighted.

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report on PUCCH and semi-persistent CSI report on PUSCH] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instance FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 for inference is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information Component 12 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 58-1-3, similar change to FG 58-1-2 is suggested as follows.

Proposal 3: Support the following update to FG 58-1-3 with red highlighted.

| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP for inference | - Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 for inference 2. Supported maximum number of predicted beams in each reporting instance FFS: other components | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP for inference is not supported | FFS | FFS | FFS | FFS | Component 2 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 58-1-4, similar changes to FG 58-1-2 are suggested. In addition, supported maximum number of predicted time instances should be reported with value range of 1,2,3,4.

Proposal 4: Support the following update to FG 58-1-4 with red highlighted.

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case2] [4. Maximum number of inference report(s) activated for BM-Case2] [5. Maximum number of inference report(s) triggered for BM-Case2] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report on PUCCH and semi-persistent CSI report on PUSCH] [10. Supported options for performance monitoring for beam case 2 with UE side model] 11. Supported maximum number of predicted beams in each reporting instance 12. Supported maximum number of predicted time instances [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [14. Supported value(s) of time gap between predicted time instances] [15. Supported value(s) of setB periodicity] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection FFS: whether/how to report each of supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) FFS: whether/how to merge some of above components to report combination(s) of supported values | FFS | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information Component 11 candidate values: {1,2,3,4} Component 12 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 58-1-5, similar changes to FG 58-1-3 are suggested. In addition, component 2 is unnecessary and should be removed.

Proposal 5: Support the following update to FG 58-1-5 with red highlighted.

| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) for inference [2. Maximum number of future time instance] FFS: other components | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 58-1-7, for UE-side data collection, UE does not need to report whether the collected data is for BM-Case1 and/or BM-Case2. It is up to UE implementation to process the collected data.

Proposal 6: Support the following update to FG 58-1-7 with red highlighted.

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] FFS: components for maximum number of resources | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | FFS | FFS | FFS | FFS | FFS: separate rows/FGs for BM case 1 and BM case 2 Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

2.2 AI/ML based positioning

According to last meeting agreement, the following UE features are agreed with yellow highlighted. So in this section, the yellow highlighted part will be discussed.

Regarding the name of the feature, i.e., with or without “for inference”, our stance is to not include it since it will create non-necessary limitation of understanding of such feature.

Proposal 7: Support the following update to FG 58-2-1 with red highlighted.

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | FFS | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Only thing for 58-2-2 is that it should be inherited from 2-1.

Proposal 8: Support the following update to FG 58-2-2 with red highlighted.

| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS58-2-1 | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

.

2.3 AI/ML based CSI prediction

Proposal 9: Support the following update to FG 58-3-1 with red highlighted.

| 58. NR_AIML_Air | 58-3-1 | AI/ML based CSI prediction for UE-sided model when N4=1 | 1. Support of AI/ML based CSI prediction 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 5. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 6. Support parameter combinations with L=2,4 7. Support for rank = 1,2 8. Value for CPU and APU occupation, when P/SP-CSI-RS is configured for CMR 9. Value for CPU and APU occupation, when A-CSI-RS is configured for CMR 10. Support for the size of DD-basis, N4=1 11. Scaling factor for active resource counting Kp 12. Supported types of performance monitoring | FFS | yes | n/a | CSI prediction for N4=1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | AI/ML based CSI prediction for UE-sided model when N4>1 | 1. Support of CSI prediction 2. Supported values of the number of future time instance. 3. Supported values of the maximum number of observation number 4. Support for reporting predicted PMI with N4>1 5. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Support for the size of DD-basis, N4>1 8. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 9. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 10. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 11. Supported values of the maximum number of resources for measurement 12. Supported values of the maximum number of future time instance | FFS | yes | n/a | CSI prediction for N4>1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

Proposal 10: Support the following new FG 58-3-3

| 58. NR_AIML_air | 58-3-3 | Data collection for AI/ML based CSI prediction for UE-sided model | 1. Support of data collection for AI/ML based CSI prediction 2. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR for data collection | FFS | yes | n/a | Data collection for UE-side CSI prediction is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 11: Support the following new FG 58-3-4

| 58. NR_AIML_air | 58-3-4 | Performance monitoring for AI/ML based CSI prediction for UE-sided model | 1. Support of performance monitoring for AI/ML based CSI prediction 2. Value for APU and CPU occupation, when P/SP-CSI-RS is configured for CMR for data collection 3. Support for SGCS reporting for baseline CSI | FFS | yes | n/a | Performance monitoring for UE-side CSI prediction is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

The proposals made in this contribution are summarized below.

Proposal 1: Support the following update to FG 58-1-1 with red highlighted.

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management [for NW-sided model inference] | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 FFS: whether/how to have separate FG for some of above components or how to merge components | FFS | yes | n/a | Increased number of reported beams for beam management [for NW-sided model inference] is not supported | FFS | FFS | FFS | FFS | Component 3 candidate values: {5,6…64} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Support the following update to FG 58-1-2 with red highlighted.

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report on PUCCH and semi-persistent CSI report on PUSCH] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instance FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 for inference is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information Component 12 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Support the following update to FG 58-1-3 with red highlighted.

| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP for inference | - Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 for inference 2. Supported maximum number of predicted beams in each reporting instance FFS: other components | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP for inference is not supported | FFS | FFS | FFS | FFS | Component 2 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Support the following update to FG 58-1-4 with red highlighted.

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case2] [4. Maximum number of inference report(s) activated for BM-Case2] [5. Maximum number of inference report(s) triggered for BM-Case2] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report on PUCCH and semi-persistent CSI report on PUSCH] [10. Supported options for performance monitoring for beam case 2 with UE side model] 11. Supported maximum number of predicted beams in each reporting instance 12. Supported maximum number of predicted time instances [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [14. Supported value(s) of time gap between predicted time instances] [15. Supported value(s) of setB periodicity] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection FFS: whether/how to report each of supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) FFS: whether/how to merge some of above components to report combination(s) of supported values | FFS | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information Component 11 candidate values: {1,2,3,4} Component 12 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: Support the following update to FG 58-1-5 with red highlighted.

| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) for inference [2. Maximum number of future time instance] FFS: other components | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: Support the following update to FG 58-1-7 with red highlighted.

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] FFS: components for maximum number of resources | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | FFS | FFS | FFS | FFS | FFS: separate rows/FGs for BM case 1 and BM case 2 Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: Support the following update to FG 58-2-1 with red highlighted.

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | FFS | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8: Support the following update to FG 58-2-2 with red highlighted.

| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS58-2-1 | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

.

Proposal 9: Support the following update to FG 58-3-1 with red highlighted.

| 58. NR_AIML_Air | 58-3-1 | AI/ML based CSI prediction for UE-sided model when N4=1 | 1. Support of AI/ML based CSI prediction 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 5. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 6. Support parameter combinations with L=2,4 7. Support for rank = 1,2 8. Value for CPU and APU occupation, when P/SP-CSI-RS is configured for CMR 9. Value for CPU and APU occupation, when A-CSI-RS is configured for CMR 10. Support for the size of DD-basis, N4=1 11. Scaling factor for active resource counting Kp 12. Supported types of performance monitoring | FFS | yes | n/a | CSI prediction for N4=1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | AI/ML based CSI prediction for UE-sided model when N4>1 | 1. Support of CSI prediction 2. Supported values of the number of future time instance. 3. Supported values of the maximum number of observation number 4. Support for reporting predicted PMI with N4>1 5. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Support for the size of DD-basis, N4>1 8. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously 9. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 10. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 11. Supported values of the maximum number of resources for measurement 12. Supported values of the maximum number of future time instance | FFS | yes | n/a | CSI prediction for N4>1 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

Proposal 10: Support the following new FG 58-3-3

| 58. NR_AIML_air | 58-3-3 | Data collection for AI/ML based CSI prediction for UE-sided model | 1. Support of data collection for AI/ML based CSI prediction 2. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR for data collection | FFS | yes | n/a | Data collection for UE-side CSI prediction is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 11: Support the following new FG 58-3-4

| 58. NR_AIML_air | 58-3-4 | Performance monitoring for AI/ML based CSI prediction for UE-sided model | 1. Support of performance monitoring for AI/ML based CSI prediction 2. Value for APU and CPU occupation, when P/SP-CSI-RS is configured for CMR for data collection 3. Support for SGCS reporting for baseline CSI | FFS | yes | n/a | Performance monitoring for UE-side CSI prediction is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

- R1-2502959,Session Notes on 9.15.1, RAN1#120bis.