3GPP TSG RAN WG1 #121		                                    R1-2503592

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.15.2

Source:			Samsung

Title:			UE features for NR MIMO Phase 5

Document for:		Discussion and decision

- Introduction

This contribution provides Samsung’s view regarding UE features for supporting Rel-19 MIMO functionality [1]. During Rel-19 MIMO discussion, RAN1 agreed on remarkable enhancements possibly requesting more complicated UE operation. Some of features are already agreed to bring new UE capabilities. Also, there are RAN1 agreements put FFS or working assumption on introduction of new UE capabilities, and some of the agreed or discussed UE parameters are aligned on their existence or configuration. In this contribution, we discuss all the Rel-19 MIMO related UE capabilities together, and propose whether and how to define new UE capabilities.

- Discussion

  - UE-initiated/event-driven beam management (UEI/ED BM)

In RAN1#120bis, the following UE features for Rel-19 UEI/ED BM were provided as a baseline for further discussions [2].

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management [for Event-2 and Mode A] | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 4 candidate values: {1, 2, … , 64}] Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Maximum number of the configured RS(s) for new beam in the RS resource set] [3. Supported values of X] symbols between the last symbol of sending first PUCCH and the first available symbol of transmission occasion of second PUSCH] | 59-1-1 FFS for other potential FG(s) | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-2 | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to have separate FGs for event 2/1/7 for mode B | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window. | 1. Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) for at least one new beam within a time window [with M>1 per one new beam], where M > 1 | 59-1-1 FFS for other potential FG(s) | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | FFS Per band | FFS n/a | FFS | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] [4. Support of Mode A for Event-1 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported report [for Mode A] | Per Band | n/a | FFS | n/a | [Component 3 candidate values: {FFS}] Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] [2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set] [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] [4. Support of Mode A for Event-7 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS | n/a | Component 1 candidate values for Q: FFS [{1, 2, ..., 8}] [Component 2 candidate values: {1, 2, ..., 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In the following, we provide our views on the initial list of UE features presented above.

First of all, we support to remove the brackets from [for Event-2 and Mode A] in 59-1-1. RAN1 has agreed Mode A as a basic UE feature, and Event-2 as the baseline for Event-1 and Event-7 – hence both of them should be supported in part of the basic UE FG.

Proposal 1. support to remove the brackets from [for Event-2 and Mode A]

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management [for Event-2 and Mode A] | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 4 candidate values: {1, 2, … , 64}] Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For Event-1 and Event-7, in addition to Mode-A, Mode-B should also be supported.

Proposal 2. Add the following two separate rows related to Mode-B for Event-1 and Event-7, respectively

| 59. NR_MIMO_Ph5 | 59-1-x | UE-initiated/event-driven beam management for Event-1 Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Supported values of X] | [59-1-4] | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-1 | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-y | UE-initiated/event-driven beam management for Event-7 Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Supported values of X] | [59-1-5] | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-7 | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

For 59-1-4 and 59-1-5, we support 59-1-1 as their pre-requisite. Then,

- For 59-1-4, component 2 can be removed which can be referred to that in 59-1-1; in addition, the square bracket of component 3 can be removed – note that different from that in 59-1-1 which will be used for both event evaluation and L1 measurements, the new beam RSs configured for event-1 is used for L1 measurements only.

- For 59-1-5, similarly, component 2 can be removed which can be referred to its corresponding counterpart in 59-1-1.

Proposal 3. Support the following updates in red for FGs 59-1-4 and 59-1-5.

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] [4. Support of Mode A for Event-1 UE-initiated/event-driven beam report] | FFS 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported report [for Mode A] | Per Band | n/a | FFS | n/a | [Component 3 candidate values: {FFS}] Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] [2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set] [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] [4. Support of Mode A for Event-7 UE-initiated/event-driven beam report] | FFS 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS | n/a | Component 1 candidate values for Q: FFS [{1, 2, ..., 8}] [Component 2 candidate values: {1, 2, ..., 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - UL 3TX

In RAN1#120bis, the updated list of UE feature for UL 3TX has been discussed and agreed as follows [2]:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3[, 4]} Component 3 candidate values: {1,2,3} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-1a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP | Support the association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH | 59-3-1 | yes | n/a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | FFS | yes | n/a | 3TX 3T6R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3-1 | Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T6R antenna switching | 59-3-3 | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T6R is supported | FFS | FFS | FFS | FFS | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a | 3T3R antenna switching | 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | FFS | yes | n/a | 3TX 3T3R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, supportedSRS-TxPortSwitch or 59-3-3 to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx Note: ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a-1 | Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T3R antenna switching | 59-3-3a | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T3R is supported | FFS | FFS | FFS | FFS | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4 | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based | 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2,3[.4]} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5 | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2,3[,4]} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission | Number of supported PTRS ports for PUSCH transmission | [59-3-1 or] 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission | FFS | FFS | FFS | FFS | Candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-7 | UL full power transmission mode of fullpower | Support of full-power Mode 0 for codebook-based for 3 Tx operation | 59-3-2 | yes | n/a | UL full power transmission mode of fullpower is not supported | FFS | FFS | FFS | FFS | Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply | Optional with capability signalling |

Regarding FG 59-3-1 (Non-codebook based PUSCH transmission for 3TX for single-TRP),

- For Component 2 ([2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)]), based on the following conclusion, it is basically following the legacy UE feature which is up to 4. Hence, we would like to make a corresponding candidate value range from 1 to 4.

Conclusion in RAN1#118bis To support non-codebook-based precoding by a 3TX UE, for SRI indication, re-use the legacy-based solution according to NSRS ≤ maxNumberSRS-ResourcePerSet.

- For pre-requisite, we would like to add FG 2-15 which is a basic non-codebook based PUSCH transmission defined in Rel-15.

Regarding FG 59-3-1a (Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP),

- For Type, we would like to define per band which is same as component 1 of FG 2-15a (i.e., srs-AssocCSI-RS).

Regarding FG 59-3-2 (Codebook based PUSCH transmission for 3TX for single-TRP),

- For pre-requisite, we would like to add FG 2-14 which is a basic codebook based PUSCH transmission defined in Rel-15.

Regarding FG 59-3-3 (3T6R Antenna switching),

- We think that either having new component for downgrade antenna switching configurations or having new note is fine. To align with the design for 3T3R, we slightly prefer to deleting component 4 and taking new note.

- For pre-requisite, we would like to add FG 2-55 which is a basic SRS Tx switch defined in Rel-15.

Regarding FG 59-3-3a (3T3R Antenna switching),

- For pre-requisite, we would like to add FG 2-55 which is a basic SRS Tx switch defined in Rel-15.

Regarding FG 59-3-3-1 (Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching) and FG 59-3-3a-1 (Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching),

- For Type, we would like to define per FS which is same with FG 23-8-4 (srs-AntennaSwitching2SP-1Periodic-r17) and FG 40-5-5 (srs-AntennaSwitching8T8R2SP-1Periodic-r18).

Regarding FG 59-3-4 (M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based),

- For pre-requisite, we would like to add FG 2-14 which is a basic codebook based PUSCH transmission defined in Rel-15.

Regarding FG 59-3-4a (M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based),

- As commented in Component 2 in FG 59-3-1 ([2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)]), we would like to make a corresponding candidate value range from 1 to 4.

- For pre-requisite, we would like to add FG 2-15 which is a basic non-codebook based PUSCH transmission defined in Rel-15.

Regarding FG 59-3-5 (M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based),

- For pre-requisite, we would like to add FG 2-14 which is a basic codebook based PUSCH transmission defined in Rel-15 and FG 11-5 (PUSCH repetition Type B).

Regarding FG 59-3-5a (M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based)

- As commented in Component 2 in FG 59-3-1 ([2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)]), we would like to make a corresponding candidate value range from 1 to 4.

- For pre-requisite, we would like to add FG 2-14 which is a basic codebook based PUSCH transmission defined in Rel-15 and FG 11-5 (PUSCH repetition Type B).

Regarding FG 59-3-6 (PTRS of 3-antenna-port PUSCH transmission),

- We think that FG 59-3-1 could be one of pre-requisites since PTRS is also supported with non-codebook based transmission.

- For pre-requisite, we would like to define per band which is similar with FG 2-48 (twoPortsPTRS-UL) or FG 2-49 (ptrs-DensityRecommendationSetUL).

Regarding FG 59-3-7 (UL full power transmission mode of fullpower),

- For Type, we would like to define per FS which is same as FG 16-5a (ul-FullPwrMode-r16).

Considering above comments on top of the initial list of UE capabilities, agreements and some basic features which have been agreed in normative phase and/or Rel-19 MIMO work item description without explicit description on UE capability, we would like to propose the following list of UE capabilities.

Proposal 4. Support the following list of UE capabilities for Rel-19 UL 3TX.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol | 2-15 | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3, 4} Component 3 candidate values: {1,2,3} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-1a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP | Support the association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH | 59-3-1 | yes | n/a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH is not supported | Per band | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | 2-55 | yes | n/a | 3TX 3T6R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3-1 | Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T6R antenna switching | 59-3-3 | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T6R is supported | Per FS | FFS | FFS | FFS | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a | 3T3R antenna switching | 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | yes | n/a | 3TX 3T3R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, supportedSRS-TxPortSwitch or 59-3-3 to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx Note: ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a-1 | Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T3R antenna switching | 59-3-3a | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T3R is supported | Per FS | FFS | FFS | FFS | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4 | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based | 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | 2-14 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | 2-15 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5 | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | 2-14 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | 2-14 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission | Number of supported PTRS ports for PUSCH transmission | 59-3-1 or 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission | Per band | FFS | FFS | FFS | Candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-7 | UL full power transmission mode of fullpower | Support of full-power Mode 0 for codebook-based for 3 Tx operation | 59-3-2 | yes | n/a | UL full power transmission mode of fullpower is not supported | Per FS | FFS | FFS | FFS | Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply | Optional with capability signalling |

- Conclusion

In this contribution, the following observation and proposals are given:

Proposal 1. support to remove the brackets from [for Event-2 and Mode A]

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management [for Event-2 and Mode A] | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 4 candidate values: {1, 2, … , 64}] Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2. Add the following two separate rows related to Mode-B for Event-1 and Event-7, respectively

| 59. NR_MIMO_Ph5 | 59-1-x | UE-initiated/event-driven beam management for Event-1 Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Supported values of X] | [59-1-4] | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-1 | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-y | UE-initiated/event-driven beam management for Event-7 Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Supported values of X] | [59-1-5] | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-7 | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

Proposal 3. Support the following updates in red for FGs 59-1-4 and 59-1-5.

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] [4. Support of Mode A for Event-1 UE-initiated/event-driven beam report] | FFS 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported report [for Mode A] | Per Band | n/a | FFS | n/a | [Component 3 candidate values: {FFS}] Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] [2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set] [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] [4. Support of Mode A for Event-7 UE-initiated/event-driven beam report] | FFS 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS | n/a | Component 1 candidate values for Q: FFS [{1, 2, ..., 8}] [Component 2 candidate values: {1, 2, ..., 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4. Support the following list of UE capabilities for Rel-19 UL 3TX.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol | 2-15 | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3, 4} Component 3 candidate values: {1,2,3} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-1a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP | Support the association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH | 59-3-1 | yes | n/a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH is not supported | Per band | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | Per FSPC | n/a | n/a | n/a | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | 2-55 | yes | n/a | 3TX 3T6R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3-1 | Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T6R antenna switching | 59-3-3 | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T6R is supported | Per FS | FFS | FFS | FFS | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a | 3T3R antenna switching | 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | yes | n/a | 3TX 3T3R antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, supportedSRS-TxPortSwitch or 59-3-3 to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx Note: ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-3-3a-1 | Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T3R antenna switching | 59-3-3a | yes | n/a | Only maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T3R is supported | Per FS | FFS | FFS | FFS | Note: The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4 | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based | 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | 2-14 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | 2-15 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based | Per FS | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5 | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | 2-14 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | 2-14 | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based | Per FSPC | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission | Number of supported PTRS ports for PUSCH transmission | 59-3-1 or 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission | Per band | FFS | FFS | FFS | Candidate values: {1,2} | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-7 | UL full power transmission mode of fullpower | Support of full-power Mode 0 for codebook-based for 3 Tx operation | 59-3-2 | yes | n/a | UL full power transmission mode of fullpower is not supported | Per FS | FFS | FFS | FFS | Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply | Optional with capability signalling |

References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105.

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#120bis.