3GPP TSG RAN WG1 #121		                                    R1-2503596

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		9.15.6

Source:			Samsung

Title:			UE features for NR mobility enhancements Phase 4

Document for:		Discussion and decision

- Introduction

This contribution provides Samsung’s view regarding UE features for supporting Rel-19 LTM functionality [1].

- Discussions on agreements made in both 9.9.1 and 9.15.6

In the following, we provide our views on the UE capabilities that need to supported for Rel-19 LTM, based on at least the following agreements made in the agenda item 9.9.1 of Rel-19 LTM.

Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study. Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported Agreement The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios Agreement For the identification of the serving cell RS for event evaluation, If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell. - UE does not expect the following configuration: - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and - CSI-RS is configured as measurement resource for the candidate cell(s). Agreement CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 - Send an LS to RAN2 to inform this agreement. Final LS in R1-2501500. Agreement An LTM report configuration for L1-RSRP is associated with a single resource config that includes: - Alt.A: a single resource set containing CSI-RS resources corresponding to multiple candidate cells. i.e., the same design as that of SSB in Rel-18 LTM. - FFS: how to associate between the measurement CSI-RS resources and candidate cells - explicit or implicit signaling of candidate cells Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 Send an LS to RAN2 and RAN3 to inform this agreement Agreement For the RS type determination for event triggered reporting with event LTM2, - Alt 1-2: At least one candidate RS shall be configured Agreement Regarding CSI acquisition, for a candidate cell, - A single CSI report configuration is configured - Multiple CSI-RS resources for CMR can be associated with the CSI report configuration - The number of CSI-RS resources for CMR is subject to UE capability Agreement Following restrictions are introduced - For the codebook configurations in report configuration, typeI-SinglePanel is supported for LTM CSI acquisition - For report frequency configuration in report configuration, wideband CQI and wideband PMI are supported for LTM CSI acquisition - For the report quantity in report configuration, cri-RI-PMI-CQI is supported for LTM CSI acquisition - The supported max rank is up to separate UE capability - For the number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for a candidate cell for LTM CSI acquisition - Up to 128 ports is supported The supported max number of CSI-RS ports is up to separate UE capability

In RAN1#120bis, the following agreements were made regarding UE capabilities/features for Rel-19 LTM:

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources of candidate cell(s) for CSI measurement before LTM CSC MAC CE | 63-6 | Yes |  | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 63. NR_Mob_Ph4 | 63-3 | CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes |  | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-3a | CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 63. NR_Mob_Ph4 | 63-4 | CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 63. NR_Mob_Ph4 | 63-4a | CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |

#### For online checking

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-3 | CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated jointseparate DL/UL LTM TCI states FFS: potential other component(s) | FFS | Yes |  | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated jointseparate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Proposals

For gNB scheduled measurement/reporting for LTM, it was agreed in RAN1 that periodic, semi-persistent and aperiodic reporting are supported, where are missing from the current UE features list. Hence, we propose to reflect this reporting behaviors in both 63-1 and 63-2 with corresponding modifications (highlighted in red) as in Proposal 1.

Proposal 1. Introduce support of periodic, semi-persistent and aperiodic reporting for CSI-RS based NW scheduled measurement/reporting in Rel-19 LTM.

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement/reporting based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 5. Support of periodic, semi-persistent and aperiodic reporting | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement/reporting based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement/reporting based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 5. Support of periodic, semi-persistent and aperiodic reporting | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement/reporting based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

As 63-6 focuses on the UE to perform measurement on a (target) cell after receiving the CSC MAC CE, where the (target) cell is indicated by the CSC MAC CE, the total number of RRC configured candidate cells are irrelevant here. Furthermore, after receiving the CSC MAC CE, only the number of CSI-RS resource(s) and CSI report configuration(s) of the indicated (target) cell would be of UE’s concerns.

In addition, RAN1 has agreed to introduce the following two UE capabilities for CSI acquisition:

- number of CSI-RS resources for CMR (for a candidate cell configured with one CSI report configuration)

- supported max number of CSI-RS ports

Based on the above, we present the following proposal with modifications to the corresponding components in 63-6 in red.

Proposal 2. Support the following updates in red for 63-6.

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources for the indicated cell 3. Maximum number of CSI report configs for the indicated cell 4. Maximum number of CSI-RS resources for CMR for the indicated cell 5. Maximum number of CSI-RS ports | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For event-triggered measurement/reporting for Rel-19, support to introduce the following FG based on RAN1’s agreements:

Proposal 3. Support to introduce the following FG for event-triggered L1 measurement/reporting for Rel-19 LTM

| 63. NR_Mob_Ph4 | 63-x | Event-triggered L1-RSRP measurement/reporting for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of SSB and periodic CSI-RS based L1-RSRP measurement 2. Support of serving cell RS by using QCL RS in the indicated TCI state or the corresponding QCL SSB 3. Maximum number of beams in a single event triggered reporting |  | Yes |  | Event-triggered L1-RSRP measurement/reporting for L1-L2 Triggered Mobility (LTM) procedure is not supported |  |  |  |  | Component 1: the periodic CSI-RS is configured in an NZP CSI-RS resource set configured with repetition | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, the following proposals are provided regarding UE features for Rel-19 LTM:

Proposal 1. Introduce support of periodic, semi-persistent and aperiodic reporting for CSI-RS based NW scheduled measurement/reporting in Rel-19 LTM.

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement/reporting based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 5. Support of periodic, semi-persistent and aperiodic reporting | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement/reporting based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement/reporting based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 5. Support of periodic, semi-persistent and aperiodic reporting | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement/reporting based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |

Proposal 2. Support the following updates in red for 63-6.

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of the RRC configured candidate cells and CSI-RS resources for the indicated cell 3. Maximum number of CSI report configs for the indicated cell 4. Maximum number of CSI-RS resources for CMR for the indicated cell 5. Maximum number of CSI-RS ports | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3. Support to introduce the following FG for event-triggered L1 measurement/reporting for Rel-19 LTM

| 63. NR_Mob_Ph4 | 63-x | Event-triggered L1-RSRP measurement/reporting for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of SSB and periodic CSI-RS based L1-RSRP measurement 2. Support of serving cell RS by using QCL RS in the indicated TCI state or the corresponding QCL SSB 3. Maximum number of beams in a single event triggered reporting |  | Yes |  | Event-triggered L1-RSRP measurement/reporting for L1-L2 Triggered Mobility (LTM) procedure is not supported |  |  |  |  | Component 1: the periodic CSI-RS is configured in an NZP CSI-RS resource set configured with repetition | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

- RP-242356, RAN#105, September, 2024.