3GPP TSG RAN WG1 #121		                                  R1-2503597

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.15.7

Source:			Samsung

Title:			UE features for XR for NR Phase 3

Document for:		Discussion and decision

- Introduction

This contribution discusses UE features for enabling TX/RX for XR during RRM measurements.

- Discussion

In [1], following agreements were made.

Agreement: Introduce following basic FG for XR Phase 3, and FFS on some potential components (that may be separate FGs). X-1; Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1]; 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. [4. The minimum time offset from the DCI carrying the gap/restriction cancellation to the cancelled gap/restriction] FFS: other component (which may be separate FG); FFS; YES; n/a; UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1]; [Per UE or Per band]; Candidate values of component 3: {Option 1, Option 2} [Candidate values of component 4: {5ms, 3ms}]; Optional with capability signalling. Agreement: Confirm the following component 4 in X-1 for XR - [4. The minimum time offset from the end of PDCCHDCI carrying the gap/restriction cancellation to the start of cancelled gap/restriction] Agreement: Confirm the following candidate values for component 4 in X-1 for XR - [Candidate values of component 4: {5ms, 3ms}]

Regarding whether to introduce a separate UE feature per DCI format or not, there is no apparent reason why a UE can support MG skipping indication when it is provided by, e.g., DCI format 0_1/1_1, but cannot support it when it is provided by e.g., DCI format 0_2/1_2. In such a case, only a single FG would be needed—the one for the offset value. Thus, [by DCI format 0_1/1_1] can be deleted in the column of ‘feature group’ and the column of ‘consequence if the feature is not supported by the UE.’

As for the prerequisite FG, it can consider:

- 3-1 (Monitoring DCI formats 0_0, 1_0, 0_1, 1_1)

- 11-1 (Monitoring DCI format 1_2 and DCI format 0_2)

- 49-1 (Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with the same SCS between the scheduling cell and cells in the set)

- 49-2 (Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with the same SCS between the scheduling cell and cells in the set)

Regarding the first component, “is enabled” can be confirmed. Or, if there is no consensus for this part, considering endorsed spec text in TS38.213 [2] can be considered. For example: “Explicit indication by a DCI format in PDCCH reception is used to cancel an earliest RM measurement gap occasion where the UE would perform RRM measurements.”

Regarding the third component, it can be updated to have similar wording to the spec text in TS38.213 [2]. For example: “For explicit indication by DCI, a bit value equal to ‘0’ of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception, where the UE would perform RRM measurements on cells in the same frequency band as the one or more scheduled cells associated with the DCI format, is as described in [10, TS 38.133].”

With these understandings, the following proposal is suggested.

Proposal 1:  Consider the following UE capabilities for enable TX/RS for XR during RRM measurement

| X-1 | Enabling TX/RX during measurement gap scheduling restriction [by DCI format 0_1/1_1] | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled fully cancelled based on explicit indication by DCI 2. DCI format 0_1/1_1[/0_2/1_2/0_3/1_3] indication for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 per legacy behaviour. - Option 2: For explicit indication by DCI, bit value equal to “0” of the XYZ field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [TS 38.133]. 4. The minimum time offset from the end of PDCCHDCI carrying the gap/restriction cancellation to the start of cancelled gap/restriction FFS: other component (which may be separate FG) | FFS 3-1, 11-1, 49-1, 49-2 | YES | n/a | UE does not support Enabling TX/RX during measurement gap scheduling restriction based on explicit indication [by DCI format 0_1/1_1] | [Per UE or Per band] |  |  |  | Candidate values of component 3: {Option 1, Option 2} Candidate values of component 4: {5ms, 3ms} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Reference

[1] R1-2501396, Session Notes of AI 9.15.7

[2] R1-2501663, Introduction of XR (eXtended Reality) for NR Phase 3