3GPP TSG RAN WG1 #121		                                  R1-2503599

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.15.9

Source:			Samsung

Title:			UE features for NTN for Internet of Things (IoT) Phase 3

Document for:		Discussion and decision

- Introduction

This contribution considers UE features for IoT NTN Phase 3.

- Discussion

In [1], following is agreed for OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED.

| 1. IoT_NTN_Ph3 | 1-1 | OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED [2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa] [3. Dynamic activation/deactivation of OCC feature] [FFS: Whether to have other component(s)] |  | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | [Per band or Per UE] | [N/A] | [N/A] | [FFS: Whether to separate this FG for GSO and NGSO] [Note: This FG is applicable only to [IoT-NTN band]] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Component: For the second bullet, it seems sufficient to capture only TDM DMRS without explaining details, as it will be captured in other RAN1 specifications. For the third bullet, it is preferable to capture “OCC indication by DCI field” rather than “activation/deactivation.”

- Need for FDD/TDD differentiation: No differentiation is needed. However, this does not imply that special handling is required for IoT NTN TDD mode if the OCC scheme is enabled.

- Note: It is open to making separate UE capability for this FG for GSO and NGSO if the motivation is clear. Furthermore, it is preferred that this feature applies only to the IoT-NTN band, as it is based on the IoT NTN WID. It can be further extended if there are strong commercial needs in TN later.

Proposal 1: Consider the above updates for OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED.

In [1], following is agreed for OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED.

| 1. IoT_NTN_Ph3 | 1-2 | OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED [2. Support of CDM DMRS with the pattern supported for NPUSCH format 1 with 15 kHz SCS] [3. Dynamic activation/deactivation of OCC feature] [FFS: Whether to have other component(s)] |  | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | [Per band or Per UE] | [N/A] | [N/A] | [FFS: Whether to separate this FG for GSO and NGSO] [Note: This FG is applicable only to [IoT-NTN band]] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Component: For the second bullet, it seems sufficient to capture only CDM DMRS without explaining details, as it will be captured in other RAN1 specifications. For the third bullet, it is preferable to capture “OCC indication by DCI field” rather than “activation/deactivation.”

- Need for FDD/TDD differentiation: No differentiation is needed. However, this does not imply that special handling is required for IoT NTN TDD mode if the OCC scheme is enabled.

- Note: It is open to making separate UE capability for this FG for GSO and NGSO if the motivation is clear. Furthermore, it is preferred that this feature applies only to the IoT-NTN band, as it is based on the IoT NTN WID. It can be further extended if there are strong commercial needs in TN later.

Proposal 2: Consider the above updates for OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED.

- Reference

[1] R1-2502968, Session Notes of AI 9.15.9